Bharat Heavy Electricals Ltd.

Ramachandrapuram, Hyderabad-32

Specifications for Canteen Dining Table

Broad Description :	Eight seater dining table for industrial canteen with SS top (AISI 304 non-magnetic SS sheet), cast iron frame & legs, four foldable MS arms on each side bearing freely rotating SS seats. Only FOUR cast iron legs to be in contact with the floor. No support required below the seats.

Height, Length and Width of table        		:	750 mm x 2320 mm x 860 mm

Thickness of side beading / edge thickness (vertical) :  50 mm with additional 20 mm

bent below the table top

Thickness of SS sheet forming top of table             :	18 swg, matte finish; No plastic sheet or plywood  between SS table top and MS frame

Support below SS sheet forming top of table          : 	MS rectangular section frame (50 x 20, 40 x 30, 25 x 20 mm) with spot welding between SS top and frame support.

Suitable Cast Iron support frame along with 4 legs  :	Joined by MS channel (75 x 40 x 6 mm) and two MS plates 10 mm below the table top

SS seat, 300 mm dia, 20 swg, mirror finish              :  	SS Seat with out beading, welded to inner MS plate of 200 mm dia and 4mm thickness plus MS wheel support of 20 mm width and 3.5mm thick, 32 mm dia solid bush below seat with 16 mm bore with SS screw.

Handle MS pipe 300 mm length, outer dia 32 mm   :	inside bore dia 25 mm, wall thickness 3.5 mm with one MS bush 50 x 40 mm at the table end and another MS bush 32 x 40 mm at the seat end. Inside both MS bushes there should be another inner bush of gun metal with SS pin.

Painting of all non-SS parts                                       :	Two coats of red oxide followed by one coat of

Grey Hammer Stone paint

Number of Tables required                                       :    Total 50 nos. with 8 seats each

Delivery within 50 days

of receipt of PO

Pre-qualification				     :     Please enclose TWO Purchase

orders for minimum 10 tables

each as proof of having supplied

similar tables in the past other

wise offer is liable for rejection.

Model Table                                                               :    Tenderer is advised to visit our Canteen and see the table before preparing their quotation.

Warranty      						     :   Tables to be warranted for 5 yrs

against any manufacturing defects