MPR NO. ZETH 19315015

SPECIFICATIONS OF 20T MOBILE CRANE

- Type :

Rough – terrain diesel hydraulic mobile crane.

- Lifting Capacity :

- 20 T at 3m radius (85% rating) in blocked condition (360deg. Slew)

- 11T at 3m radius (85% rating) free on wheels over front.

- The crane should be able to rotate 360 degree with this load.

- Drive :

Four Wheel drive , four wheel steer & hydraulic power  steering.

4.0	Axles :

Both front & rear axles should be drive and synchronized steerable.

- Outriggers :

Independent hydraulic horizontal & vertical movement and control from operators cabin.

- Boom :

- Hydraulic telescopic fully powered boom with minimum extendable length 20.0 m.

- The boom should be suitable for rated lifting capacity.

- Slew System :

360 degree continuous rotation powered by hydraulic motor & foot applied

multidisc brake with locking device.

- Brakes :

Pneumatic / hydraulic service brakes on all wheels & parking brakes.

- Safety Devices :

All in built safety devices like pressure valves in hydraulic circuits , automatic

safe load indicator, over hoist & overload cut outs, etc.

- Work Lights :

360 Degree beacon cabin lights etc.

- Operator’s Cabin :

Totally enclosed, fully round vision, windows fitted with toughened safety glass, lockable sliding door, electric horn, interior light & fan, fully adjustable upholstered seat for operator, controls for crane movements, travel & outrigger operations.

- Paint Finish :

High gloss finishes cabin, super structure, chairs, safety markings on front & rear of

chassis and counter weight.

13.       Speed :

32-35 kmph maximum with full counter weight.

14.	Tool Kit :

Tool Kit for normal maintenance of the crane, should be provided in a tool box and a list of  tools should be given in the offer.

- Spares :

- Recommended spares for 2 years smooth working including 3 sets of hydraulic and engine filters.

16.	(i)	Pre-dispatch inspection at suppliers works.

(ii)	Maintenance training for BHEL personnel at supplier’s works.

17.	(i)	Commissioning & job proving for rated load at BHEL’s works.

(ii)	The Crane should be got load tested at BHEL works by an approved agency of

Govt. of Uttarakhand and certificate to be submitted to BHEL.

- Guarantee for 24 months after successful job proving.

- PRE-QUALIFICATION REQUIREMENT (PQR) :

  - Vendor should have been manufacturing the offered model or higher capacity diesel hydraulic rough terrain mobile crane for at least 5 years.

  - The offered model should be from current manufacturing range.

  - Vendor to submit customers list for above mentioned supply with contact persons & telephone numbers.

  - Vendor to submit performance certificates from customers for offered or higher capacity diesel hydraulic cranes.

( A.K. MATHUR )                                                                   ( A.K. AHUJA )

AGM (CSX)                                                                        SR. MANAGER (CSX)