Technical Specification of Chassis for Water Tender

| Item SL No. | Technical specification |  |
| --- | --- | --- |
| 01 | Item Description | TATA CHASSIS MAKE NO- 1613 EX LPT OR ASHOK LEYLAND 1612 H6E OR EQUIVALENT CHASSIS WITH POWER STEERING ENGINE EXHAUST BRAKE AND COWL FOR THE FABRICATION OF FIRE TENDER. |
|  | Material | CHASSIS FOR FABRICATION OF FIRE TENDER. |
|  | Model/ Type | TATA CHASSIS MAKE NO- 1613 EX LPT OR ASHOK LEYLAND 1612 H6E OR EQUIVALENT |
|  | STANDARD | TATA CHASSIS MAKE NO- 1613 EX LPT OR ASHOK LEYLAND 1612 H6E OR EQUIVALENT |
|  | ENGINE | Turbo charged inter-cooled diesel engine |
|  | ENGINE OUT PUT | 129HP or 114 PS @ 2400 rpm |
|  | CLUTCH | Single plate dry friction type. Facing dia- minimum 330 mm. |
|  | GEAR BOX | Synchromesh 5 forward 1 reverse, synchromesh on all forward gears and constant mesh on reverse gear. |
|  | FRONT AXLE | Heavy duty forged i beam, reverse elliot type capacity 6000 kgs. |
|  | REAR AXLE | Single reductions, fully floating axle shafts i beam, reverse elliot type capacity 10200 kgs. Ratio – 5.857(41/7) or 6.167:1. |
|  | STEERING | Integral power steering. ratio- 20.2:1 or 20.4:1 |
|  | GROSS VEHICLE WEIGHT | Not more than 16200 kgs. |
|  | GROUND/ROAD CLEARANCE | The Road/Ground clearance shall be not less than 260 mm. |
|  | FUEL TANK CAPACITY | Not less than 165lts. |