TECHNICAL SPECIFICATION OF ASSEMBLY OF PICKLING TANKS SPECIFICATION NO. EMT - 6757 REV. 00.

| S. N. | DESCRIPTION OF BHEL REQUIREMENT | REQUIRED | OFFERED | REMARKS / DEVIATION |
| --- | --- | --- | --- | --- |
| 1.0 | PURPOSE : |  |  |  |
|  | Assembly line of Pickling Tanks will be used for Pre-Cleaning of the non-ferrous components before brazing these items on stator winding bar of large size Turbo generator set. |  |  |  |
| 2.0 | SCOPE : |  |  |  |
|  | The scope of supply includes Design, Manufacturing, Supply, Erection, Commissioning of the pickling tanks and Job Proving of the complete Installation at BHEL Haridwar works. | 01 set |  |  |
| 3.0 | JOB DESCRIPTION, & TECHNICAL REQUIREMENT OF JOB : |  |  |  |
| 3.1 | Job Description : The items such as Contact Sleeve, Water Box Bottom Part, Water Box Top Part, Spacer, Taper Wedge etc. will be dipped in the pickling tanks to remove the oxide film from the surfaces as well as cleaning of these small components, before brazing of these components on stator winding bar of large size Turbo Generator. NOTE : The material of these components as referred above, is COPPER only. |  |  |  |
| 4.0 | TECHNICAL SPECIFICATION OF ASSEMBLY LINE OF PICKLING TANKS : Attached Annexure I, II and III may be referred as the General Layout of the Assembly of Pickling Tanks |  |  |  |
| 4.1 | PICKLING TANKS : Six no. tanks as shown in attached Annexure I, will constitute the assembly line of Pickling Tanks for the respective solutions as stated below. Tank no. 1 : Trichloro Ethylene Tank no. 2 : Running water Tank no. 3 : Pickling solution Tank no. 4 : Neutralizing solution Tank no. 5 : Pickling solution Tank no. : Spirit |  |  |  |
| 4.1.1 | Pickling solution (tank no. 3 & 5) will be the mixture of following : Water : 40.0 kg Sulphuric Acid : 14.0 kg Phosphoric Acid : 7.0 kg Hydrogen per oxide : 7.0 kg |  |  |  |
| 4.1.2 | Neutralizing solution will be mixture of following : Water : :120.0 kg Ammonia solution : 10.0 kg |  |  |  |
| 4.2 | All the tanks will be aligned in one line along the length of bay of the shop as shown in attached Annexure I. | 06 nos. |  |  |
| 4.2.1 | Inner dimension of each tank : Width : 600 mm approx. Width : 600 mm approx. Depth : 600 mm approx | Vendor shall submit the details of the tanks, in their offer |  |  |
| 4.2.1.1 | Material of the Pickling Tanks : All the 6 tanks shall be fabricated from 4 – 5 mm thick stainless steel plate. |  |  |  |
| 4.2.2 | Top surface of these tanks, will be approx. 800.00 mm from the floor of the shop. Accordingly bottom base of these tanks, will be supported on suitable designed with 4 legs as shown in the attached Annexure I. Material of this fabricated structure, shall be resistant to the Pickling solution (tank no. 3 & 5), Neutralizing solution (tank no. 4) and Ttrichloro ethylene (tank no. 1). |  |  |  |
| 4.2.3 | Suitable and detachable Top Cover Plate (refer Annexure – I) shall be supplied for each tank for blanking these tanks from top, when these tanks are not in use. Top cover should have provision to put lock so that unauthorized can not remove the top cover. |  |  |  |
| 4.2.4 | On rear side and bottom portion of each tank, suitable Drain Valve as shown in the attached Annexure – III, is to be provided to drain the complete the solution from the respective tank as and when necessary. Material of each valve should be resistant to the solution of respective tank as specified in sl. no. 4.1, 4.1.1 and 4.1.2. | Make and Technical Specification of these valves should be clearly mentioned in the offer, against respective tank. |  |  |
| 4.3 | CASE TYPE IMMERSION HEATER : | 02 nos. |  |  |
| 4.3.1 | One no. each reputed make and effective case type immersion heater shall be supplied to heat the pickling solution (tank no. 3 & 5 – ref. sl. no. 4.1.1) at temperature 50 + / - 5 Degree Centigrade. Heating capacity of each immersion heater will be approx. 2.0 kw. Outer case of immersion heater must be of suitable material which will not be affected by the pickling solution. | Make, Technical Specification details, kw capacity etc. of the immersion heater, shall be submitted by the vendor, in their offer. |  |  |
| 4.3.2 | Suitable clamping / fixing device shall be supplied to facilitate the holding and dipping of immersion heater in the tank of pickling solution ( tank no. 3 &5) |  |  |  |
| 4.4. | THERMOCOUPLE TO MEASURE THE TEMPERATURE OF PICKLING SOLUTION : | 02 Nos. |  |  |
| 4.4.1 | Reputed make and suitable thermocouple shall be supplied for the control and indication of the temperature of pickling solution (tank no. 3 and 5) shall be supplied to maintain the temperature 50 + / - 5 Degree Centigrade. | Make, Specification details, of the thermoco – uple, shall be submitted by the vendor, in their offer. |  |  |
| 4.4.2 | Suitable clamping / fixing device shall be supplied to facilitate the holding and dipping of thermocouple in the tank of pickling solution ( tank no. 3 &5) |  |  |  |
| 4.5 | At the rear side of running water tank (tank no. 2 – sl. no. 4.1) minimum 8 no. of holes diameter 12.0 mm are to be provided, for the outlet of running water - refer Annexure - III. All the 8 holes are to be made 50 mm below the top level of this tank. These 8 holes will facilitate the outlet of running water above these drilled holes. Water outlet through these 8 holes, will be collected in collection tray which will be welded below the level of these holes. One pipe connection is to be provided below this collection tray for further drainage. |  |  |  |
| 4.6 | EXHAUST DUCTING : |  |  |  |
| 4.6.1 | At the rear side of these tanks, one common exhaust duct along the length of 6 tanks, shall be supplied. Sketch as shown in attached Annexure II, can be referred. | 01 set |  |  |
| 4.6.2 | Subsequently, one no. exhaust pipe each corresponding to the location of Pickling Tank (Tank no. 3 and 5) are to be connected to common exhaust duct (refer cl. no. 4.6.1) |  |  |  |
| 4.6.3 | Both these pipes (cl. no. 4.6.2) shall be connected to one pipe which is further connected to the suction side of a suitable capacity and effective exhaust fan. | Make, Specification details, of the exhaust fan, shall be submitted by the vendor, in their offer. |  |  |
| 4.6.3.1 | Maximum Noise Level of exhaust fan, must be less than 85db(A) at normal running operation, one meter away from the exhaust fan with correction factor for background noise. |  |  |  |
| 4.6.3.2 | The impeller and inner casing of fan, must be resistant to the fumes of pickling solution, neutralizing solution and Trichloro Ethylene as stated in sl. no. 4.1, 4.1.1 and 4.1.2 above. |  |  |  |
| 4.6.4 | Fume from the delivery side of exhaust fan, shall be released through the roof of shop structure. Length (from delivery side of exhaust fan to roof structure) of this exhaust pipe, will be around 10.0 meter. |  |  |  |
| 4.6.5 | Main line duct (cl. no. 4.6.1), Exhaust pipe (cl. no. 4.6.2 & 4.6.3) Delivery pipe (cl. no. 4.6.4) shall be of suitable material which is resistant to the fumes of pickling solution (cl. no. 4.1.1), Neutralizing solution (cl. no. 4.1.2) and Trichloro ethylene (cl. no. 4.1). | Vendor shall furnish the material grade and specification detail of the ducting, in their offer. |  |  |
| 4.7 | CONTROL PANEL : | 01 No. |  |  |
| 4.7.1 | Control panel shall be supplied for the following utilities and function ; |  |  |  |
| 4.7.1.1 | Electrical supply to immersion heaters (sl. no. 4.3) |  |  |  |
| 4.7.1.2 | Indication and control of the temperature of pickling solution (tank no. 3 & 5) with the help of thermocouple (sl. no. 4.4) |  |  |  |
| 4.7.1.3 | Switch ON – Switch OFF the exhaust fan (sl. no. 4.6.3) |  |  |  |
| 4.7.1.4 | DOL starter, over voltage protection etc. for the exhaust fan (4.6.3) |  |  |  |
| 4.8. | GENERAL ARRANGEMENT & LAYOUT DRAWING FOR THE ASSEMBLY OF PICKLING TANKS : |  |  |  |
| 4.8.1 | General Arrangement & Layout Drawing for Assembly Line of Pickling Tanks, shall be submitted along with the offer. General Arrangement & layout drawing shall clearly indicate the following features : | Vendor shall submit the following information and documents, in their offer |  |  |
| 4.8.1.1 | Total plan area required for the pickling tanks, exhaust ducting and control panel |  |  |  |
| 4.8.1.2 | Size of the tanks |  |  |  |
| 4.8.1.3 | Location of drain valve (sl. no. 4.2.4) |  |  |  |
| 4.8.1.4 | Location of holes and collection tray etc. (sl. no. 4.5) |  |  |  |
| 4.8.1.5 | Total Power Required |  |  |  |
| 4.8.1.6 | Scheme for exhaust ducting (sl. no. 4.6) |  |  |  |
| 4.8.1.7 | Location of exhaust fan (sl. no. 4.6.3) with reference to the location of pickling tank |  |  |  |
| 4.8.1.9 | Location of control panel (sl. no. 4.7) with reference to the location of pickling tank |  |  |  |
| 4.8.2 | On receipt of Purchase Order, General Arrangement & Layout drawing as per sl. no. 4.8.1, shall be submitted within 4 weeks after placement of order, for BHEL approval. | Vendor to confirm |  |  |
| 5.0 | OPERATION AND MAINTENANCE MANUAL FOR ASSEMBLY LINE OF PICKLING TANKS : |  |  |  |
| 5.1 | 4 sets of Operation and Maintenance Manual in English language, shall be supplied. Operation & Maintenance Manual should contain following aspects along with other features. | Vendor shall furnish confirmation in their offer |  |  |
| 5.1.1 | Erection manual for the Assembly Line of Pickling Tanks |  |  |  |
| 5.1.2 | BHEL approved General Arrangement & Layout Drawing (sl. no. 4.8.2) |  |  |  |
| 5.1.3 | Constructional and Dimensional details of pickling tanks (sl. no. 4.1) |  |  |  |
| 5.1.4 | Make and Specification of Case type immersion heater (sl. no. 4.3) |  |  |  |
| 5.1.5 | Make and Specification of Thermocouple (sl. no. 4.4) |  |  |  |
| 5.1.6 | Make and Specification of Exhaust fan (sl. no. 4.6.3) |  |  |  |
| 5.1.7 | Make and Specification of Drain Valve (sl. no. 4.2.4) |  |  |  |
| 5.1.8 | Electrical circuit diagram of control panel (sl. no. 4.7) |  |  |  |
| 5.1.9 | Constructional and layout of exhaust ducting (sl. no. 4.6) |  |  |  |
| 6.0 | POWER SUPPLY AND ENVIRONMENT CONDITION : |  |  |  |
| 6.1 | BHEL will provide Electrical supply as detailed out below : : Three phase, 3 wire, 415 + / - 15% volt, 50 + / - 3% Hz to Isolator Switch ( BHEL scope) nearby control panel of Assembly Line of Pickling Tanks. Cable connection from isolator switch to control panel and remaining cabling / wiring for the Assembly line of pickling plant and other auxiliaries / equipment, will be in the scope of vendor. There will be no neutral wire and earthing will be provided through shop structure column. | Vendor to note |  |  |
| 6.2 | Environment Condition |  |  |  |
|  | Control and its system should be suitable for tropical condition with ambient temperature from 0 to 45 Degree Centigrade and relative humidity maximum 95 % | Vendor to confirm |  |  |
| 6.3 | Noise Level |  |  |  |
| 6.3.1 | Maximum noise level of exhaust fan (sl. no. 4.6.3.1), should be below 85dB(A) one meter away from the exhaust fan with correction factor for background noise. | Vendor to confirm |  |  |
| 6.3.2 | Vendor will demonstrate the noise level as per International Standard like DIN 45636-16, if asked for. |  |  |  |
| 7.0 | PRE-ACCEPTANCE OF ASSEMBLY LINE OF PICKLING TANKS AT VENDOR'S WORKS : |  |  |  |
| 7.1 | Pre-acceptance of the Assembly Line of Pickling Tanks will be done by BHEL engineer at vendor’s works. Following points will be verified by BHEL engineer at vendor’s works. | Vendor shall furnish concurrence in their offer |  |  |
| 7.1.1 | Verification of all the items as per total list of Bill of Materials |  |  |  |
| 7.1.2 | Calibration certificate of Thermocouple (sl. no. 4.4) |  |  |  |
| 7.1.3 | List of spare items as per Purchase Order (sl. no. 8.0) |  |  |  |
| 7.1.4 | Completeness of Operation & Maintenance Manual (sl. no. 5.0) |  |  |  |
| 8.0 | SPARES : Following items are to be included in the list of spares along with the recommended items of spares : |  |  |  |
| 8.1 | One no. Case type Immersion Heater (sl. no. 4.3) |  |  |  |
| 8.2 | One no. Thermocouple (sl. no. 4.4). |  |  |  |
| 8.3 | One no. Impeller for exhaust fan (sl. no.4.6.3) |  |  |  |
| 9.0 | PAINTING : |  |  |  |
| 9.1 | Vendor shall indicate the colour of the Control Panel for BHEL approval |  |  |  |
| 9.2 | Vendor shall indicate the colour of entire ducting for exhaust line for BHEL approval |  |  |  |
| 9.3 | Vendor shall repair the paint with original paint in case the same is damaged in transit / erection, before commissioning the Assembly Line of Pickling Tanks, at BHEL Haridwar works. | Vendor to note |  |  |
| 10.0 | ERECTION AND COMMISSIONING OF THE ASSEMBLY LINE OF PICKLING TANKS AT BHEL HARIDWAR WORKS : |  |  |  |
| 10.1 | Vendor shall take full responsibility for carrying out the erection, commissioning of the Assembly Line of Pickling Tanks at BHEL Haridwar works. | Vendor shall furnish their confirmation in their offer |  |  |
| 10.2 | Service requirement like overhead crane facility, unskilled man power, compressed air and water shall be provided by BHEL during erection of the Assembly Line of Pickling Tanks. Details of these requirements should be informed / discussed by the vendor and agreed with BHEL in advance. | Vendor to note |  |  |
| 10.3 | Welding sets and welding consumables required during erection of the Assembly Line of Pickling Tanks, shall be arranged by the vendor. | Vendor shall furnish their concurrence in their offer |  |  |
| 10.4 | Special equipment / instrument if any required for the erection / commissioning of the Assembly Line of Pickling Tanks, shall be brought by the vendor, on returnable basis. |  |  |  |
| 11.0 | FINAL ACCEPTANCE OF THE ASSEMBLY LINE OF PICKLING TANKS : |  |  |  |
| 11.1 | Demonstration of all features such as Heating of pickling solution (sl. no. 4.3.1), Automatic Temperature Control of pickling solution (sl. no. 4.4.1), Effective fume extraction (sl. no. 4.6) and noise level of exhaust fan (sl. no. 4.6.3.1), to the satisfaction of BHEL, at BHEL Haridwar works. | Vendor shall furnish their concurrence in their offer |  |  |
| 12.0 | GUARANTEE : |  |  |  |
| 12.1 | Guarantee for complete Assembly Line of Pickling Tanks, Control Panel and other supplied items for 18 months from the date of final acceptance ( refer sl. no. 11.0) of the Assembly Line of Pickling Tanks. | Vendor shall confirm in their offer |  |  |
| 12.2 | Any spare required during guarantee period shall have to be arranged by the vendor free of cost (up to BHEL Haridwar stores) and duty levied have to be borne by the vendor. | Vendor shall confirm in their offer |  |  |
| 13.0 | PACKING : |  |  |  |
| 13.1 | Rigid wooden packing box shall be provided for all the items of Assembly Line of Pickling Tanks to avoid any damage / loss in transit. All small and loose items like spares, Case Thermometers, Thermocouples etc. should also be suitably packed in separate wooden boxes. | Vendor shall confirm in their offer |  |  |
| 14.0 | GENERAL INFORMATION : |  |  |  |
|  | All the information and drawing attached with the tender document are exclusive property of BHEL Haridwar. Under any circumstance, these information as stated above, must not be passed on to any third party without prior permission of BHEL and must not be used directly or indirectly detrimental to the interest of BHEL. |  |  |  |
| 15.0 | REFERENCES : The vendor shall furnish the following reference such as Purchase Order, Year of Manufacturing and Commissioning, Name of their customers to whom such or bigger size of Assembly of Pickling Tanks and System has been supplied. | Vendor shall furnish the relevant details, in their offer |  |  |