LIST OF STD. TOOLS; TOOL HOLDERS; BORING BARS; SLEEVES ETC. FOR                             CNC VTL – 6MTS.

ANNEXTURE – I

Refer the Indent No:                   (Matl.Code.No.CG2014296014)

- STANDARD TOOL HOLDERS: (2 Nos. each of the following.)

  - PCL NR	4040 S19

  - PCLNL	4040 S19

  - PDJNR	4032

  - PDJNL	4032

  - PCBNR	4040 S19

  - PCBNR	4040 S19

  - PSBNR	4040

  - PSBNL	4040

  - PSSNR	4040

  - PSSNL	4040

(Corresponding 5 sets of spares viz. lever, screw, key, shim each.)

- INSERTS (50 NOS. EACH FOR THE FOLLOWING)

a)	      CNMG	190608

b)	      SNMG	190608

c)         DNMG

(Note: Insert grade suitable for machining of carbon steel, stainless steel)

- BORING BARS:

a)         LONG BORING BAR DIA. 60 x 400 mm	2nos.

b)	      LONG BORING BAR DIA. 80 x 600 mm	2nos.

c)	      LONG BORING BAR DIA. 100 x 1000 mm	2nos.

- SLEEVES:

a)         SLEEVE DIA. 180 x 400		2nos.

b)         SLEEVE DIA. 200 x 600 		2nos.

- BORING HOLDERS

a) MT100 x 400				2nos.

b) MT100 x 600				2nos.

6.	Long studs M30 size of lengths(for job clamping):

1300mm(8no’s),1700mm(4no’s),2300mm(4no’s),2600mm(4no’s)

7.	Supporting pipes(for job clamping): Dia110X6mm of lengths L=2300mm(2no’s),

L=2900mm(2no’s),         L=1000mm(2no’s)

Note: pipe min dia is 100mm

# JOB PROVE OUT DETAILS FOR CNC VTL 6 MTS.

ANNEXURE – II

Refer the Indent No:                  (Matl.Code.No.CG2014296014)

Type OF CASING 				:	150MW Steam Turbine HP-IP Inner casing

Material Specification		:	Hy 19573 (Gs17CrMoV511)

Mechanical properties		:	Ts : 590~780;	Ys : 440; E   : 15

Weight of job both 			:	36300 kgs (Gross)

upper & lower parts			:	34300 Kgs (Net)

VOLUMETRIC DIMNSOF CASING

IN COMBINED CONDITION (L X B X H):	3675x 2240x2120 mm.

Chemical composition		:	C = 0.15~0.2; Si = 0.3~0.6;

Mn = 0.5~0.8; S = 0.03; P = 0.03;

Cr = 1.2~1.5; Mo = 0.9~1.1; V = 0.2~0.3.

INNER CASING  (WZ) MACHINING          :   	01050262011-00 (2shts)

Groove plan drawing		 :	91010762023-00 (1sht)

mACHINING SCOPE:

All internal dias, blade grooves including suspension & sealing fin grooves finish machining as per drawing.

Note:	A Typical casing component to be machined is mentioned, however this job Prove out (on any

one casing) will be done on any other nearly similar casing available at the Time of Erection &

commissioning. Modifications to carryout the same including CNC programming, Tooling,

Machining time etc.. Shall be part of contract.