Technical specifications : Video lower GI Endoscope

t. lt should have high definition endoscopy digital video processor with the integrated primary

xenon light source with Narrow band/multi band imaging. work within power supply 230 volts'

2. lt should have the DVI and PIP'

3. lt should be supplied with the standard accessories and other. additional accessories including

Polypectomy snare, connecting cord and foreign body removal forceps.

4, Colonoscope should have the 12.8mm to 13mm diameter with 3'7mm channel diameter and

forward viewing direction.

5. Field of view is 120 to 140 degree'

6. Working length -1650mm or more'

7. Total length - of colonoscope should be t800mm to 2000mm in range'

8.Right and left 160/160 degree and up and down- 180/180 degree.

9-. lt should be supplied with the 19 inch DVI monitor of medical grade'

10, lt should be supplied image software and computer system with proper 2kva servo stabilizer'

11. The bidder should arrange the training for the doctor and technician'

12. The colonoscope should be of international reputed manufacturer'

13. lnstallation base in lndia should be given'

14. Bidder should have service center in Hyderabad. Address of the same to be provided in technical bid.

15- Vendor has to support the specifications with manufacturer’s brochure failing which offer may be rejected. Vendor has to demonstrate the equipment at BHEL General Hospital, Hyderabad, within  specified time limit, if asked for;   failing which offer will be rejected.

16- Installation(Erection & commissioning) and training to be provided by vendor .