# BHARAT HEAVY ELECTRICALS  LIMITED

HEAVY POWER EQUIPMENT PLANT

RAMACHANDRAPURAM :: HYDERABAD-502032 (INDIA)

NOTICE INVITING EXPRESSION OF INTEREST  ( EOI )

FOR SUPPLY OF GEARED VARIABLE SPEED FLUID COUPLINGS

FOR BOILER FEED PUMPS OF 250MW AND 600MW POWER STATIONS

EOI REFERENCE NUMBER : F6/10/PUMP/PUR/HC/PN/1

(This document contains 17 pages)

Pumps Division of Bharat Heavy Electricals Limited,  Ramachandrapuram, invites :

Expression Of Interest ( EOI ) from Original Equipment Manufacturers (OEMs)  for supply  of  Geared Variable Speed Fluid Couplings for Speed Control of Boiler Feed Pumps of 250MW and 500MW Power Stations

DETAILS OF THE CONTACT PERSON  AT BHEL

NAME : P. NANCHARIAH

DESIGNATION : SENIOR MANAGER / PURCHASE DEPT (PUMPS)

POSTAL ADDRESS  :

Bharat Heavy Electricals Limited, Ramachandrapuram, Hyderabad - 502032, India

PHONE : +91 40 23182679

e-mail ID  : pnr@bhelhyd.co.in

Fax: 91(0) 40 – 23021874

SCHEDULE OF EVENTS

Last date for receipt of any queries from prospective OEMs :  5th November 2010

Consolidated Reply to all queries/clarifications by BHEL : 15th November 2010

(All queries and replies will be available only on BHEL websites on this date – www.bhel.com and www.bhelhyderabad.com)

Last Date for Receipt of Response to EOI at BHEL : 30th November 2010

MODE OF SUBMISSION OF RESPONSE TO EOI

Response should be sent  in a sealed cover to the above address by courier mail. Alternately, same may be e-mailed  to the e-mail ID given above, so as to reach BHEL on or before the date mentioned above. The cover shall be super-scribed with EOI Reference number and the words “Expression of Interest – Geared Fluid Couplings”. In case of offer by e-mail, the words “Expression of Interest – Geared Fluid Couplings” shall be in the Subject field of e-mail.

In case of submission of response through e-mail to the e-mail ID, hard copy of the same shall be enclosed along with printout of e-mail and sent by courier mail to the above person so as to reach BHEL on or before 30th November 2010.

NOTICE INVITING EXPRESSION OF INTEREST  ( EOI )

FOR SUPPLY OF GEARED VARIABLE SPEED FLUID COUPLINGS

FOR BOILER FEED PUMPS OF 250MW AND 600MW POWER STATIONS

EOI REFERENCE NUMBER : F5/10/PUMP/PUR/HC/PN/1

TABLE OF CONTENTS

SECTION – 1: SUMMARY OF REQUIREMENT

SECTION – 2: TECHNICAL SPECIFICATIONS

SECTION – 3: QUALIFYING REQUIREMENTS

SECTION – 4: COMPANY PROFILE  OF OEM

SECTION – 5: CHECKLIST OF DOCUMENTS

SECTION - 1

SUMMARY OF REQUIREMENT

1.1 	ABOUT BHEL

Bharat Heavy Electricals Limited (BHEL) (www.bhel.com), a Government of 	India Undertaking, is a major electrical equipment manufacturer for both the 	Indian and export markets.  It is one of the largest engineering and 	manufacturing enterprises in India and is one of the leading international 	companies in the field of power generating equipment with an annual revenue 	of over INR 275 Billion (US$ 5.5 Billion).

BHEL offers a wide spectrum of products and services for core sectors like 	Power, Transmission, Industry, Transportation, Oil & Gas, Non Conventional 	Energy Systems etc.

1.2 	ABOUT HYDERABAD UNIT OF BHEL

The Hyderabad unit (www.bhelhyderabad.com) of BHEL was formed in 1976 	mainly for the design and manufacture of Steam Turbine, Generator, Heat 	Exchangers and Pumps required for thermal power plants

1.3 	PUMPS DIVISION OF BHEL HYDERABAD

Boiler Feed Pumps along with matching Booster Pumps, Condensate 	Extraction Pumps and Condenser Circulating Water Pumps suitable for 	thermal power plants of unit ratings upto 1000MW are designed and 	manufactured at the Pumps Division of BHEL Hyderabad

1.4 	EXPRESSION OF INTEREST (EOI)

This EOI is published for seeking response from qualified and experienced 	Original Equipment Manufacturers (OEMs) who are willing to be associated 	with BHEL for supply of  geared variable speed fluid couplings meeting the 	technical and qualifying requirements as specified in this document.

1.5	 RESPONSE TO THE “EXPRESSION OF INTEREST” – (EOI)

BHEL will analyze the responses received towards this EOI to shortlist 	prospective partners.

BHEL reserves the right to choose their  prospective partners based on the 	responses received.

A separate Request for Quotation (RFQ) along with detailed Technical and 	Commercial Specifications will be issued to the shortlisted  prospective 	partners for submitting Techno-Commercial and Price offers.

1.6  	TECHNICAL AND QUALIFYING REQUIREMENTS

Only OEMs meeting both the Technical Requirements as described in 	Section-2 and the Qualifying Requirements (QR) as described in Section–3 	may respond to this Expression of Interest and they only will be considered for   	further evaluation.

1.7	 CHECKLIST OF DOCUMENTS

The information required to be submitted along with the EOI by the interested 	OEMs are given in Section–5.

SECTION - 2

TECHNICAL SPECIFICATION REQUIREMENTS

| S.No. | Technical Parameter | For 250MW sets | For 600MW sets |
| --- | --- | --- | --- |
| 1 | Fluid coupling input speed (rpm) | 1485 | 1485 |
| 2 | Maximum ouput speed (RPM) | 5300 | 6000 |
| 3 | Maximum input power (KW) | 4000 | 8000 |
| 4 | Max ambient temperature( DegC) | 50 | 50 |
| 5 | Max CW temperature ( Deg C) | 45 | 45 |
| 6 | Relative Humidity | < 95% | < 95% |
| 7 | Power supply frequency (Hz) | 50Hz ± 5%, | 50Hz ± 5%, |
| 8 | Drive Motor Voltage | 6.6/11 kV ± 10% | 6.6/11 kV ± 10% |
| 9 | Type of AC Motor | 4 pole, AC | 4 pole, AC |
| 10 | Permissible noise level | <80dbA | <80dbA |
| 11 | Oil quantity required for external lubrication ( LPM) | 150 | 300 |

1.0.0	SCOPE OF SUPPLY :

1.1.0	Design, Engineering, manufacture, inspection, testing, packing and forwarding 	the Hydraulic Couplings and accessories to site.

Each hydraulic coupling shall be supplied with following accessories.

a) 2 x 100% Working oil coolers and 2 x 100% Lube oil coolers(duplex type)

along with change-over device on oil side.

b) 1 No. AC motor driven auxiliary lube oil pump to cater for external

lubrication.

c) An electrical actuator and linkages along with the necessary power 	    	    amplifier unit shall be provided by the Bidder to operate the fluid coupling 	    scoop tube. The time required by the actuator to operate the scoop from its 	    minimum position to the maximum position shall be indicated in the Bid.

The 	actuator and the power amplifier unit shall be compatible with                	   microprocessor based Distributed Digital Control & Monitoring system     	   (DDCMIS) procured by Owner.

d) Instrumentation as specified at Cl.No. 1.1.2

e) Complete lube oil piping, working oil piping and vent piping to be connected 	     between hydraulic couplings and coolers and drain piping with a valve to     	     drain the oil from oil tank of hydraulic coupling. All necessary flanges, 		     packings, 	reducers, elbows, bolts and nuts required in these lines shall be 	     included in the scope. 25 meters of pipe and 20 Nos. of elbows per straight   	     run per cooler shall be supplied.

f)  Duplex strainer in lube oil line.

g) Holding down bolts to secure Hydraulic coupling to common foundation  	    frame.

h) Counter flanges, bolts, nuts & gaskets for cooling water flanges.

i)  J-type foundation bolts & nuts for coolers.

k) Rapid start device to accelerate the pump from stand by condition to 	   	    normal running speed, within 15 seconds. Solenoid valves of rapid start 	 	    device shall be suitable for 24 V,DC power supply.

l) Vibration pads for mounting the vibration probes shall be provided on 	 	    bearing housings.

1.1.2	The following instrumentation shall be included in the scope of supply. 	However, any additional instrumentation required for the safe operation of the 	equipment shall also be included.

3 nos. of Electronic Pressure Transmitters in the lubrication oil supply 		     piping down stream of filter shall be provided for interlocks & annunciation    	     for 	AOP Motor & BFP Drive Motor operation.

1 No. Pressure indicator on lube oil supply header downstream of filter.

1 No. Pressure indicator for control oil.

3 Nos. Electronic Pressure  transmitters for control oil.

Capillary dial thermometers at all journal bearings and thrust bearings.

4 Wire type temperature elements (Duplex RTDS) for each journal and thrust bearing.

1 No. Capillary dial thermometer at lube oil inlet to cooler.

1 No. Capillary dial thermometer at lube oil outlet from cooler.

1 No. 4 wire duplex RTD  at lube oil inlet to cooler.

3 Nos. 4 wire duplex RTDs  at lube oil outlet from cooler.

1 No. Capillary dial thermometer at working oil inlet to cooler.

1 No. Capillary dial thermometer at working oil outlet from cooler.

3 Nos. 4 wire duplex RTDs at working oil inlet to cooler.

1 No. 4 wire duplex RTD  at working oil outlet from cooler.

1 No. differential pressure indicator across oil filter.

1 No. differential pressure switch across oil filter.

1 No. Level gauge on oil tank for local indication.

1 No. Level switch on oil tank for annunciation.

Position feed back transmitter (In actuator).

Signaling contacts (In actuator) suitable for giving 4-20 mA galvanically isolated position feed back signal.

2 nos of speed transduceres with necessary accessories to give galvanically isolated 4-20ma DC out put signal for owners  DDCMIS      ( Distributed digital control, monitoring and information system)

1 No. Limit switch for 0% scoop tube position.

2.0.0	SPECIFIC TECHNICALREQUIREMENTS:

Hydraulic Coupling :

Hydraulic coupling shall be of full duty, generously designed, infinitely 	variable, remote controlled and totally enclosed type. It shall be 	adequately rated to meet the requirement of the boiler feed pump 	characteristics during start-up and load fluctuations. Operating range on 	lower side shall be upto 20% of the maximum output speed.

The hydraulic coupling shall be complete with Stainless steel impellers & 	casing, and  Kingsbury or Michell Tilting pad, type thrust bearings for 	input output shafts. An enclosing steel housing with removable covers, oil 	sump, duplex filters to remove all particle upto 5 microns, oil 	temperature and 	pressure gauges, control pump for oil regulation if  provided, regulating 	valves, stainless steel scoop tube, oil coolers etc., shall be furnished to 	from a centralised lubrication system for the total pump set. Bidder shall 	also state in their offer the effect on fluid 	coupling if a reversal of pump 	rotation occurs through the failure of the 	non return valve.

The starting time from minimum scoop position to 100% scoop position 	shall be less than 15 seconds after receipt of starting signal by motor, 	which operating single or in parallel with another Boiler Feed Pump. A 	quick start device shall be incorporated to meet the above requirement 	for 	auto stand by operation of Motor Driven Boiler Feed Pump.

Step-Up Gear Unit :

Gear boxes shall be totally enclosed and oil lubricated. Positive 	lubrication 	shall be provided to all the bearings and particularly to the 	working surfaces 	of the gear teeth.

The gear unit shall be specifically designed for continuous service at the 	speeds required by the application. Precision and accuracy of manufacture is 	essential in order to minimize problems of vibration. Impact-stresses, noise 	distortion and lubrication.

Double helical gears cut by the hobbing process , or approved equal, and 	dynamically balanced shall be used. The high speed pinion shall be of heat 	treated forged steel with integral shaft. The low speed gear shall be of forged 	steel securely keyed to the input shafts. Bidders shall submit detailed 	specification of the materials used including factor of safety etc.

The gearing shall be enclosed in an oil and dust proof gear case made of 	close grained cast iron or fabricated steel.

The gear case shall be horizontally split and stress relieved before machining. 	The faces of adjoining sections shall be accurately machined to provide oil 	tight joints. Two oil level gauges, drain, fill vent, thermometer dipstick and 	other required connections equipped with approved mountings, valves and 	fittings shall be furnished for the gear casing.

The housing shall be generously proportioned and of rigid construction to 	provide permanent alignment of rotating parts and protection from possible 	external vibration. The arrangement shall be such that it will be possible to lift 	gear case cover without disturbing the alignment of shaft, gear and pinion.

Service factors for the gearing shall not be less than the following values :-	Motor driven pumps (Combined gearing/Hydraulic coupling)   …1.4

These two gear boxes shall be in accordance with AGMA 420 and 421, as 	applicable. The gearing and bearing shall be pressure lubricated with oil 	supplied from the central lubrication system of the respective pump sets.

The gear box shall be provided with separate thrust bearings of conservative 	design to meet the worst duties under transients.

Bearings :

The bearings shall be amply proportioned and designed to withstand all 	operating conditions. They shall be of split sleeve type, bronze/steel backed 	with high grade centrifugally cast tin base babbit lining. Bearings shall be 	accurately machined to assure perfect alignment. Gear journal bearings shall 	have babbited thrust faces next to the gear to act as locating surfaces for the 	gear train.

Lubrication & Working Fluid System :

Each hydraulic coupling shall be provided with built in lubrication system to 	also cater for external lubrication of 300 lpm. It shall comprise oil reservoir, 	main oil pump, auxiliary oil pump, duplex oil filter, and oil coolers with 	interconnecting piping and fittings.

The main oil pump shall be driven by the motor shaft and shall be capable to 	supply lube oil for external requirement also.

Aux. Oil pump shall be driven by an AC motor mounted on the hydraulic 	coupling itself AC motor shall be suitable for 415v, 3-phase, 50HZ supply.

These motors shall comply with the Owner’s specification for electrical motors 	annexed to this specification. Discharge system shall be common for main oil 	pump and aux. Oil pump with suitable NRVs in each line. The oil to be used 	shall be the same for lubrication and torque transmission. Oil piping shall be 	sch. 80 and shall be supplied in pickled condition.

Oil Coolers :

Each hydraulic coupling  set  shall be provided with 2 x 100% working oil coolers and 2 x 100% lube oil coolers.

The coolers shall be adequately rated and shall have the necessary arrangements for change over the cooler without interrupting the oil flow.

Both the coolers (Lubricating oil and working oil) shall be utilized in a closed loop system employing demineralized water of condensate quality as the cooling medium. The coolers shall be designed to accept the above system arrangement.

The shell of the coolers shall be of mild steel construction. Suitable corrosion allowance shall be added to all the carbon steel and cast iron parts including shell, flanges etc.

The tubes shall be of type 304 stainless steel. The tube bundle shall be of removable type. The tubes shall be roller expanded and flared at the ends. Provision for differential expansion shall be suitably incorporated.

The coolers shall be so designed that the cooling medium will flow in the tube side. Suitable fouling allowance shall be taken into the design consideration based on the relevant standards.

The design pressure of the coolers on the water side not be less than 10.0 kgs/sq.cm.

Provision shall be made for complete venting and draining of all the parts of the heat exchangers. Connections along with temperature and pressure indicators shall be provided on the inlet and outlet branches of the water side and also on the shell side of each heat exchanger.

Scoop Tube Actuator :

Electrical actuator shall be provided for positioning the scoop tube. Following accessories shall be provided with the actuator.

- Servo Motor

- Position feed back transmitter.

- Four signalling contacts.

- Electronic power positioner with IP-55 enclosure and     suitable for field mounting.

- Transformer (if required)

- Mains rectifier (if required)

- Handwheel arrangement to manually position the actuator.

The servomotor of the actuator should be designed for 240v, Single phase, 50HZ supply with possible variation of ±10% on voltage ±5% variation on frequency. If necessary a transformer can be supplied to suit the actuator voltage requirement. The motor shall  be capable of  withstanding  electrodynamic stresses and heating if started 	at 110% voltage. The motor shall be capable of running at 80% voltage for a period of atleast 5 minutes.

The actuator shall have IP55 enclosure. Position feed back transmitter shall be of 2 wire, 4-20 mA D.C output type and suitable for 24v D.C supply. Actuator shall have all metal gear trains self linked gearing shall be provided. Electronic power positioner shall be suitable to accept 4-20 mA control signal.

Motors shall have class-B, non hygroscopic  insulation scaled with two additional dips and bakes of epoxy finish. Motors shall be furnished with totally enclosed self ventilated, weather proof, dust tight enclosures suitable for outdoor service. Double shielded, grease prelubricated, regreasable, antifriction bearings having minimum life rating of 40,000 hours shall be furnished. Motor leads shall be terminated in the limit switch compartment.

3.0.0   MATERIALS OF CONSTRUCTION :

Materials of construction of the coupling components shall be those with 	which supplier has had previous 	successful operating experience in similar 	applications for a period of at least 5 years. Material standards shall be 	indicated.

4.0.0  CATALOGUES  :

Detailed catalogues covering range of manufacture and salient 	design  	features may be submitted along with the response

5.0.0  CONFIRMATIONS /DEVIATIONS TO SPECIFICATIONS

Point-wise confirmation to this specification to be given along with response to 	this Expression of Interest. Deviations if any, shall be indicated separately. 	This shall be furnished in the format below:

Sl.No. Point No. Confirmation / Deviation  If Deviation,Details  Remarks, if any

SECTION - 3

QUALIFYING REQUIREMENTS

3.1  Technical Capability

The respondent shall be an Original Equipment Manufacturer (OEM) who should have designed geared fluid couplings of similar type, of same or higher size, and which are in satisfactory operation at minimum  two power stations for the past two years or more as on the date of response to this EOI.  This data is to be furnished as per the format indicated below:

PROFORMA FOR  QUALIFYING REQUIREMENT

SL. NO. :

CUSTOMER NAME, ORDER REFERENCE & DATE  :

ITEM DESCRIPTION, MW  RATING, APPLICATION  :

NUMBER OF COUPLINGS SUPPLIED :

CUSTOMER’S CONTACT DETAILS :

• NAME

• DESIGNATION

• PHONE NO.

• FAX NO.

• EMAIL ID

DATE OF SUPPLY  :

DATE OF COMMISSIONING :

NOTE :  Performance certificate from customers regarding  satisfactory performance

of the couplings should be attached

SECTION – 4

COMPANY PROFILE

4.1   GENERAL INFORMATION:

4.1.1  NAME OF COMPANY:

4.1.2   DETAILS OF HEAD OFFICE:

ADDRESS:

TELEPHONE:

FAX:

E-MAIL:

WEB SITE:

4.1.3   DETAILS OF FACTORY / WORKS:

ADDRESS:

TELEPHONE:

FAX:

E-MAIL:

4.1.4   DETAILS OF MARKETING AGENT (OUTSIDE INDIA, IF ANY):

ADDRESS:

TELEPHONE:

FAX:

E-MAIL:

4.1.5   DETAILS OF INDIAN AGENT, IF ANY:

ADDRESS:

TELEPHONE:

FAX:

E-MAIL:

4.1.6   CHIEF EXECUTIVE OF THE COMPANY :

4.1.7  CONTACT PERSON(S) FOR PRODUCT OFFERED:

NAME(S):

DESIGNATION:

ADDRESS:

TELEPHONE:

FAX:

E-MAIL:

4.1.8  YEAR OF ESTABLISHMENT:

4.1.9   PRODUCTION CAPACITY PER ANNUM FOR VARIABLE FREQUENCY 	DRIVES:

4.1.10  PARTICULARS OF PRODUCT INCLUDING SPECIFICATION AND RANGE:                                     	(attach brochures and catalogues)

4.2   COUNTRY OF ORIGIN FOR OFFERED PRODUCTS AND TECHNOLOGY

4.3  FINANCIAL INFORMATION:

4.3.1  ANNUAL TURN OVER AND PROFIT AFTER TAX FOR LAST 3 YEARS: 	(attach copies of audited Balance Sheet and P&L Account)

YEAR – 2009-10:

YEAR – 2008-09:

YEAR – 2007-08:

4.3.2   DUN AND BRADSTREET REPORT FOR THE COMPANY

4.4    QUALITY AND ENVIRONMENTAL MANAGEMENT SYSTEM:

4.4.1 :   IS THE COMPANY ISO:9001 CERTIFIED : YES / NO.

(  IF YES, ENCLOSE COPY OF CERTIFICATE )

4.4.2 :   IS THE COMPANY ISO:14001 CERTIFIED : YES / NO.

( IF YES, ENCLOSE COPY OF CERTIFICATE )

4.4.3 :   IS THE COMPANY OHSAS 18001 CERTIFIED : YES / NO.

( IF YES, ENCLOSE COPY OF CERTIFICATE )

4.5  :    EXPERIENCE LIST FOR OFFERED/SIMILAR ITEMS

4.6 :     ANY OTHER INFORMATION

SECTION – 5

CHECKLIST OF DOCUMENTS TO BE SUBMITTED AS RESPONSE TO EOI

Following documents are to be provided along with response to Expression of Interest:

1   Covering Letter signed by an Authorised Signatory on Company letterhead,

listing clearly the Enclosures.

2   Catalogue  indicating design features, sizes, weights etc

3  Reference list  of fluid couplings  supplied/commissioned

4  Confirmation / Deviation to Technical Specifications – Section–2

5 Filled-up Qualifying Criteria Format – Section–3

6  Filled-up Company Profile – Section–4