PR NO: M2/12/6000000822/BRK				Specifications

| - CONFERENCE HALL TABLE (14 SEATER) QTY: 1 NO Material Code No. HY7802161754 |  |
| --- | --- |
| TOP:- Top work surface Thickness : 37.5 mm (18mm +18mm baton <skirting>+1mm DL+0.4mm membrane). Edge profile: Water fall edge with radius of 18mm (approx.) on top edge. Substrate MDF. Skin: PVC membrane foil (0.4 mm thick clad on the substrate of MDF using PU glue for better scratch resistance |
| UNDER STRUCTURE:- |  |  |
| LEGS:- Made from 36mm pre laminated particle board (18+18) having a straight profile with half round edges and clad with 0.6mm thick post forming laminate. Overall thickness of leg=38mm |  |  |
| MODESTY:- Modesty panel has to be made from pre laminated twin board of 18mm thick. |  |  |
| SIZE:- 5400mm (L) x 1400mm (W) x 750mm (H) Approx. size of two seater = 1350mm (W) x 600mm (D) Approx. size of Half round (2 seater) = R 700 mm - VISITOR CHAIR NON REVOLVING TYPE – QTY – 20 Nos. Material Code No. HY7802161665. SEAT / BACK ASSEMBLY:- The seat / back are to be made of 12 mm thick hot pressed ply wood, upholster with changeable upholstery fabric covers and moulded polyurethane foam, together with moulded back - spine cover. The back foam is to be designed with counter lumbar support for extra comfort. The Polyurethane foam is moulded with density - 45 ± 2 Kg/Cu. m and hardness = 20 ± 2 on Hamden machine at 25% compression. Rear side of the Back rest and bottom of the Seat shall be covered with Black Polypropylene covers. Seat Height : 440 mm, Height of Chair: 805 mm Width of Chair: 550 mm, Depth of Chair: 610 mm. ARM REST:- Arm rest are to be made of Black integral skin polyurethane with 50 to 70 shore hardness and reinforced with MS insert. The arm rest should be scratch and weather resistant. The arm rest are to be fitted to the seat and back with arm rest connecting strip assembly, made of 0.5 Cm thick HR steel. Base of the chair shall be fixed to the frame made of black powder coated MS Pipe with suitable fixtures. Pipe outer dia shall be 25 mm. Suitable Nylon bushes shall be fixed to the bottom of the MS Pipe frame.. |
| - CUSHION CHAIRS (REVOLVING TYPE) - QTY: 16 NOS. Material Code No. HY7802161762 Seat/ Back Assly: The seat and Back are to be made of 12 mm thick hot pressed plywood, upholstered with changeable fabric upholstery covers and moulded polyurethane foam, together with moulded back –spine cover. The back foam is to be designed with counter lumbar support for extra comfort. Seat ply size (Approx): 45 cm (W) x 53 cm (H) Polyurethane foam : The polyurethane foam is moulded with density=45+/- 2Kg /cu.mtr and Hardness= 20+/- 2 on Hampden machine at 25 % compression. Armrests: The armrests are to be made of black integral skin polyurethane with 50-70 shore hardness and reinforced with MS insert. The armrests should be scratch and weather resistant. The armrests are to be fitted to the seat with seat/ armrest connecting strip assly made of 0.5 cm thick HR steel. Permanent contact mechanism: The permanent contact mechanism with 360 degree revolving facility and provision for back tilt arrangement. Required facility for tilt tension adjustment has to be provided. Height adjustment: The gas lift for height adjustment may be of personalized choice with upright position locking arrangement. Telescopic below assly: The below with 3 piece telescopic type and injection moulded in black polypropylene. Pedestal Assly: The pedestal has to be fabricated using 0.2cm thick CR. Steel, powder coated and fitted with an injection moulded in black polypropylene hub cap. 5 Nos of twin wheel castors are to be provided. The castor wheels made of nylon material and twin wheel castors with hood for extra protection may be used. Dimensions (Approx.): Seat height = 41.5 - 53.5 cm. Height of chair = 93.5 – 105.5 cm Width of chair = 70 cm Depth of chair = 70 cm - HIGH BACK REVOLVING CHAIR – (with armrest) – QTY: 1 NO Material Code No. HY7802161738 SEAT / BACK ASSEMBLY:- The seat / back are to be made of 12 mm thick hot pressed ply wood, upholsted with changeable upholstery fabric covers and moulded polyurethene foam, together with moulded back - spine cover. The back foam is to be designed wih counter lumbar support for extra comfort. The Polyurethene foam is moulded with density - 45 ± 2 Kg/Cu. m and hardness = 20 ± 2 on hampden machine at 25% compression. Rear side of the Back rest and bottom of the Seat shall be covered with Black Polypropelene covers. Seat Height : 415 mm to 530 mm Height of Chair: 1000 mm to 1120 mm Back Rest Size: 500 mm (W) x 650 mm (H) ARM REST:- Arm rest are to be made of Black integral skin polyurethane with 50 to 70 shore hardness and reinforced with MS insert. The arm rest should be scratch and weather resistant. The arm rest are to be fitted to the seat and back with arm rest connecting strip assembly, made of 0.5 Cm thick HR steel. PERMEANENT CONTACT MECHANISM:- The permeanent contact mechanism with 360 degree revolving facility and provision for back tilt arrangement. Required facility for tilt tension adjustment has to be provided. HEIGHT ADJUSTMENT:- The Gas lift for height adjustment may be of personalised choice, with upright position locking arrangement. TELESCOPIC BELLOW ASSEMBLY:- The bellow with three piece telescopic type and injuction moulded in Black poly propylene. PEDESTAL ASSEMBLY:- The pedestal has to be fabricated using 0.2 Cm thick CR Steel, powder coated and fitted with an injuction moulded in Black Polypropylene hub cap. 5 Nos. of twin wheel castors are to be provided. The castor wheels made of Nylon material and twin wheel castors with hood for extra protection may be used. - SIDE DESK (RH/LH) - TOP PARTICLE BOARD 25MM – QTY: 1 NO Material Code No. HY5111762019 Side Desk material consists of attached side top made of high density particle board with melamine lamination, with post forming, free standing legs and consists key board tray and CPU Stand TOP:- Side Desk Top material consists of high density particle board with malamine lamination, 25 mm thickness. One side of Side desk top should be fixed to the main office desk top, with powder coated iron brackets made of HR Steel. Size: 1000 mm (L) x 500 mm (W) EDGE PROFILE:- Edge shall be full round. UNDER STRUCTURE: LEGS:- Made from 25 mm thickness pre-laminated particle board, having straight profile with full round edges and clad with post forming laminate. Size: 400 mm (W) x 650 mm (H) - 1 No. MODESTY:- Modesty panel has to be made from pre-laminated board of 18 mm thick having straight profile with post forming laminate. One side of modesty shall be fixed with powder coated iron brackets made of HR Steel to the leg of Main Office Desk and other side shall be fixed with powder coated iron brackets made of HR Steel to the leg of Side Desk. Size: 1050 mm (L) x 700 mm (W) - 1 No. NOTE:- Round shape openings (60 mm dia) shall be made in the corners of Top panels of Main Office Desk and Side Desk, along with PVC round caps for running of power, data and voice cables. |  |  |