- THE CABINETS SHOULD BE MANUFACTURED WITH PRIME  QUALITY CRCA SHEET OF GAUGES AS GIVEN BELOW:

A.	BOX UNIT (TOP, BOTTOM & SIDE)	- 18G

B.	ALL SUPPORTS				- 14G

C.	DRAWER FRONT				- 14G

D.	DRAWER BODY, BOTTOM & RACK	- 18G

E.	RAILING					- 10G

F.	LOCKING DOGS				- 14G

G.	FIXED CHANNEL				- 10G

H.	TELESCOPIC CHANNEL 			- 10G

I.	HANDLES					- MS EXTRUSION

- HEAVY DUTY DRAWERS SHOULD HAVE LOAD CAPACITY OF 200 KGS & LIGHT DUTY DRAWER OF 100 KGS.

- ALL THE DRAWERS SHOULD BE PROVIDED WITH 3 SLITTED PARTITIONS & 8 NOS. DIVIDERS THEREBY MAKING 12 SEPARATE COMPARTMENTS IN EACH DRAWER.

- THE HEAVY DUTY DRAWER SHOULD BE PROVIDED WITH TELESCOPIC CHANNELS MOUNTED ON HEAVY DUTY TELESCOPIC SLIDES. IT SHOULD HAVE 8 SHIELDED ANTI-FRICTION BEARINGS (LUBRICATED LIFELONG) MOUNTED ON HARDENED CARBON STEEL PINS/ROLLERS. THERE SHOULD ALSO BE 2 NOS. OF NYLON IDLE ROLLERS IN EACH DRAWER MOUNTED ON HARDENED STEEL PIN/ROLLERS. IN ALL, THERE SHOULD BE 10 NOS. OF ROLLERS IN EACH DRAWER. IT SHOULD HAVE FULL EXTENSION SLIDES & SHOULD BE RETRACTABLE.

- LIGHT DUTY DRAWER SHOULD BE RETRACTABLE TO THE EXTENT OF 85% OF THEIR LENGTH. IT SHOULD BE PROVIDED WITH 4 NOS. OF ANTI-FRICTION BEARING, LUBRICATED LIFE LONG, IN EACH DRAWER.

- ALL THE DRAWERS (SAME HEIGHT) SHOULD BE INTERCHANGEABLE. NO PROBLEM SHOULD BE FELT DURING INTERCHANGING.

- PRIME QUALITY OF LOCKING ARRANGEMENT SHOULD BE PROVIDED. THE MECHANISM SHOULD ENABLE USER TO KEEP ONE DRAWER OPEN & ALL OTHERS CLOSED. THE OPEN DRAWER CAN BE LOCKED LATER WITHOUT HAVING TO UNLOCK OTHER DRAWERS BY MERELY PUSHING IT. KEYS SHOULD BE PROVIDED IN TRIPLICATE.

- ALL STEEL COMPONENTS SHOULD BE GIVEN 7 STEP ANTI RUST TREATMENT. AFTER PRE-TREATMENT & PRIMER COATING, ALL THE ITEMS SHOULD BE PAINTED WITH ELECTROSTATIC POWDER COATED PAINT.

- THE BODY SHOULD BE DESIGNED TO TAKE A LOAD OF 2500 KGS PLUS AN ADDITIONAL LOAD OF A 2500 KGS OF A FULLY LOADED CABINET TO SAVE SPACE.

12.	THE TOOL CABINETS SHOULD BE COMMISSIONED AT BHEL BY SUPPLIER'S REPRESENTATIVE.

13. RIGID ALUMINUM EXTRUSION HOLDER CUM LABEL HANDLE TO BE

PROVIDED FOR EACH DRAWER.

- FASTENERS USED DURING ASSEMBLY MUST NOT RUST.

15. TC FOR MATERIAL USED & 7 TANK PROCESS FOR SHEET METAL USED

DURING MANUFACTURING MUST BE FURNISHED.

16. GC FOR TROUBLE FREE SERVICE OF ALL COMPONENTS & ACCESSORIES

AFTER INSTALLING MUST BE FURNISHED. GC FOR PAINT NOT PEELING

OFF DURING USE IS ALSO REQUIRED.

17. DETAILED PRE DISPATCH INSPECTION MAY BE CARRIED OUT BY BHEL

REPRESENTATIVE AT PARTY’S WORKS.

18. THE OFFER SHOULD BE COMPLETE IN ALL RESPECTS WITH TECHNICAL   DETAILS SHOWING PHOTOGRAPH, DIMENSIONS, WEIGHT ETC. OF OFFERED ITEMS. THE OFFER SHOULD BE ENCLOSED WITH COMPANY'S PRINTED CATALOGUE SHOWING THEIR PRODUCTS AND TECHNICAL DETAILS RELEVANT TO ABOVE MENTIONED ALL REQUIREMENTS.

19. DRAWER COMBINATION, WITH APPROX. DIMENSIONS, IS AS FOLOWS:

50MM-3 NO. LIGHT DUTY

75MM-3 NO. LIGHT DUTY

100MM-1 NO. HEAVY DUTY

125MM-1 NO. HEAVY DUTY

150MM-2 NO. HEAVY DUTY