ANNEXURE –I

SCOPE OF WORK

1.  To carry out Breakdown and Preventive Maintenance of vacuum plants including Vapour Phase Drying Plants and Oil treatment Plants in CPM ,BCM AND UHV Transformer BLIII ( 52  Nos. of preventive maint.) as per Schedule (Annexure-I) ,Annexure –II and instructions of P.M. Card .

2.  Checking for leak if any in the system and to take corrective measure to maintain the plants for leak proof functioning.

3.   Regular checking of the oil level of the vacuum pumps and booster pumps and also heating oil of the respective plants.

4.   Periodic checking of pumping down time in order to ascertain the efficiency of   vacuum pumping system.

5.   Replacement of damaged rubber parts.

6.   Checking and rectification of power drives and coupling.

7.   Checking and rectification of condensers for sealing and cooling.

8. Checking and rectification of requisite amount of cooling water for the various

Equipments.

9.   Checking and rectification of vacuum for accurate functioning.

10.  Checking and rectification of proper functioning of electrical circuit.

11.  Periodic checking of leakage of the plants and systems to carry out the leak rate test to achieve the required value of leak rate of the plants.

12. You will have to modify the pumping system of vacuum line to accommodate the vacuum pumps of the other make and model to restore the efficiency of the plants if needed.

13. Checking and repairing of Vacuum Pumps.

14.   Gas cutting and welding work if needed will be done by you free of cast.

15.   All spares parts, consumables, lifting tackles wherever necessary,  will be  provided  by BHEL free of cost.

16.  You have to attend the breakdown of mechanical and electrical nature whatsoever, may be during the operation of the plants..

17.   If the B/D is major then you will have to continue the repair work during the night shift, Sundays, and holidays also.

18.  You have to depute, minimum 08 Nos. of   Skilled Fitters and 04 nos of unskilled fitters to maintain three shift for whole of the year .

19.  If any critical plants are reported under breakdown at any point of time the contractor have to detain their staff and attend all the pending break downs after 16.00 hrs. and after 1.00A.M. and on Sunday and holiday also. BHEL will not pay any thing extra to compensate it. It will be on contractor s account only.

20.  List of vacuum plants and oil treatment plants including vapour phase plants are given in Annexure-II.

DY MGR (FRX)