BHARAT   HEAVY ELECTRICALS LIMITED

IVP GOINDWAL – 143422

CIVIL ENGINEERING DEPARTMENT

# PREAMBLE

01.	The scope of work includes Extension of shop floors, and other infrastructure works under Valves Capacity Augmentation in IVP complex, BHEL, Goindwal,Punjab involving all Civil, Structural steel, Water supply, Drainage, Approach road and other allied works.

02.	Time is the essence of the contract.  Being a time bound expansion scheme with capital expenditure, the contractor should make all efforts to complete the work in time.  Eventhough the overall completion period is indicated as  07 months, the shop floor and other works shall be completed progressively and handed over as per agreed split up  schedule.

03. 	Usually, working hours are limited to day time only.  In emergency cases to carryout works during nights, it may be done so with the specific prior permission of BHEL. In such case, it is to be carried out in the presence of BHEL officials or their authorized persons.

04.	The tenderers are advised to visit BHEL Factory at Goindwal and get themselves acquainted with the site conditions before submitting the offer.

05.	The following eligibility criteria shall be complied to fulfill the Qualification Bid:-

- Contractor should have separate Registration Code No. for EPF & ESI .

- Average annual financial turnover of similar works in the last three years should be 30% value of respective work.

- During last seven years should have successfully completed works either i)Three similar works each not less than 40% value of respective work or ii)Two similar works each not less than 50% value of respective work or iii)One similar work not less than 80% value of respective work.

- Value of structural steel works completed in the last three years should be atleast 30% value of the minimum required annual turn over as given in criterion 05b..

- Solvency shall be produced for atleast 40% value of respective work.

06. 	The norms for qualification with prescribed score is attached in the Qualification Bid Document. Evaluation will be done accordingly. Minimum score required for qualification is 60.

07.	The value of turnover and the value of similar works mentioned above excludes cost of cement, reinforcement steel and structural steel. However if the turnover value furnished by the tenderer includes the above materials, then the tenderer has to furnish the cost of the same separately. If it is not furnished, the materials cost will be presumed as follows: -

Civil & structural works combined                                        -  50 %

Civil works alone                                                                  -  35 %

Structural works including cloaking items                            -  70 %

Other minor maintenance works                                          -  20 %

08. 	The works executed in the own name of the tenderer only will be considered for eligibility criteria.

09. 	Dissimilar / irrelevant works will not be considered for eligibility criteria.

10. 	Documentary evidences (Xerox copies - attested) for turnover, works experience, P.F, ESI registration etc., all as indicated & required in the tender document should be furnished without which it will not be taken into account.

11. 	The tender value excludes cost of cement, reinforcement steel & structural steel which will be supplied by BHEL at Stores at free of cost. Other free issue materials by BHEL are indicated in Schedule B of the Price bid document.

12.	Tender Value is only approximate and liable for variation without entitling the tenderer to any compensation, till the total value of contract vary by more than 20% (twenty percentage).

13.	Quoted rate shall be firm throughout the contract period  of 07 months and extended  contract period also and  no  cost escalation is allowed on any account.

14.	The work shall be carried out as per drawings released then and there, Civil Engineering Department Work & Safety procedure, AWS / BIS specifications, standard code of practice and as per the instructions of Engineer-in-charge.  The brief description of items of work is given in the bill of quantities provided in the Price Bid. Tenderer has to quote rates both in figures and in words for all the items given in the Bill of Quantities provided in the Price Bid. (From page No.24   to  62  ).

15. 	The item rate offered is for finished item of works and shall provide for the complete cost towards fuel, tools, tackles, plant & machinery, temporary works, labour, materials, levies, taxes, transport, lay-out, repairs, rectifications, maintenance till handing over, supervision, labour colonies, establishment, services, roads, revenue expenses, overheads, profits & all other incidentals etc., complete.  However if the service tax is applicable for this contract, the same will be reimbursed on production of valid document proof for having paid service tax by the tenderer.

16. 	For any item of work not covered in Bill of Quantities, the rate will be arrived at based on the conditions given in BHEL General Conditions of Contract.

17. 	After award of work the contractor has to furnish the security deposit, as per Clause 13 of Tender Notice, attached in the Price Bid.  Also it is to be noted that after award of work the contractor has to furnish 50 % of security deposit before the commencement of work.

18.	The contractor should bring the construction earth moving machineries like Poclain,  Paver, Road Roller, JCB etc., as and when required for the work without any delay.  Also required number of the following tools & Plants / instruments shall be made available always at site for the works.

a)	Precise Leveling instruments and theodolite.

b)	Full load mixture machine / mini batching

c)	Petrol / Kerosene driven vibrator/ mechanical vibrat

d)	Cube moulds

e)	Inter carting vehicle (tractor, hand trolley etc)

f)	Hydra cranes

g)	Power operated winches

h)	Mobile / Hydraulic crane

- Welding machine etc.

19.	The contractor has to arrange sufficient number of lorries to collect & transport the surplus earth, construction debris generated, etc. at site. Otherwise BHEL would clear the debris at the contractor’s risk and cost.

20. 	The contractor has to execute any item of work irrespective of the quantity without any reservation.

21. 	The contractor has to carryout the work in production shops without affecting the production by planning such work on holidays / obtaining proper shut down.

22. 	The decision of Engineer-in-charge shall be final and binding on the contractor regarding clarification of items in this tender schedule.

23. 	The contractor shall strictly adhere to various labour laws in force.

24. 	The contractor shall follow norms of BHEL security system for movement of men & materials within the complex.

25.	Since the responsibility for the quality, workmanship and accuracy of any work being carried out under this contract lies with the contractor, the contractor should ensure that no work is done without the presence of contractor’s representative at the work spot.

26.	The contractor has to submit the organization chart of their set up for the  works and any change thereafter in the organization set up shall have the prior approval of BHEL.

27.   	Statement of completed works with detailed measurements along with material reconciliation statements shall be submitted by the contractor in the last week of every month for processing bill.

28. 	The contractor should establish his own site office, fabrication yard, handling facility, storage facility to stock about 300 MT of cement etc., for which vacant land will be allotted on specific request for which the rent  will not be collected.

29.	The contractor is required to carry out construction activities as directed by the department officials.  It is required to engage certain minimum strength of staff for effective supervision of works as indicated below:

Site in-charge / Construction Manager 				-	1 No.

Planning Engineer and co-ordinator				-	1 No.

Qualified Surveyor						-	1 No.

Civil Engineer (Graduate in Civil Engineering)	          	 	-	2 Nos.

Structural Engineer (Graduate in Mechanical / Civil Engg.)	- 	2 Nos.

Supervisor/Engineer (Diploma Holder)              			-           4 Nos.

Level-II NDT Inspector						-	1 Nos.

In addition to the above, Quantity surveyors have to be engaged to prepare measurements and submit    the bills.

30.	Adequate number of Quality Engineer and qualified Safety Engineer should be deployed at site. Experienced  Engineers / Supervisors are  to be engaged in Railway and water supply works.

31. 	To safeguard the persons working at height in roof, wall etc., sufficient number of Industrial Safety nets shall be provided at tenderer’s cost in appropriate level and locations.  The working hand including Supervisors, Engineers should wear the personal protective items and safety measures such as helmets, safety belts, shoes, etc., before entering into working place.

32.	The tenderer has to deploy adequate labour of required categories such as Unskilled, Skilled, Mason, Carpenter, Plumber, Welder, Fitter, Mistry, Technically experienced, etc. so as to execute the works simultaneously in all areas of work.   The Technical persons with experience shall have to produce valid certificate for verification.

33.	Expertised labour only to be engaged for specialized items of work like laying of ceramic tiles, marbles, cuddapah slabs, granite slabs and false ceiling, partition, wall paneling, architectural finishing etc. and work experienced persons shall be engaged for fabrication, water supply, railway track laying and aligning works, sewerage system work, etc.

34.	Exclusive Stores personnel should be engaged who would co ordinate with department official for clearance and collection of BHEL supplied materials required for works.

35.	Separate non-technical persons should be engaged for arranging daily gate passes for labours and vehicles entry in all the gates of Factory Complex.

36. 	The tenderer shall carry out health performance test at his cost for all the workmen engaged  in the work through a registered medical practitioner and produce certificate on demand.

37. 	It shall be the responsibility of the contractor to see that the workmen do not utilize the departmental canteen facilities.  Contractor has to make his own arrangements to provide refreshment for the workmen.

38.	All the works shall be executed as per the standard specifications as provided in CPWD / BIS.

39.	The works contract to be entered into with the successful tenderer will be governed by the BHEL Revised General Conditions of Contract in force.

40.	Some changes are likely in the quantities furnished as well as in the layout, design and specifications of the work.  The rates quoted shall be deemed to be inclusive of all such contingencies.

41.	All the materials to be used in the work and the nature of work shall conform to the respective CPWD & BIS and National Building Organisation, Standard Specifications forming part of "ALL INDIA STANDARD SCHEDULE OF RATES" specifications and shall be got approved by the Engineer-in-charge before actual incorporation in the work.

42.	The contractor should maintain proper accounts for cement, reinforcement steel and structural steel sections and other materials if any supplied by this organisation and these should be available at the site of work for verification and check by the Officials of this Organisation at any time.

43.	All materials brought by the Contractor for incorporation in the work shall be got inspected and approved by the Engineer-in-charge before they are incorporated in the work.

All the structural works including fabrication, erection etc., are subjected to inspection and clearance by Third Party Agency ,IF ANY,fixed by BHEL.

44.	All tools and plants including jack hammer with silencer  & bits, rock drilling machine, etc., required for all the works covered under this contract will have to be brought by the contractor.  Compressed air alone will be provided at viable locations at free of cost by BHEL. From that point the contractor has to provide further arrangements for site works.

45.	The contractor should be in a position to make his own arrangement for compressed air without any extra claim for the uninterrupted operation of jackhammer with silencer wherever BHEL is not in a position to supply compressed air.

46.	All safety measures are to be followed during execution of work, particularly during blasting and only licensed blaster should be engaged for this purpose.  Sufficient care shall be taken by the contractor during excavation to avoid damages to the buried pipe lines, cables and other infrastructure like railway lines if any etc. Controlled blasting including muffling can be carried out with prior permission from safety department.

47.	The contractor should use only the materials of brand and quality as approved by BHEL.

48.	The contractor should submit the programme for the completion of work and the list of machineries and site personnel to be deployed for the work along with tender.

49.	All materials and consumables brought by the contractor should have manufacturer’s certificate.

50.	The contractor should use only weigh batch for all reinforced cement concrete  works (controlled concrete).  The tenderer is advised to install concrete mini batching plant at site considering the quantum of concrete work involved.

51.	Contractor’s materials and tools & plants shall have to be brought inside factory with proper invoice / voucher and make necessary entry in the security gate.  They should maintain proper record for tools and plats, materials, etc., brought inside the factory complex.

52.	The contractor should submit the mix design for RCC for different mix of concrete for different grade of cement at his cost through reputed institution.

53.	The contractor should extend fullest co-operation with the third party agencies engaged, if any by BHEL to adhere the Quality Control Procedures ensuring quality.

54.	The contractor should extend full co-operation to the other contractors who may be doing other works in the same areas to enable them to execute their portions of work without any delay or difficulty.

55.	With regard to specifications not covered by the General and Special Conditions of Contract, those contained in the CPWD Specifications or other specifications approved by Bharat Heavy Electricals Limited, shall apply.

56	Water required for the work may be provided by this organization at only one point at the     site of the work at free of cost. In case of failure of water supply, the Contractor will have to make his own arrangements for water without any extra claims until supply is restored. BHEL does not accept any liability whatsoever for non-supply or delay in the supply of water under any circumstances. The contractor shall ensure that there is no wastage of water otherwise supply of water is liable to be stopped at contractor’s risk and cost.

57.	Electrical energy required for the work may be given  by this organization at any one point at the site of work at free of cost. BHEL does not accept any liability whatsoever for non-supply, delay in supply or failure of supply of electrical energy.  Contractor shall ensure that there is no wastage of electrical energy otherwise supply is  liable to be stopped at contractor’s risk and cost.  The contractor shall make leak proof / fire proof shed and provide control panel board of required capacity and lay the required cables at their own cost for further distribution.

58.	Purchase preference policy will be extended to CPSE bidders subject to other terms & conditions as provided in the office memorandum issued by DPE.

59.	No advance / mobilization advance will be given.

60.	For the agencies who had already been qualified and enlisted by BHEL under various money category, price bid document for the eligible value of work  can be obtained after paying  the cost of tender document.  They already enlisted and eligible agencies may submit document duly signed in all pages and no Technical evaluation is applicable for item.

61.	LD / Penalty clause is applicable as per General Conditions of Contract.

- BHEL reserves its right to reject a tender  due to unsatisfactory past performance in the execution of  a contract at another project/unit awarded against a different enquiry.

63. 	BHEL reserves its right to accept/reject any or all the tenders and to award the work in full or split to more than one agency without assigning any reasons.

# NORMS FOR QUALIFICATION

(VALUE OF WORK: Rs. 253 LakhS)

| # I | ELIGIBILITY:- |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1. | Separate Registration for EPF & ESI. |  |  |
|  | 2 | Average annual turnover in the last 3 years shall be atleast 30% value of the respective work. (Rs.76L) |  |  |
|  | 3. | Three similar works not less than 40% value of the respective work.(Rs.102L) |  |  |
|  |  | or |  |  |
|  |  | Two similar works not less than 50% value of the respective work. (Rs.127L) |  |  |
|  |  | or |  |  |
|  |  | One similar work not less than 80% value of the respective work.(Rs.203L) |  |  |
|  | 4. | Value of structural steel works completed in last 3 years should be atleast of value Rs. 30L |  |  |
|  | 5. | Solvency shall be produced for atleast 40% value of the respective work. (Rs.102L) |  |  |
| II | NATURE OF COMPANY |  | 5 |  |
|  | Public Limited |  | 5 |  |
|  | Private Limited/Partnership firm |  | 3 |  |
|  | Sole Proprietor |  | 2 |  |
| III | Similar Experience (Civil Works) |  | 10 |  |
|  | Total value of civil works executed in the last three years |  |  |  |
|  |  |  | 10 | (Rs. 107L) |
|  | (Pro-rata for in between cases) |  | 6 | (Rs. 54L) |
| IV | Similar Experience (Structural steel works) |  | 20 |  |
|  | Value of structural steel works executed related to Civil Engg. in last three years |  |  |  |
|  |  |  | 20 | (Rs. 120L) |
|  | (Pro-rata for in between cases) |  | 10 | (Rs.60L) |
|  | (Pro-rata for in between cases) |  | 6 | (Rs.30L) |
| V | Performance on previous works |  | 10 |  |
|  | Successful completion of three major works in time. |  | 10 |  |
|  | Successful completion of two major works in time. |  | 8 |  |
|  | Successful completion of one major work in time. |  | 6 |  |

| VI | Highest value of single work completed | 10 |  |
| --- | --- | --- | --- |
|  |  | 10 | (Rs.203L) |
|  | (Pro-rata for in between cases) | 6 | (Rs.101L) |
| VII | Average Annual Turnover in last 3 years | 10 |  |
|  |  | 10 | (Rs.154L) |
|  | (Pro-rata for in between cases) | 6 | (Rs.76L) |
| VIII | Equipments owned | 10 |  |
|  | Welding & erection equipments, crane, concreting equipments etc. (Mixer machine with Hopper, Vibrator, Shuttering, Pumps etc. (Min. two sets of equipments) | 10 |  |
|  | Welding & erection without crane & concreting equipments. (Min. two sets) | 8 |  |
|  | Welding & erection equipments only available. (Min. two sets) | 6 |  |
|  | Concreting equipments only available.(Min. two sets) | 4 |  |
| IX | Qualified staff availability | 5 |  |
|  | If adequate Graduate Engineers, Supervisors & Tech. Staff available. | 5 |  |
|  | If adequate Graduate Engineers & Supervisors available. | 4 |  |
|  | If adequate Graduate Engineers & Tech. Staff only available. | 3 |  |
|  | If adequate Supervisors & Tech. Staff only available. | 2 |  |
| X | Financial stability | 10 |  |
|  | Solvency |  |  |
|  |  | 10 | (Rs.203L) |
|  | (Pro-rata for in between cases) | 6 | (Rs.102L) |
| XI | PAN & ITCC for the past three years. | 10 |  |

NOTE:

- Minimum score required for qualification is 60 out of 100.

- All the above financial value excludes cost of cement, steel reinforcement and structural steel.

BHARAT HEAVY ELECTRICALS LIMITED

IVP GOINDWAL- 143422

CIVIL ENGINEERING DEPARTMENT (FACTORY)

QUALIFICATION PROFORMA

1.	Name of applicant 	:	Shri / M/s

2.	Nationality		:	Indian		      other

3.	Address		:	………………………………………………………..

………………………………………………………..

………………………………………………………..

………………………………………………………..

4.	Telephone No.	:

Fax No.		:					          E-mail :

5.	Constitution		Individual			Sole Proprietorship concern

Partnership Firm		Private Ltd. Company

Public Ltd. Company

- In case of Limited Liability Company or Company 	:

Limited by Guarantees.

A)  amount of paid-up-capital

| 7. | Enclose a chart showing Company Organisation indicating various factories / workshops / sites / divisions etc., with levels and names of key personnel and clearly depicting the reporting and functional relationships. Also give brief write-up on the functions and responsibilities of each entity. |
| --- | --- |
| 8. | Details of Owner / Partners / Directors (Strike out whichever is not applicable) |

| Sl. No. |  | Name & Designation | Occupation Address | Telephone No. |  | Fax No. | E-mail |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | To be furnished in separate sheet - |  |  |  |  |  |
| 9. | Value of property owned, Certificate to be produced from the Revenue Department. |  |  |  |  |  |  |
| 10. | Does the applicant maintain any office for preparing designs, drawings, bills, etc. |  |  |  |  |  |  |
| 11. | Does the applicant posses facility to set up laboratory for Quality Control at site of work. If so please furnish full details of organisation, equipments, tools, etc. |  |  |  |  |  |  |

| 12. | Is the individual/sole proprietor/any partner/ directors of company: |  |
| --- | --- | --- |

| (a) | Dismissed Government Servant | Yes |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| (b) | Having business banned/suspended by any | Yes |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | government in the past |  |  |  |  |  |  |

| (c) | Convicted by a court of law | Yes |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| (d) | Retired Engineer / Official from Engineering | Yes |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Departments of Govt. of India within last two years |  |  |  |  |  |  |

| (e) | Director or partner of any other company / firm | Yes |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | enlisted with CPWD or any other department |  |  |  |  |  |  |

| (f) | Member of Parliament or any State Legislative |  |  |  |  |  |  | Yes |  |  |  |  | No |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Assembly If answer to any of the above is ‘Yes”, furnish details on a separate sheet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13. | Name of person holding power of attorney. (Copy to be enclosed) |  |  |  |  |  | …………………………………………………… |  |  |  |  |  |  |  |  |  |  |  |
|  | (a) | Nationality | Indian |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |
|  | (b) | Liabilities |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. | Name of Banker with full address (Note: Banker’s report in original preferably in sealed cover, giving the financial capacity to handle works of the required magnitude should be enclosed) |  |  |  |  | …………………………………………………………………………………………………………………………………………………………………………………………….. |  |  |  |  |  |  |  |  |  |  |  |  |
| 15. | Place of business |  |  |  |  | ……………………………………………… |  |  |  |  |  |  |  |  |  |  |  |  |
| 16. | Date of commencement of business |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17. | Details of Income Tax paid during last three years. |  |  |  | 1. 2006-07 2. 2005-06 3. 2004-05 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 18. | State whether Income Tax Clearance |  | Yes |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Certificate from the appropriate authority in the prescribed form enclosed. |  |  |  |  |  |  |  |
| 19. | Contractor’s capital in the business. (in case of partnership, please mention percentage of shares and amount) |  |  |  |  |  |  |  |
| 20. | Quantum of business done during last three financial years | 1. 2006-07 2. 2005-06 3. 2004-05 |  |  |  |  |  |  |
| 21. | Value of fixed assets of the business in the last three years | 1. 2006-07 2. 2005-06 3. 2004-05 |  |  |  |  |  |  |
| 22. | Guarantee limits (if any) enjoyed by the firm. |  |  |  |  |  |  |  |
| 23. | Over-draft limits (if any) enjoyed by the firm. |  |  |  |  |  |  |  |

| 24. | State whether Audited report for Profit and | Yes |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Loss Account & Balance Sheet for last three years enclosed. |  |  |  |  |  |  |

| 25. | Details of Technically qualified staff :- |  |  |  |
| --- | --- | --- | --- | --- |
| Sl. No. | Name and Designation | Qualification | Experience and Specialisation | Remarks if any |

| 26. | Whether the details of T & P, Machinery, | Yes |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Equipments and work shop as per Annexure – I given. |  |  |  |  |  |  |

| 27. |  |  | Whether enlisted with any other Department |  |  |  | Yes |  |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (a) |  |  | If yes, give details: |  |  |  |  |  |  |  |  |  |
|  |  | (i) |  |  | Name of Department & address | …………….…………………..………………. |  |  |  |  |  |  |  |
|  |  | (ii) |  |  | Money limit | ………..……………………………………. |  |  |  |  |  |  |  |
|  |  | (iii) |  |  | Enlistment No. & date | ………………………………………………… |  |  |  |  |  |  |  |
|  |  | (iv) |  |  | Valid upto |  |  |  |  |  |  |  |  |

| 28. | Licence No. and validity of licence obtained from Dy. Chief Inspector of Factories / Assistant Commissioner of Labour |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 29. | Whether the applicant has registered his workmen under Employees’ State Insurance Act. If so, code number may be furnished. If applied, attested copy of application for registration acknowledged by ESI Authorities. |  |  |  |  |
| 30. | Whether the applicant has registered his workmen under Employees’ Provident Funds and Miscellaneous Provisions Act ?. If so, the code number may be furnished. If applied, attested copy of application for registration with acknowledged by PF Authorities. |  |  |  |  |
| 31. | Indicate Central / Local Sales Tax, Excise Duty code Numbers and PAN. | 1. 2. 3. 4. |  | CST LST ED PAN |  |

| 32. | Is any person working with the applicant as a | Yes |  |  | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | near relative of the Officer / Official of BHEL |  |  |  |  |  |  |
| (a) | If yes, give details |  |  |  |  |  |  |

|  | (i) | Name | …………………………………………… |  |
| --- | --- | --- | --- | --- |
|  | (ii) | Staff No. | …………………………………………… |  |
|  | (iii) | Designation & Department | …………………………………………… |  |
|  | (iv) | Unit |  | …………………………………………… |

| 33. | Details of similar works completed during the last seven years (To be submitted in separate sheet as per Annexure-II.) |  |
| --- | --- | --- |
| 34. | Certificates from clients in original as per proforma given in Annexure -III for all eligible works. |  |
| 35. | Certificates: |  |
| (i) | I/We (including all partners) certify that I/We have read the Preamble & Terms and conditions and shall abide by them. |  |
| (ii) | I/We certify that the information given above is true to the best of our knowledge. I/We also understand that if any of the information is found wrong, I/We am/are liable to be debarred. |  |
| (iii) | I/We certify that I/We will not get myself / ourselves registered as contractor(s) in BHEL under more than one name. |  |
| (iv) | (a) | I certify that I did not retire as an Engineer of Gazetted rank or as any Gazetted Officer employed on Engineering or Administrative duties in any Engineering Department of the Government of India during the last two years. I also certify that I have neither such a person under my employment nor shall I employ any such person within two years of his retirement except with the prior permission of the Government. (For Individuals seeking enlistment in their own name). |
|  | (b) | We certify that none of the partners/directors retire as an Engineer of Gazetted rank or as any Gazetted Officer employed on Engineering or Administrative duties in last two years. We also certify that we have neither under our employment any such person nor shall we employ any person within two years of his retirement except with the prior permission of the Government. (For partnership firms and limited companies). |

Signature(s) of the applicant(s)

Name					Signature		         Address

(Seal in case of  Firm)

| 1. | ……………………………………... | …………………………. | ………………………………… |
| --- | --- | --- | --- |
| 2. | ……………………………………... | …………………………. | ………………………………… |
| 3. | ……………………………………... | …………………………. | ………………………………… |
| 4. | ……………………………………... | …………………………. | ………………………………… |
| 5. | ……………………………………... | …………………………. | ………………………………… |

Date:

NOTE: 1) All the relevant certificates, details etc. should be attached with the application.

2) The terms that are not applicable may be scored out.

Details of documents attached:-

1.

2.

3.

4.

5.

6.

7.

8.

9.

10.

ANNEXURE - I

DETAILS OF PLANTS AND EQUIPMENTS OWNED

| Sl. No. | Name of Equipments | Nos. | Capacity or Type | Age | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | ##### Earth moving equipments |  |  |  |  |
| 1. | Excavator (Various sizes) |  |  |  |  |
|  | ##### Equipments for hoisting & lifting |  |  |  |  |
| 1 | Mobile crane |  |  |  |  |
| 2. | Tower crane |  |  |  |  |
| 3. | Builder’s hoist |  |  |  |  |
|  | ##### Equipments for concrete works |  |  |  |  |
| 1. | Concrete batching plant |  |  |  |  |
| 2. | Concrete pump |  |  |  |  |
| 3. | Concrete transit mixer |  |  |  |  |
| 4. | Concrete mixer (diesel) |  |  |  |  |
| 5. | Concrete mixer (electrical) |  |  |  |  |
| 6. | Concrete vibrator (electrical) |  |  |  |  |
| 7. | Concrete vibrator (petrol) |  |  |  |  |
| 8. | Table vibrator (elect./petrol) |  |  |  |  |
|  | ##### Equipments for building works |  |  |  |  |
| 1. | Block making machine |  |  |  |  |
| 2. | Bar bending machine |  |  |  |  |
| 3. | Bar cutting machine |  |  |  |  |
| 4. | Wood thickness planer |  |  |  |  |
| 5. | Drilling machine |  |  |  |  |
| 6. | Circular saw machine |  |  |  |  |
| 7. | Welding generators |  |  |  |  |
| 8. | Welding transformers |  |  |  |  |
| 9. | Cube testing machine |  |  |  |  |
| ##### 10. | ##### Steel shuttering |  |  |  |  |
| 11. | Steel scaffolding |  |  |  |  |
| 12. | Grinding/polishing machine |  |  |  |  |
|  | ##### Equipments for road works |  |  |  |  |
| 1. | Road roller |  |  |  |  |
| 2. | Bitumen paver finisher |  |  |  |  |
| 3. | Hot mix plant |  |  |  |  |
| 4. | Spreaders |  |  |  |  |
| 5. | Earth rammers |  |  |  |  |
| 6. | Vibratory road roller |  |  |  |  |
|  | ##### Equipments for transportation |  |  |  |  |
| 1. | Tipper |  |  |  |  |
| 2. | Truck |  |  |  |  |
|  | ##### Pneumatic equipments |  |  |  |  |
| 1. | Air compressor (diesel) |  |  |  |  |
|  | Dewatering equipments |  |  |  |  |
| 1. | Pump (diesel) |  |  |  |  |
| 2. | Pump (electrical) |  |  |  |  |
|  | Power equipments |  |  |  |  |
| 1. | Diesel generator |  |  |  |  |
|  | Any other plants/equipments |  |  |  |  |

ANNEXURE - II

| DETAILS OF SIMILAR WORKS COMPLETED DURING THE LAST SEVEN YEARS (2001 – 2002 TO 2007-2008) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Name of work & Agreement No. | Date of commencement | Date of completion |  | Reasons for delay & compensation levied, if any | Work order Value | Gross cost of completion |  | Name, designation & complete address of the authority for whom the work was done |
|  |  |  | Stipulated | Actual |  |  | Including cost of cement, steel reinforcement & strl. steel | Excluding cost of cement, steel reinforcement & strl. steel |  |
| DETAILS OF WORK COMPLETION CERTIFICATES, WORK ORDERS ETC. ARE TO BE FURNISHED |  |  |  |  |  |  |  |  |  |

ANNEXURE - III

#### CLIENT’S  CERTIFICATE  REGARDING  PERFORMANCE OF CONTRACTORS

Name & Address of the Client

…………………………………………………………… ………….

……………………………………………………………………….

Details of works executed by Shri . M/s   ………………………………………………

………………………………………………

1.	Name of work with brief particulars		:

2.	Agreement No. and date			:

3.	Date of commencement				:

4.	Stipulated date of completion			:

5.	Actual date of completion			:

6.	Details of compensation levied for delay, if any:

7.	Tendered amount				:

8.	Gross amount of the work completed		:

9.	Name and address of the authority under	:

whom work executed

10.	Whether the contractor employed qualified	:

Engineer/Overseer during execution of work?

11.	(i)  Quality of work (indicate grading)		:	Outstanding/V.Good/Good/Poor

(ii)  Amount of work paid on reduced rate	:

basis, if any

12.	(i)    Did the contractor go for arbitration ?	:

(ii)   If yes, amount of claim			:

(iii)  Amount received				:

13.	Comments on the capabilities of the contractor

- Technical Proficiency			:  Outstanding/V.Good/Good/Poor

(b)	Financial Soundness			:  Outstanding/V.Good/Good/Poor

(c)	Mobilisation of adequate T & P		:  Outstanding/V.Good/Good/Poor

- Mobilisation of manpower		:  Outstanding/V.Good/Good/Poor

- General behaviour			:  Outstanding/V.Good/Good/Poor

NOTE:  All columns should be filled in properly.

Signature of the Certifying Officer

with Official seal.

### GENERAL AND SPECIAL SPECIFICATIONS

1. Site Clearance

All the area upon which the construction is to be carried out and areas which are required by the Contractor  for his construction facilities are to be cleared off all rubbish and objectionable matter at Contractor’s own cost. Trees, if any, shall not be uprooted or cut without the prior approval of the Engineer-in-charge. All spoils, unserviceable materials and rubbish shall be burnt or removed from site. Usable materials, saleable timber, fire-wood etc., shall be stocked properly at work site in the manner as directed by the Engineer-in-charge.  The cost of clearing the areas shall be deemed to have been included by the tenderer in his general rate.

2.  Earthwork for Foundations

Earthwork excavation for foundations and filling in foundations shall conform to CPWD  CLAUSES 2.8 AND 2.9 respectively.The area to be excavated or filled in with excavated materials shall be clearly demarcated in the field by the Contractor. Excavation shall be done to lines and levels defined.  Excavation shall be carried out to such widths, lengths, depths, profiles and grades as shown  in the drawing or as may be specified by  the Engineer – in – charge.   Rough Excavation may be carried out up to a depth 15 cm less than the final level but the balance shall be excavated to precise level with special care. All soft pockets of soil met with even below the final level shall be removed and the excavation filled up as directed  by Engineer-in-charge. The methods of excavation shall in every case be subject to the approval of the Engineer-in-charge and the contractor shall ensure the safety and stability of the excavations, being executed by him as well adjacent buildings, structures, services and other works in the vicinity of the site of work. Wherever necessary Engineer-in-charge may direct that the sides of the excavation should be timbered and shored at the contractor’s own cost, adopting a proper method approved by the Engineer-in-charge. Not withstanding the above, should any slip occur, the contractor shall remove all the slipped materials from the excavated pit, at his own cost. He shall also make good at his own all damages caused to the work as well as adjacent buildings, structures etc., as a  result of the slip, referred to.

All excavation work shall be subject to inspection and approved by the Engineer-in-charge before any further works  in the excavated areas allowed to commence. Should any excavation be carried out beyond the specified depth, the contractor should fill it up at his own cost with the same type  and class of material as it is proposed to be laid over the excavated portion. No payment will however be made to the contractor on his account.

The contractor shall ensure that the excavations and the structures under construction are kept free  from water logging at all stages of construction. He shall take all necessary precautions and `streams, acquifers, springs, surface flows etc., are excluded effectively so as to ensure that the works are carried out in a reasonable dry conditions in accordance with the construction schedule.

Back filling around the foundations, trenches, plinth and under the floor shall be done in accordance with CPWD CLAUSE 2.21.The finished level of the plinth filling shall be trimmed to the slope required to be given to the finished floor.  Back filling, watering and consolidation of excavated earth in layers etc., complete as per specifications shall be done unless  otherwise stipulated in the tender schedule.

Removal of Hard rock by Blasting

This shall include all rock occurring in large masses which cannot be removed except by blasting. Blasting shall be done in conformity with CPWD clauses 2.6 and 2.8  as instructed by the Engineer-in-charge. When rock blasting has to be done adjacent to structures, the following  precautions shall be observed.

    - All blasting should be completely muffled to prevent damage by flying pieces.

    - Blasting within 3 meters of the structures shall be avoided

    - No blasting should be done within 1.5 meters of concrete / masonry structures.

    - An isolated boulder extending under the existing structure but projecting within the area of blasting should be blasted.

    - The contractor shall be responsible for all damages  caused  by blasting and shall replace or repair the damaged structures at his own cost.

3. Plain and Reinforced Cement Concrete Works :

All design and construction shall be performed in accordance with the Indian standard code of Practice for plain and reinforced concrete – IS 456. Any Special requirements noted on the drawings or bill of quantities shall govern over the provisions of this specifications. Controlled concrete  shall be used wherever specified in the schedule of items complying with all requirements of IS : 456 and as per special specifications appended herewith.

The coarse aggregate to be used shall be of hard broken granite stone jelly of various sizes as specified  under respective items in the Bill of Quantities, conforming to IS 383 latest edition. The Engineer-in-charge may require the contractor to carryout moisture content tests in both fine and coarse aggregates. For determination of moisture content IS – 2386 shall be referred to. The amount of water to be added  shall be then adjusted to compensate for any observed variation in the moisture contents. Proper control of mixing water is deemed of paramount importance. Mixtures with automatic water measuring drums shall be used or else. Water should be measured by volume in calibrated buckets. All measuring equipments shall be maintained in a clean serviceable condition and the accuracy periodically checked and got certified by the Engineer-in-charge. The contractor shall carryout slump tests apart from taking test cubes at regular intervals. All such methods of sampling and analysis of concrete shall be in accordance with IS – 1199.

Mixing of concrete shall be strictly carried out in an approved type mechanical mixer. The mixing equipment shall be capable of combining the aggregates, cement and water within the specified time (not less than 2 Minutes) into a thoroughly mixed and uniform mass and of discharging the mixture without segregation. Mixing shall be continued until there is a uniform colour and consistency, Concrete shall be handled from the place of mixing to the place of final deposit as rapidly as practicable by methods which will prevent segregation or loss of any of the ingredients. Before depositing the concrete, all debris and dirt shall be removed from the space to be occupied by concrete.  Concreting shall not be done unless the formwork conform to the shapes, lines and dimension as shown in the drawings.

Unless otherwise approved, concrete shall be placed on single operation to the full thickness of slabs and beams and similar members and not exceeding 1 metre deep in walls, columns and similar members. Concrete shall be placed continuously until completion of the part of the work between construction joints or as directed by the Engineer-in-charge. The concrete after being laid shall be compacted by means of vibrators of approved type under proper supervision as directed by the Engineer-in-charge.   Care should be taken to avoid segregation and formation of air bubbles.  The whole process starting from the mixing of concrete to the placing and compaction shall not take more than 20 minute and the process shall be completed before the initial setting takes place.  Curing shall be accomplished in accordance with IS-456 by keeping the concrete covered with a layer of sacking, canvas or similar absorbent materials and kept constantly wet for the period as directed by the Engineer-in-charge.

Reinforcement

All reinforcement shall be clean and free from pitting, loose mill-scales, dust, loose rust and coats of paint, oil or other coatings which may destroy or reduce bond.  General construction details   and  workmanship  relative  to  reinforcement  including  bar  bends,  lap  splices  and installation shall be in accordance with the IS-2502 as well as IS-456.  All bars be bent as per the bar bending schedules indicated in the drawings or supplied separately relevant to particular drawing.  The contractor shall in all cases verify himself the correctness of schedules, giving the number, length and the bending details of the bars. The numbers, sizes, shape and position of all the reinforcement shall, unless otherwise, directed or authorized by the Engineer-in-charge be strictly in accordance with the drawing. The reinforcement shall be adequately held in position by 18 / 20 SWG soft black annealed binding wire. The contractor must obtain the approval of Engineer-in-charge for the reinforcement placed, before any concrete is placed in the form.  All reinforcing bars shall be so tied as to form a rigid cage to prevent displacement before or during concreting. Rate quoted for reinforcement should include cost of transporting M.S. Rods / CTD bars from BHEL Store to site of work, cleaning, cutting, bending, placing, binding with contractor’s own binding wire and providing necessary cover blocks of concrete but excluding cost of steel which shall be supplied free of cost at BHEL Stores.

##### Form Work

Formwork shall conform to the shape, lines and dimensions of concrete and RCC structures as shown in the drawings and shall be well within the permissible tolerance. Formwork for concrete shall be of plywood, steel, good seasoned timber or other approved materials, properly designed easy for removal and cleaning. They shall be of sufficient strength and rigidity to maintain their position and shape under loads incidental to placing concrete. The number of props, their sizes and dispositions shall be such as to able safely carry the full dead load area constructional loads. The arrangement  and alignment of formwork shall be got approved by the Engineer-in-charge prior to concreting. However this shall not relieve the contractor from his responsibility for proper work and safety. Formwork shall be sufficiently tight to prevent loss of cement slurry from the concrete. All joints and holes in the formwork shall be caulked with putty jute cloth or other approved materials to the satisfaction of the Engineer-in-charge. The stripping time for the shuttering and the formwork shall in general conform to the provisions in the relevant clauses of IS 456. Unless otherwise specified rates for reinforced cement concrete shall include cost of centering, shuttering charges also.

##### Expansion and Other Joints

Expansion Joints in concrete structures shall be provided at specified places as indicated in the drawings. The material shall be as specified by the Engineer-inc-charge, expansion joints, with or without metal strip shall be as shown on drawings the filler shall be “ EXPANDEX JOINT FILLER”  premoulded non – extrudent type fibrous joint  filler impregnated with bitumen conforming to IS : 1838. The filler shall be durable, waterproof, compressible and shall have a high degree recovery after compression is released, ensuring thereby that no free space develops in the expansion joints. The top 25 mm, or as specified in the drawings, shall be sealed with “SHALITEX SEALING COMPOUND” or equivalent after application of approved primer.

4.  Stone Masonry

General: Stone masonry shall conform to CPWD specification. Stone shall be obtained from the approved quarry and shall be free from decay and weathering.

All stones shall be thoroughly wetted before use. The mortar used for jointing shall be as specified under the respective items of the ‘ Bill of Quantities’.

The walls shall be carried up truly plumb.  Every stone shall be carefully fitted to the adjacent stones so as to form neat and closed joints.

To give sufficient lateral bond, vertical joints shall be avoided. Prescribed number of headers as required shall be provided to give sufficient transverse bond. At junctions of wall the stones each alternate course shall be so carried into each of the respective walls as to unite the work thoroughly.  Where  breaks are unavoidable  in  carrying up the work continuously in  horizontal course sufficiently long steps shall be left to joint the old and new work building of two thin faces and filling up the middle with small stuff or dry packing shall be strictly avoided.  When plastering or raised pointing is not required to be  done joints shall be struck flush and finished simultaneously.

##### Random rubble masonry

Random rubble masonry work shall conform to CPWD specification clause 7.1. Stones shall be hammer dressed at faces and joints to enable them to come into close proximity with each other. The face stone shall be laid headers and stretchers alternately, so as to break joint by at least 75mm. Thickness of joint shall not exceed 20mm. No pinning shall be used on the face and face stone shall extend well back into the headers. Stones less than 150 mm. in height shall not be used on the face.

Bond stones running through the wall shall be provided at 1.8M intervals in walls up to 0.60 M. thickness and if the wall is more than 0.60 M. thick a line of headers shall be laid from face to back each header overlapping the other by at least 150 mm.

5. Brick Masonry:

First class Bricks: Brick shall be sound, hard, tough, rectangular in shape and size, well burnt of uniform deep red or copper colour and conform to IS – 1077.

Brick shall be free from cracks, chips, flaws, stone or humps of any kind. Bricks shall be homogeneous in texture and evict a clear ringing sound on being struck and shall have a minimum compressive strength of 50KG / sq.cm. and shall not absorb water more than 20% of its weight, when soaked cold water for 24 Hrs. All bricks shall be table moulded.

Second Class Bricks: These shall be ground moulded but should otherwise conform to the specifications of first class bricks except for some surface cracks are allowable. These shall have minimum compressive strength of 50 Kg./sq.cm.

Samples of  each type of brick, shall be got approved by the Engineer-in-charge before being used. All subsequent deliveries shall be up to the standards of the approved samples.

Brickwork shall conform to IS –2212.

Brick works shall be classified as first or second class according to the classification of bricks used and the method of laying. The thickness of joints shall not exceed 6 mm.  In first class brickwork and 10 mm in second class brick work.

Bricks shall be well soaked before use on works for at least 6 hrs. The soaked bricks shall be kept on wooden planks or platform. Brick required for masonry with clay or lime mortars shall not be soaked.

Brickwork shall be laid with specified mortar to be prepared in accordance with IS –2250. Brick works shall be laid in English Bond unless otherwise specified. Half or cut bricks shall not be used except when needed to complete the bond. Each course shall be taken up truly plumb, if battered, the batter is to be truly maintained.  The level  of  brickwork  shall  be  checked  up  at every one metre interval. Bricks shall be laid with frogs upward, while laying bricks shall be thoroughly bedded and flushed in mortar and taped into position with a wooden mallet and the superfluous mortar removed. Walls of all structures shall be carried up regularly in all cases, leaving no part, one metre lower than the other. Where the masonry of one part has to be delayed, the work shall be raked back according to bond (and not toothed) an angle not exceeding 45 Deg. But the raking back should not start within 60cm. of a corner vertical joints in alternate courses shall come directly over one another. The brickwork shall not be raised more than 14 courses per day. All iron fixtures, pipes, conduits, drains, sleeves, bolts, holdfasts of doors and windows etc. which are required to be built in walls shall be embedded in cement mortar or cement concrete as specified, in their correct position as the work proceeds.

##### Joints

Joints shall be restricted to 6 mm in first class brickwork and 10mm in second class brick work. All bed joints shall be normal to the pressure upon them, radial in arches and at right angles to the face in battered retaining walls. Care shall be taken that all joints are fully mortared (proportion as specified in the schedule items) well flushed up and in case where no pointing to be done, neatly struck at the work proceeds.  The joints in faces which are to be plastered or pointed shall be squarely raked out to  a depth of 12 mm  while the mortar is still green. The raked joints shall be well brushed to remove loose particles. After the work the faces of work shall be cleared well by brush so as to remove any splashed mortar during the course of raising the brickwork.

##### Curing

Green work shall be protected from rain by suitable covering. Masonry work shall be kept thoroughly well watered on all faces for atleast 10 days after completion. In case of fat lime mortar curing shall commence two days after laying of masonry and shall continue for seven days.

##### 6. Scaffolding

Scaffolding will generally be single but may be double if warranted for the particular work as approved by the Engineer-in-charge. The contractor shall take all measure to ensure safety of work and the working people.

Payment for brickwork shall be made on cubic metre basis on the volume of actual work done. Half brick wall and brick on edge wall shall be paid on square metre basis. The rate of brick work shall include scaffolding and all items mentioned above and no extra payment will be made for cutting bricks if required either for openings or for rounding or insertions or for recesses at the time of brick wall construction.

7. Damp Proof Course

Damp proof course shall either be with cement concrete or with cement mortar of specified thickness as mentioned in the relevant item of schedule. Damp proof course shall not be carried across doorways. It shall be laid for all walls except verandah retaining wall or for particular wall only directed by the Engineer-in-charge. It shall be laid flush with floor level or as instructed by the Engineer-in-charge.

Damp proof course with cement concrete shall be of 25 or 38 mm thick in cement concrete M15/M20 as specified using 12mm. and down size aggregate well rammed and smoothened with trowel. It will be kept wet for 40 hours and after it has dried, two coats of hot bitumen shall be applied over it and allowed it to dry after which sand shall be sprinkled over it.

Damp proof course with cement mortar shall be 12 or 20 mm thick in cement mortar 1:3 well mixed with crude oil at 5% by weight of cement used.

8.  Cement Plastering:

Cement plastering shall be in accordance with IS – 1661, Cement mortar shall conform to IS – 269. The mortar of specified mix and thickness shall be used.

The surface to be plastered shall be thoroughly cleaned so that it is free from dust, oil, salts etc., The joints of masonry shall be racked out to a depth of atleast 12 mm. On cement concrete surfaces the surface shall be cleaned with wire brush and scarified by lines with trowel or hacking done. The surfaces in both cases shall be washed properly and kept wet for 4 hours before plastering is commenced.

Plastering shall be started from top and gradually worked down towards the floor. It shall not at any place be thinner than specified. To ensure even thickness plaster of about 15 cm X 15 cm shall be first applied horizontally and vertically at not more than 2 metres intervals over the entire surface to serve as gauges. The surface of these gauges shall be truly in the plane of finished surface. The mortar shall then be laid in the wall or other surfaces between the gauges and finished even. All corners shall be rounded to a radius of 24 mm unless otherwise directed. The contractor shall not be paid for any extra thickness of plaster done than as specified.

Plaster, when more than 15 mm thick shall be applied in two coals, a base coat followed by the finishing coat. Thickness of the base coat shall be just sufficient to fill up unevenness in the surface, no single coat, however, shall exceed 12mm in thickness. The under coat shall be roughened or scratched before it is fully hardened.

Curing shall start 24 hours after the plaster is laid. It shall be kept wet for 14 days. During this period it shall be suitably protected from all damages at the contractor’s cost by such means as approved by the Engineer-in-charge.

Any cracks  which appear in the surface shall be cut out in rectangular shape and redone as directed by the Engineer-in-charge. Wherever specified standard waterproofing compound as approved by the Engineer-in-charge shall be added to the mortar at the rate of 2% or as specified by the manufacturer by the weight of cement for which the rate shall be paid separately. The rate for plastering shall include cost of  scaffolding, swing etc., needed for the work with labour and material all complete.

9.   Steel Door, Windows and Ventilators:

All steel doors, windows and ventilators shall conform to IS – 1038, IS – 1361 and IS 1081 or equivalent as mentioned in the bill of quantities and as approved by the Engineer-in-charge.

Rolled steel sections shall conform to Is – 226.  The sections shall be cold straightened and finished goods shall be free from bends and other defects. Materials used in the fabrication shall be the best procurable and conforming to relevant IS specification. Thickness and specification of the glass to be provided shall be as indicated in the relevant item of the bill of quantities. Glass shall be free from flaws, specks, bubbles, etc., Bolts, nuts, screws, peg stays and other mild steel fittings shall be treated for corrosion as per relevant Indian Standards.   Putty for glazing shall conform to IS – 420.

Doors, windows and ventilators, etc., shall be truly square and free from twist and warp. They shall be constructed  of sections which have been cut to the required lengths and welded or riveted at the corners as per standard specifications.

All steel surfaces shall first be thoroughly cleaned free of rust scale or dirt and mill scale by approved means and shall be painted with one coat of approved primer conforming IS – 102 before despatch. Alternatively if specified they may be galvanised by the “Hot dip” zinc spray or electro galvanising process described IS – 1361. Doors, windows and ventilators shall be fixed in positions, as specified under IS 1081.

Whenever contractor is required to supply the doors, windows and ventilators etc., he shall first submit to the Engineer-in-charge, the details about source of supply, detailed drawing and specifications etc.,. for prior approval.

10. M.S Rolling Shutters :

It shall be of approved quality, made out of 18 gauge 75 mm black laths mechanically operated by reduction gear type mechanism. It shall be fitted with two self-aligning ball bearing with locking arrangements (both inside and outside) including M.S  pressed side guides bottom rails brackets and top rolling spring pressed etc., complete  provided with locking arrangements for padlocks, pulling hooks, handles, top cover etc., It shall be painted with one coat of approved primer conforming to Is – 102 before despatch.

11.  Wooden Doors, Windows and Ventilators :

All wood work for doors, windows, ventilators cup board, shelves, etc., conform to relevant IS specification and CPWD specification clause 9.1and 9.3 and shall be well seasoned teak wood or well seasoned country wood (pillamarudu or karumarudu) as the case may be. Timber shall be best quality and shall be free from knots, injurious open shales, bore holes, decay, soft  or spongy spots, hollow pockets and all other defects and blemishes. Timber shall conform to  IS 1003 (Part 1).  Size of doors, windows and  ventilators shall as specified in the relevant item of schedule and detailed drawings and generally in conformity with IS – 1003, part I and part II.  The rates for doors, windows and ventilators  etc., shall be for the finished work inclusive of fixing them in position with necessary iron hold fasts and furniture fitting of oxidized iron or aluminium or oxidized brass as stated in the description of the relevant  items of the bill of quantities and as directed by the Engineer-in-charge. Fittings and furniture shall be of best quality and machine made and robust type. Wherever glazed shutters are to be provided, the cost of glass panes  of specified thickness and fixing them in position should be included in the quoted rate. All glass shall be of superior quality from approved  manufacturer. In case of solid  core flush doors, they should conform to IS:2202 Part I and Part II. The flush door shutters should have a finished thickness as specified  in the scheduled item. Flush door shutters shall be obtained from firms of repute as approved by the Engineer-in-charge.

12.  ROOFING : (A) R.C.C Slabs :

Roof slab shall be of RCC of specified mix conforming to IS : 456, with adequate main tensile, transverse and adhesive reinforcement of ribbed steel bars of  mild steel rounds. Unless otherwise mentioned the rates for RCC slab shall include cost of concrete, centering and shuttering charges, vibration charges, rounding of corners, curing and finishing etc., complete. Construction joints and Expansion joints in reinforced cement slabs shall be as per CPWD specification clauses 5.4.4 and 5.4.5 respectively.

The ceiling should be finished as per CPWD specification. No extra payment shall be made towards cost of ceiling plaster necessitated on account of defective centering materials used or poor workmanship.

The top of roof slab shall be finished with weathering course treatment if so specified. The weathering course work shall conform to CPWD specification and consist of concrete with broken brick in neat slacked lime of specified thickness finished  with one course of pressed split tiles / pressed tiles of specified thickness and size as described in the bill of quantities.

(B)  Roofing and Side Cladding with A.C. sheets :

The A.C. sheets shall be of specified and approved quality and shall  conform to IS 459- 1962 in all respects. The sheet shall be laid with the smooth side upwards and with a minimum end lap of 15cm. and for every flatter slopes this should be 20cm. The laying operation shall include scaffolding works involved. Sheets shall be secured to the purlins by means of 8mm galvanized iron J or L hook bolts and nuts. The  grip of  the hook bolt on the side of the purlin shall be not less than 25mm.  Each bolt  shall  have a bitumen  washer and galvanized  iron washer  placed over the sheet before the nuts are screwed down from above. Hole for hook bolts etc., shall be drilled and not punched in the ridges of the corrugations in the exact positions. The diameter of these holes shall 1.5 mm more than the diameter of the fixing bolts. The payment will be square metre  basis of the laid area.

13. A.C. Rain Water Pipes

The pipes shall be of standard quality conforming to IS 1628. These shall be straight, true smooth and regular  in thickness. They shall be free from cracks and other flaws. The supply shall include all necessary pipe fittings and accessories.

All pipes shall be fixed to wall or columns by standard M.S butt holder clamps of approved make. The spigot of the upper  pipe shall be properly fitted into the socket of the lower pipe, such that there is uniform annular space for filling with the jointing materials. One third depth of this annular space is to be filled with spun yarn soaked  in bitumen of approved quality and properly pressed with caulking tool. The remaining two, third depth of the joint is to be filled with Cement Mortar 1:2 ( 1Cement : 2 coarse sand) and shall be pressed with caulking tool and finished smooth at the top at an angle of 45 Deg. sloping up.

The rate shall include supplying and fixing pipes with specials and accessories, including sizing as required, jointing, testing, cutting of walls and making good necessary scaffolding etc., complete.

14.  Flooring :

Flooring shall consist of a base  course of cement concrete of specified thickness and proportion laid over the compacted earth or sand filling as specified and a finishing layer  of concrete, mosaic, glazed tiles or any other material as specified to be laid. Flooring work for Factory shop floors as well as mosaic flooring shall be done as per the special specifications.

The bed flooring shall be prepared either level or sloped as per relevant drawing or as instructed by the Engineer-in-charge. Filling in basement with earth or sand shall be in accordance with  CPWD specification clause 2.21.  On the prepared  bed, cement concrete of  specified mix and thickness shall be laid and  well consolidated.

15.  White Washing and Colour Washing

White washing and colour washing shall be done as per  specifications laid down in I.S 6278 and CPWD specification clause 13.25 and 13.28  . The surface shall be thoroughly cleaned off mortar drops and foreign matter. All patchings must be scraped properly. The white washing shall be done from pure shell lime / Janathacem.  Samples of lime shall be got approved by the Engineer-in-charge. The wash shall be applied with a brush, the coats being laid on vertically and horizontally alternatively, each coat being allowed to dry before next coat is applied. For colour washing the desired shade shall be obtained by mixing approved quantity of colouring matter  or distemper with shell lime solution and applied as per white. The contractor shall take every precaution to prevent  white wash being splashed on  wall, floor and other places and articles not to be white washed. No colour wash shall be done unless a sample pattern of the mixed colour has been approved by the Engineer-in-charge.  The rates  shall be inclusive of scaffolding charges, cost of ladder etc.,

16.  Painting

All painting work shall be done in accordance with CPWD specification clause 13.33 and the relevant Indian Standard Specifications. Paints, varnishes, cement paints etc., shall be the highest  grade products of well known approved manufacturer and shall be delivered to site in original sealed containers. It is desired that materials of one manufacturer only shall be used as far as possible. Colours shall be uniform and nonfading.  Samples of all colours  selected shall be submitted to the Engineer in charge for approval before bulk  purchase is made. All finished work shall match corresponding samples kept with the Engineer.

Preparation of Surface :

Before painting wooden surfaces, protruding timber fibres shall be removed  and nail  marks  shall  be    covered with putty. The surface shall be thoroughly  cleaned and sand prepared. In case of steel work it shall be scraped, well brushed and cleaned  free of rust, scale dirt. Base  preparation  for painting concrete, masonry and plastered surfaces shall be carried out as per IS 2395 – Part I. Before actually proceeding with the work  of painting the concrete, masonry and plastered surfaces, it shall be verified that the surfaces shall be completely dry, free from efflorescence and alkaline effect.

##### Application

The primer shall be applied with brushes and spread as evenly and as smooth as possible. For steel work a priming coat  of Red Oxide / Zinc Chromate paint shall be applied. Painting  shall be done by skilled labourers in a work-man like  manner. All coats shall be of proper consistency and shall be well brushed out, so that no brush marks are visible. The under coating should be nearest to the specified colour of the finishing coat. Unless  otherwise specified ready mixed synthetic enamel  paints shall be used  for painting, wood and steel work. Under coats should be completely dry before finishing coat is taken up. Priming coat and under coat shall be rubbed with sand paper and dusted clean. The finished coat of approved paint shall then be applied.

17.  Cement Painting

Cement Paint solution shall be applied to the surface with hair brushes in a number of coats to get uniform finish. Specification for cement paint laid down in I.S. 5410 and CPWD specification clause 13.32  shall apply. After the  first coat of paints has hardened, it shall be cured with water atleast for 24 hours. The surface shall be wetted again  before the application of the second coat. Atleast 24 hours should lapse  between the  two coats, number of coats shall be as specified in the schedule of quantities. It shall be kept  damp atleast  for seven days.

18.  French Polish

French  polish to be used shall comply with IS 348.   Polishing shall be obtained by dissolving 1 lbs, of shellac in one gallan of methylated spirit without applying any source of heat. After the shellac has dissolved ¼ lbs of cobalt, ¼ lbs of lobano and 0.4 oz of crystals of desired pigment shall be added. The solution shall be applied with a pad of fine muslin cloth tied as per general practice. The pad shall be dipped into the solution and wrung with  fingers and be rubbed  hard on the surface in this way, the first coat is to be given after this gets dried up, the successive coats shall be given in the same fashion till the mirror like surface  is obtained. The wood to be polished shall be  first  applied  with a filler composed of 1.25 Kg. of whiting mixed with one litre of methylated spirit and  then sand papered when dry.  The finished surface shall have uniform texture and gloss. Approved transparent sealer shall only be used  in base preparation.

SPECIAL SPECIFICATION FOR CONTROLLED CONCRETE

a)	PROPORTIONING MIX:

The proportion of aggregate, Cement and water to be used for controlled concrete shall be designed by preliminary tests of the materials to be actually used   to obtain the densest to workable concrete requiring the minimum quantity of cement paste for binding the materials to give the required strength. However the maximum total quantity of aggregate by weight per  50 kg. of cement shall not normally exceed 450kg All proportions shall be by weight.  However water may be added by weight or volume.

b)	MIX DESIGN:

Immediately  upon  the  receipt of the  award  of  the contract, the Contractor shall inform the  Engineer-in-charge  the exact location of the sources of  materials  which  he  proposes  to use  and  get  the  materials  approved.   The mix with the actual approved  materials to  be used shall be got designed by the Contractor in  an approved laboratory chosen by BHEL and got  approved   by the Engineer-in-charge.  These proportions shall  be   used  so long as the materials continue to be the  same   quality  and  from  the same sources.   If  during  the  progress  of work, the Contractor wishes to change  the  materials, the proportions shall be fixed on the  basis   of  fresh  preliminary  tests to  give  the required  strength.   No  change of materials  shall  be  allowed  unless fresh tests with new materials show satisfactory  results.

c)	STRENGTH REQUIREMENTS OF CONCRETE:

Controlled  concrete shall be in the following  grades,  M 15, M 20,  M 25, M 30, M 35 &      M 40.  The concrete mix shall  be  designed to produce the  grade  of  concrete  having  the  required  workability  and  characteristic  strength  not less than the appropriate values (as  per I.S. 456) as given in table below:

| Sl. No | Grade of. Concrete | Specified Characteristic Cube Strength |  |
| --- | --- | --- | --- |
|  |  | Preliminary test N/mm2 | Works test N/mm2 |
| 1. | M 15 | 20 | 15 |
| 2. | M 20 | 26 | 20 |
| 3. | M 25 | 32 | 25 |
| 4. | M 30 | 38 | 30 |
| 5. | M 35 | 44 | 35 |
| 6. | M 40 | 50 | 40 |

d)	WORKABILITY OF CONCRETE:

The proportions chosen shall be such that the  concrete is of adequate workability for the conditions  prevailing on the work in question, and  shall  be properly  compacted.   Water shall be added to the mixer to give the  required workability.  The water content  of  each batch  shall be adjusted as necessary to  maintain  the  required workability with simultaneous adjustment of cement concrete such that the water cement ratio is not changed.  The workability tests shall be carried out in  accordance  with  IS : 1199 - Latest,  "Methods  of   sampling and analysis".

Workability of concrete shall be controlled by direct measurement of water  content, making allowance for any surface water in the fine and coarse aggregates.  Allowance shall be made for surface  water present in the aggregate when computing the water  content.   Surface water shall be determined by one  of the  field  methods described in IS : 2386,  Part  III.

e)	CONSISTENCY AND SLUMP:

The concrete shall have a consistency such that it will  be  workable in the required position.  It shall be  of such  consistency that when properly vibrated  it  will  flow around reinforcing steel and all embedded parts.  The slump for concrete as determined by slump tests  as  per  Indian  Standard 456 (latest  edition)  shall  not  exceed the maximum slumps indicated below for each type  of construction as approved by the Engineer-in-charge.

----------------------------------------------------------------------------------------------------------	Workability		Slum in MM			Type of Construction

Min.	 Max.

-----------------------------------------------------------------------------------------------------------

Medium			40	80			All RCC works

-----------------------------------------------------------------------------------------------------------

The contractor shall not place concrete having a  slump outside  the limits specified without the  approval  of  the Engineer-in-charge.

Atleast  one  slump  test  shall  be  made  for   every compressive  strength test carried out.  More  frequent  tests  shall be made if there is a distinct  change  in    job  conditions  or  if required  by  the  Engineer-in-charge.

f)	TESTS:

Test  shall conform to the specifications laid down  in  IS  : 456.  These tests shall be got done in  an   approved  laboratory and in accordance with IS : 516 - Latest  at  the  cost of  the  Contractor  (for  making necessary  cube moulds, transporting the cubes  to  the  Laboratory and all other incidental etc.).  The test  fees for the cubes shall be borne by the  contractor.   In  order  to  ensure  proper  quality  control  sampling  of the concrete of each specification  placed  on any day in work shall be done in accordance with the  following table.

-----------------------------------------------------------------------------------------------------------

Quantity of Concrete in work		Total Number of samples(each

sample to consist of 6 test cubes)

-----------------------------------------------------------------------------------------------------------

Upto 5.00 Cu.m.						1

Greater than 5.00 Cu.m. and  less than 15.00 Cu.m 		2

Greater than 15.00 Cu.m. and less than 30.00 Cu.m.		3

Greater than 30.00 Cu.m. and  less than 50.00 Cu.m.		4

Greater  than  50.00 Cu.m.		4 +  one  additional  sample   for  each

additional 50  Cu.m. or part thereof.

-----------------------------------------------------------------------------------------------------------

i)	The  sampling  shall be spread as  evenly  as  possible   throughout  the  day.   When wide  changes  in  weather  conditions occur during concreting, additional  samples  may  be taken as desired by  Engineer-in-charge.   Each   sample  shall  consist of 6 cubes of 15 x 15  x  15  cm  concrete.   Test cubes shall be kept immersed in  water until required for test which will be usually at 7 days  and minimum crushing strength of not less than the following:

| Grade of Concrete | M15 | M20 | M25 | M30 | M35 | M40 |
| --- | --- | --- | --- | --- | --- | --- |
| Crushing strength in N/mm2 at the end of |  |  |  |  |  |  |
| a) 7 days | 10 | 13.50 | 17 | 20 | 23.50 | 27 |
| b) 28 days | 15 | 20 | 25 | 30 | 35 | 40 |

All costs connected with the preliminary tests for proportioning the mix, sampling, curing, handling, other incidental charges, labour charges and cost of materials  shall be borne entirely by the  Contractor.   Testing charges for cubes (work tests) shall be borne by contractor.   Such  testing  shall  normally be conducted at the BHEL Central Laboratory or any  other approved laboratory as directed by  the  Engineer-in-charge.

All samples for tests shall be taken in the presence of Engineer-in-charge and the Contractor or his authorised  agents.

A set of six specimens from random mixer batches, shall constitute  a test, three being tested for 7  days  and  three being tested for 28 days strength.

The strength test result shall be the average  strength of  the  three companion test specimens, tested  at  28 days,  except  that, if one specimen in  a  test  shows manifest  evidence  of improper sampling,  moulding  or testing, the result shall be discarded and the  remaining two strengths averaged.

ii)	Normally, 7 days  and 28 days tests shall be made on specimens.  For any mix, a correlation between  7  days  and  28  days strengths may be made in  the laboratory.   Soon after a job starts, a similar correlation will  be  evolved for samples of a concrete taken from the mixer.   After  that  correlation  has  been established, the results  of the 7 days test may be used as an indicator of the compressive strength which should be expected at 28 days,  provided such results are consistent, if  7 days  tests shown compressive strength that are too low, corrective measures shall  be taken  atonce, at the Engineer's direction, without waiting for the  results of the 28 days tests.

iii)	Each class  of concrete shall meet the following strength requirements:

The  average  of any three consecutive  strength  tests shall have a value equal to or greater than the  specified  strength subject to the condition that  only  one  out  of three consecutive tests may give a  value  less  than the specified strength but this shall not be  less   than 90% of the specified strength.

iv)   In  the event that concrete tested in  accordance  with  the  requirements of this specification, fails  to  meet  the  requirement, the Engineer shall have the right  to require any one or all the following.

1.	Changes in the concrete mix proportions for  the remaining work.

- Curing  and testing of the concrete represented by  the tests which

failed.

3.	Replacement  of  any  such portions of the structure.  (No  payment for dismantled concrete,  associated  from worker or  reinforcement  shall  be made. Embedded fixtures, reinforcement and adjoining structures, damaged during dismantling shall be made good by the Contractor at his  own expense).

4.	Extended  curing  of the concrete  represented  by  the specimen.

5.	The  contractor  shall carry out all such  measures  as directed at his own expense.

Load tests of structural members may be required by the  Engineer  when  the strength of the job control tests falls below the required strength and is not acceptable  as  per  "Acceptance  Criteria"  stated  earlier.   The entire  cost  of the load test shall be  borne  by  the  Contractor.   If  the load testing is  decided  by  the  Engineer, the  member  under  consideration  shall  be subjected to a  superimposed load equal  to  one  and  quarter  (1.25) times the specified superimposed  load  used for design and this load shall be maintained for a  period of 24 hours before removal.

The detailed procedure of the test shall be decided  by the Engineer-in-charge.

If,  within  24 hours of the removal of the  load, the  structure  does not show a  recovery  of  atleast  75  percent of the deflection after the 24 hours under load, the  test should be repeated.  The structure should  be  considered  to have  failed to pass the  test  if  the  recovery  after  the  second test  is  not  atleast  80 percent  of the maximum deflection occuring during  the  second test.

If  the member shows evident failure, such  changes  as are necessary to make the structure adequately  strong,  shall  be  made by the contractor free of cost to  the Department.  If on the other hand, the failure  becomes apparent, the Engineer under special circumstances, can  retain  the  portion  of  the  structure  under   test, provided  suitable strengthening and or load  dispersed  arrangement is feasible. Cost of such strengthening or  load  dispersed  arrangement  shall  be  borne  by  the Contractor.

Load test shall not be made until the expiry of 56 days  of effective hardening of the concrete.

If a portion of the structure is found to be  unacceptable  it  shall be dismantled and replaced by  a  fresh structure  as  per  specification.  The  cost  of  such dismantling  and reconstruction should be borne by  the  Contractor.

The  proportions  of  cement,  fine  aggregate,  coarse aggregate and water necessary to produce a concrete mix which will fulfill the requirements of this specification  for  each grade  of concrete shall be determined on  the basis of trial mixes conducted with the samples of  the material  to  be used in the work by  the  Engineer-in-charge in the field Laboratory.

All  mix  design  and test data and  results  shall  be  maintained  as part of the record of the  contract  and  shall  be  signed  by the  Engineer-in-charge  and  the  Contractor.   A register showing such record  shall  be  maintained at site of work as shown below.

NAME OF WORK:

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Sl.	Date &		Sample		No. of		Identifi-		Proportions	Description		Initials of 	Date of

No.	Time		No.		Cubes		cation		of mix		of portion		S.O/A.E. in	test

marks		 		of work 			whose

represented 		presence

by sample and		sample is

quantity of		taken

concrete

represented

by the sample

(1)	(2)		(3)		(4)		(5)		(6)			(7)		(8)		        (9)

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

7 days		Average	Standard	Date	28 days 	Average	Standard	Remarks	Review		Review

test		7 days		7 days		of	last result	28 days		28 days		(the		and		and

result		strength	strength	test 	of each		strength	strength	reference	Remarks	Remarks

of each						cube 	cube						Nos. of		by		by

cube 		 											test		Asst. 		Execu-

report		Engineer	tive

should 				Engineer

also be

recorded

here)

(10)		(11)		(12)		(13)	   (14)		    (15) 		     (16) 		    (17)		 (18)                      (19)

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

SPECIAL CONDITIONS OF CONTRACT

STRUCTURAL STEEL WORKS

1.	Broadly, the work under this contract comprises of conveying of raw structural steel sections from  BHEL Store to site fabrication shop, fabrication of steel work at the site fabrication shop including one priming coat of Zinc Chromate/red oxide primer, transportation of fabricated steel structures to the erection site including loading,  unloading, leading, stacking as required and all handling costs, erection of steel work over prepared foundations or on RCC brackets as the case may be and painting of steel work two coats with approved synthetic enamel ready mixed paint or as specified in the Bill of quantities carrying from BHEL stores and erection of cloaking items etc. complete.

2.	If found necessary the tenderer may also be required to supply some raw structural steel sections also as required for this work in accordance with the latest editions of the Indian Standard Specifications noted below:-

IS 2062 & IS 1977.  He will be required to submit the necessary test certificates for the materials so supplied for use on this work.

3.	The detailed fabrication drawings as approved for fabrication will be the responsibility of BHEL.  The Detailed Shop Drawings will be supplied by BHEL to the contractor progressively to suit the fabrication and erection sequence.  The Contractor shall not depart from the drawings approved by BHEL without the written permission of the Engineer-in-charge.

4.	Based on the detailed shop Drawings approved by the BHEL the Contractor shall prepare at his cost, the Drawing Office Despatch Lists (abbreviated as D.O.D.L.'s) and get them approved by BHEL.  These shall contain the drawing number, the designation of items, number of pieces, based on the section weights as adopted for supply of raw materials without deduction for bolt holes and skew  cuts.  The DOD Lists shall form the basis for payment.

5.	Rate quoted against item(s) of the Bill of Quantities cover the fabrication of all items involved and shall therefore be an Overall-average-rate.  The actual quantities for payment shall be based on the drawing office despatch lists to be prepared by the Contractor and got approved by BHEL as aforesaid.

- All Civil works like preparation of foundations for columns, including embedding of holding down bolts etc., also have to be carried out  and hence  included in the present enquiry.  All works shall be performed and completed in a thoroughly workmen like manner and the contractor shall follow the best modern practice in the manufacture of high grade structures not withstanding any omission in the specifications.

7.	(a)	Fabrication shall generally be in accordance with IS 800 (latest issue) entitled "Code of practice for use of structural steel in general building constructions".  Welding shall be in accordance with IS 816 entitled "Code of practice for use of metal arc welding for general construction in mild steel".  The contractor shall provide necessary splicing as approved by the Engineer-in-charge, to suit the available lengths of raw steel, and no extra amount shall be paid on this account.  Any specifications not covered by the Relevant Indian Standard Codes of practice shall be in accordance with the relevant BS or in its absence in accordance with the well established standard Engineering practice to be acceptable to BHEL.

  - Under the contract, site fabrication is to be adopted for which a suitable site near and inside the factory premises will be made available by BHEL at  free of ground rent.  The Contractor shall provide all plant and equipments, tools covered  sheds and other facilities required for site fabrication work at his own cost.

(c)	The Contractor shall take into consideration the  existing foundation structures and make necessary provisions in the fabricated components with the approval of the Engineer-in-charge so that proper alignment, vertically and easy connection / erection of structures is ensured, should this involve any extra work on the part of the contractor beyond his normal scope, the same will be paid for at mutually agreed rates.

8.	All fabricated steel work shall be given one coat of Red Oxide / Zinc Chromate primer conforming to Relevant IS code before erection. All fabricated steel work shall be match marked suitably to facilitate their erection in position without any difficulty.

9.	Materials to be supplied by this organisation will be supplied in standard/random sizes/lengths as stocked by BHEL.  Hence the Contractor will be required to receive extra materials to cover conversion, wastage, alteration etc.  He shall do so without any claim for extra payment, whatsoever on this account.

10.	All breakages after the A.C. materials are handed over to the contractor shall be to Contractor's account.  As far as possible partially broken A.C. materials should be cut suitably and used for smaller sizes with the approval of BHEL, without any extra claim.  A breakage allowance of 3% of the consumed quantity is permissible for A.C. materials, if size of sheets supplied vary with actual requirement the actual size as supplied shall be allowed, for material accounting.

11.	The Contractor should prepare in advance cutting lists for materials so as to obtain efficient and economical use of all types of materials including steel.

- Normally no night work will be permitted.  But in case of emergency and urgent in nature where night work is warranted, the Contractor shall arrange for night works providing all facilities including illumination at his own cost, after obtaining written permission from Engineer-in-charge under intimation to Security and Safety Departments.

- The Contractor shall closely scrutinise all the drawings for the work issued by this organisation and bring to the notice of the Engineer-in-charge any discrepancies/ omissions noticed in the drawings before undertaking the actual work.

- M/s BHEL through their duly authorised representative shall have all reasonable times access to the Contractor's premises or works and shall have the power at all reasonable times to inspect any portion of the work or examine the materials and workmanship of the structures during their manufacture and test.  The  Contractor shall give notice in writing to the inspecting Engineer of BHEL when the materials to be supplied are ready for inspection and test, no materials shall be used on the fabrication  work  until  the  inspecting  Engineer  has  certified  in  writing that such materials have been inspected and approved by him.  The contractor shall provide at his own cost all facilities for the required tests on fabricated structures as required by the Engineer-in-charge.  Defective fabrication structures shall be made good/replaced by the Contractor at his own cost to the full  satisfaction of BHEL.  This also applies to loss or damage of raw materials, if special tests only will be borne by BHEL.

15.	Weekly progress reports shall be submitted giving in detail the position of receipt of raw materials, progress of fabrication and completion of fabricated structures etc.  The contractor shall submit at such times as may be requested by the Engineer-in-charge, schedules showing the programme and order in which the Contractor proposes to carryout the fabrication and erection works with dates and estimated completion times for various portions of the work.  Such schedules shall be approved by the  Engineer-in-charge prior to the starting of the relevant works.

16.	BHEL  would expect  the  contractor  to  take  up  simultaneously  wherever  possible   both fabrication and erection of structures especially where a good deal of assembly work, which is in fact a continuation of the fabrication work, is required to be done at site.  The painting and cloaking items of works, wherever possible, should  be done simultaneously  to save time.  The  Contractor may with the prior approval of BHEL sublet the work.  The main contractor shall however, be responsible for all works executed on his behalf by the sub-contractors.

17.	SETTING OUT AND ERECTING OF STEEL WORK:

a)	Erection of steel work shall generally be in accordance with the provision of IS 800 (Code of practice for use of structural steel in General Constructions).

b)	Site assembly of members on the ground by welding, bolting or otherwise as specified in   shall be inspected thoroughly by the Engineer-in-charge, or his authorized representative and approved before erection.

c)	All equipments, facilities and consumables for site fabrication as well as erection plant requirement, etc., such as derricks, cranes, lifting tackles, wire ropes, chain pulley blocks, jacks, winches etc. as necessary shall be arranged for by the Contractor at his own cost.  Care shall be taken to see that all equipments, tools and tackles and wire ropes etc. in use are always in good working conditions and fit for use.   For all the tools and plants periodical calibration certificate from approved agency should be obtained.  The above lists should be enclosed along with the tender.

- Frames shall be lifted at such points that they will not buckle or  deform.   Trusses  shall  be lifted only at  nodes.  Temporary bracing shall, if required, be provided at no extra cost, to relieve erection stresses.

e)	In the case of trusses and similar roof structures all or  atleast a majority of the purlins and wind bracings, shall be erected side by side with the erection of these structures.  Columns shall be erected true to plumb, (no screed bars provided and fixed by the Contractor over the prepared pedestals), true to center  line, level and gauge of traveling cranes.  Alignment of the columns, crane girders and rails shall be done very carefully using high precision survey instruments and necessary adjustments made to suit actual requirements.  A list of survey instruments proposed to be used shall be furnished in the tender.

f)	All damages to the steel work caused during the transit or otherwise shall be made good to the full satisfaction of BHEL at Contractor’s own cost, before erection.

g)	It shall be the sole responsibility of the Contractor to ensure accuracy of level, plumb, span and alignment of steel work before erection of other components.

18.	PAINTING AFTER ERECTION:

a)	All steel work shall be given two coats of approved brands of first quality synthetic enamel ready mixed paint after erection (over the priming coat already provided).  Painting shall be done as per  IS 800.  The shade, make, quality and other particulars of the  paint proposed to be used, shall be subject to prior approval by BHEL.

b)	It is the responsibility of the Contractor to procure sufficient quantities of the approved paints well in advance to ensure their availability in time.

19.	TIME ALLOWED (Period of Contract)

The following programme should be strictly followed.

(i)	Fabrication of structural  steel  works }   Progressively from the date of award of

to be  completed            	       	        }   work

(ii)	Erection, painting  of the  above 	 	}  Progressively   from the  date on

structures, supply and erection of  	}  which  completion of civil work and

cloaking items etc. to be completed	} the readiness of site declared as fit for

} erection

20.	The contractor will have to provide and fix the necessary screed bars etc. at his own cost in proper line, level and position to facilitate the erection work.  Further, the contractor should lay and maintain necessary temporary approaches within his working areas at his own cost to facilitate his work and for easy movement of vehicles, cranes etc. deployed on the work.   Only reasonable areas will be provided by BHEL.

21.	Till the expiry of the maintenance period of SIX MONTHS after the completion of the entire contract work and handing over to BHEL, the contractor should retain the minimum equipments and staff required and should carry out the maintenance works with the least possible interference to the routine works of the new shop.  Till the expiry of the maintenance period the contractor shall be responsible for all damages occurring due to any fault on his part or on the part of his workmen, sub-contractor or other agencies engaged by him.

22.	BHEL shall have the right to take possession of or use any completed or partially completed part of the work.  Such possession or use shall not be deemed to be acceptance of any work not in accordance with the contract.

23.	The contractor shall observe all safety regulations and take necessary safety precautions as called for under the Factories Act or other relevant statute as applicable including the use of safety boots, safety belts, helmets and other equipments and accessories for ensuring safe execution of the contract and freedom from accidents.

24.	The rates quoted in the Bill of Quantities of the tender for the fabrication item shall cover also the cost of preparation of DODL and similar incidental items.  Fabrication works shall be undertaken only after the issue of approval of shop drawings.

- The Contractor should co-operate with other contractors who may be executing their work in the same area in order to facilitate efficient execution of the entire project work in this area.

- BHEL is registered as an indentor for materials on rate contract with DGS & D.  Tenderers who are on DGS & D List of rate contracts should mention their rate contract number and furnish a copy of the rate contract document with the tender.

27.	The Guarantee / Warranty period for this contract shall be SIX MONTHS from the date of completion of the entire work and handing over to BHEL.  During this period the successful tenderer shall be at site on his own expense for replacement or repair of all defects arising out of faulty materials and/or workmanship.

28.	Further particulars relating to design  and fabrication and clarifications, if any, may be obtained on reference to the Sr. Manager/Civil / Factory, BHEL, goindwal-22 or his nominee.

29.	The contractor shall engage a level-II NDT Inspector for inspection of NDT works.

30.	Inspection will be done by BHEL Staff/Agencies appointed by BHEL for the works covered in the tender.

31.	Painting, DFT of primer should be 25 micron.

32.	Finish coat Painting, DFT 20 micron for each coat.  Paint shall be of first quality product of approved brand as per list of approved brands for materials enclosed in the price bid.

33.	Pre heating and post heating required shall be as per AWS (latest).

- Welded qualification is  to be done before commencement of the work and approved & qualified welder  only shall be engaged in work.  Necessary testing charges shall be borne by the contractor.

35.	For the supply of material the manufacturer’s test certificate is to be produced.

GENERAL QUALITY  CONTROL  PROCEDURE  FOR  STRUCTURAL STEEL  WORKS

The following quality standards are required to be maintained in all the structural steel fabrication components.

| 1. | All the raw materials will be generally conform to IS 2062-1999 or equivalent. |  |  |
| --- | --- | --- | --- |
| 2. | The welding electrodes shall be kept in oven only. |  |  |
| 3a. | Submerged Arc welding has to be done for crane girder flange to web joints and built up column as per the details given in the approved drawings. |  |  |
| 3b. | E7018 Welding electrode rods shall be used where the thickness of plates are 20mm and more and as specified in the drawings issued. |  |  |
| 3c. | All the other weld in crane girder and butt welds in columns and beams shall be done with E 7018 welding electrodes. |  |  |
| 3d. | All the welds in Nodal points of trusses and lattice girders shall be done with E 7018 welding electrodes. |  |  |
| 4. | Pre heating shall be done for plates having thickness 25mm & more and the temperature shall be maintained by checking with thermal chalk, as per quality standards. |  |  |
| 5. | All erection welding, of any component shall be done using E 7018 Welding Electrode. |  |  |
| 6. | 100% visual check shall be carried out for raw materials and welding on all components and the welding areas during inspection, to be tested for LPI / MPI / UT / Radiography / PWHT as provided in the Quality Control Procedure. |  |  |
| 7. | Base plate to column shall be perfectly checked for perpendicularity. |  |  |
| 8. | All the components shall be checked for dimension at fit-up stage itself before the commencement of welding operation. |  |  |
| 9. | Contractor shall buy & use Black Hex. bolts and nuts as per IS Specifications as detailed below for fabrication & erection of steel structures. Black Hex Bolts IS 1363 - 1984 Part – I (Class 5.6/5.8) Black Hex Nuts IS 1363 - 1984 Part - III Plain MS washer IS 2016 - 1975 Taper washer for beams IS 5274 - 1975 Taper washer for channel IS 5372 - 1975 Spring washer IS 3063 - 1964 |  |  |
| 10. | Contractor shall buy & use the following welding consumables as detailed below: |  |  |
| a) | SMAW No. | Electrodes | Manufacturer |
|  | 1. | E 6013 & E 7018 | D & H Secheron |
|  | 2. | E 6013 & E 7018 | ESAB India Ltd. |
|  | 3. | E 6013 & E 7018 | Modi Arc Electrode Co. |
|  | 4. | E 6013 & E 7018 | D & H Welding Electrode (I) Ltd. |
|  | 5. | E 6013 & E 7018 | Mantek Electrods (P) Ltd. |
|  | 6. | E 6013 & E 7018 | Ador Welding Ltd. |

| b) | SAW Wire |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Sl.No. | Brand |  |  | Manufacturer |
|  | 1. | AUTO melt Grade A |  |  | Ador Welding |
|  | 2. | Mantek Grade A |  |  | Mantek Wires |
|  | 3. | Metaspool S1 dia. 4mm |  |  | Precision Wires |
|  | 4. | OK Aut Rod 12.08 L |  |  | ESAB India Ltd. |
| c) | Flux |  |  |  |  |
|  | Sl.No. | Brand |  |  | Manufacturer |
|  | 1. | Super S41 |  |  | Super Elec Inds |
|  | 2. | Mantek 22 |  |  | Mantek Welded Flux Co. |
|  | 3. | RMH Auto Weld Grade 1 |  |  | RMH Chemicals |
|  | 4. | SWP 40 T |  |  | Super Weld Prod |
| d) | GMAW ( Co2) |  |  |  |  |
|  | Electrode |  | Brand | Manufacturer |  |
|  | ER70S-6 Co2 WIRE |  | Automig-1 | Ador Welding Ltd. |  |
|  | ER70S-6 Co2 WIRE |  | Esab MW1 | Esab India Ltd., Chennai |  |
|  | ER70S-6 Co2 WIRE |  | Mantek Mig | Mantek Wiress |  |
|  | ER70S-6 Co2 WIRE |  | Nouvarc | Nouveax Industries (P) Ltd., |  |
|  | ER70S-6 Co2 WIRE |  | Autofil-6 | Voltrac Electrode Pvt. Ltd., |  |
| 11. | Rolled steel sections of depth greater than 450mm need to be visually checked for lamination before the process of fabrication. The doubtful area to be tested for LPI and if lamination is found UT Test to be carried-out. |  |  |  |  |
| 12. | The welding code of practice, procedures, specifications for electrodes / consumables etc., for manual welding and auto welding shall conform to AWS D1.1/D1.1M:2006 (Latest) and as per the QCP issued by BHEL component-wise. |  |  |  |  |
| 13. | All the fabrication and erection works shall have to be executed as per the QCP issued by BHEL. |  |  |  |  |
| 14. | All the fabrication and erection works will be inspected for Quality Control by a third party inspection agency as fixed by BHEL. |  |  |  |  |

TERMS AND CONDITIONS REGARDING COMPLIANCE WITH VARIOUS LABOUR LAWS BY THE CONTRACTORS FOR BHEL

- The Contractor shall not employ in connection with  the work any person who has not completed 18 years of age.

- The Contractor shall in respect of labour employed by him either directly or through subcontractors, comply with or cause to be complied with the following statutory provisions and rules and in regard to all matters provided therein.

The Contract Labour (Regulation & Abolition) Act 1970

  - The Minimum Wages Act 1948 and the related Punjab Rules.

  - The  Payment of Wages Act 1936 and the related Punjab Rules.

  - The Factories Act 1948 and the related Punjab Rules.

  - The Employee’s Provident Fund & Miscellaneous Provisions Act 1952.

  - The Employees State Insurance Act 1948.

  - The Workmen Compensation Act 1923.

  - The Industrial Disputes Act 1947.

and any other  law or modifications to the above or  to the Rules made thereunder from time to time.

##### REGISTRATION AND LICENSING

- Every Contractor shall register his name with the Welfare Section of BHEL before taking up the work awarded to him by giving the following information and getting a Code Number :

  - The Name of the Contractor

  - Nature of Contract Work

  - Period of work

  - Number of maximum labour employed by him on any one day

  - License No. & Date (Applicable in case of contractor employing 20 or    more workers)

  - Whether enrolled for PF, ESI, etc., and enrolment No.

This information is called for, for the purpose of informing the Inspectorate of Factories whenever  they call for information regarding contracts.

- The Contractor employing 20 or more workmen is required  to obtain license  from the authorities ( The Deputy  Chief  Inspector of Factories / Assistant  Commissioner of Labour as the case may be ). The license shall be amended and / or renewed wherever, there is  an increase in the workmen employed by him  or in the event of contract being extended or renewed. The Contractor shall inform the licence number to the BHEL Management before taking up the work.

- The Contractor (Licensed or unlicensed) shall promptly furnish every information and document required by BHEL authorities for the purpose of fulfilling their obligations as Principal Employer and / or Occupier of the Factory and shall render all necessary assistance for the same.

##### WAGES

- The Contractor shall pay wages to the workmen employed by him at the rate which shall not be less than the minimum wages applicable under Law from time to time.

- The Contractor shall fix wage periods in respect of which wages shall be  payable. No wage period  shall exceed one month.

- The Contractor shall ensure payment of wages to the contract labour employed by him within three days from the end of wage period in case the wage period is one week or a fortnight and in all other cases before 10th day  of the following month.

- All Payment of wages shall be made on working days at the work site and during the working time and on date notified in advance. In case the work is completed before the expiry of the wage period final payment shall be made within 48 hours of the last working day.

- Where the employment of any worker is terminated by or on behalf of the Contractor, the wages earned by him shall be paid before the expiry  of the second working day  from the day on which his employment is terminated.

- Wages due to every worker  shall be paid to him direct or to the person authorized  by him in this behalf. All wages shall be paid in current coin or currency in both.

- The Contractor shall ensure the disbursement of wages in the presence of such authorized representative of BHEL Management.

- The above payment shall be verified by the authorized officer / representative of BHEL with the following  certificate of the payment sheet “Certified that  the amount shown in Column No…….…. has been paid to the workmen concerned in my presence on ………..at…………….

- A certificate of payment shall be furnished in duplicate  by the Contractor to the Engineer in charge each month in Form ‘A’.

- A notice showing  the wage period and the place and time of disbursement of wages shall be displayed at the place of work and a copy to be sent to the Welfare Department by the Contractor under acknowledgement.

- Notices showing the rate of wages, weekly rest  days, hours of work, wage period, date of  payment of wages, names and addresses  of the Inspector having jurisdiction, the date of unpaid wages shall be displayed  in Hindi and English in conspicuous  places at the establishment and  at work site by the Contractor. The Contractor shall inform  the BHEL Management every month the details of contract labour  engaged  for contract in this following form :

  - Serial Number

  - Location

  - Period of work

  - No. of contract labour engaged during the month

  - No. of days worked

  - No. of men worked

  - Wages paid to workers

The above statement shall be furnished to BHEL Management at the end of every month.

##### REGISTERS AND RECORDS AND COLLECTION OF STATISTICS

- The following documents / formats  under Contract Labour ( Regulation & Abolition ) Act 1970 and Punjab Rules  thereunder shall be maintained by each contractor.

  - Register  of persons employed by the Contractor

  - Employment Card

  - Service Certificate

  - Muster Roll, Wage Register, Deduction Register, Wage slip, Overtime Register, Register of  Fines, Register  of Advances etc.,

- The Contractor shall display the abstract of the Contract Labour (Regulation&Abolition )  Act and the Rules  thereunder both in English and Hindi.

- Half yearly Return shall be sent by the Contractor in duplicate to the Licensing Officer.

- The Contractor shall submit the returns required under the Contract Labour (Regulation & Abolition ) Act 1970 periodically to BHEL Management.

- The Contractor shall without fail give upto date information in writing of the attendance of the workers  employed by him.

- The Contractor shall ensure that his workers keep and produce their Employment Card when coming to duty and take them back when leaving duty.

- All the above registers and records shall be preserved in original  for a period of three years. All the registers, records and notice maintained under the Act and rules shall be produced on demand  by Inspector or any authority under the Act.

##### WORKING HOURS AND WORKING CONDITIONS

- No worker shall be required or allowed to work on Sunday unless he has or will have a holiday on anyone of the three days before or after the said day.

- The Contractor shall inform BHEL Management in the prescribed form details of the contract workers scheduled to work on Sunday, the day of rest and also indicate the substituted holiday in lieu thereof. This shall be intimated two days in advance before his workmen are booked for work  on Sunday.

- The contract labour working  for more than nine hours in any day or for more than 48 hours in any week shall be paid wages at the rate of twice  the ordinary rate of  wages in accordance with the provisions of Sections 59 of the Factories Act 1948.

- The Contractor  shall provide  all safety devices and personal protective equipment to his workmen at his own cost and shall ensure  that his workmen wear / use such devices or equipment provided to them while doing the work and there should not be any relaxation on this.

- The Contractor shall give four paid National  Holidays  to his workers, viz., 26th January, 1st May,  15th August and 2nd October.

- The Contractor shall ensure that his workmen vacate the premises after the shift is over.

- The Contractor shall give leave with wages to his workmen who have worked for a period of 240 days or more in the Factory premises during a calendar year.  This leave shall be allowed during the subsequent calendar year at the rate of one day for every 20 days of work performed by the worker during the previous calendar year.  The worker whose services commences on a day other than the first of January shall be entitled to leave with wages at the above rate (One day for every 20 days of work) only if he had worked for a minimum of 2 /3 of the total  number of days in the reminder of the calendar year. This leave will be admissible only during the subsequent calendar year.

- No woman worker shall be required or allowed to work in the Factory except between the hours of 6.00 A.M. and 7.00 P.M.

- The Contractor shall comply with the provisions relating to Welfare and Health facilities as  provided in the Contract Labour (Regulation and Abolition) Act 1970.

##### NOTICE OF ACCIDENTS

- Not withstanding anything contrary to this, in the event of accident the contractor shall be required to fill injury  report and submit the Engineer in charge immediately and ensure  the compliances of ESI / Workmen’s compensation Act, Factories Act and Rules made thereunder. He shall also maintain  a register of accident as per the Act.

- The Contractor shall get the contract  labour engaged by him insured under  Workmen’s Compensation policy from General  Insurance Corporation of India before actually starting the work of contract.  The insurance coverage should be for the entire period of Contract. The Contract shall comply with the provisions of the Workmen’s Compensation Act 1923. (This should be read in conjunction with the provisions of ESI Act )

##### COVERAGE UNDER THE ESI ACT / PF AND MISCELLANEOUS PROVISIONS ACT

- The contractor shall ensure that all his workmen are covered under the Employee’s State Insurance Act and produce to BHEL such Registration Number / Enrolment Number before executing the contract work.

- The Contractor shall regularly pay the amount of contribution. i.e., employer’s  contributions as well as employees’ contribution pursuance of the above scheme as fixed from time to time. The Contribution payable presently is 1.75% wages to be recovered from his workmen and 4.75% of wages to b e contributed by the Contractor. Contributions recovered from employee and contribution made by the contractor may be rounded to the next higher multiples of five paise.

- The Contractor shall take note of any amendment that may be brought forth in the above contribution rate and act accordingly.

- The contractor shall ensure that his workmen are covered under the EPF & Miscellaneous Provisions Act 1952 and accordingly produce to the BHEL Management the registration / enrolment number before awarding of contract work.  As per the existing provisions every worker shall be entitled and required to become a member of the fund. The employee’s contribution payable at present is 12% of wages which will be recovered by the contractor from the wages of his workmen and the contractor should pay equal contribution. The contractor is also liable to pay any administrative charges in this behalf that may be decided  from time to time. It will be the responsibility of the contractor to ensure such contribution payable in respect of workmen employed through sub-contractors also.

- The Contractor shall take note of any amendment in the rate of contribution payable under the scheme from time to time.

- The Contractor shall within seven days of the close of every month submit to BHEL a statement showing  the amount of contribution payable / paid  for employees engaged by him or  through him and shall also furnish to BHEL such information as Principal Employer is required  to furnish under the provisions of the ESI Act and PF as well as the schemes made thereunder to the authorities concerned.

- Whenever any sum of money is found to be recoverable from or payable  by the contractor under the above Act, the sum shall be deducted from any sum that may be due or which at any time thereafter may become due to the Contractor under this contract or under any other  contract or from his security deposit.  In case the recoveries are not sufficient to satisfy the claim, the contractor shall pay  the balance thereof on demand.  In case any recoveries are made under this clause from security deposit, the contractor shall immediately thereafter  pay such further sums as may be required to replenish the shortage caused by such recoveries in amount of security deposit.

- The Contractor shall abide by all the labour and other laws applicable to contract labour / worker under this contract  and shall at all times keep BHEL indemnified against all loses, claims, prosecutions under any law.

- In case of non-compliance of any of the provisions of the Acts and in case BHEL having complied with the same, BHEL will be entitled to recover the same from the contractor / sub-contractor.

- Non-exercise  of any of the powers  or rights available to BHEL  hereunder or under any law, shall not in any way  operate as waiver thereof.

Note :  The Specimen forms for the following are available in BHEL.

1) 	Form ‘A’		-	Payment Certificate

2)	Form IV 		-	Application for License

3)	Form XIII		-	Register of Workmen employed by contractor

4)	Form XIV		-	Employment Card

5)	Form XV		-	Service Certificate

6)	Form XVI		-	Muster Roll

7)	Form XVII		-	Register of wages

8)	Form XIX 		-	Wage slip

GENERAL CONDITIONS OF CONTRACT FOR LUMPSUM, ITEM RATES AND PERCENTAGE CONTRACT

CONTENTS

| Condition No |  | Description |
| --- | --- | --- |
|  | CHAPTER - I |  |
| 1 | DEFINITIONS |  |
|  | CHAPTER - II |  |
|  | SCOPE OF CONTRACT |  |
| 2 | HEADING TO THE CONTRTACT |  |
| 3 | CONTRACT DOCUMENTS |  |
| 4 | WORKS TO BE CARRIED OUT |  |
| 5 | PROVISIONAL ITEMS |  |
| 6 | DEVIATIONS |  |
| 7 | TIME |  |
| 8 | STORES & MATERIALS |  |
| 9 | DELAY & EXTENSION OF TIME |  |
| 10 | PATENT RIGHTS |  |
| 11 | OCTROI AND OTHER DUTIES |  |
| 12 | ROYALTIES |  |
| 13 | PLANT & EQUIPMENT |  |
| 14 | ASSIGNMENT OR TRANSFER OF CONTRACT |  |
|  | a) SUB-CONTRACT |  |
| 15 | COMPLIANCE TO REGULATIONS AND BYE-LAWS |  |
|  | CHAPTER - III |  |
|  | PERFORMANCE OF THE CONTRACT |  |
| 16 | SECURITY DEPOSIT |  |
| 17 | ORDERS UNDER THE CONTRACT |  |
| 18 | ADMISSION TO SITE |  |
| 19 | CONTRACTOR'S SUPERVISION |  |
| 20 | LABOUR |  |
| 21 | WATER |  |
| 22 | TEMPORARY WORKSHOPS, STORES ETC. |  |
| 23 | STORES & MATERIALS ON SITE |  |
| 24 | TOOLS AND PLANTS ON SITE |  |
| 25 | STATEMENT OF HIRE CHARGES |  |
| 26 | PRECAUTIONS AGAINST RISKS |  |
| 27 | NOTICE & FEES |  |
| 28 | SETTING OUT OF THE WORKS & PROTECTION & MAINTAINING SIGNALS AND WORKS |  |
| 29 | SITE DRAINAGE |  |
| 30 | EXCAVATIONS, RELICS ETC. |  |
| 31 | FOUNDATIONS |  |
| 32 | COVERING-IN WORKS |  |
| 33 | APPROVAL OF WORKS BY STAGES |  |
| 34 | EXECUTION OF WORKS |  |
| 35 | DAY WORKS |  |
| 36 | INSPECTION OF THE WORKS |  |
| 37 | RESPONSIBILITY FOR BUILDING |  |
| 38 | INSURANCE OF WORKS |  |
| 39 | DAMAGE AND LOSS TO PRIVATE PROPERTY & INJURY TO WOKMEN |  |
| 40 | COMPLETION |  |
| 41 | COMPENSATION FOR DELAY |  |
| 42 | LAWS GOVERNING THE CONTRACT |  |
| 43 | CANCELLATION OF CONTRACT FOR CORRUPT ACT |  |
| 44 | CANCELLATION OF CONTRACT FOR INSOLVENCY,ASSIGNMENT OR SUB LETTING OF CONTRACT |  |
| 45 | CANCELLATION OF CONTRACT FOR DEFAULT |  |
| 46 | TERMINATION OF CONTRACT FOR DEATH |  |
| 47 | SPECIAL POWERS OF DETERMINATION |  |
| 48 | FAIR WAGE |  |
|  | CHAPTER – IV |  |
|  | VALUATION AND PAYMENT |  |
| 49 | RECORDS & MEASUREMENTS |  |
| 50 | VALUATION OF DEVIATIONS |  |
| 51 | REIMBURSEMENT/ REFUND ON VARIATION IN PRICE |  |
| 52 | ADVANCES ON ACCOUNT |  |
| 53 | FINAL BILL |  |
| 54 | PAYMENT OF BILLS |  |
| 55 | RECOVERY FROM CONTRACTOR |  |
| 56 | POST TECHNICAL AUDIT OF WORKS AND BILLS |  |
| 57 | REFUND OF SECURITY DEPOSIT |  |
| 58 | ARBITRATION |  |
|  | ANNEXURE |  |
|  | BHEL CONTRACTOR'S LABOUR REGULATIONS |  |
|  | MODEL RULES FOR LABOUR LAWS |  |
|  | BHEL SAFETY CODE |  |

CHAPTER- I

1. DEFINITIONS

In these General Conditions of Contract, the following terms shall have the meaning hereby assigned to them except where the context otherwise requires:-

- The “CONTRACT” means the documents forming the tender and acceptance thereof, together with all documents referred to therein including General and Special Conditions of Contract, Schedules ‘A’, ‘B’, ‘C’, ‘D’, ‘E’, and  / or General Summary attached to the form of tender, the Bharat Heavy Electricals Limited, Schedule of Rates as amended  and in force the Specifications and the Drawings.  All these documents as applicable taken together shall be deemed to form one Contract and shall be complementary to one another.

- The “TENDER DOCUMENTS” means the form of Tender the applicable Schedules ‘A’, ‘B’, ‘C’, ‘D’, ‘E’, and  / or General Summary, General and Special Conditions of Contract and the Specification and / or Drawings as given to Contractors on payment for the purpose of preparing their tenders.

- The “WORK” means the work described in the tender documents in individual work orders and/or accompanying Drawings and Specifications as may be issued from time to time to the Contractor by the Engineer–in– charge within the powers conferred upon them, including all modified or additional works and obligations to be carried out either at the site or at any Factory Workshop or other place as required for the performance of the Contract.

- The “SITE” means the lands and/or other places on, in into or through which the work is to be executed under the Contract or any adjacent land, path or street which may be allotted to or used for the purpose of carrying out the contract.

- The “CONTRACTOR” means the individual, firm or Company, whether incorporated or not undertaking the work and shall include the legal personal representatives of such individuals or the persons composing the firm or Company, or the successors of the firm or Company and the permitted assigns of such individual or firm or Company.

- The “ Engineer–in–charge” means the Engineer who is incharge for the works referred.

### CHAPTER II

# SCOPE OF CONTRACT

2. 	Heading to the Contract:

The heading to these conditions shall not effect the interpretation thereof.

3.	Contract Documents

The Accepting Officer shall furnish to the Contractor on demand “FREE OF COST” three copies of signed drawings and one copy of the signed agreement comprising of preamble to agreement, General and Special Specifications, Schedule ‘A’, ‘B’, ‘C’, & ‘E’, etc., (but excluding General Conditions of Contract and Drawings) and three copies of all further drawings issued during the progress of work.

However, for any additional copies of the agreement or drawings required by the Contractor the same will be supplied on payment at the specified cost.

The Contractor shall keep one copy of all the Drawings and the Specifications at the site and the Engineer-in-charge or his representative shall have access to them at all reasonable times.

None of these documents shall be used by the contractor for any purpose other than that of this contract.

The Contractor shall take necessary steps to ensure that all persons employed on any work in connection with this contract have noticed that the Indian official Secrets Act 1923 (XIX of 1923) applies to them and shall continue so to apply even after the execution of such works under the contract.

4.	Works to be Carried Out

The Contract shall, except as provided under Schedules ’B’ and ‘C’ include all  labour, materials, tools, plants equipment and transport which may be required in preparation for, and in the entire execution and full completion of the work.  Schedule ‘A’ shall be deemed to have been prepared in accordance with good practice and recognized principles and unless otherwise stated, the descriptions given therein shall be held to include waste on materials carriage and cartage, lead, return of empties, hoisting, setting, fitting in position and all other labour necessary in and for the entire execution and full completion aforesaid.  Any error in description or quantity in schedule ‘A’ or any omission there from shall not vitiate the Contract or release the Contractor from the execution of the whole or any part of the work comprised therein according to the Drawings and Specifications, or from any of his obligations under the Contract.  The insertion of the name of any firm of suppliers in the Tender Documents is for the purpose of obtaining a particular class or quality of materials or workmanship but the articles or materials specified may be obtained from any other firm subject to prior written approval of the Engineer – in – charge.

In the case of a discrepancy between Schedule ‘A’ the specification and  / or the Drawing, the Accepting Officer shall be the sole deciding authority as to which shall prevail and his decision shall be final and conclusive.  If neither Drawings nor Specifications contain any mention of minor details of construction, which in the opinion of the Accepting Officer whose decision shall be final and conclusive, are reasonable and obviously and fairly intended for the satisfactory completion of the work, such details shall be provided by the Contractor without any extra cost as if they were specially mentioned and shall be deemed to be included in the contract.

The contractor will be deemed to have satisfied himself as to the nature of the site, local facilities of access and all matters affecting the execution and completion of the work.  No extra charges consequent on any mis-understanding in these respects or otherwise will be allowed.

5.	Provisional Items

The full amount of provisional lumpsums and the value annexed to each provisional item inserted in the tender documents shall be deducted from the  contract sum and the value of work ordered and executed thereunder shall be  ascertained by measurement or valuation as for deviations.

No work under these items is to be begun without instructions in writing from the Engineer-in-charge.

The extent of quantities or items described as “Provisional” shall not be held to guarantee or limit the amount and description of the work to be executed by the contractor either in respect of the items concerned or the work as a whole.

No addition or deduction shall be made by the Contractor to the amount of the provisional lumpsums as included in the tender documents.

6.	Deviations

The contractor shall not make any alteration in addition to or omission from the work as described in the tender documents except in pursuance of the written instructions of the Engineer-in-charge.  No such deviation from the work described in the tender documents shall be valid unless the same has been specifically confirmed and accepted by the Accepting Officer in writing and incorporated in the contract.

The Accepting Officer may deviate either by way of addition or deduction, from the work so described, provided that the contract sum be not thereby varied on the whole by more than the percentage set out in the tender documents.  The value of all addition and deductions will be added to, or deducted from the contract sum.  Whenever the Accepting Officer intends to exercise such a right, his intention shall specify the deviations which are to be made, the lumpsum assessment or the proposed basis of payment, the extra time allowed, if any, and the date for completion of the entire contract.

Any objection by the Contractor to any matter concerning the order shall be notified by him in writing to the Engineer-in-charge within Seven days from the date of such order, but  under no  circumstances  shall  the work be stopped (unless so ordered by the Engineer-in-charge) owing to differences or controversy that may arise from such an objection. In the absence of such a notification of objection by the contractor, he will be deemed to have accepted the order and the conditions stated therein.  In the event of the contractor failing to agree with the Engineer-in charge regarding the terms of the proposed deviation, the objection shall be referred to the Project Manager whose decision shall be final conclusive and binding on the Contractor.

7.	Time

Time is the essence of the contract and is specified in the tender document or in each individual Work Order.

As soon as possible after the contract is let or any substantial Work Order is placed and before work under is to begin, the Engineer-in-charge and the Contractor shall agree to a Time and Progress Chart.  The Chart shall be prepared in direct relation to the time stated in the Tender Documents or the Work Order for the completion of the individual items there of and/the contract or order as a whole.  It shall indicate the forecast of the dates for the commencement of the various trade processes or sequences of the work, and shall be amended as may be required by agreement between the Engineer-in-charge and the Contractor within the limitation of the time imposed in the Tender Documents or  Order

In the absence of any specific Time and Progress chart to be agreed to between the Contractor and the Engineer–in–charge, the contractor shall ensure and maintain uninterrupted progress of the work such that the entire work shall be completed within the time imposed in the Tender Documents or Order and that the proportion of work completed upto any time in relation to the entire work to be under the Contractor Order shall not be less than the proportion that the time elapsed bears to the total time of completion provided in the Tender Documents or Order.

The contractor shall suspend the execution of the work, or any part or parts thereof whenever called upon in writing by the Engineer–in–charge to do so, and

shall not resume work thereon until so directed in writing by the Engineer –in–charge.  The Contractor will be allowed an extension of time for completion not less than the period of suspension.  However, no other claim in this respect for compensation or otherwise however will be admitted.  Provided the cause for suspension is not attributable to any default on the contractor’s part to proceed with or fulfill the contractual obligations.  This may also be extended to allow for alteration of work made by the deviation order.

8.	Stores and Materials

The Contractor shall, at his own cost and expense, provide all materials required for the works, other than those listed in Schedule ‘B’, which are to be supplied by Bharat Heavy Electricals Limited.  All materials to be supplied by the Contractor shall be of the best kind as described in the specifications and the Contractor shall, if requested by the Engineer–in–charge, furnish proof to the satisfaction of the Engineer-in-charge, that the materials so comply with the specifications.

The contractor shall, at his own expense and without delay, supply samples of materials proposed to be used in the execution of the work for approval of the Engineer–in–charge, who may reject the materials not corresponding either in quality or character to the approved samples.

In the case of stores provided under Schedule ‘B’ the Contractor shall bear the cost of loading, transporting to site, unloading, storing under cover as required assembling and jointing the several parts together as necessary and incorporating or fixing these stores materials in the work, including all preparatory work of whatever description as may be required, and of closing, preparing, loading and returning empty cases or containers to the place of issue without any extra charges.

9.	Delay and Extension of Time:

if, in the opinion of Engineer–in–charge the work is delayed:

i)  	by reason of abnormally bad weather,  OR

ii) 	by reason of serious loss or damage by fire, OR

iii) 	by reason of Civil commotion, local combination of workmen strike   or  lockout, affecting any of the trades employed on the work OR.

iv) 	by delay on the part of the agency or tradesman engaged by BHEL in  executing  work not forming part of this contract, OR

v) 	by reason of any other cause which in the absolute discretion of the Engineer–in–charge is (when he is the Accepting Officer of the Contract) beyond the   Contractor’s  reasonable  control, than in such case the Accepting Officer on the recommendation of the Engineer-in-charge (or higher authority) may make fair and reasonable extension in the completion dates of the individual  items of work or the contract as a whole.  Such extension which will be communicated to the Contractor by the Engineer–in–charge in writing shall be final and binding on the Contractor.  No other claim in this respect for compensation or other–wise howsoever is admissible. Upon the happening of any such event causing delay, the Contractor shall immediately given notice thereof in writing to the Engineer-in-charge but shall nevertheless use constantly his best endeavour to prevent or make good the delay and shall do all that may reasonably be required to the satisfaction of the Engineer–in–charge to proceed with the work.

10.	Patent Rights:

The Contractor shall fully indemnify B.H.E.L or the agent, servant, or employee of B.H.E.L against any action, claim or proceeding relating to infringement or the use of any patent or design or any alleged patent or design rights, and shall pay any royalties which may be payable in respect of any article / or part there of included in the contract.

In the event of any claims being made or action brought against B.H.E.L or any agent, or servant or employee of BHEL in respect of matters aforesaid the Contractor shall immediately be notified thereof for taking necessary action provided that payment of  indemnity shall not apply when such infringement has taken place in complying with the specific directions issued by the BHEL but the Contractor shall pay any royalties payable in respect of any such use.

11.	Octroi and Other Duties:

All charges on account of Octroi, Terminal or Sales Tax and/or other duties on material obtained for the work (excluding materials provided by B.H.E.L on payment) shall be borne by the contractor.

12.	Royalties:

Royalties fixed from time to time as per prevalent local rules will be recovered for materials, which the Contractor may be allowed to remove from quarries situated on land which is in charge of the B.H.E.L authorities.

13.	Plant and Equipment:

The Contractor, shall at his own expense, supply all tools, plant and equipment (here-in-after referred to as T & P) required for the execution of the contract other than those listed in Schedule ‘C’ which subject to their availability may be hired by B.H.E.L., to the Contractor or issued free for use in the execution of the work as specified in Tender Documents.

14.	Assignment or Transfer of Contract:

The Contractor shall not, without the prior written approval of the Accepting Officer, assign or transfer the Contract or any part thereof, or any share, or interest therein to any other person.  No sum of money which may become payable under the Contract shall be payable to any person other than the Contractor unless the prior written approval of the Accepting Officer to the assignment or transfer of such money is given.

14.	(a)  Sub Contract:

The Contractor shall not sub–let any portion of the Contract without the prior written approval of the Accepting Officer.

15.	Compliance to Regulations and Bye Laws:

The Contractor shall conform to the provision of any statute relating to the work and regulations and bye-laws of any local authority and of any water and lighting Companies or Undertakings with whose system the work is proposed to be connected.  He shall, before making any variation from the drawings or the specifications that may be necessitated for such connections give the Engineer–

in– charge notice, specifying the variation proposed to be made and the reasons there for and shall not carry out any such variation until he has received instructions from the Engineer–in–charge in respect thereof.  The contractor shall be bound to give all notice required by Statute Regulations or Bye-laws as aforesaid and to pay all fees, and taxes payable to any authority in respect thereof.

CHAPTER III

## PERFORMANCE OF THE CONTRACT

- Security Deposit

16.1	Security Deposit should be collected from the successful tenderer.  The rate of Security   Deposit will be  as below:

Upto    Rs. 10 lakh			10%

Above Rs. 10 lakh upto Rs.50 lakh  	1 lakh + 7.5% of the amount Exceeding

Rs.10 Lakh

Above Rs. 50 lakh	4 lakh + 5 % of the amount exceeding Rs.50 Lakh

At least 50% of the security Deposit should be furnished  before start of the work by the contractor in the form of Demand Draft / Bank Guarantee.

Security Deposit may be furnished in any one of the following forms :-

i)	Cash (as permissible under the Income Tax Act)

ii)	Pay Order, Demand Draft in favour of BHEL.

iii)	Local cheques of scheduled banks in the  name of BHEL subject to realization.

iv)	Securities available from Post Offices such as National Savings Certificates, Kisan Vikas Patras etc. (Certificates should be held in the  name of Contractor furnishing the security and duly  pledged in  favour of BHEL and discharged).

v)	Bank Guarantee from  Scheduled Banks /  Public Financial Institutions as defined in the Companies Act subject to a maximum of 50% of the total security deposit value.  The balance 50% has to be remitted either by cash or in the other form of  security. The Bank Guarantee format should have the approval of BHEL.

vi)	Fixed Deposit Receipt issued by  Scheduled Banks /  Public Financial Institutions  as defined in the Companies Act . The FDR should be in the name of the contractor,  A/C BHEL, duly discharged on the back.

vii) 	Security deposit can also be recovered at the rate of 10% from the running bills. However in such cases at least 50% of the Security Deposit should be collected before start of the work and the balance 50% may be recovered from the running bills.

viii) 	EMD of the successful tenderer  shall be converted and adjusted against the security deposit.

ix)	The security deposit shall not carry any interest.

NOTE:   Acceptance of Security Deposit against Sl. No. (iv) and (vi) above will be subject to hypothecation or endorsement on the documents in favour of BHEL. However, BHEL will not be liable or responsible in any manner for the collection of interest or renewal of the documents or in any other matter connected therewith.

All compensation or other sums of money payable by the Contractor to BHEL, under the terms of this Contract or under any other contract with BHEL, may be deducted from the Security Deposit or realized by the sale of the Securities or from the interest arising there from or from any sums which may be due or may become due to the Contractor payable by BHEL, on any account whatsoever against this Contract or any other Contract with BHEL, and in the event of his Security Deposit being reduced by reason of such deductions or sale as aforesaid, the Contractor shall, within seven days thereafter, make good in cash or in securities endorsed as aforesaid, any sum or sums by which the Security Deposit has been so reduced.

50% of the Security Deposit / may be refunded on completion of the work after    payment of the final bill and the balance 50% of the Security Deposit is refundable only after the expiry of the maintenance period of six (6) months from the date of completion of work as stipulated in the Contract concerned.

17.	Order under the contract

All orders, notices etc., to be given under the contract shall be in writing typescript or printed and if sent by registered post to the address given in the tender of the contractor, shall be deemed to have been served on the date when in the ordinary course they would have been delivered to him.

The contractor shall carry out without delay all orders given to him.

18.	Admission to site

The Contractor shall not enter on (other than for inspection  purposes) or take possession of the site unless permitted to do so by the Engineer-in-charge.  The portions of the Site to be occupied by the Contractor will be clearly defined and marked on the site plan, and the Contractor will on no account be allowed to extend his operations beyond these areas.

The Contractor shall provide, if necessary or required at the Site, temporary access there to and shall alter, modify and maintain the same as required from time to time.  He shall take out and clear away the access route when no longer required and restoring the area to its original condition.

The Engineer-in-charge shall have power to execute other works (whether or not connected with the work in the contract agreement) on the site contemporaneously with the execution of the original work and Contractor shall give reasonable facilities for this  purpose.

B.H.E.L reserves the right of taking over, at any time, any portion of the site which they may require and the Contractor shall at his own expense clear such portion forthwith.  No photographs of the Site or of the work or any part there of shall be taken, published or otherwise circulated without the prior approval of the Engineer-in-charge.

No such approval shall however exempt the contractor from complying with any statutory provisions in regard to the taking and publication of such photographs.

B.H.E.L Officials connected with the Contract shall have the right of entry to the Site at all times.

Engineer - in charge shall have the power to exclude from the site any person whose admission there to may, in his opinion be undesirable for any reason whatsoever.

19.	Contractor’s Supervision

The Contractor shall either himself supervise the execution of the Contract or shall appoint a competent Agent approved by the Engineer–in–charge to act in his stead.  The contractor shall employ an Engineer/Agent having at least a ‘Degree of Bachelor of Civil Engineering’ from a recognized University/on any work with a Contract value exceeding rupees two lakhs, and having at least a Diploma in civil Engineering from a recognised college, on work with a contract value exceeding Rs. 50,000/- but not exceeding  rupees two lakhs.

The Employment of an Engineer/Agent as aforesaid shall not be necessary if the Contractor himself in possession of a recognized technical qualification and is in opinion of the Engineer-in-charge capable of receiving instructions of the Engineer–in–charge and of executing the work to the satisfaction of the Engineer–in–charge.  If the Contractor fails to appoint a suitable Engineer/ Agent as aforesaid, the Engineer–in–charge shall have full powers to suspend the execution of work and stop payment of any advances that may have become due until such date as a suitable Engineer/Agent is appointed and the contractor shall be held responsible for the delay caused to the work and no extension of time on this account shall be given to him as stipulated in condition (9) above.

Orders given to the Contractor’s Agent/Engineer shall be considered to have the same force as if they had been given to the Contractor himself.

The contractor or his  Agent shall be in attendance at the site during all working hours and shall superintend the execution of work with such additional assistance in each trade as the Engineer–in - charge may consider necessary.

The contractor or his accredited agent shall attend when required and without making any claim for doing so, either the Office of the Engineer–in–charge or the work site to receive instructions.

The Engineer–in–charge shall have full powers, and without assigning any reason to require the Contractor immediately to cease to employ in connection with the  Contract any Agent, servant or employee whose continued employment is, in his opinion undesirable.

The Contractor shall not be allowed any compensation on this account.

LABOUR

20.	The Contractor shall employ labourer in sufficient numbers either directly or through sub-contractors to maintain the required rate of progress and of quality to ensure workmanship of the degree specified in the Contract and to the satisfaction of the Engineer-in-charge.  The Contractor shall not employ in connection with the works any person who has not completed his fifteen years of age.

The Contractor shall furnish to the Engineer-in-charge at the intervals specified by him, a distribution return of the number and description by trades of the work people employed on the works.  The Contractor shall also submit on the 4th and 19th of every month to the Engineer-in-charge a true statement showing in respect of the second half of the preceding month and the 1st half of the current month              (i) the accidents that occurred during the said fortnight showing the circumstances under which they happened and the extent of damage and injury caused by them and (ii) The number of female workers who have been allowed maternity benefit as provided in the Maternity Benefit Act, 1961 or Rules made thereunder and the amount paid to them.

The Contractor shall pay to labour employed by him either directly or through  sub–contractors wages not less than fair wages as defined in the Contractor’s Labour Regulations.

The contractor shall in respect of labour employed by him either directly or through sub – contractors comply with or cause to be complied with Contractor’s Labour Regulations in regard to all matters provided therein.

The Contractor shall comply with the provisions of the Payment of Wages Act 1936, Minimum Wages Act 1948, Employers liability Act 1938, Workmen’s Compensation Act 1923, Industrial Disputes Act 1947, Maternity Benefit Act 1961 and Mines Act 1952, Contract Labour Regulation and Abolition Act 1970 or any modifications there of or any other law relating thereto and rules made thereunder from time to time.

The contractor shall be liable to pay his contribution and the employees’ contribution to the State Insurance Scheme in respect of all labour employed by him for the execution of the contract, in accordance with the provision of “ The Employees” State Insurance Act, 1948” as amended from time to time.  In case the contractor fails to submit full details of his account of labour employed and the contribution payable, the Engineer–in–charge shall recover from the running bills of contractor an amount of contribution as assessed by him.  The amount so recovered shall be adjusted against the actual contribution payable for Employees’ State Insurance.

The Engineer–in–charge shall on a report having been made by an Inspecting Officer as defined in the Contractor’s labour Regulations have the power to deduct from the moneys due to the Contractor any sum required or estimated to be required for making good the loss suffered by the worker or worker by reason of non-fulfillment of the Conditions of the Contract for the benefit of workers, non-payment of wages or of deductions made from his or their wages which are not justified by the terms of the Contract or non-observance of the said Contractor’s Labour Regulations.

The Contractor shall indemnify the B.H.E.L against any payments to be made under and for observance of the Regulations aforesaid without prejudice to his right to claim indemnity from his sub-contractors.

In the event of the Contractor committing a default or breach of any of the provisions of the aforesaid Contractor’s Labour Regulations, as amended from time to time or furnishing any information or submitting or filling any form / Register/Slip under the provisions of these Regulations which is materially incorrect then on the report of the Inspecting Officers as defined in the Contractor’s Labour Regulation, the Contractor shall without prejudice to any other liability pay to the B.H.E.L a sum not exceeding Rs. 50/- as liquidated damages for every default breach or furnishing, making submitting, filling materially incorrect statement as may be fixed by the Engineer–in–charge and in the event of the Contractor’s default continuing in this respect, the liquidated damages may be enhanced to Rs. 50/- per day for each day of default subject to a maximum percent of the estimated cost of works put to tender.

The Engineer in charge shall deduct such amount from bills or security deposit of the Contractor and credit the same to the Welfare Fund constituted under Regulations.  The decision of the Engineer-in-charge in this respect shall be final and binding.

Model Rules for Labour Welfare

The Contractor shall at his own expense comply with or cause to be complied with Model Rules for Labour welfare as appended to these Conditions or rules framed by Government from time to time for the protection of health and for making sanitary arrangements for workers employed directly/or indirectly on the works.  In case the Contractor fails to make arrangements as aforesaid, the Engineer–in– charge shall be entitled to do so and recover the cost thereof from the Contractor.

Safety Code

The Contractor shall at his own expense arrange for the safety provisions as appended to these conditions or as required by the Engineer – in – charge, in respect of all labour directly or indirectly employed for performance of the works and shall provide all facilities in connection therewith.  In case the Contractor fails to make arrangements and provide necessary facilities as aforesaid, the Engineer– in–charge shall be entitled to do so and recover the cost thereof from the Contractor.

Failure to comply with model Rules for Labour  Welfare, Safety Code, or the provisions relating to report on accidents and to grant of maternity benefits to female workers shall make the  Contractor liable to pay to the B.H.E.L as liquidated damages an amount not exceeding Rs. 50/- for each default or materially incorrect statement.  The decision of the Engineer–in–charge in such matters based on reports from the Inspecting Officers as defined in the Contractor’s Labour Regulation as appended to these conditions shall be final and binding and deductions for recovery of such liquidated damages may be made from any amount payable to the Contractor.

WATER

- The Contractor shall allow in his Tender and provide at his cost all water required for the work or his employees on the work, together with all pipes and fittings or other means that may be necessary or required to ensure a proper and ample supply of water for all purposes connected with the work.

Water will be supplied from the BHEL supply system, or other sources at one point fixed by the Engineer-in-charge on the site of work at free of cost.  The Contractor shall make necessary arrangement for lifting pumping, carrying or conveying the water as required at his own cost.

22.	Temporary Workshops, Stores Etc.

The Contractor shall, during the progress of the work provide, erect and maintain at his own expense all necessary temporary workshops, stores, offices, etc., required for the proper and efficient execution of the work.  The planning, siting and erection of these buildings shall have the approval of the Engineer-in-charge and the Contractor shall at all times keep them tidy and in a clean and sanitary condition to the entire satisfaction of the Engineer-in-charge.

On completion of the work all such temporary buildings shall be cleared away and the site restored and left in a clean and tidy condition to the entire satisfaction of the Engineer-in-charge.

23.	Stores and Materials on Site

All stores and materials required for the work are to be deposited by the Contractor only in places to be indicated by the Engineer-in-charge.

Where in accordance with the contract stipulations certain Stores & Materials  (for incorporation in the work) are to be issued to the Contractor by the BHEL as detailed under Schedule ’B’ such items will be so issued only to the extent required for the actual completion of the work  as stipulated in the contract.  The decision of the Engineer-in-charge regarding the quantities to be issued as above shall be final and binding on the contractor.  For any excess quantities consumed on the work upto 5% over the theoretical consumption will be charged at issue rates and excess consumption beyond this limit, their cost will be recovered from the Contractor at punitive rates which will be 100% (Hundred Percent) more than the issue rates of the BHEL as specified in the Instructions to the Tenderers.

In regard to the materials and stores which may be issued to the Contractor by BHEL the Contractor shall give the Engineer-in-charge reasonable notice in writing of his requirements of such stores and materials and on the approval of his demand being notified to him, he shall make immediate arrangements for drawing the same.  Such stores and materials shall be transported by the Contractor at his own expense direct from the place of issue to the site of the work, unless prior written approval is obtained from the Engineer-in-charge to take them to a Store or Workshop elsewhere.

The Contractor shall have to build a weather proof shed for the storage of cement required for 15 days consumption of the work.

BHEL Officers connected with the Contract shall have the power at any time to inspect and examine any stores or materials intended to be used in or on the work, whether on the site or at any factory or workshop or other place where such stores or materials are being fabricated or manufactured or at any place where the same are lying and the contractor shall give necessary facilities for such inspection and examination.

The Engineer-in-charge shall be entitled to have tests made of any stores or materials supplied by the Contractor who shall provide at his own expense all facilities which the Engineer-in-charge may require for this purpose.  If at the discretion of the Engineer-in-charge an independent expert is employed to make any such tests his charges shall be borne by the Contractor only if the test discloses that the said stores or materials are not in accordance with the provisions of the Contract.

Should the Engineer-in-charge consider at any time during the construction or re-construction, on prior to the expiry of the “MAINTENANCE PERIOD” that the stores or materials provided  by the Contractor are unsound or of a quality inferior to that contracted for or otherwise not in accordance with the contract (in respect whereof the decision  of the Engineer-in-charge shall be final and conclusive) the Contractor, shall on demand, in writing from the Engineer-in-charge specifying the Stores or materials complained of, notwithstanding that the same may have been inadvertently passed, certified and paid for, forthwith remove the stores or materials so specified and provide other proper and suitable stores or materials  at his own expense; to the entire satisfaction of the Engineer-in-charge and in the event  of his failing to do so within a period to be specified by the Engineer-in-charge in his demand aforesaid  the Engineer-in-charge may replace within others the stores or materials complained of at the risk and expense in all respects of the Contractor.  The liability of the contractor under this condition shall not extend beyond the maintenance period aforesaid except as regards stores or materials which the Engineer-in-charge shall have previously given notice to the contractor to replace.  (*MAINTENANCE PERIOD for any work under this Organisation will be SIX MONTHS FROM THE DATE OF ACTUAL COMPLETION of the particular work and handing over to B.H.E.L).

All stores and materials brought to the Site shall become and remain the property of B.H.E.L and shall not be removed from the site without prior written approval of the Engineer-in-charge.  However, when the work is finally  completed, the  Contractor  shall  at  his  own  expense  forthwith remove from the site all surplus stores or materials originally supplied by him and upon such removal, the same shall revert in and become the property of Contractor.  All B.H.E.L Stores and materials issued to Contractor for in-corporation or fixing in the work and which, making due allowance for reasonable wear and tear/or waste, have not on completion of the work been so incorporated or fixed, shall be returned by the Contractor at his own expense to the place of issue.

Credit for surplus stores and/ or materials returned by the contractor to B.H.E.L will be given to him at a price based on the prevailing market rate but not exceeding that at which the said stores and materials were originally issued to him but due consideration shall be given to the allowance claimed by B.H.E.L in respect or any depreciation or damage suffered by the stores and/or materials whilst in the custody of the Contractor regarding which the decision of Engineer-in-charge shall be final and conclusive.

If, in the opinion of the Engineer-in-charge (which shall be final and conclusive)  any stores, supplied by B.H.E.L have either during currency of the work or after completion of the work whilst under the custody of the Contractor, become damaged to such an extent that they cannot be usefully utilized, either in the same work or in other works, the Engineer-in-charge shall not accept the stores and in the event of his rejection the contractor shall be charged for the said Stores at a rate as fixed by the Accepting Officer.  The Contractor shall not be entitled to any claim whatsoever on this account.

24	Tools and Plants on site:

All tools, plants and equipment brought to the site shall become the property of B.H.E.L and shall not be removed from the site without the prior written approval of the Engineer-in-charge when the work is finally completed or the Contract is determined for reasons other than the default of the Contractor he shall forthwith remove from the site all tools, plants, equipments etc., (other than those as may have been provided by B.H.E.L) and upon such removal, the same shall in, and become the property of the Contractor.

25.	Statement of Hire Charges:

A monthly detailed statement of the hire charge incurred in respect of B.H.E.L tools, plants, equipments etc., shall be given to the Contractor by the Engineer-in-charge.

26.	Precaution Against risks:

The Contractor shall be responsible for providing at his own expense, for all precautions to prevent loss or damage from any and all risks and to minimize the amount of any such loss or damage and for the necessary steps to be taken for the said purpose until the works have been handed over complete in all respect of the Engineer-in-charge.

The Contractor shall provide all watchmen necessary for the protection of the site, the work, the materials, tools, plants, equipments and anything else lying on the Site during the progress of the work.  He shall be solely responsible for and shall take all reasonable and proper steps for protecting, securing, lighting and watching, all places on or about the work and the Site which may be dangerous to any person whomsoever.

27.	Notices and Fees:

The Contractor shall give all notices required by any Statutory provision or by the regulations and/or bylaws of any local Authority and/or of any Public Service, Company or Authority affected by the work or with whose system the same are or will be connected.  The Contractor shall pay and indemnify B.H.E.L against any fees and charges payable under such Acts.  Regulation and/or byelaws in respect of the work and shall make and supply all drawings and plans required in connection with any such notice.

28.	Setting out of the Works and Protective and Maintaining Signals and Works:

The Engineer-in-charge shall supply dimensioned drawings, levels and other information necessary to enable the contractor to set out the work.  The Contractor shall at his own expense set accurately according to the drawings and figured dimension thereon, all the work comprised in the contract and any extras or additions there-to and shall be solely responsible for their being so set out and executed.  All bench marks, pegs, signals on the surface, alignment stones, milestones and all similar marks whether put in by B.H.E.L  Authorities for the purpose of checking the Contractor’s  work or  in the nature  of permanent survey marks will during the tenure of the  contract, be  under the care of  the Contractor who shall, at his own expense, take,all proper and reasonable precautions and care to preserve and maintain them in their true position.  In the event of these marks being disturbed or obliterated by accident or due to any other cause whatsoever, the same may, if deemed necessary, be replaced by the Engineer-in-charge at the Contractor’s expense and the cost thereof deducted from any money then or thereafter becoming due to the Contractor.

Where requested by the Contractor, the level marks, center line and chainage pegs corresponding to those shown on the Drawing will be pointed out to the Contractor on the ground but all bench marks or chainage pegs additional to those shown on the Drawing will be set out by BHEL authorities.

29.	Site Drainage:

All water that  may accumulate on the site during the progress of the work or in trenches and excavations shall be removed to the entire satisfaction of the Engineer-in-charge and at Contractor’s expense.

30.	Excavations, Relics Etc.

Material of any kind obtained from excavation on the site shall remain the property of BHEL and shall be disposed off as  Engineer-in-Charge  directs.

All gold, silver, oil and other minerals of any description and all precious stones, coins, treasures, relics, antiques and other similar items which may be found in  or upon the site shall be the property of Bharat Heavy Electricals Limited and the Contractor shall duly preserve the same to the satisfaction of the BHEL and shall from time to time deliver the same to such person or persons as the B.H.E.L may appoint to receive the same.

31.	Foundations

The Contractor shall not lay any foundations until the excavations for the same have been examined and approved in writing by the Engineer-in-charge.

32.	Covering-in Work

The Contractor shall give reasonable notice in writing to the Engineer-in-charge whenever any work is to be permanently covered up or concealed, whether by earth or other means so that it can finally be inspected or measured if necessary.  In default of so doing, the Contractor shall, if required by the Engineer-in-charge uncover such work at his own expense.

33.	Approval of works by Stages:

All work embracing more than one process shall be subject to examination and approval at each stage thereof and the Contractor shall give due notice in  writing  to  the Engineer-in-charge when each stage is ready. In default of such notice being received, the Engineer-in-charge shall be entitled to approve the quality and extent thereof at any time he may choose and in the event of any dispute, the decision of the Engineer-in-charge thereon shall be final and conclusive.

34.	Execution of the Work:

The work shall be executed in a workman-like manner and to the satisfaction in all respects of the Engineer-in-charge.

The Engineer-in-charge will communicate or confirm his instructions to the Contractor in respect of the execution of the Work in a “Work Site Order Book” maintained at his office and the Contractor shall visit this office daily and shall confirm receipt of such instructions by signing the relevant entries in this book.  Such entries will rank as order or notices in writing within the intent and meaning of these conditions.

35.	Day Work:

No day-work shall be performed without the prior written instructions of the Accepting Officer.

The Contractor shall give to the Engineer-in-charge reasonable notice of the start of any work ordered to be executed by day-work and shall deliver to the Engineer-in-charge within two days of the end of each pay week a return in duplicate giving full detailed accounts of labour and materials for that pay-week.  One copy of each of these returns, if found correct, will be certified by the Engineer-in-charge and returned to the contractor and must be produced at the time of adjustment of accounts.

An invoice in duplicate signed by the Contractor or his agent shall be sent with each delivery of materials for day-work and the Contractor will be furnished with a receipt signed by the Engineer-in-charge specifying the description, quantities weight or measurement (as the case may be) of the articles approved, reference will be made in this receipt in the return aforesaid and the Contractor’s Bill.

In the case of Lumpsum Contracts, the rates to be charged and the percentage addition for profit and establishment charges, etc., will be agreed upon between the Accepting Officer and the Contractor prior to the execution of the work.

36.	Inspection of the Work:

B.H.E.L Officers concerned with the Contract shall have power at any time to inspect and examine any part of the work and the Contractor shall give such facilities as may be required to be given for such inspection and examination Should Engineer-in-charge consider, at any time during the expiry of the

maintenance period, that any work has been executed with unsound, imperfect or unskilled workmanship or of a quality inferior to that contracted for or not otherwise in accordance with the contract (in respect) whereof the decision of the Engineer-in-charge shall be final and conclusive the Contractor shall on demand in writing from the Engineer-in-charge specifying the fault notwithstanding that the same may have been inadvertently passed, certified  and paid for, forthwith rectify or remove and reconstruct the work so specified in whole or in part as the case may be required at his own expense to the entire satisfaction of the Engineer-in-charge and in the event of his failing to do so within a period to be specified by the Engineer-in-charge in his demand   as aforesaid, the Engineer-in-charge may carry out the work by other means at the risk and expense in all respects of the Contractor.  However, the liability of the Contractor under this condition shall not extend beyond the maintenance period except as regards workmanship which the Engineer-in-charge shall have previously given notice to the Contractor to rectify.

37.	Responsibility for Building:

In the event of any building or part of any building being handed over to the Contractor for the execution of work thereto under the provisions of the Contract, he shall give a written receipt for all fixtures, glass etc. and he shall be required to make good at his own expense all damages resulting from any cause whatsoever while in his charge and on  completion of the work to deliver the said building or part thereof in a clean state complete in every particular to the entire satisfaction of the Engineer-in-charge.

38.	Insurance

The contractor shall within one month after the date of the acceptance of the contract,  insure the work against loss or damage to the contract works, temporary work and materials erected in performance of the contract on “all risks” basis from the time of arrival on site until taken over by BHEL on completion of the contract.

The cover shall also include wherever necessary the risks of testing including breakdown or explosion of plant and machinery undergoing testing, trail and commissioning operations.  The insurance shall also specifically cover removal of debris cost.  The sum insured shall represent the estimated full value of the contract works inclusive of value of free supply materials by BHEL, transport charges, customs dues, express freight, overtime charges,  cost of erection,  value  of constructional  plants  and  machinery,  removal  of  debris  and escalation of costs where the contract includes a maintenance period, the insurance cover shall specifically include the contractors’ liabilities during the maintenance period.  The insurance shall also  be extended to cover third party personal injury and property damage for a sum to be specified by BHEL.  The insurance shall be effected in the name of BHEL and the contractor shall submit to BHEL a draft of the insurance policy for approval.  The policy when issued will be lodged with BHEL together with receipts of premium for such insurance and the contractor shall maintain such policies in force until the obligations of the contractor are fully discharged.

If the contractor fails to comply with the terms of this condition the Accepting officer may insure the work and may deduct the amount of premiums from any money that may become payable to the contractor or may at his discretion refuse payment of any advances to the contractor until the contractor shall have complied with the terms of this condition.  This provision does not, however, absolve the contractor of his responsibility for taking up the insurance. The contractor is, therefore, primarily   responsible for taking up the insurance in time.

39. 	 Damage and loss to private property and injury to workmen

The contractor shall at his own expense reinstate and make good to the satisfaction of the Engineer–in-charge and pay compensation for any injury, loss or damage occasioned to any property  or  rights whatever including property and rights of  B.H.E.L, (or agents, servants or employees of B.H.E.L)  the injury loss or damage arising out of or in anyway in connection with the execution or purported execution of the  contract and further the contractor shall indemnify B.H.E.L, against  all claims enforceable against B.H.E.L, or any agent, servant, or employee of B.H.E.L a private person, in respect of any such injury (including injury resulting in death loss or damage to any person) whosoever or property, including all claims which may arise under the workmen’s Compensation Act or otherwise, or which would be enforceable against B.H.E.L.

40.	Completion

The works shall be completed to the entire  satisfaction of the Engineer-in-charge and in accordance with the Contractor’s forecast of Time and Progress where operative, and all unused stores and materials, tools, plants, equipments, temporary buildings and things shall be removed and the site and work cleared of rubbish and all waste materials and delivered up clean and tidy to the satisfaction of the Engineer-in-charge at the Contractor’s expense and/or before the Scheduled date of completion.

The B.H.E.L shall have power to take over from the Contractor from time to time each sections of the work as have been completed to the satisfaction of the Engineer-in-charge.

In case the Contractor fails to remove any of his properties, assets or fails to clear the rubbish and waste materials within 30 days of the completion of the contract, it is lawful for the contractee, that is BHEL to take such action as it deems fit to clear dispose of such properties, assets or such waste materials and charge the contractor any expenses incurred thereon.

The Engineer-in-charge shall certify to the Contractor the date on which the work is completed and the state thereof.

The Engineer-in-charge shall also certify to the Contractor the state of the work at the end of maintenance period,  where applicable.

41.	Compensation for Delay:

If the contractor fails to maintain the required progress in terms of condition 7 or to complete the work and clear the site on or before the contracted or extended period of completion, he shall, without prejudice to any other right or remedy of the B.H.E.L on account of such breach, pay as agreed compensation an amount calculated as stipulated below or such smaller amount as may be fixed by the BHEL on the contract value of the work for every week that the progress remains below that specified in condition 7 or that the work remains incomplete.

This will also apply to items or group of items for which separate period of completion has been specified.

For this purpose  the term ‘Contract Value’ shall be the value at contract rates of the work as ordered.

a.  Completion period (as originally-stipulated)	 -- at 1 percent per week.

Not exceeding 6 months.

b.  Completion period (as originally-stipulated)	  -- at ½ percent per week

Exceeding 6 months and not exceeding 2 years.

c.  Completion period (as originally-stipulated)	  -- at¼ percent per  week

Exceeding 2 years.

Provided always that the total amount of compensation for delay to be paid under this condition shall not exceed the under noted percentage of the contract value or  of the contract value of the item or group of items of work for which a separate period of completion is given:

a.	Completion period (as originally-stipulated)	  --	10  percent.

Not exceeding 6 months.

b.	Completion period (as originally-stipulated)	  --	7½  percent.

Exceeding 6 months and not exceeding 2 years.

c.	Completion period (as original-stipulated)	  --	  5 percent.

Exceeding 2 years

The amount of compensation may be adjusted or set-off against any sum payable to the Contractor under this or any other contract with the B.H.E.L.

42.	Laws Governing the Contract:

This contract shall be governed by the Indian Laws for the time being inforce.

43.	Cancellation of Contract for Corrupt Acts:

The Accepting Officer, whose decision shall be final and conclusive, shall, without prejudice to any other right or remedy which shall have accrued or shall accrue thereafter to Bharat Heavy Electricals Limited, cancel the contract in any of the following cases and the Contractor shall be liable to make payment to B.H.E.L for any loss or damage resulting from any such cancellation for default.

If the Contractor shall:

a.	Offer or give or agree to give to any person in BHEL service any gift or consideration of any kind as an inducement or reward for doing or for bearing to do or for having done or forborne to do a day act in relation to the obtaining or execution of this or any other contract for BHEL service OR

- Enter into a contract with B.H.E.L in connection with which commission has been paid or agreed to be paid by him or with his knowledge, unless the particulars of any such commission and the terms of payment thereof have previously been disclosed in writing to the Accepting Officer, OR

c.	Obtain a contract with B.H.E.L as a result of ring tendering or by non-bonafide methods of competitive tendering without first disclosing the fact in writing to the Accepting Officer.

44.	Cancellation of Contract for Insolvency, Assignment or Transfer or Sub-Letting of Contract:

The Accepting Officer, without prejudice to any other right or remedy which shall accrue thereafter to B.H.E.L shall cancel the contract in any of the following cases:

If the Contractor,

a)	Being an individual, or if a firm any partner thereof shall at any time be adjudged bankrupt or have a receiving order or orders for administration of his Estate made against him or shall take any proceedings, for liquidation or composition under any Bankruptcy Act for the time being in force or make any conveyance or assignment of his effects of composition or arrangement for the benefit of his creditor or purport to do so, or if any application be made under any Bankruptcy Act for the time being in force for the sequestration of his Estate or if a trust deed be granted by him on behalf of his creditors, OR

b)	Being a Company, shall pass a resolution or the Court shall make an order for the liquidation of its affairs, or a Receiver or Manager on behalf of the debentures holders shall be appointed or circumstances shall arise which entitle the court or debentures holders to appoint a Receiver or Manager OR.

c)	Assigns, transfers, sub-lets or attempts to assign, transfer or sub-let any portion of the work without the prior written approval of the Accepting Officer. OR

d)	Shall suffer an execution being levied on his goods and allow it to be continued for a period of 21 days.

Whenever the Accepting Officer exercises his authority to cancel the Contract under this condition, he may complete the work by any means at the Contractor’s risk and expense provided always that in the event of cost of the completion (as certified by Engineer-in-charge which is final and conclusive) being less than the contract cost, the advantage shall accrue to the BHEL and that if the cost of completion exceeds the money due to the Contractor under the contract, the Contractor shall either pay the excess amount ordered by the Engineer-in-charge or the same shall be recovered from the Contractor by other means.

Engineer-in-charge will have powers to take possessions of  the site and any materials, constructional plant, implements, stores, etc, thereon and or carryout the work by any means at the risk and cost of the contractor.

In case the BHEL completes the work under the provisions of this condition the  cost of such completion to be taken  into account in determining  the excess cost to  be charged to the contractor under this Condition shall consist of the cost of materials purchased and/or labour provided by the BHEL with an addition of such percentage to cover superintendence and establishment  charges as may be decided by the Project Manager/Project Engineer whose decision shall be final and  conclusive.

If the contractor fails to pay the excess sum within a period of 30 days, the Engineer-in-charge shall have the right to sell any or all of the contractor’s unused materials, constructional plant implements, temporary buildings, etc., and apply the proceeds of sale thereof towards the satisfaction of any sum due from the contractor under the contract and if thereafter be any balance outstanding from the contractor, it shall be recovered in accordance with the provisions of the contract.

45.	Cancellation of  contract in part  or in full for contractor’s default:

If the Contractor:

- makes default in commencing the work within a reasonable time from the  date of handing over of the site and continue in that state after a reasonable notice from Engineer-in-charge, OR

(b)     in the opinion of the Engineer-in-charge at any time, whether before or after the date or extended date for completion, makes default in proceeding with the work, with due diligence and continue  in that state after a notice of  seven days from Engineer-in-charge, OR

(c)	fails to comply with any of the terms and conditions of the contract or after 7   days notice in writing with  orders properly issued there under, ( OR)

(d)	fails to complete the work order and items of work individual dates for completion and clear the site on or before the date of completion or fails to achieve the progress as set out under clauses 7 of these General Conditions of Contract.

The Accepting Officer may, without prejudice to any other right or remedy which shall have accrued or shall accrue there after to B.H.E.L cancel the contract as a whole or in part thereof or only such work order or items of work in default from the contract.  Whenever the Accepting Officer exercises his authority to cancel the contract as a whole or in part under this conditions he may complete the work at the Contractor’s  risk and cost, provided always that in the event of the cost of completion (as certified by  Engineer-in-charge which is final and conclusive) being less than the contract cost the advantage shall accrue to the B.H.E.L if the cost of completion exceeds, the money due to the contractor under this contract,  the contractor shall either pay the excess amount ordered by Project Manager or the same shall be  recovered from the contractor by other means.  Engineer-in-charge will have power to take possession of the site and any materials, constructional plant, implements, Stores, etc., thereon.

In case the B.H.E.L completes the work or any part thereof under the provisions of this conditions the cost of such completion to be taken in to  account in determining the excess cost to be charged to the contractor under this conditions shall consists of the cost of materials purchased and/or labour provided by the B.H.E.L with an addition of such percentage to cover superintendence and establishment charges   as

may be decided by the Project Manager/Engineer whose decision shall be final  and conclusive.

If the contractor fails to pay the excess sum within a period of 30 days, the Engineer-in-charge shall  have the right to sell any or all of the contractor’s unused materials, constructional plant  implements, temporary buildings, etc, and apply the proceeds of sale thereof towards  the satisfaction  of any sum due from the contractor under the contract and if thereafter be any balance outstanding from the contractor it shall be recovered in accordance with the provision of the contract.

46.	Termination of Contract for Death

Without prejudice to any of the rights or remedies under this contract if the contractor dies, the accepting Officer shall have the opinion of terminating the contract without compensation to the contractor.

- Special Powers of Determination

If at any time after the acceptance of the tender B.H.E.L shall for any reason whatsoever not require the whole or any part of the work, to be carried out the project Manager/Engineer shall give notice in writing of the fact to the Contractor who shall have no claim to any payment of compensation or otherwise howsoever on account of any profit or advantage which he might have derived from the execution of the work in full  but which he did not derive in consequence of the foreclosing of the work.

He shall be paid at Contract rates, for the full amount of the work executed  including such additional works, e.g. clearing of site, etc., as may be rendered necessary by the said fore closing.  He shall also be allowed a reasonable payment (as decided  by the Accepting Officer )  for  any expenses sustained on account of labour and materials collected but which could not be utilised on the work, as verified by the Engineer-in-charge.  Neither shall the Contractor have any claim for compensation on account of any alterations having been made in the original specifications,  drawings, designs  and instructions, involving any curtailment of the work as originally contemplated.

48.	Fair Wage

a)	The contractor shall pay not less than the “ Fair Wage” to labourers engaged by   him on the work.

“Fair Wage” means wage whether for time or piece work notified at the time of inviting tenders for the work and where such wages have not been notified the wages prescribed by the Project Manager/Engineer for the stations at which the work is done.

b)	The Contractor shall not with standing the provision of any contract to the contrary, cause to be paid a “Fair Wage” to labourers indirectly engaged on the work, including any labour engaged by the Sub-Contractors in connection with the said work, as if the labourers had been directly employed by him.

c)	In respect of labourers directly or indirectly employed on the work for the performance of  the Contractors part of this Agreement, the Contractor shall comply with or cause to be complied with B.H.E.L Contractor’s Labour Regulations (appended here to as Annexure ‘A’ to these conditions) in regard to payment of wages, wage period deduction from wages, recovery of wages not paid and deductions unauthorisedly  made, maintenance of wage book, wage-slips publication of scale of wage and other terms of employment inspection and submission of periodical returns and all other matters of alike nature.

d)	The Engineer-in-charge concerned shall have the right to deduct from the money due to the contractor any sum required or estimated to be required for  making good the loss suffered by a worker or workers by reason of non-fulfilment of the conditions of the contract for the benefit of the workers non-payment of wages or of deductions made from his or their wages which are not justified by the terms of the contract or non-observance of the regulations.

e)	The Contractor shall be liable primarily for all payments to be made under the contract and for the observance of the Regulations aforesaid without prejudice to his right to claim indemnity from his sub-contractors.

- The regulations aforesaid shall be deemed to be a part of this contract and any breach thereof shall be deemed to be a breach of this Contract.

CHAPTER IV

VALUATION AND PAYMENT

49.	Records and Measurements:

All items having a financial value shall be entered in the B.H.E.L Measurement Book so that a complete record is obtained of all works performed under the Contract.

Buildings, etc., priced in Schedule ‘A’ as a unit lumpsum will be entered by number at the unit lumpsum.

Work carried out for agreed lumpsum will be described and similarly recorded.

Lumpsum omissions will be entered for deduction.  Measurement shall be restricted to that required to ascertain the financial liability of B.H.E.L under the contract.

Work which fails to be measured in detail shall be measured physically, without reference to any local custom that may obtain excepting where it may otherwise be directed in the tender documents.  The measurements shall be taken jointly by any person duly authorized on the part of the BHEL and by the Contractor.

The Engineer-in-charge shall give reasonable notice in writing to the Contractor of appointment for measurement.

The Contractor shall, without extra charge, provide assistance with appliance and other things necessary for measurement.

The Contractor shall bear all the cost of measurement of his work.

Measurement shall be entered in the B.H.E.L Measurement Book and signed and dated by both parties each day at the Site on completion of measurement.  If the Contractor objects to any of the measurements recorded on behalf of the B.H.E.L a note to that effect will be made in the BHEL Measurement Book or against the item or items objected to; and such note shall be signed and dated by both the parties engaged in taking the measurement.

If, as a result of such objection, it becomes necessary to re-measure the work wholly or in part, the expense of such re- measurement shall be borne by the party requiring the measurement.

Measurement to be re-taken, provided that a net error is found by this  remeasurement to amount to less than 5% (five percent)  of the value as recorded by the first measurement.  But, where the net errors amount to 5% and over  of the  said value, then the cost is to be borne by the other party.  In any case, if the net value of errors found exceeds Rs. 500/- the expense of re-measurement is to be borne by the other party.  If the Contractor’s representative fails to attend when required, the Engineer-in-charge shall have power to proceed by himself to take measurement and in that case these measurements shall be accepted by the Contractor as final.

The contractor shall, once every month, submit to the Engineer-in-charge with a copy to the Civil Manager/Senior Engineer details of his claims for the work done by him up to and including the previous month which are not covered by his Contract Agreement in any of the following respects;

- Deviation from the items and Specifications provided in the contract documents.

- Extra Items/New Items of work

- Quantities in excess of those provided in the contract schedule.

- Items in respect of which rates have not been settled.  He should, in addition furnish a clear certificate to the effect that the claims submitted by him as aforesaid cover all his claim and that no further claims shall be raised by him in respect of the work done up to and including the period  under report.

Except where any  general or detailed description of the work in quantities expressly  shows to the contrary, schedule of quantities shall be deemed to have been prepared and measurements shall be taken in accordance with

the procedure set forth in the schedule of rates specification notwithstanding any provision in the relevant standard method of measurement or any general or local custom.  In the case of items which are not covered by the schedule of rates / specification, measurements shall be taken in accordance with relevant standard method of measurement issued by the Indian Standard Institution or as per standard engineering practice.

50.	Valuation of Deviations:

Rates for deviated items of work will be  fixed as follows:-

1.	For any item of work required to be carried out after the contract has been awarded and which is not covered by Contractor’s  Schedule but is covered by B.H.E.L Schedule of rates the rate payable for such a fresh item will be derived from B.H.E.L Schedule by the method of proportion as follows:

a.	In the same proportion to rate in B.H.E.L Schedule of Rates as the tendered rate for the nearest analogous item of work in Contractor’s schedule bears to rate for the particular analogous item of work in B.H.E.L Schedule of rates.

b.	If a single appropriate analogous item of work is not available in both Schedule (Contractor’s and B.H.E.L Schedule) then the method of proportion will be applied to the nearest analogous group of items available in both the Schedules referred to i.e. in the same proportion as the total tendered cost of that particular group of  items (the sum of the products of the tendered rates and the  quantities for which orders are placed bears to the total cost of the same items and quantities at the B.H.E.L Schedule of Rates.

c.	If even an appropriate analogous group of items is not available in Contractor’s Schedule and B.H.E.L Schedule, then the methods of proportion will be applied to all those items of the whole work, which are available in both the Schedules and for which orders have been placed on the contractor, i.e., in the same proportion as the total cost of all these items of work  (the sum of the products of the tendered rates and the quantities  for which orders are placed) bears to the total cost of the same items and qualities at the B.H.E.L Schedule of Rates.

II.	If any work not covered by any of the foregoing is ordered on the Contractor, the basis of payment shall be decided by the Accepting Officer whose decision shall be final and conclusive and binding on the parties.

The selection of analogous items or analogous group of items referred to above shall be done by the Engineer-in-charge.  Where the rates for deviated items or new items of work can be derived by the selection of different analogous items or analogous group of items, the lowest of all such derived rates shall be taken as the correct rate.

In the case of the contracts   for which the Engineer-in-charge  is the  Accepting Officer, all disputes regarding the settlement of rates of deviated or new items or work shall be referred to the Deputy Manager/Manager/SR.Manager whose decision shall be final and conclusive as the case may be.

51.	Reimbursement / Refund on Variation in Price, Materials:

If after submission of the tender and/or during the progress of the works, the price of any material (not being a material supplied from the B.H.E.L store in accordance with the Conditions of the Contract) is increased or decreased by an Act of Legislature (Central or State) and/or any notification there under or on account of new duties or levies such as octroi or on account of increase or  decrease in such duties affecting the price of materials required for incorporation in the works and made from materials of

which the price has increased or decreased as aforesaid and the Contractor has thereupon to pay in respect of such material or item a price which is higher or lower than the price of that material or item as prevailing immediately before the passing of such Act or levying, increasing/ decreasing of such duty, the B.H.E.L shall incase of  increase in price or  the duty reimbursed to the contractor and incase of decrease in price, the B.H.E.L shall  be entitled to a refund of the reduction price or the reduction in duty.  Provided, however no reimbursement or refund shall be made if the increase/decrease is not more than 10% of the said price, and if so, the reimbursement  or refund shall be  made  only on  the  excess over 10% provided always that any such increase shall not be payable if, in the opinion of the Deputy Manager/Manager (whose decision  shall be final and conclusive)  the increase is attributable to the delay in the execution of the contract within the control of the contractor or that any such increase  has  become  operative  after the contracted/or extended date of completion of the work or items of work in question.

The Contractor shall, for the purpose of this condition, keep such books of account and other documents as are necessary to show the amount of any increase claimed or any reduction available and shall allow inspection of the same by any duly authorized representative of the B.H.E.L and further shall at the request of the Engineer-in-charge furnish for verification such other information of the Engineer-in-charge  may require.

The Contractor shall within a reasonable time of his becoming, aware of any alteration in the prices of any such materials, give notice thereof in writing to the Engineer-in-charge stating that the rate is submitted in pursuance to this condition together with all information relating thereto which he may be in a position to supply.

52.	Advances on Account:

No payment shall be made for work estimated to cost less than Rupees FIVE  THOUSAND till the whole of the work shall have been completed and a certificate of completion given by the Competent Authority.

In the case of work estimated to cost more than Rupees FIVE THOUSAND the contractor may at intervals of not less than one month or as otherwise provided for in the Contract Documents, counting from the date on which order to commence work is given by Engineer-in-charge submit claims on B.H.E.L forms for payment of advances on account of work done and of materials delivered in connection with the Contract.

The contractor shall be paid in respect of such claims to the extent approved and passed by the Engineer-in-charge subject a maximum of 90% of the value of the work actually executed to the satisfaction of the Engineer-in-charge.  The certificate of the Engineer-in-charge regarding such approval and passing of the sums so payable shall be final and conclusive against the Contractor.

“After the full amount of Security Deposit is made up through the 10% deduction from On account” bills, 100% payment of all subsequent bills may be made to the Contractor.

The Contractor may also be paid during the progress of the work 75% of the value of any materials which are in the opinion of the Engineer-in-charge in accordance with the Contract, and are actually required for incorporation in the work and which have reasonably been brought to the site in connection therewith and are adequately stored and / or  protected against damage by weather or other causes, but which have not at the time of payment of the advance been incorporated in the work on furnishing a formal hypothecation deed.  Payment of such advances, however, shall be purely at the discretion of the Deputy Manager/Senior Engineer provided always that payments shall not be made under these periodical certificates in respect of perishable materials like lime, cement, timber, sand, kankar, etc.,  Any sums/due from the Contractor on account of Tools and Plant, Stores or any other items provided by B.H.E.L shall be deducted from the respective advances, the Engineer-in-charge shall from time to time certify the sums payable to the contractor after retaining the reserves.

Any certificate relating to work done or materials delivered may be modified  or corrected by any subsequent interim certificate or by the final certificate and no certificate of the Engineer-in-charge supporting an advance payment shall itself be conclusive evidence that any work or materials to which it relates are in accordance with the contract.  All such intermediate payments shall be regarded as advances against the final payment only and shall not be considered as an admission of the due performance of the contract or any part there of in any respect or the accruing of any claim whatsoever.

Such intermediate payment shall not conclude, determine or affect  in any way the powers of the Engineer-in-charge as to the final settlement and adjustment of the accounts or otherwise, or in any way vary or affect the contract.

53.	Final Bill

As soon as possible after the completion of the work to the satisfaction of the Engineer-in-charge, the contractor shall forward a certified final account on BHEL form, in duplicate.

It shall be accompanied by all abstracts, vouchers etc., in support thereof and shall be prepared in the manner prescribed by the Engineer-in-charge.

No claims will be entertained after the receipt of the final bill.

The Contractor shall be entitled to be paid the final sum less the value of payments already made on account subject to certification of the final bill by the Engineer-in-charge.  Any sums due from the contractor on account of Tools & Plant, Stores or any other items provided by BHEL not yet  recovered from the contractor shall be deducted from the final sum aforesaid.

No charge shall be allowed to the Contractor on account of the preparation of the final bill.

54.	Payment of Bills

All payment to be made to the Contractor under this contract shall be by “Crossed Cheque” marked ‘A/C payee only (Within a reasonable time after the Certification by the Engineer-in-charge) at the State Bank of India or their subsidiaries located in the station where either the work is executed or service rendered or at their branch nearest to the station where the Officer of the Engineer-in-charge is located.

55.	Recovery from Contractor:

Whenever under the contract any sum of money shall be recoverable from or payable by the contractor the same may be deducted from any sum then due or which at any time thereafter may become due to the Contractor under the contract or under any other Contract with BHEL or from his Security Deposit or he shall pay the claim on demand.

56.	Post Technical Audit of Work and Bills:

BHEL reserves the right to carry out a post-payment audit and technical examination of the work and final bill including all supporting vouchers, abstracts etc., and to enforce recovery of any sums becoming due as a result thereof in  the manner provided in the preceding sub-paragraphs provided however that no such recovery shall be enforced after three years of passing the final bill.

57.	Refund of Security Deposit:

50% of the Security Deposit mentioned in condition 16 above, may be refunded to the contractor in respect of all contracts on completion of work  and  after payment  of  final

bill and the  balance  50% on  expiry  of  the maintenance period, (described under clause 23) provided the contractor shall have rendered a “No-Demand” Certificate.  In case of works where maintenance period is not involved 100% of the Security Deposit may be refunded after payment of final bill provided that the contractor shall have rendered a “No-Demand Certificate”.

58. 	Arbitration:

Except where otherwise provided for in the contract all questions and disputes relating to the meaning of the specifications, designs, drawings and instructions hereinbefore mentioned and as to the quality of workmanship or materials used on the work or as to any other question, claim, right, matter or thing whatsoever in any way arising out of or relating to the contract, designs, drawings, specifications, estimates, instructions, orders or these conditions or otherwise concerning the work or failure to execute the same whether arising during the progress of the work or after the completion or abandonment thereof shall be referred to the sole arbitration of the Executive Director  / General Manager of BHEL and if  ED   is unable or unwilling to act, to the  sole arbitration of some other person appointed by the  ED / General Manager, willing to act  as such arbitrator.  The cases referred to arbitration shall be other than those for which the decision of the Manager / Senior Engineer / Engineer-in-charge is expressed in the contract to be final and conclusive.  There will be no objection if the arbitrator so appointed is an employee of B.H.E.L and that he had to deal with the matters to which the contract relates and that in the course of his duties as such he had expressed views on all or any of the matters in dispute or difference.

The arbitrator to whom the matter is originally referred being transferred or vacating his office or being unable to act for any reason, such Executive Director / General Manager as aforesaid at the time of such transfer, vacation of office or inability to act, shall appoint another person to act as arbitrator in accordance with the terms of the contract.  Such person shall be entitled to proceed with the reference from the stage at which it was left by his predecessor.

Subject as aforesaid the provision of the Arbitration & Reconciliation Act, 1996 or any statutory modification or re-enactment  thereof and the rules made there under and for the time being in force shall apply to the arbitration proceeding under this clause.

It is a term of the contract that the party involving arbitration shall specify the dispute or disputes to be referred to arbitration under this clause together with the amount or amounts claimed in respect of each such dispute.

The arbitrator(s) may from time to time with consent of the parties enlarge the time for making and publishing the award.

The work under the Contract shall, if reasonably possible, continue, during the arbitration proceedings and no payment due or payable, to the Contractor shall be withheld on account of such proceeding.

The Arbitrator shall be deemed to have entered on the reference on the date he issues notice to both the Parties fixing the date of first hearings.

The arbitrator shall give a separate award in respect of each dispute or difference referred to him.

The venue of arbitration shall be such place as may be fixed by the Arbitrator in his sole discretion.

The award of the arbitrator shall be final, conclusive and binding on all parties to this contract.

In the event of disputes or differences arising between one public sector enterprise and a Govt.  Department or between two public sector enterprises the above stipulations shall not apply the provisions of BPE office memorandum No. BPE/CL 001/ 76MAN / 2 (1.10) 75-BPE (GM-1) dated 1st January 1976 or its amendments for arbitration shall be applicable.

ANNEXURE ‘A’

B.H.E.L CONTRACTOR’S LABOUR REGLUATIONS

(See condition 20)

1.	 Definition:

In these regulations, unless otherwise expressed or indicated, the following words and expressions shall have the meaning hereby assigned to them.

a)	“Labour” means workers employed by a contractor directly, or indirectly through a sub-contractor, or by an agent on his behalf on a payment not exceeding Rs.500 per month.

b)	“Fair Wage” means wages, which shall include wages for weekly day of rest and other allowances, whether for time or piece work, after taking into consideration prevailing market rates for similar employments in the neighborhood but shall not be less than the minimum rates of wages fixed under the minimum Wages Act.

c)	“Contractor” for the purpose of these Regulations shall include an agent or Sub-Contractor employing labour on the work taken on contract.

d)	“Inspecting Officer” means any Labour Enforcement Officer, or Assistant Labour Commissioners of the Chief Labour Commissioner’s Organisation.

e)	“Form” means a form appended to these Regulations.

2.	Notice of Commencement :

The Contractor shall, within SEVEN DAYS of commencement of the work, furnish in writing to the Inspecting Officer of the area concerned the following information with copy to the Engineer-in-charge.

a)	Name and situation of the work.

b)	Contractor’s name and address.

c)	Particulars of the Department for which the work is undertaken.

- Name and address of sub-contractors as and when they are appointed

- Commencement and probable duration of the work

- Number of workers employed and likely to be employed,

- ‘Fair wages’ for different categories of workers.

3.	(i)	Number of hours which shall constitute a normal working day:

The number of hours which shall constitute a  normal working day for an adult shall be NINE hours.  The working day of an adult worker shall be so arranged that of intervals, if any, for rest it shall not spread over more than twelve hours on any day.  When an adult worker is made to work for more than NINE hours on any day or for more than FORTY EIGHT hours in any week he shall in respect of overtime work, be paid wages at double the ordinary rate of wages.

(ii)	Weekly day of rest:  Every worker shall be given a weekly day of rest which shall be fixed and notified atleast TEN days in advance. A worker shall not be required or allowed to work on the weekly rest day unless he has or will have a substituted rest day, on one of five days immediately before or after the rest day.  Provided that no substitution shall be made which will result in the worker working for more than ten days consecutively without a rest day for a whole day.

Where in accordance with the foregoing provisions a worker works on the rest day and has been given a substituted rest day he shall be paid wages for the work done on the weekly rest day at the overtime rate of wages.

NOTE:	The expression ‘Ordinary rate of wages’ means the fair wage the worker is entitled to.

4.	Display of Notice Regarding Wages, Weekly Day of Rest, Etc.:

The Contractor shall before he commences his work on contract display and correctly maintain and continue to display and correctly maintain in clean and legible condition in consipicuous places on the works, notice in English and in the local Indian languages, spoken by majority of workers, given the rate of fair wages, the hours of work for which such wages are payable, the weekly rest days workers are entitled to and name and address of the Inspecting Officer.  The Contractor shall send a copy each of such notices to the Inspecting Officers and the Engineer-in-charge.

5.	Fixation of Wage Periods:

The Contractor shall fix wage periods in respect of which wages shall be payable .

No wage period shall normally exceed one week.

6.	Payment of Wages:

- Wages due to every worker shall be paid to him direct.  All wages shall be paid in current coins or currency or in both.

Wages of every worker employed on the contract shall be paid where the wage period is one week, within, THREE DAYS from the end of the wage period, and in any other case before the expiry of the 7th day or 10th day from the end of the wage period according as number of workers does not exceed 1,000.

- When employment of any worker is terminated by or on behalf of the contractor, the wages earned by him shall be paid before expiry of the day succeeding the one on which his employment is terminated.

- Payment of wages shall be made at the work site on a working day except when the work is completed before expiry of the wage period, in which case final payment shall be made at the work site within 48 hours of the last working day and during normal working time.

NOTE: The term “Working Day”  means a day on which the work on which labour  is employed is in progress.

7.	Register of Workmen :

A register of workmen shall be maintained in the form appended to these regulations and kept at the work site or as near to it as possible, and the relevant particulars of every workmen shall be entered therein within THREE days of his employment.

8.	Employment Card:

The Contractor shall issue an employment card in the Form appended to these regulations to each worker on the day of work or entry into his employment. If a worker has already any such card with him issued by the previous employer the contractor shall merely endorse that Employment Card with relevant entries.  On termination of employment the Employment Card shall again be endorsed by the Contractor and returned to the worker.

9.	Register of Wages etc.,  :

- A register of Wages-cum-Muster Roll in the Form appended to these regulations shall be maintained and kept at the work site or as near to it as possible.

- A wage slip in the form appended to these regulations shall be issued to every worker employed by the Contractor atleast a day prior to disbursement of wages.

10.	Fines and Deductions which may be made from wages:

- Wages of worker shall be paid to him without any deductions of any kind except the following:

a.	Fines;

b.	Deductions for absence from duty, i.e., from the place or the places where by the terms of his employment he is required to work.  The amount of deduction shall be in proportion to the period for which he was absent.

c.	 Deduction for damage to or loss of goods expressly entrusted to the employed person for custody, or for loss of money which he is required to account for, where such damage or loss is directly attributable to his neglect or default;

d.	Deductions for recovery of advances or for adjustment of overpayment of a wages.  Advance granted shall be entered in a register;

and

e.	Any other deduction, which the B.H.E.L may from time to allow.

ii.	No fines shall be imposed on a worker save in respect of such acts and omissions on his part as have been approved by the Chief Labour Commissioner.

iii.	No fines shall be imposed on a worker and no deductions for damage or loss shall be made from wages until the worker has been given an opportunity of showing cause against such fines or deductions.

iv.	The total amount of fines which may be imposed in any one wage period on a worker shall not exceed an amount equal to three paise in rupee of the wages payable to him in respect of that wage period.

v.	No fine imposed on a worker shall be recovered from him in instalments, or after expiry of sixty days from the date on which it was imposed.  Every fine shall deemed to have been imposed on the day of the act or omission in respect of which it was imposed.

vi.	The Contractor shall maintain both in English and the local Indian language a list, approved by the Chief Labour Commissioner, clearly stating the acts and omissions for which penalty or fine may be imposed on a workman and display it in good condition in a conspicuous place on the work site.

vii.	The Contractor shall maintain a register of fines and the register of deductions for damage or loss in the Forms appended to these regulations which should be kept at the place of work.

11.	Register of Accidents  :

The Contractor shall maintain a register of accidents in such form as may be convenient at the work place but the same shall include the following particulars:

a.	Full particulars of the labourers who met with the accident.

b.	Rates of Wages.

c.	Sex

d.	Age

e.	Nature of accident and cause of accident.

f.	Time and date of accident.

g.	Date and time when admitted in hospital.

h.	Date of discharge from the hospital.

i.	Period of treatment and result of treatment.

j.	Percentage of loss of earning capacity and disability as assessed by Medical Officer.

k.	Claim required to be paid under Workmen’s Compensation Act.

l.	Date of payment of compensation.

m.	Amount paid with details of the person to whom the same was paid.

n.	Authority by whom the compensation was assessed.

o.	Remarks.

12.	Preservation of Registers	:

The Register of Wages-cum-Muster Roll required to be maintained under these Regulations shall be preserved for 3 years after the date on which last entry is made therein.

13.	Enforcement	:

The Inspecting Officer shall either on his own motion or on a complaint received by him carry out investigations, and send a report to the Engineer-in-charge specifying the amounts representing workers, dues and amount of penalty to be imposed on the contractor for breach of these Regulations, that have to be recovered from the contractor, indicating full details of the recoveries proposed and the reasons therefore.  It shall be obligatory on the part of the Engineer-in-Charge on receipt of such a report to deduct such amounts from payments due to the contractor.

14.	Disposal of amounts recovered from the Contractor	:

The Engineer-in-charge shall arrange payment to workers concerned within FORTY FIVE days  from receipt of a report from the Inspecting Officer except in cases where the contractor had made an appeal under Regulation, 16 of these Regulations.  In cases where there is an appeal, payments of workers, due would be arranged by the Engineer-in-charge, wherever such payments arise, within THIRTY days from the date of receipt of the decision of the Regional Labour Commissioner (R.L.C.).

15.	Welfare Fund:

All money that are recovered by the Engineer-in-charge by way of workers, due which could not be disbursed to workers within the time–limit prescribed above, due to reasons such as where-about of workers not being known, death of a worker, etc., and also amounts recovered as penalty, shall be credited to a fund to be kept under the custody of B.H.E.L for such benefit and welfare of workmen employed by contractors.

16.	Appeal against decision of Inspecting Officer	:

Any person aggrieved by a decision of the Inspecting Officer may appeal against such decision to the Regional Labour Commissioner concerned within THIRTY days from the date of the decision, forwarding simultaneously a copy of his appeal to the Engineer-in-charge.  The decision of the Regional Labour Commissioner shall be final and binding upon the Contractor and the workmen.

17. 	Representation of Parties:

i.	A workman shall be entitled to be represented in any investigation or enquiry under these Regulations by an Officer of a registered trade union of which the said trade union is affiliated or where the workman is not a member of any registered trade union, by an Officer of a registered trade union connected with, or any other workmen employed in the industry in which the worker is employed.

ii.	A Contractor shall be entitled to be represented in any investigation or enquiry under these Regulations by an officer of  an association of contractors of which he is a member or by an officer of a Federation of Associations of Contractors   to which the said association is affiliated or where the contractor is not a member of any association of employers, connected with, or by any other employer engaged in the industry in which the contractor is engaged.

iii.	 No party shall be entitled to be represented by a legal practitioner in any investigation or enquiry under these Regulations.

18.	Inspection of Books and other Documents:

The Contractor shall allow inspection of the Register and other documents prescribed under these Regulations by Inspecting Officer and the Engineer-in-charge or his authorized representative at any time and by the worker or his agent on receipt of due notice at a convenient time.

19.	Interpretation etc.

On any question as to the application, interpretation or effect of the Regulations the decision of the Chief Labour Commissioner or Deputy Chief Labour Commissioner (Central) shall be final and binding.

20.	Amendments:

Central Government may, from time to time, add to or amend the Contractor’s Labour Regulations and issue such directions as it may consider necessary for the proper implementation of the Contractor’s Labour Regulations or for the purpose of removing any difficulty which may arise in the administration thereof, based on which the B.H.E.L., Contractor’s Labour Regulations herein contained shall be subject to revision.

MODEL RULES FOR LABOUR WELFARE

(See Condition 20)

1.	Definition:

(a)	‘Workplace’ means a place at which, on an average, twenty or more      workers are employed.

(b)	‘Large Workplace’ means a place at which on an average, 500 or more workers are employed.

2.	First Aid:	At every workplace, there shall be maintained in a readily accessible place first-aid appliances including an adequate supply of sterilized dressings and sterilized cotton wool as prescribed in the Factory Rules of the State in which the work is carried on.  The appliances shall be kept in good order and, in large work places, they shall be placed under the charge of a responsible person who shall be readily available during working hours.

At large workplaces, where hospital facilities are not available within easy distance of the works First Aid posts shall be established and be run by a trained compounder.

Where large workplaces are remotely situated and far away from regular hospitals an indoor ward shall be provided with one bed for every 250 employees.

Where large workplaces are situated in cities, towns or in their suburbs and no beds are considered necessary owing to proximity of city or town hospitals, suitable transport shall be provided to facilitate removal of urgent cases to these hospitals.  At other workplaces, some conveyance facilities shall be kept readily available to take injured person or persons suddenly taken seriously ill, to the nearest hospital.

At large workplaces there shall be provided and maintained an ambulance room of the prescribed sizes, containing the prescribed equipment and in the in-charge of such medical and nursing staff as may be prescribed.  For this purpose the relevant provisions of the Factory Rules of the State Government of the area where the work is carried on may be taken as the prescribed standard.

3.	Accommodation for  Labour:	The Contractor shall during the progress of the works provide, erect and maintain necessary temporary living accommodation and ancillary facilities for labour at his own expense and to standards and scales as approved by the Engineer-in-charge.

4.	Drinking Water:  In every workplace, there shall be provided and maintained at suitable places, easily accessible to labour, a sufficient supply of cold water fit for drinking.

Where drinking water is obtained from an intermittent public water supply each workplace shall be provided with storage where drinking water shall be stored.  Every water supply storage shall be at a distance of not less than 15 meters from any latrine drain or other source of pollution. Where water has to be drawn from an existing well, which is within such proximity of latrine drain or any other source of pollution, the well shall be properly chlorinated before water is drawn from it for drinking. All such wells shall be entirely closed in and be provided with a trap door which shall be dust and water-proof.

A reliable pump shall be fitted to each covered well, the trap door shall be kept locked and opened only for cleaning or inspection which shall be done at least once a month.

5.	Washing and Bathing places:	Adequate washing and bathing places shall be provided separately for men and women.  Such places shall be kept in clean and drained condition.

6.	Scale of Accommodation in Latrines and Urinals:	These shall be provided within the precincts of every workplace latrines and urinals in an accessible place, and the accommodation, separately for each of these, shall not be less than at the following scales:

No.of Seats

a)	Where number of persons does not exceed 50		2

b)	Where number of persons exceed 50 but does

not exceed 100						3

c)	For additional persons					3

Per 100 or part thereof.

In particular cases, the Engineer-in-charge shall have the power to increase the requirement, where necessary.

7.	Latrines and Urinals:	Except in workplaces provided with water flushed latrines connected with a waterborne sewage system, all latrines shall be provided with receptacles on dry-earth system which shall be cleaned at least four times daily and at least twice during working hours and kept in a strictly sanitary condition.  Receptacles shall be tarred inside and outside at least once a year.

If women are employed, separate latrine and urinals, screened from those for men and marked in the vernacular in conspicuous letters “For Women only” shall be provided on the scale laid down in rule 6.  Those for men shall be similarly marked “For Men only” A poster showing the figure of a man and of a woman shall also be exhibited at the entrance to latrines for each sex.  There shall be adequate supply of water close to latrines and urinals.

8.	Construction of Latrines:	Inside walls shall be constructed of masonry or other non-absorbent material and shall be cement-washed inside and outside at least once a year.  The dates of cement washing shall be noted in a register maintained for the purpose and kept available for inspection.  Latrines shall have at least thatched roof.

9.	Disposal of Excreta:	Unless otherwise arranged for by the local sanitary authority, arrangement for proper disposal of excreta by incineration at the workplace shall be made by means of a suitable incinerator approved by the local medical health and municipal or cantonment authorities.  Alternatively local excreta may be disposed off by putting a layer of night soil at the Bottom of pucca tank prepared for the purpose and covering it with a 15 cm. layer of waste or refuse and then covering it with a layer of earth for a fortnight (when it will turn into manure).

The contractor shall, at his own expenses, carry out all instruction issued to him by the Engineer-in-charge to effect proper disposal of soil and other conservancy work in respect of Contractor’s work-people or employees on the Site.  The Contractor shall be responsible for payment of any charges which may be levied by municipal or cantonment authority for execution of such work on his behalf.

10.	Provision of shelters during rest:  At every workplace there shall be provided, free of cost, four suitable sheds, two for meals and two others for rest, separately for use of men and women labour.  Height of each shelter shall not be less than 3 meters from floor level to lowest part of roof.  Sheds shall be kept clean and space provided shall be on the basis of at least 0.5 sq.m. per head.

11.	Crèches:	At a place at which 20 or more women workers are ordinarily employed, there shall be provided at least one hut for use of children under the age of 6 years of such women.  Huts shall not be constructed to a standard lower than that of thatched roof, mud floor and walls with wooden planks spread over mud floor and covered with matting.

Huts shall be provided with suitable and sufficient openings, for light and ventilation.  There shall be adequate  provision of sweepers to keep the places clean.  There shall be two ‘dais’ in attendance.  Sanitary utensils shall be provided to the satisfaction of local medical, health and municipal or cantonment authorities.  Use of huts shall be restricted to children, their attendants and mothers of children.

When the number of women workers is more than 25 but less than 50, the Contractor shall provide at least one hut and one Dais to look after children of women workers.

Size of crèche (s) shall vary according to the number of women workers employed.  Creche(s) shall be properly maintained and necessary equipment like toys, etc. provided.

12.	Canteen:	A cooked food canteen on a moderate scale shall be provided for the  benefit of workers wherever it is considered necessary.

13.	Planning, setting and erection of the above mentioned structures shall be approved by the Engineer-in-charge, and the whole of such temporary accommodation shall at all times during the progress of the works be kept tidy and in a clean and sanitary condition to the satisfaction of the Engineer-in-Charge and at the Contractor’s expense.  The Contractor shall conform generally to sanitary requirements of local medical, health and municipal or cantonment authorities and at all times adopt such precautions as may be necessary to prevent soil pollution of the site.

On completion of the works the whole of such temporary structures shall be cleared away, all rubbish burnt, excreta or other disposal pits or trenches filled in and effectively sealed off and the whole of site left clean and tidy to the entire satisfaction of the Engineer-in-Charge, and at the Contractor’s expenses.

14.	Anti-malarial precautions:	 The Contractor shall, at his own expense, conform to all anti-malarial instructions given to him by the Engineer-in-charge, including filling up of any borrow pits which may have been dug by him.

15.	Enforcement: The Inspecting Officer mentioned in the Contractors Labour Regulations or any other officer nominated in his behalf by the Engineer-in-Charge shall report to the Engineer-in-Charge all cases of failure on the part of the Contractor and or his sub-contractors to comply with the provisions of these Rules either wholly or in part and the Engineer-in-Charge shall impose such fines and other penalties as are prescribed in the conditions.

B.H.E.L SAFETY CODE

See Condition-20

1.	Suitable scaffolds shall be provided for workmen for all work that cannot safely be done from the ground, or from solid construction except such short period of work as can be done safely from ladders.  When a ladder is used an extra mazdoor shall be engaged for holding the ladder and if the ladder is used for carrying materials as well, suitable footholds and hand-holds shall be provided on the ladder and the ladder shall be given an inclination not steeper than ¼ to 1 (¼  horizontal and 1 vertical)

2.	Scaffolding or staging more than 3.25 meters above the ground or floor, swung or suspended from an overhead support or erected with stationary support, shall have a guard rail properly attached, bolted, braced  and otherwise secured atleast 1 meter high above the floor or platform of such scaffolding or staging and extending along the entire length of the outside and ends thereof with only such openings as may be necessary for the delivery of materials.  Such scaffolding or staging shall be so fastened as to prevent it from swaying from the building or structure.

3.	Working platform, gangways, and stairways shall be so constructed that they do not sag unduly or unequally, and if height of a platform or gangway or stairway is more than 3.25 meters above ground level or floor level, it shall be closely bordered have adequate width and be suitably fenced, as described in 2 above

4.	Every opening in floor of a building or in a working platform shall be provided with suitable means to prevent fall of persons or materials by providing suitable fencing or railing with a minimum height of 1 meter.

5.	Safe means of access shall be provided to all working platforms and other working places.  Every ladder shall be securely fixed. No portable single ladder shall be over 9 meters in length.  Width between side rails in a rung, ladder shall in no case be less than 30 cm, for ladders upto and including 3 metres in length.  For longer ladders this width shall be increased by atleast 6 mm for each additional 30 cm. of length.  Uniform step spacing shall not exceed 30 cm.

Adequate precautions shall be taken to prevent danger from electrical equipment.  No materials on any of the sites shall be so stacked or placed as to cause danger or inconvenience to any person or the public.  The Contractor shall provide all necessary fencing and lights to protect public from accidents and shall be bound to bear expenses of  defence  of  every suit action or other proceedings at law that may be brought by any person for injury sustained  owing  to neglect  of  the  above precautions,  and pay any damages and costs which may be awarded in any such suit, action or proceeding to any such person or which may with the consent of the Contractor be paid to compromise any claim by any such person.

6.	Excavation and Trenching:

All trenches, 1.5 metres or more in depth, shall at all times be supplied with atleast one ladder for each 30 m length or fraction thereof.  Ladder shall be extended from bottom of trench to atleast 1 metre above surface of the ground.  Sides of a trench 1.5 metres or more in depth shall be stepped back to give suitable slope or securely held by timber bracing, so as to avoid the danger of sides collapsing.  Excavated materials shall not be placed within 1.5 metres of the edge of trench or half the depth of trench, whichever is more.  Cutting shall be from top to bottom.  Under no circumstances shall undermining or undercutting be   done.

7.	Demolition: Before any demolition work is commenced and also during			the process of the work:

a.	All roads and open areas adjacent to the work site shall either be closed or suitably protected:

b.	No electric cable or apparatus  which is liable to be a source of danger over a cable or apparatus used by the operator shall remain electrically charged.

c.	All practical steps shall be taken to prevent danger to persons employed, from risk of fire or explosion, or flooding.  No floor, roof, or other part of a building shall be so overloaded with debris or materials as to render it unsafe.

8.	All necessary personal safety equipment as considered adequate by the Engineer-in-Charge shall be available for use of persons employed on the site and maintained in a condition suitable for immediate use and the Contractor shall take adequate steps to ensure proper use of equipment by those concerned.

a.	Workers employed on mixing asphaltic materials cement and lime mortars/ concrete shall be provided with protective footwear and protective gloves.

b.	Those engaged in handling any material which is injurious to the eye shall be provided with protective goggles.

c.	Those engaged in welding works shall be provided with welder’s protective eye-shields.

d.	Stone breaker shall be provided with protective goggles and protective clothing and seated at sufficiently safe intervals.

e.	When workers are employed in sewers and manholes which are in use, the

Contractor shall ensure that manhole covers are opened and manholes are ventilated atleast for an hour before workers are allowed to get into them.  Manholes so opened shall be cordoned off with suitable railing and provided with warning signals or boards to prevent accident to public.

The Contractor shall not employ men below the age of 18 and women on the work of painting with products containing lead in any form.  Whenever men above the age of 18 are employed on the work of lead painting, the following precautions shall be taken:

i.	No paint containing lead or lead products shall be used except in the form of paste or ready-made paint.

ii.	Suitable face masks shall be supplied for use by workers when paint is applied in the form of spray or a surface having lead paint is dry rubbed and scraped.

iii.	Overalls shall be supplied by the Contractor to workmen and adequate facilities shall be provided to enable working-painters to wash during on cessation of work.

9.	When work is done near any place where there is risk of drowning, all necessary equipment shall be provided and kept ready for use and all necessary steps taken for prompt rescue of any person in danger and adequate provision made for prompt first aid treatment of all injuries likely to be sustained during the course of the work.

10.	Use of hoisting machine and tackles including their attachments, anchorage and supports shall conform to the following:

a.	i. These shall be of good mechanical construction, sound material and adequate strength and free from defects and shall be kept in good working order.

ii.  Every rope used in hoisting or lowering materials or as a means of suspension shall be of durable quality and adequate strength, and free from defects.

b.	Every crane driver or hoisting appliance operator shall be properly qualified and no person under the age of 21 years shall be in charge of any hoisting machine including any scaffolding winch or give signals to operator.

- In case of every hoisting machine and of every chain, ring, hook, shackle, swivel and pulley block used in hoisting or lowering or as means of suspension, safe working load shall be ascertained by adequate means.  Every hoisting machine and all gear referred to above shall have the safe working load plainly marked there on, In case of a hoisting machine having a variable safe working load, each safe working load and the conditions under which it is applicable shall be clearly indicated.  No part of any machine or any gear referred to above in this paragraph shall be loaded beyond the safe working load except for the purpose of testing.

d.	Incase of departmental machine, safe working load shall be notified by the Engineer-in-Charge.  As regards contractor’s machine the Contractor shall notify safe working load of each machine to the Engineer-in-Charge whenever he brings it to site of work and get it verified by the Engineer-in-Charge.

11.	Motors, gearing, transmission, electric wiring and other dangerous parts of hoisting appliances shall be provided with efficient safeguards; hoisting appliances shall be provided with such means as will reduce to the minimum risk of accidental descent of load.  Adequate precautions shall be taken to reduce to the minimum risk of any part of a suspended load becoming accidentally  displaced.  When workers are employed on electrical installations which are already energized, insulating mats, working apparel such as gloves, sleeves and boots, as may be necessary shall be provided.  Workers shall not wear any rings, watches carry keys or other materials which are good conductors of electricity.

12.	All scaffolds, ladders and other safety devices mentioned or described herein shall be maintained in a safe condition and no scaffold, ladder or equipment shall be altered or removed while it is in use.  Adequate washing facilities shall be provided at or near the places of work.

13.	These safety provision shall be brought to the notice of all concerned by display on a notice board at a prominent place at the work spot.  Persons responsible for ensuring compliance with the Safety Code shall be named thereon by the Contractor.

14.	To ensure effective enforcement of the rules and regulations relating to safety precautions, arrangements made by the Contractor shall be open to inspection by the Engineer-in-Charge or his representatives and the Inspecting Officers as defined in the Contractor’s Labour Regulations.

15.	Notwithstanding the above conditions 1 to 14, the Contractor is not exempted from the operation of any other Act or Rule in force.

FORM OF REGISTER OF WORKMEN

(Regulation-7)

i.	Name and address of the Contractor-------------------------------------------------------------------------------------------------------------------------------

  - Number and date of the WORK ORDER & CONTRACT AGREEMENT -------------------------------------------------------------------------

iii.         Name and address of the department awarding the contract-------------------------------------------------------------------------------------------

iv.	Nature of the Contract and location of the work----------------------------------------------------------------------------------------------------------------

v.	Duration of the Contract--------------------------------------------------------------------------------------------------------------------------------------------

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Sl.	 Name and surname	Age &	Father’s /	Nature		Permanent /	Present		Date of		Date of	   	Signature	 Remarks

No	  of the workers		Sex	Husband’s	of emp-		Home address	 address     	commen-    	termina-     	or thumb

Name		loyment		of Employee    		 	cement       	tion or           	impression

Desig-		(Village,			of emp-       	leaving	    	of the

nation. 		Distt. Thana).		  	loyment       	of emp-          	employee.

loyment

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

1	            2		3		4		5			6		7	        8		9		10	         11

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

FORM OF EMPLOYMENT CARD

(Regulation-8)

i	Name and Sex of the Worker------------------------------------------------------------------------------------------------------------------------------------

ii	Father’s / Husband’s  Name ----------------------------------------------------------------------------------------------------------------------------------------

iii	Address         -----------------------------------------------------------------------------------------------------------------------------------------------------------

iv	Age  or Date of birth------------------------------------------------------------------------------------------------------------------------------------------------

v	Identification mark -------------------------------------------------------------------------------------------------------------------------------------------------

Particulars of next of kin (wife/husband and children, if any, or of dependent next of kin in case the worker has no wife/ husband or child):-

Name-----------------------------------------------------------------------------------------------

Full address of Dependents

(Specify Village, Dist., and State--------------------------------------------------------------------------------------------------------------------------------------------------------------------

---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Sl.    Name & Address of     Particulars of	Total period	Actual		Leave    	Nature of	Wage        Total      REMARKS   Signature

No   employer (specify         location of		for which the  	number  	taken (No.	Work done	rate	     Wage		 of the

Whether a contractor   worksite and	worker is	of days		of days		by the		With           earned		 Employee

or a sub contractor).    description of	employed	worked		should be	worker.		Parti-	     by the

work done		from………..			specified).			culars	     Worker

to……………)							of	     during

unit in         the

Case of      period

Piece	      shown

work.	      Under

Col.5.

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

1	            2		      3		         4	        	    5		      6	                 7	               8	        9		   10	      11

--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

N.B:- 	For a worker employed at one time on piece-work basis and at another on daily wages, relevant entries in respect of each type of employment

should be made separately.

FORM OF WAGE SLIP

(Regulation-9)

i.	Name of the Contractor ------------------------------------------------------------------------------

ii.	Place -----------------------------------------------------------------------------------------------------

--------------------------------------------------------------------------------------------

1.	Name of the worker with

father’s / husband’s name.

2.	Nature of employment.

3.	Wage period.

4.	Rate of Wages payable

5.	Total attendance / Unit of work done.

6.	Dates on which overtime worked

7.	Overtime Wages.

8.	Gross Wages payable.

9.	Total deductions (indicating

nature of deductions )

10.	Net wages payable.

---------------------------------------------------------------------------------------------------------------

Contractor’s Signature /						Employees’ Signature/

Thumb impression.						Thumb impression.

FORM OF REGISTER OF FINES

(Regulations No.10 vii)

| Sl. No. | Name | Father’s / Husband’s name | Sex | Department | Nature and date of the offence for which fine imposed | Whether workmen showed cause against fine or not, if so, enter date | Rate of wages | Date and amount of fine imposed | Date on which fine realised | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

FORM OF REGISTER OF WAGES-CUM-MUSTER ROLL

(Regulation – 9)

i.	Name and address of the Contractor-------------------------------------------------------------------------------------------------------------

ii.	No. & Date of the Contract Agreement /Work Order----------------------------------------------------------------------------------------

  - Name and address of the department awarding the Contract-----------------------------------------------------------------------------

  - Nature of the Contract and location of the work----------------------------------------------------------------------------------------------

  - Duration of the Contract-----------------------------------------------------------------------------------------------------------------------------

  - Wage period--------------------------------------------------------------------------------------------------------------------------------------------

FORM OF REGISTER OF DEDUCTIONS FOR DAMAGES OR LOSS CAUSED TO THE B.H.E.L.

BY THE NEGLECT OR DEFAULT OF THE EMPLOYED PERSONS

Regulation No 10 (vii)

| Sl.No | Name | Father’s / Husband’s Name | Sex | Department | Damage or loss caused with date | Whether worked showed cause against deduction if so, enter date | Date & amount of deduction imposed | Number of Instal- ment, if any | Date on which total amount realised | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |

EXTRACT FROM  N.B.O.

Blasting and storage of explosives.

APPENDIX A

GENERAL

  - The manufacture, storage, transport and use of explosives all be covered by a license as required  by the current rules and legislature in regard  to storage and handling of explosives, Instructions given hereunder wherever they are repugnant to the requirements laid down  by the  Central and the State  Governments, shall be deemed to have been superseded by these rules. All departments officials and the contractors in charge of the use and storage of explosives, shall acquaint  themselves with the current rules legislation issued by the Government, and ensure that they are strictly complied with.

  - No child under 16 years of age and no person who in a state of intoxication, shall  be employed on the loading, unloading or transport of explosives, or be employed in, or allowed to enter in premises where explosives are handled and / or stored

  - For issue of instructions on use of explosives, refer paragraph 3601 of the Indian Railways and Works Manual.

2. Carriage of explosive

2.1.	For detailed instructions about carriage of explosives, reference should be made  to the I.R.C.A. Red Tariff No. 18 for “Conveyance by rail, of explosive and other dangerous goods”.  Special attention is invited to the following sections in the Red Tariff : -

104.2		..	..	Powers to return to consignor

105.1		..	..	Pass required to accompany Consignments

106.1		..	..	Notice of dispatch compulsory

107.1.		..	..	Documents required

107.1 to 110.6 	..	..	Packing

111.1 to 111.4	..	..	Marking and leveling

115		..	..	Handling

115		..	..	Precautions

116		..	..	Storage

119 to 124 	..	..	Trains by which dispatch is permission

124		..	..	Types of wagons and quantities permitted

125 and 126	..	..	Carriage in brake vans, passenger compartments, etc.

  - The transport of explosives by rail, ship  or other public conveyance, is regulated by the rules framed  by Government on the subject. The civil authorities  should be  contacted to obtain the rules in force.

  - Every consignment to or from a “Storage Magazine” must be accompanied by a Guard and a responsible person experienced in the handling of explosives.

  - Detonators and explosives are not to be conveyed in the same consignment. Inflammable materials such as matches, kerosene etc., are not  to be carried in the same consignment with either explosives or detonators.

2.5	Before explosives  are removed  each case, barrel or package is to  be carefully examined to ascertain that it is properly closed and shown no sign of leakage.

Should any case,  barrel or package, appear to  be damaged or in a dangerous condition is to  be removed separately and with special care.

2.6	Should any case, barrel or package, appear to be damaged or in a dangerous condition it is to removed separately and with special care.

2.7. 	Cases containing dynamite should not be exposed to the sub, or allowed to get damp.

2.8.	Where the weight of the explosives transported in any vehicle exceeds 900 kg. They shall be  placed in the interior of the vehicles which shall be enclosed on all sides with wood or metal, so as to effectually protect the explosives from communication of fire and the vehicle shall be locked. When the  weight of explosives does not exceed 900 kg. And unless  they are carried in the manner laid down above, they must be completely covered with fire proofed cloth  , tarpaulins or any other  suitable material. So as to effectually protect explosives  from communication of fire.

2.9. 	Explosives should not be moved  during the thunder storm or dust storm.

  - No person except the drier should be allowed to travel on a vehicle conveying explosives, He shall not drive, conduct or maneuver the vehicle in a dangerous negligent manner.

2.11. No carriage of vessel shall be used for transporting explosives unless all iron or steel therein with which a package containing any explosives is likely to come in contact, is effectually covered with lead, leather, wood, cloth or other suitable material. No lights  should  be carried on the vehicle carrying the explosives.

2.12. No person is to be allowed  to smoke or even  to carry matches when accompanying  a consignment of explosives.

2.13. No operation connected with loading, unloading and handling of explosives shall be conducted between sunset and sunrise.

2.14. No explosives shall be brought to any place of loading until the carriage or vessel into which it is to be loaded, as at the place in readiness to receive it. A consignment of explosives should be  conveyed direct without delay  or unnecessary stoppage on the way.

2.15.	In moving explosives, the cases should be carried or passed from hand to hand, not thrown dragged or dropped. Barrels are to be carried, not rolled.

  - When explosives are being put into or taken out from a storage magazine the men should be divided into two gangs, one working inside the magazine and the other outside; no persons from either gang being allowed to cross threshold  while the work is going on .

2.17 	The maximum quantity  of explosives permitted to be carried by each consignment is fixed by the “Explosive Rules, 1940” as under -

  - 4500 kg. In any one Railway wagon

  - 2700 Kg. In any one carriage other than a railway wagon

  - 22,500 Kg. In any one boat

Custody and storage of explosives

3.1.      The explosives shall not be stored anywhere but in a special magazine to be built for the purpose. The site of the magazine shall be fixed  by the Engineer-in-charge in consultation with civil authorities according to the rules  in force. Explosives  must not be exposed  to the sub but must always be  kept under cover

  - Magazines may be considered as of two kinds viz., “Storage Magazines” and ‘expense Magazines”.  The former  may be in  any convenient position but the latter should be near the site of the work  on which the explosives are to be used.

  - A service supply shall be stored by the Supervisor in charge in an “express  magazine” to be built at site  according to the Government rules in force on the subject, and should if practicable be erected in such a position  that in the event of an explosion s neither the work nor the men employed on the same are likely to be affected, it must in no case be placed inside a tunnel mine or quarry.

  - For the site of a magazine , an isolated position  should be selected a  considerable distance form  any town, railway station, or place  of public resort ,  complying  with the table of  distance to be  kept  clear around  explosive  magazines, as per rules in force.

  - Under no circumstances should  magazine be erected  with 400 m  of any  working   kiln  of furnace,  and  similarly after a magazine has been constructed such  operations  as lime or charcoal burning could not be allowed  within 400 m of the magazine.

  - The site for a magazine should, if practicable, be so arranged  that in the event of explosion, any neighbouring  town, station or other place of public resort, would be sheltered by intervening high ground. Where the natural surface of the ground  is not shifted to this arrangement, a thick belt of tress may sometimes be utilised as a screen. An  artificial shelter may also be made by surrounding  the magazine, by a bank  of earth as high as the eaves of the magazine. When this  arrangement is adopted  care must be taken that proper  provision is made  for drainage, and the slopes of the bank  towards the magazine should be as  easy as practicable.

  - The space surrounding the magazine should be fenced in. The ground inside  this fence should be kept clear and free from tress, bushes, etc., the admission  to this fenced space should be by one gate only and no person should be allowed inside this fence, without permission of the officer in-charge. In the case of “Storage Magazine” the clear space between the fence and the Magazine should nowhere be les than 100m.

  - The door of magazine should be sheltered by a screen wall or other  erection so arranged as to protect the doorway from direct bullet fire. The door and its fastenings should be strong enough to resist forcible entry, and should preferably be a thick wooden one, faced on the outside with plate iron about 6 mm. Thick and   opening outwards. The window should be  of the same construction. All fittings and keys should be of brass or fun metal (not iron). The door should be fastened with two separate  locks, the key of one being kept by the guard and the key of  the other by the officer – in – charge.

  - The roof  of the magazine should be arched, and where considered desirable, may be made bomb proof  by thick  covering earth above  arch. Timber  should not be used in the construction of either the walls or the roof.

  - Two thoroughly efficient  lightning conductors should be provided  to the magazine, one at each end. The lightning conductors shall be tested at least once every year.

  - Ventilators should be provided, and so designed as to secure good ventilation, but not admit of the insertion of man’s arm or any article being passed through them from outside. They should be protected by a grating on the outside and be fitted inside with copper wire gauze.

  - Every magazine should be perfectly well-drained

  - No iron or steel is to be used in a magazine for nails, fastenings, fittings or any other purpose.

  - The  interior of the magazine, that is, the floor and walls, should be plastered  smooth with cement. This is preferable to  lining them with wood, which attacked  by insects and creates dust.

The magazine should at all times be kept scrupulously clean  High explosives like dynamite should be stored in a dry,  clean well – ventilated, bullet proof and the fire proof building, on an isolated site.

  - No one hall enter a magazine except  in the presence of the Officer-in-charge, with his permission. The person in charge of the magazine is to take care that the magazine is well and securely locked.

  - Magazine  shoes without nails should be kept at all time in the magazine, and a wood tub or cement trough about 30 cm, high and 45 cm. In diameter, filled with water should be fixed  near the door of the magazine.

Persons  entering the magazine must put on the magazine shoes provided for the purpose and be careful.

- Not to put their feet on the clean floor unless they have the magazine shoes on :

- Not to allow the magazine shoes to touch the ground outside the clean floor :

- Not to allow any dirt  or grit to fall on the clean floor

Persons with bare foot will, before entering the magazine, dip their feet in water, and then  	step direct from the tub over the barrier (if there is one ) on to the clean floor.

  - A brush or broom should be kept  in the lobby of the magazine for cleaning on each  occasion it is opened, for the receipt, delivery on inspection of explosive.

  - No matches  or inflammable materials should be allowed  in a magazine.

  - No person having articles of steel or iron  on him is to be allowed to enter the magazine.  Workmen or any other persons any admitted  into the magazine should not have on their person  any articles mentioned in paragraphs 3-18,3-19, and 3-0.

  - Oily  cotton rags, waste and articles liable to spontaneous ignition, should not be taken into the magazine.

  - No tools or implements other than those of copper, brass gunmetal or wood should  be allowed inside the magazine. Tools should  only be used with great gentleness  and care/

  - A  guard should be provided to every magazine for its safe  custody.  The strength of the guard should be settled by the Engineer in consultation with the civil authorities.

  - Detonators, for signals, fuses etc., are not to be kept in the same magazine with  gun-powder, dynamite  or other explosives. Detonators can however be kept in an annex adjoining the magazine provided that their number does  not exceed 25,000 and  that the annex is so constructed that not less than 60 cm of masonry and 100 cm of air space shall intervene between any detonators in such annex and the interior of the main magazine.

  - Barrels or cases containing  explosives are not to be opened  in a magazine. Explosives in open cases are not to be received into a magazine.

  - Explosives  which appear to be in a damaged  or  dangerous  condition are not to be kept in any magazine, but must  be removed without delay to a safe  distance and destroyed. Any necessary destruction of explosives such as might be required  owing to deterioration of the  explosives  or exudation of nitroglycerine, should be  carried out in accordance with the instructions  given by the Engineer in-charge.

  - Artificial  light is not to be allowed in any magazine. Electric storage battery torches may however be allowed.

  - No smoking shall be allowed within 100 m of a magazine.

  - In “Expense Magazines” the mallets, levers, wedges, etc., for opening barrels  or cases are to be of wood. Tools should only be used with great  gentleness and care.

  - Packing or unpacking, or any operation involving the moving of explosives among one another in an opened case, is not to be permitted in a magazine.

  - Barrels or cases to moved in a magazine are to be carried  by hand, not rolled, dragged, or turned over and over on the floor.

  - No person is to be allowed to sleep, to take food, or to  loiter in a magazine.

  - Should  repairs or alteration be necessary in a magazine every precaution should be taken, and if  practicable, the officer in-charge  should be present while the work is in progress.

  - Account should kept in every magazine of all explosive  received and issued.

  - Barrels and cases are to be properly labeled to indicate the nature of the contents, quantity, date of receipt, from whom received, and other particulars, so that their history can be traced.

  - Explosives which have been issued and  returned to the magazine are to be issued first ; otherwise those which have been longest in store are to be first issued.

  - The quantity of explosives allowed to be kept  in an “Expense magazine” should be limited, and a notification defining that limit should be  posted up in a conspicuous  position, both inside and outside the magazine.

  - Barrels and cases must be kept clear of the walls and floors so as to allow the air to circulate freely on all sides. It is recommended that where practicable, they should be stacked in the middle with a clear space all round.

  - Neatness and order in arrangements of a magazine are to be strictly  enforced. No dirt or rubbish is to be allowed, and special care is to be taken to keep the floor  free from grains of powder or portions of explosive matter fall on the floors due to leakage of cases, etc.

  - A brush or broom should  be kept in the magazine or in the  lobby if there is one, for  cleaning out the magazine on each occasions it is opened for the receipt, delivery or inspection of the explosives.

  - Magazine  should be opened occasionally in dry weather, so as to keep them well aired. When open they are to be carefully guarded.

  - The ventilators of a magazine are to be closed during a dust storm or thunder storm, and the magazine  is not  to be opened during the prevalence of an  atmospheric disturbance of this nature, not should any person be allowed in the vicinity of the magazine.

  - Should there be a difficulty in keeping  the magazines free from damp, freshly  burnt quick lime, exposed in wooden trays recommended to be kept in the magazines.

  - In every “storage magazine”  a maxima and minima thermometer should be put up  in a suitable position, and a note of the temperature recorded by each instrument should be made at regular intervals not exceeding a week. It is recommended that this be also  done in an “expense magazine”.

  - A record should be kept of all occasions on which a “storage magazine” is opened, with  such particulars as may appear desirable.

  - All magazines are to be officially inspected at definite intervals and a record kept of the results of such inspections.

  - At each inspection, the conditions of the lightning conductors should be carefully examined. The lightning conductors  have to be got tested once every year by an Inspector of Explosives in the manner prescribed by the Chief Inspector of Explosives, and a certificate showing the result and date of test obtained from him on payment of requisite fee, shall be  hung up in the magazine in a conspicuous place.

  - The rules in force in each magazine should be  printed in clear type  and hung up  in a conspicuous position both in and outside the building. The officer-in-charge of the magazine should be responsible to ensure that the rules are strictly observed.

  - When high explosives are stored, a sample should be taken atleast once a year and forwarded to the nearest chemical examiner for test. High explosives are those which are habitually fixed by detonation, and used where a destructive rather than a propellant effect is aimed at, and  include such explosives as :

Blasting gelatine, carbonite, celitite, dynamic, geignite, monober powder, phoenix 	powder, Negro powder, reburite, tenite ammonal, gun cotton slab, picric acid, tri-nitro-to-	vol (TNT), etc.

An adequate slot must be cut on the side of the plug to hold the fuse, and prevent it from damage when the plug is being  tapped down. The top of the plug should be painted red.

### Use of Explosives

4.1. The explosive shall not be used on the works or on the site by the contractors  without the written   permission of the Engineer-in-charge.

  - Blasting shall only be carried out at  certain specified times to be jointly  agreed to by the contractor and the Engineer-in-charge within the hours of day specified by the licensing authorities, and an area of 200 m radius from the firing point is to be specially flagged out, and all workmen excluded there from atleast  10 minutes before the hour for firing, a warning bell being sounded for the purpose.

  - Explosives shall not be brought  on to the works in quantities exceeding the amount enough for  the particular amount of firing  to be done  and any surplus explosive left, when all the holes have been charged, must be carefully removed at least 300 m away from the firing point.

  - A properly appointed agent on behalf of the contractor shall personally superintend the firing, and not  more than  eight holes  at a time shall be sent off ( fired successively and not simultaneous). Account shall be kept  of the blasts : if these do not tally with the number  fired, the difference indicates misfires, in which case  no person shall be allowed to return to the site of the firing for at least half an hour after firing, when the misfire shall be carefully looked  for by the authority’s  agent.  In the event of misfire, a portion of tamping may be sludged out with compressed air or water under pressure, but no kind of tool shall be used for this purpose. The hole shall thereafter be reprimed and fixed. If a reliving hole shall run parallel to the misfired hole. If a misfired is detected during  working hours and it is necessary to leave it charged  till firing time, a sludge of red paint must be cut and around the drill hole.

  - If misfire has been found to be due to defective fuse, detonators or explosives, special notice should be sent to the officer-in-charge of the works, and  to the persons in charge of the storage and “expense  magazines”. So that steps may be taken to inspect the whole quantity or box from which the defective article  was taken. No more of the defective stock is to be used without  permission of the Engineer-in  charge

  - As each hole is drilled and loaded, a small wooden plug (not more than 8 cm.in length). Is to be driven in at the top, confining the fuse and also helping to indicate the position of the hole if it misfires. An adequate slot must be cut on the side of the plug to hold  the fuse, and prevent it from damage  when the plug is being tapped down. The top  of the plug should be painted red.

  - Dynamite should be kept  dry  until used. If exposed to damp, the nitroglycerine exudes and becomes most dangerous. It shall not be exposed to direct rays of the sun, also it becomes highly dangerous if placed near fires, stoves, steam pipes or heated metal.

  - Frozen dynamite is a most dangerous substance, as simple  friction of the crystals of nitro-glycerine due to rough  or careless  handling, dropping a cartridge or any sudden  jolt ore jar is liable  to  cause detonation. None but the most experienced men should therefore handle it in a frozen state. It should be thawed by placing the cartridges in a pan floating in warm water.

        - (i)  A wooden  tamping rod should be used to push the cartridge home. No metal rod or rammer shall be used in tamping or brought to the site of   blasting work.

  - The charge  should not be rammed or pounded  but pressed firmly into place.

  - Dry earth should be used for tamping. If water is used, great care should be taken to press the cartridges into contact with each other, as water coming in between two cartridges will prevent the explosion of the lower one, and lead to a hold being only partially blown out  and remaining charged with the balance  of the dynamite.

  - The dynamite is exploded by means of a detonator pinched in to a fuse by means of special nippers, and put into the primer or cartridge. Instructions accompany each box and these should be carefully adhered to.

  - All fuses must be out to the lengths required before being inserted into the holes.

  - Any slackness, carelessness or the infringement of the rule will  render all responsible parties liable to prosecution.

  - Further detailed instructions for blasting  operations are given in standard specification

Rules for opening an closing boxes of dynamite and blasting gelatine.

  - Wooden wedges, about 15 cm. Long 3 cm, thick at the head and 8 cm. Wide, also a small wooden  mallet, weighting about 0.70 kg, and  a wooden  lever 60 cm, long x 5 cm. Wide x 2.5 cm, thick should  be provided as the tools for opening boxes.

  - The boxes should be opened on a wooden table or plank, free from grit  or metal nails.

  - To open the box, the  lid should be forced up by inserting two wooden wedges in the end, and driving them in by the wooden mallet. When raised sufficiently, the wooden lever should be inserted  and the lid prized off. After removing the lid, the water  proof lining at the joint  should be rent open, when the packets containing the cartridges will be accessible for taking out.

  - To re-close the box, all the brass nails should be first with drawn from the lid, the lid replaced in its proper position, and nailed with a wooden mallet.

  - Metal tools shall not under any circumstances be used for opening or re-closing : -

Account of explosives

    - To prevent explosives being disposed off unlawfully, following order must be strictly enforced :

- The superior in charge of the work shall personally responsible for all explosives, and shall personally attend to the charging, tamping and firing of all holes.

- The supervisor in charge shall maintain a register in which the explosives used  each day will be entered regularly the same evening, and copy of the entry shall be forwarded to the Engineer- in – Charge and IN ADDITION, A STATEMENT AT THE END OF EACH MONTH SHOULD BE PREPARED AND SENT TO THE ENGINEER-IN-CHARGE.

- The Engineer – in – Charge shall personally check the balance once a quarter

    - The Engineer-in-Charge shall be responsible for implicit compliance with the instructions herein given and with such as may emanate from Government from time to time, with regard to the storage and use of such explosives on the work.

    - All losses, shortage of stock and thefts of explosives, shall be reported without delay to the nearest Police Station.

Code of  practice for earth work on canals.

APPENDIX – B

TYPICAL METHODS OF DRILLING AND BLASTING ROCK IN  EXCAVATION

B-3. Blasting with powder

B – 3.1. In case of blasting with powder bore holes will be jumped  or drilled into the rock face. The direction and spacing of the holes are very important for efficiency and economy. The line  of least resistance ( that is the distance of the bulk  of the powder from the nearest rock face ) must never be in the direction of the hole bored. The depth of the bore hole should be about the same as the line of the least resistance

B-3.2.  The bore holes shall be dried  before being charged. The powder shall be enclosed in a waterproof cartridge and introduced into each bore hole by a funnel  or a copper tube. Safety  fuses shall  then be passed into the powder and taken outside to the required distance.

B-3.3.  A wadding of hay or dry turf shall be placed on the powder and around the fuse. An inch or two of the wadding shall be pressed down on the powder and the remainder of the hole shall be filled in with tamping materials, that is , dry clay. The filling material shall be rammed or tamped with a copper or brass  rod until it becomes  compact. Care shall be taken  to avoid  any possibility of an air hole around the fuse.

B-3.4.  The charge shall be fired by lighting the fuse. Fuses shall  be cut to the required length before being inserted into the bore holes. Joints in fuses shall be avoided. Where they are necessary a semicircular  niche shall be out in each fuse about one inch from the end and the two pieces  shall be superimposed and bound together with the string. All joints exposed  to damp  shall be wrapped with a rubber tape.

B-3.5The number of blasts to be fired and the actual number of shots heard  shall be compared   and person responsible shall satisfy himself by examination that all the blasts have exploded  before workers are permitted  to approach the same. The withdrawal of the charge which has  to exploded is under no circumstances to be permitted. The charge shall flooded  with water and the hole marked in a distinguishing manner. Another  hole shall be jumped  at a distance of about 50 cm. Of the old  hole and fired in the usual way.  The results shall be carefully examined   by the persons  in charge of blasting and the operations continued until the original blast is exploded.

### B-4. Blasting with dynamite

B-4.1. Dynamite is procurable, made up in cartridges of two kinds, one 22.2 mm. Diameter, 88.9 mm. Long, weighing 56.7 g, the other 25.4 mm. Diameter, 88.9 mm. Long, weighing 66.1g.

B-4.2. Dynamite is exploded by means of detonators. These are procurable in tin boxes and are laid in saw dust which should be blown out  with a dry blow of the mouth before using. One end (cut square ) of a Bickford’s fuse is pushed into the detonator, till  it touches the white  fulminate within it. The open end of the cap  is then pinched  in with pincers to attach it to the fuse, care being  taken not to break the powder cone of  the fuse by pinching too tightly. If the detonator is to be used to damp or wet places the junction should be made  watertight with grease, white lead or tar. A primer is then opened and  the cap gently pushed up  to its head into the dynamite, care being taken not to bury  the detonator entirely. The paper of the primer  is then closed up and securely bound with wire or twine to prevent displacement of the cap.

B-4.3. One cartridge is first placed in the bore  hole and pressed not rammed down with a wooden hammer, never with an iron  or metal rod, so as to form a proper contact with the sides of a rock, other cartridges are then added  as may be required to make up the necessary charge. The primer is then  gently pushed, not rammed down on the top of the charge. The space for about 20 cm. Above the charge is then  gently filled with dry clay pressed home and the rest of the tamping is formed of any convenient  material gently paced with  a wooden rammer.

B-4.4. Following general principles should be followed in case of blasting rock with dynamite . the mode of proceeding is very much the same as for powder, but  the holes of similar depth should be rather apart but will have smaller diameter. Continuity of circuit shall be checked with galvanometers so that  all the detonators are correctly connected before blasting.

B-4.5. The following are the diameters of drills used for different depth of drills used  for different depths of bore holes : -

Depth. 			Drill Diameter

m				mm

1 to 2 	..	..	..	25.4

2 to 3 	..	..	..	38.1 to 50.8

3 to 4.5 ..	..	..	50.8 to 63.5

B-4.6. The depth of bore  hole should be about the same as the line of least resistance and if possible  the bottom of the hole should never descend  below the face of the rock. The bore holes should generally be not more than 1.5 m deep and their distance apart should be from one and half to two times their depth.

B-4.6.1.  If the required charge  is so great that it cannot be held in a holes should be made close together, the total charge being slightly increased and exploded  simultaneously. By using delay detonators over cutting and undercutting can be minimized.

B-4.7. Cracks and fissures in the rock to be blasted should be carefully studied to ascertain the best  position for bore holes. The charge should always be placed in a sound piece of rock and if possible not nearer to a crack than 30 cm. If it is desired to shatter rock, close connections between the dynamite and the rock is essential and the points of contact should be  multiplied  as much as possible. For this reason several  bore  holes of moderate diameters are preferable to one hole of a larger diameter. Sometimes by having sloping holes s excavation  can be controlled in a particular direction.

B-4.8.  In case  of gently sloping rock with no face, dynamite should be used  as powder  is to be used, only with fewer and shall power bore holes. As the line of least resistance is not so important in dynamite as in powder the necessity for sloping  the holes is not so great. But, if a face is required on an almost level rock  sloping holes should be used.

Safety code for  blasting and related drilling operations

2.  Terminology-

2.0. For the purpose of this standard, the following definitions shall apply.

2.1. Blasting cap – A shell closed at one end and containing a charge  of detonating compound, which  is ignited from a spark it  is used for detonating explosives.

  - Detonator – The term shell include igniters, blasting caps, electric blasting caps, or other similar  devices  used to explode commercial  explosives.

  - Electric blasting cap – A shall containing a charge of detonating compound which  is ignited by an electric current from two projecting insulated leg wires.

  - Explosive  - Any mixture or chemical compound which is capable of producing  an explosion by its own energy. This includes black powder, dynamite, nitroglycerine compounds, fulminate, or explosive substance  having explosive  powder equal to or greater than black powder.

  - Magazine – Any building or other structure used for the storage of explosives.

  - Missed hole – A drilled hole containing an explosive charge  that failed to explode

  - Permanent blasting wires – Those wires between the firing switches ; for use in blasting where the power source  is an electric circuit.

  - Primer : An explosive cartridge with a detonator or igniting agent inserted therein.

  - Safety fuse : - A medium manufactured especially for firing  explosive charges, that conveys a flame at uniform  rate  rather than one that is in itself a detonator or one that operates on some other principle

  - Stemming – Means  material used for confirming a charge of  explosives in a hole or to cover explosives  in mud capping.

3.  Transportation of explosives

  - All the relevant central state and local laws and rules and regulations if any, framed thereunder shall be complied with loading, unloading and handling of explosives  will be supervised by qualified  personnel.

  - Where the magazine is located near the construction site and blasting operations continue daily, actual requirement of explosives shall be drawn from the magazine and transported to ,,,,the site.  Any left over should be returned back to the magazine each time after the blast.  In case of work at scattered places and for a small duration,  portable magazine shall be used and kept within a fence in a safe place and properly guarded.

  - For carrying small quantity (approximately 5 kg. Of explosives) specially designed insulated containers may be used.  These containers shall be constructed of finished wood not less than 5 cm thick or plastic not less than 6mm. Thick or pressed fiber not less than 10 mm. thick.  There shall be no metal parts (not even nails bolts, screws, etc.) and the container shall be water proof and provided with a lid.  The containers shall be provided with suitable non-conductive carrying device, such as rubber , leather or canvas handle or strap.

  - Vehicles  to be used for  transporting explosives shall be in good working condition and shall have  a tight wooden or non-sparking metal ( copper, brass and the like ) floor with sides and ends high enough to prevent the explosives from falling off the vehicle. In open – bodied vehicles, the explosives shall be covered with a water proof and fire-resistant tarpaulin.

  - Electrical wiring in vehicle shall  be fully insulated so as to prevent the danger of short circuiting and atleast two fire extinguishers of  carbon tetra choloride type shall be carried. The vehicle shall be  properly  marked so as to give adequate warning to the public in regard to the nature of cargo.

  - No metals  except approved metal truck bodies shall be allowed  to come in contact  with          cases of  explosives.  Metal ,flammable or corrosive substances shall not be transported with explosives, as far as possible, transportation of any other material along with explosives shall be prohibited.

  - Smoking shall be prohibited in the vehicle carrying explosives.

  - No un authorised person shall be allowed in the vehicle carrying explosives.

  - Loading and unloading of explosives shall be done carefully.

  - Explosives and detonators or blasting caps should not be permitted to be transported in the same vehicles.

  - Detonators and to the site of work in the original containers or in securely locked separate non-metallic container and shall not be carried loose or mixed with other materials.

- Storage of explosives-

  - Storage of explosives is registered by the Indian Explosives act and provision thereunder should be strictly observed.

  - Explosives shall be stored only in a magazine which is clean dry, well ventilated, reasonably cool, correctly located, substantially constructed, bullet and fire resistant and securely locked.

  - Blasting caps, electric blasting caps or primers shall not be stored in the same box, container or room with other explosives.

  - Explosives, fuse or fuse lighters shall not be stored in a damp or wet place or near oil, gasoline, cleaning solutions or solvents or near radiators, steam pipes, or other sources of heat.

  - Smoking  shall not be permitted within the fencing around  the explosive magazine, nor matches, open lights, or other fire or flame shall be allowed near the magazine. Warning notice to this effect should be pasted at conspicuous places.

  - No leaves, grass, brush or debris of any kind  shall be allowed to accumulate  within 8m of an explosive magazine.

  - No sparking metal or sparking metal tools  shall be stored in an explosives magazine. Persons entering the magazine shall not have shoes with iron nails or other sparking metal.

  - If nitroglycerine from  deteriorated explosives has leaked down on to the floor of the explosive magazine, the floor shall be desensitized by washing thoroughly with an agent obtained before hand from the supplier of the explosives.  For this purpose, desensitizing agents and the instructions for using them shall always be obtained along with the supply of nitroglycerine.

5.  Use of explosives

  - Precautions against lightning  shall be provided in accordance  with Indian Electricity Act and India Explosives  Act and Rules  and regulations framed thereunder.

  - Any package containing explosives shall not be dragged dropped or handled roughly. The explosives packages shall be opened only at a safe distance  and properly shielded from the packages of explosives in bulk  storage.

  - Sparking metal tools  shall not be used to open legs or wooden or fiber  board cases of explosives.

  - Smoking shall not be permitted nor matches, open lights, fire, flame, or any other device capable of producing sparks or flame shall be carried while handling  or using  explosives

  - Explosives shall not be placed where they may be exposed to flame excessive heat, sparks or impact.

  - The covers of the explosive cases or packages shall be replaced every time soon after taking out part of the contents as long as any explosives are left in them

  - Explosives  shall not be carried in the pockets of any clothing or elsewhere on any person.

  - Primers shall not be made up in a magazine, or near excessive quantity of explosives, or in excess of immediate needs

  - Nothing shall be inserted in the open end of a blasting cap except fuses.

  - No person shall strike, tamper with, or attempt to remove of investigate the contents of a blasting cap or an electric blasting cap or attempt to pull out the crimped  safety fuse cut of a blasting cap.

  - Children, unauthorised or unnecessary persons shall not be present where explosives are being handled or used.

  - No person shall handle, user or be near explosives  during the approach or progress of any electrical storm.  All persons shall retire to a place of safety.

  - Deteriorated or damaged explosive or blasting equipment shall not be used, but shall be disposed of as stated in 11.

  - No attempt shall be made to reclaim or use fuses, blasting caps, electric blasting caps or any other explosives which have been water soaked, even if they have been dried out.  The manufacturers shall be consulted.

  - No. attempt shall be made to often hard set explosives by heating over a fire or by rolling the explosive on the ground.

6. Drilling and loading

6.1. Before planning out the drilling operations for blasting purposes, nature  of stratum and the over burden should necessarily  be examined to avoid possibilities of land sides after blasting.

6.2 The face of rock shall be carefully examined before drilling to determine the possible  presence  of unfired explosive. No attempt shall be made  to drill at a site if un detonated  explosives are suspected and action shall be taken according  to 10.4.

6.3. The bore hole shall be carefully checked for length, presence of water, dust, etc., with a wooden tamping pole or a measuring tape before loading

6.4. Surplus explosives shall not be stacked near working area during loading.

6.5. The line of detonating fuse extending into a bore hole shall be cut from cut from the spool before loading the remainder of the charge.

  - A bore hole shall not be loaded with explosives after springing (enlarging the hole with explosives) or upon completion of drilling without making sure that it is cool and that it does not contain my hot metal, burning or smoldering material.  Temperature in excess of 65  C. are dangerous.

  - A bore o near another hole loaded with explosives shall not be sprung.

  - No force, shall be used  for inserting cartridges or any explosive into a bore  or pass any obstruction in a bore hole.

  - No force shall be used for inserting a blasting cap or an electric blasting cap into explosive. The cap shall be inserted into a hold made with a pricket designed for the purpose.  A hitch of the electric  blasting cap leading wire shall be made on the primer cartridge so as to prevent pulling out of the electric blasting cap from the explosive charge. In case of fuse, the fuse shall be tied to the explosive cartridge so that the blasting cap is not pulled out. Care shall be taken so that the electric blasting cap, leading wire  or the  length the fuse does not get damaged during loading  of the charge.

  - No attempt shall be made to slit, drop, deform or abuse the primer

  - Blasting  caps or electric blasting caps shall not be connected to detonating  fuse          except non-sparking metal connectors for jointed poles shall be used. Violent tamping shall be avoided. Primer  shall not be tamped.

  - Care shall  be taken to confine the explosives in the bore hole with sand, earth clay or other suitable combustible steaming material.

  - Kinking or injuring  or fuse or electric blasting cap wires shall be avoided when tamping

7.  Electrical short firing  Circuit

7.1.  In deciding  the sizes of wires , fuses,  circuits, blasting , switches , etc.  instructions   issued   by  the  manufacturers of  these  articles  should be consulted.

7.2.No personal shall  tempt to uncoil the wires  and open  out  the short  circuited bare  leading  wires of the electric  blasting  cab  during  approach  of dust storm or near any source of  large charge of static electricity  or near a radio  transmitter. The manufacturer  of the cap or the Inspectorate of Explosives shall be consulted regarding the distance from the  transmitter, beyond which electric short  firing shall be conducted.

7.3 Firing circuit shall be kept completely insulted from the ground or other  conductors, such as bare wires, rails, pipes, or other paths or stray current.

  - There shall not be any electric live wires or cables of any  kind near electric blasting  caps or other explosives except at the  time and for the purpose of firing the  blast.

  - All electric blasting caps shall be tested singly and also when connected  in a circuit  in series using only an approved type of circuit continuity tester or ohm meter.

  - No attempt shall be made  to use in the same circuit  either electrical blasting caps made by more than one manufacturer or electric blasting caps of different design of function even if made by the same manufacturers unless such use is approved by the manufacturers.

  - No attempt  shall be made  to fire  a circuit  of electric blasting caps with less than the minimum cu rent specified by the manufacturer of that electric basing cap.

  - Care shall be taken to ensure that all wire ends to be connected are bright and clean.

  - The electric cap  wires or leading wires shall be kept short  circuited until ready to fire.

  - When energy for blasting is taken from power  circuits, the  voltage shall not exceed 220. the wiring and controlling  arrangements shall conform to the following :

- The blasting switch  shall be strictly according  to the specifications, externally operated  double – pole  double throw switch, which when locked in the open position will short circuit and ground the leading wires The switch shall be installed at the location where the firing  is to be controlled

- A ‘Safety’  switch  of the same  type as the blasting switch shall be installed  between the blasting switch and the firing  circuit and lead lines, at a distance not to exceed 180cm ., from the blasting switch.

- Both the safety switch and the blasting switch shall be locked in the open  position immediately after firing the short and before any person  is permitted to return to the blasting  area. Key to the switches shall remain in the possession of the blaster at all times.

  - Rubber covered or other  adequately  insulated copper wires in good condition  shall be used for firing  lines and shall have solid  cores of appropriate gauge. Sufficient firing line shall be provided to permit the blaster to be located at a safe distance from the blast. Single conductor lead lines shall be used.

  - Blasting operations  in the proximity of overhead power lines, communication lines, utility lines, or other structures shall not be carried  on until the operator or the owner, or both of such lines has been notified  and  precautionary  measures deemed necessary have been taken.

  - All holes loaded on a shift shall be fired on the same shift.

  - As far as possible blasting shall be carried  out using suitable exploder with 25 per cent excess capacity. Electric power from the mains shall be used only when it is absolutely necessary.

8.  Short  firing with  safety fuse :

  - The fuse shall be  carefully handled  to avoid damaging the covering. In very cold weather the fuse  shall be slightly warmed before using so as to avoid cracking the waterproofing.

  - Short fuse shall not be used. The length  of a fuse shall be not less than 120 cm. The rate of burning  of the fuse shall be known and it would be necessary to make sure that it will take sufficient time in burning so as to enable all persons to reach a place of safety. The burning rate of the fuse shall not be more than 60 cm / min.

  - The fuse shall not be cut until the operation to insert the fuse  into a blasting cap is ready. The fuse shall be cut off about 2.5 to 5 cm to ensure a dry end. It shall be cut squarely across with a clean sharp  blade. The fuse shall be seated lightly  against the cap  charge and care shall be taken to avoid twisting after it has been placed in position.

  - Blasting caps shall not be crimped by any means  except by a cap cimper designed for the purpose.  It shall be necessary to make sure that the cap is squarely crimped to the face.

  - The fuse shall be lighted with a fuse  lighter designed for  purpose. If a match is used, the fuse shall be slit at the end and the match head held in the slit against the powder core  and then the match head rubbed against an abrasive surface to light the fuse.

  - The fuse shall not be lighted   until sufficient stemming has been placed over the explosives to prevent sparks of live match heads from coming into contact with the explosives.

  - The  explosives shall not be held in hands when lighting the fuse.

9.  Underground work :

9.1. Only permissible  explosives  and in the manner  as specified  by the appropriate authority shall be used

9.2. Excessive quantities of explosives shall not be taken underground at any time. Black blasting  powder or  pellot powder  shall not be used with any other explosives in the same bore hole.

9.3. For blasting in tunnels and shafts precautions details in relevant Indian standard shall be followed. The poisonous gases shall be promptly removed by using exhaust fans in shafts and reversible axial flow fans in tunnels. The air duct shall be of such size as not to allow too much pressure drop so that the velocity  of air at the delivery end is not less than 0.2 m/s.

- Before and after firing

10.1. Before firing, sufficient warning shall be given to enable the people working in the blasting area to get off the danger zone. The danger zone shall be suitably cordoned off and flag men  posted at important points.

10.2 No loose materials, such as tools, drilling implements, etc., shall be left on the rock surfaces to be blasted.

10.3 Blasting in the  open shall be carried out during the fixed hours every day or on fixed days in the week. This information shall be  amply publicized and the following  precautions observed: -

- On the projects sites, where regular blasting operations are carried our, daily  blasting hours  should be clearly printed  on Sign-boards on all the roads approaching that area.

- Road closing barriers should be provided to close the traffic on these roads, at least 400 metre away when the firing is to take place.

- The beginning of the firing should follow loud sirens and similarly the completion of the firing should  be succeeded by loud sirens.

  - The bore holes shall be thoroughly cleaned before  a cartridge  is inserted. Wooden tamping rods (not pointed, but cylindrical throughout) shall be used in  charging the holes. The cartridges should be gently  placed and not rammed. The primer cartridge  will on the top .

  - The shot firer shall not return to the blasting site after firing, until at least 5 min, have elapsed.  In case of electric shot  firing, the shot holes shall be examined after firing and in case of misfire no person shall be allowed to approach the blasting for  atleast 5 min. In case of shot firing with safety  fuse utmost care  shall be taken to count the number of loud reports to  ensure that all the shots have fired and in the event of misfire, no person shall be allowed to approach the blasting site for atleast 30 minutes. In any case,  careful inspection for remaining of un detonated explosives shall be made after firing the shots.  All misfired  shot holes shall be cross marked. No other person than those duly authorized shall  be made after firing the shots. All misfired shot holes  shall be cross marked. No other person than these duly authorized shall approach the holes until one of the following operations  has been performed in respect of each of the misfired  holes ;

- If the misfire  is due to a faulty cable or  faulty electrical connection the defect shall be  remedied and shot  fired.

- The stemming shall be floated out of  use of water  or air jet from those  until the hole  has been opened to within 60 cm. Of the charge, whereupon water  will be siphoned or pumped out, then a fresh new charge placed and duly detonated.

- A new hole shall be drilled 60 cm. Away from the old bore and parallel to it and about 30 cm. Less in depth and the new hole charged and duly fired.

  - A careful  search shall be made of an exploded material in the debris of the second charge

NOTE :  If a shift change is unavoidable, the person in charge of one shift before leaving the work  shall inform the person relieving him for the next shift of any cases misfired and shall point out their  positions duly cross marked and also state clearly what action has to be taken  in the matter.

11.  Explosives disposal :

  - No explosive shall be abandoned. They shall be disposed off destroyed strictly in accordance with the approved methods and in doing  so the manufacturers or the appropriate authority shall  be consulted.

  - Explosives, caps, boxes lines  or material used in packing of explosives shall not be left lying  around in places to which children or un authorised  persons or livestock can have access.

  - No paper or fiber  material employed in packing explosives shall be put to any subsequent use.  Such material shall be destroyed by burning in the presence of a responsible person.

SCHEDULE B

SCHEDULE OF CONDITIONS OF CONTRACT

  - Works referred to : - The measurement entered in detail in Schedule  A show  the area over which the clearance of prickly pear bushes, trees, etc., has to be done  they shall be carefully followed :

  - Works and specification : - The work shall be carried out to the satisfaction of the Executive Engineer, Division ( hereinafter called the Executive Engineer) in accordance with his directions and in compliance with the said measurements, specification and written instruction in explanation of the same as may from time to time be given  by the Executing Engineer or his authorized representative at the spot.

  - Power of Executing Engineer to order removal of improper work : In case of default  on the part of the contractor to carryout  the work in accordance with the specifications, the Executing Engineer shall have power to employ and pay other persons to carry out the same at the contractor’s  risk and all expenses  consequent thereon or incidental thereto shall be borne  by the contractor and shall be recoverable from him by the Government of Punjab (hereinafter called the Government ) or may be deducted  by the Government from the security deposits mentioned in clause 8 below  or from any moneys due or that may become due to the contractor.

  - Defects after completion : - Any defects or other faults which may appear within one month after completion of the work shall upon the directions in writing of the Executive  Engineer, be amended and made good by the contractor at own cost,  failing which Executing Engineer shall pay him such reduced amount as he think fit or deal with him as provided for under clause 8.  The final payment for the work shall therefore be deferred  for such period as herein mentioned after the date of completion.

  - Date of commencement and completion : - The contractor shall forthwith commence work and complete the same within a period of calendar  month after the date of commencement subject  nevertheless to the provision  or extension of time hereinafter be considered as of  essence of this agreement.

  - Extension of time : - If in the opinion of the Executing Engineer the works shall have been delayed  on account of any reasonable cause, the Executing Engineer may grant in writing a fair extension of time for completion.

  - Payment and Certificates : - the Executing Engineer will cause only one final payment to be made to the contractor if the total amount of the contract is Rs. 300 and less. But if the contract amount exceeds Rs. 300 he may cause intermediate  payments to be made upon certificates to be issued at reasonable frequent intervals, either by himself or by an Officer of rank not less than  that of an Assistant Executing Engineer to the extent of 75 per cent will be dealt with as

provided below. The contractor when applying for a certificate shall  furnish to the Executive  Engineer a statement of measurements  of the works executed based on the original detailed  estimate of quantities include in Schedule A together with a bill.

In calculating  the amount of each item due to the contractor in every bill  submitted  for payment  under the contract, sums of less  than 2 ½   percent shall be omitted and sum of 2 ½ percent and more but not exceeding 5 per cent shall be reckoned as 5 per cent. The amount shall be rounded to the nearest rupees, sums of less than 50 paise being omitted and sums of 50 paise and more being reckoned One rupee.

  - Security deposits : - In cases where the contract amount exceeds Rs. 300 the 25 per cent of the value of the executed work withhold from intermediate bills under clause 7 above, will be treated as further security in addition to the security  deposit  mentioned in the agreement. These security  deposits  or so much thereof as may be returnable will be refunded after the expiration of the period fixed in clause 4 above. If the contractor fails to start work  forthwith or to executive the work within the period fixed in clause 5 or within such extended time as may  be allowed under Clause 6 or if the contractors shall be guilty  of a breach or non – observance of any of the conditions herein contained and to be performed or observed by him it shall be lawful for the Executive Engineer, in addition and without prejudice to the other rights and remedies of the Government, to determine the contract and such  determination shall carry with it the forfeiture of the security deposits mentioned above together with the value of a such work  as may have been executed and not paid for  or such portion of such  sums as may be decided by the Executive Engineer.

TYPICAL  MARKING  DETAILS FOR WALLS

  - For excavation in loose solid I.S. 3764  shall be followed

  - Where different foundation levels are necessary I.S. 1904  may be followed.

  - If, by the contractor’s mistake the excavation is made deeper or wider than shown on the plans, the extra width or depth shall be made  up with concrete – and not with earth or other material – at the cost of contractor.

- Should rocks, or remains of old buildings, be met with, the material shall be removed with wedges and levers. Blasting  will not be allowed, without the permission in writing  of the Executive Engineer. In cases  where the Execute Engineer considers blasting is essential, the extra rate, if any to be paid  shall be fixed in writing  before the work is put in hand.

- If bad ground or loose soil be met with, the contractor, will be responsible for reporting the fact to the Executing Engineer who will issue  such order as may be necessary. For extra excavation, concrete and masonry arising from bad ground, the contractors shall be paid extra  according to the schedule of rates  agreed to.

- Before concrete  is laid in the trenches, they shall be passed  by the Executive Engineer.

- All excavated earth, which is unfit or surplus to requirements for filling in, shall be spread as instructed in the standard specification for “Filling in foundation” at the contractor’s expense. If the excavated  earth is to be removed beyond  50 metres, then the rte in the agreement for extra lead will be paid or in the absence of such rate, the contractor will be paid  in accordance with the schedule of rates of the Division in which in the work is being executed. If the cartage is cheaper than head loads, then cartage shall be done and all rate shall be fixed in writing by the Executing Engineer before cartage is begun.

- The contract rate for excavation of foundation shall include the following :

    - Setting out works, profiles, etc.

    - Site clearance, preparation of bed

    - Forming deadman  or thandoos and their removal after

    - Forming (or leaving ) steps inside of deep excavation and their removal

after measurements.

    - Excavation for insertion of planking and strutting

    - Unless otherwise specified,  removing slips or falls in excavation.

    - Bailing out water on excavation from rains, sub-soil water, etc.

    - Shifting  or supporting  pipes, electric cables, etc., met during excavation.

NOTE : pumping out water caused by springs, tidal or river seepage broken water mains or drains, well point pumping and the like shall be measured in kilolitres of water and paid for separately.

- Shoring, sheeting, planking, strutting, etc., needed for  supporting sides of excavation shall be paid  for separately unless otherwise specified in the description of relevant  schedule items and in addendum Extra rate for work in or under water, in or under foul conditions , under tides and in snow shall be paid for unless  otherwise specified in the description of relevant schedule  items and in addendum specifications. The levels of high and low water tides where occurring shall stated in the tender schedule :

- Classifications of soils for earth work I.S. 1200 Part I shall apply.

NOTES : 1. In all such cases, the classification as made by the Executing Engineer shall apply and the contractor shall make the excavation under the most  favourable conditions, when so instructed  by the Executive Engineer. No revision of classification will be made by the Executive Engineer.

  - When rock is to be excavated by blasting, it will always be specified before work starts. In no case is blasting to be done  before a rate has been agreed on and the approval  of the Executing Engineer to such action obtained.

  - Contract rate for any classification of soil assumes that wedges, levers, picks, mamooties and manual tools  only are to be used in excavation and in no case will the contractor be allowed to obtain a revision of classification  on the ground that blasting is necessary unless such necessity  is approved by the Executing Engineer prior to commencement of blasting. In lump sum  contracts, the contractor assumes the risk of meeting with hard rock, requiring removal without extra compensation unless separate provision for  such contingency is specifically entered in the conditions of contract.

At places of foundation where existence of colonies of subterranean  termites are found or where the structure is likely to be attacked frequently chemical barriers which prevent the termite from reaching the superstructure of the building shall be introduced. The soil  treatment for anti-termite measures in buildings i.e. in basement excavation, column  pits and wall trenches shall be as per I.S. 6313     (Part II) .

.

- Protection during  construction – Attention is directed to the  relevant clauses  of the standard general  conditions of contract.

- Rate : - In the case of lump sum contracts, the work will be executed for a definite contract amount but  completed according to the plans and specifications.

  - Tools and plant shall be supplied by the Contractor at his  own expense. Both for dredging and dewatering where such is required – Unless other arrangements are definitely prescribed  in the relevant specifications or tender notice. The contractor shall quote his tender rate accordingly. The contractor’s attention is directed in this connection to the relevant clauses  in the standard general conditions of contract, but it is to be understood that Executive  Engineer does not bind  himself  to supply any tools and plant on hire.

  - If the schedule prescribes payment for  separate items of the well – then

- Earth work for open excavation will be paid  only for its quantity actually to be excavated taking  into account sloping back as mentioned in the relevant  specification for earth work in foundation. The excavation made  in excess of the above requirements shall not be measured  and paid for not withstanding that  the contractor may find  it more  convenient to take out the excavations at slopes other than those shown  in the plan  to avoid shoring. In every case the contractor shall make excavations perfectly safe at all stages, for the workmen, for whose  safety he shall be wholly  responsible

- Curbs  will be paid  for per curb  or per 0.01 cubic metre as indicated  in the relevant schedule  item.

- Masonry shall be paid for on the quantities  as shown  in the schedule or deduced form the plan

EXTRACT FROM  I.S. 4701

Code of practice for earthwork on canals.

4.  Clearing : -

4.1. The land   over which  embankments are  to be formed and other excavation is to be carried out shall be cleared of all trees, bushes, rubbish, ant hills and other objectionable matter.

  - It is desirable  to protect the trees outside the outer edge of the canal embankments. However, the presence  of trees in the vicinity of a canal can accentuate  variation of moisture  content  in the substratum. In the case of expansive soils such  excessive moisture variation  can result in damage to the lining. Such influence is believed  to extend  to distances equal to twice the height  of the tree

SECTION IV

CONCRETE

INDIAN STANDARDS EXTRACTS

| I.S. Number (1) | (2) |
| --- | --- |
|  | Notes on R.C.C. design |
| 2541 | Code of practice for use of lime concrete is buildings |
| 269 | Specification for ordinary, rapid-hardening and low heat Portland cement |
| 1791 | Specification for batch type concrete mixers |
| 3370 | Code of practice for concrete structures for storage of liquids pre stressed concrete structures |
| 432 1139 | Specification for mild steel and medium tensile steel bars and hard drawn steel write for concrete reinforcement – Part I – Mild steel and medium tensile steel bars Part II – Hard drawn steel wire |
| 456 | Code of practice for plain and reinforced concrete (II revision) |
| 5525 | Recommendations for details of reinforcement is reinforced concrete |
| 2751 | Code of practice for welding of mild steel bars used for reinforced concrete construction |
| 1199 | Methods of sampling and analysis of concrete |
| 2502 | Code of practice for bending and fixing of bars for concrete reinforcements |
| 3370 | Code of practice for concrete structures for the storage of liquids Part I – General requirements Part II – Reinforced concrete structures |
| 1346 | Code of practice for water proofing of roofs with bitumen felts |
| 4365 | Code of practice for application of bitumen mastic for water proofing of roofs |
| C.B.R.I. | Cored Unit for Roof / Floor – Data sheet No.3 |
|  | Channel unit for floor / roof – Data sheet No.5 |
|  | Pre cast R.C. Plank C.B.R.I. flooring roofing – Data sheet No.7 |
|  | Waffle unit for roof / floor – Data sheet No. 6 |
| 2911(Part III) | Code of practice for design and construction of pile foundations under-reamed pile foundation. |

(i) 	Slump test – The slump, is the amount of settlement in cms. Obtained after  filling with concrete a standard metal cone form, of 10 cm. Top diameter, 20 cm. Bottom diameter, and 30 cm. High and then removing the form. The filling should  be done in layer  approximately 7.5 cm high, each layer being punned 25 times with a 16 mm in diameter rod, bullet pointed at one end. The less the slump and consequently the stiffer the mixer provided that it is workable, the stronger will be the concrete, other conditions being equal. The smallest slump that will give a consistency suitable for the work in hand should be used. The recommended values for the  slumps are :

|  |  | When vibrator used | Not used |
| --- | --- | --- | --- |
| 1. | Mass concrete in foundations footings, retaining walls and pavements | 10 mm to 25 mm | 30 mm to 75 mm |
| 2. | Thin flooring of less than 75 mm thickness | 25 mm to 40 mm | 75 mm to 100mm |
| 3. | Mass concrete in R.C.C. foundation footings and retaining walls | 10mm to 25mm | 80mm |
| 4. | Beams slabs and columns simply reinforced | 25 mm to 40 mm | 100mm to 125 mm |
| 5. | Thin R.C.C. section or Section with congested steel | 40mm to 50 mm | 125 mm to 150 mm |

- In the case of pipes or similar articles where the space to  be filled is narrow and confined, a concrete  that will flow readily has to be used, but care must be taken to eliminate the  entrapped air bubbles.

- Some experiments  indicate that every ½ Kg. Weight of excess water used has the same effect on strength , as to the omission two kilograms (or more) of cement.

- Excess of water not  only causes loss of strength, but also involves increased shrinkage and  liability to cracking.

- A working mean to get best strength and workability has to be adopted to enable the use of the direst possible concrete that can be worked successfully around and between the steel bars so as to thoroughly  envelop  all of them, and leave no voids and hollows.  With insufficient water, the chemical action in setting will be adversely affected.

- The diagram appended is illustrative of the effect of water on the strength of concrete.

- Mix design : -

  - For works in which water tightness is required the specifications laid in I.S. 3370 I to II shall apply.

- Forms and centering – (a) A constructional load of 400 Kg / m2  in addition to the dead  weight  of the concrete may be assumed in designing the centering. The forms shall be carefully  designed and the  design shall be subject  to the approval of the Executive Engineer, who may require  the contractor to alter the designs in accordance with instructions, or adopt  designs prepared by him  without extra cost. If wooden centering are adopted only well seasoned timber should be used in their construction. It is better  use too thick, than too thin timber  as it is false economy to use light forms, which will not lend itself for frequent removal and re erection. Under no circumstances  should green  timber be used, as it is liable to shrink  after erection.

  - If the forms are held together by bolts or wires  they shall be so fixed that no iron will be left  exposed on the face of  the finished work.

  - All centering and supports shall be properly braced and cross – braced in two directions and well  stayed  or strutted on all sides rigidly enough  to resist storm, or high winds during execution. The forms should be strong enough to withstand the weight  or pressure when  a considerable height of wet concrete is being poured, as in  walls or columns. Many authorities suggest that pressure may be taken as caused by a liquid of  half the weight  of concrete, namely 1,200 Kg / m3

  - For joints and  gaps, paragraph 20 : 2-1 of I.S. 456   shall apply

  - To ensure that all faces of concrete  which will be exposed to view on completion of the work have smooth finish, beveled edge timber ( tongued and grooved, if so specified or ordered ) shall be used, giving smooth tight joints.

  - In special cases, the surfaces that are to come in contact with concrete should be planed flat and smooth and be free from know holes  and other imperfections. Oiling or greasing of forms shall not be done if the concrete  is to receive a coat  of plaster after removal of the forms. I.S. 456  shall also apply

  - Old forms, or forms left long exposed to weather should be examined carefully before re-use.

  - The centering should be so  designed and arranged that  the sides of columns  can be first removed, then the sides of beams, then boarding under the floor slabs and lastly the soffits of beams

  - Where the centering posts rest on soft ground, the load should be got  distributed by means of sleepers or short piles; otherwise the supports may punch into the ground, causing a sag and settlement and possibly cracks in beams and floor  slabs. All uprights should be provided with a double set of wedges or sand boxes, so that they may be gently removed. The section of the scantling from which the wedges are cut should  be at least 10 cm by 10 cm.

  - When the concrete is placed in layers’ no calculation necessary as it has been found in practice that for beams, the bottom boards should be 5 cm to 6 cm thick with sides 4 cm to 5 cm thick, for columns  sides should be 4cm to 5cm thick and for walls 4 cm boards are generally used. The thickness  can be  varied according to the spacing of the clamps or braces but on this account the material should be made too thin.  For slab panels 2.5 cm . board is generally used which require staying every 60 cm. 4 cm. Boarding will require  staying every 90 cm. And 5 cm. Boarding every 1.2 m to 1.5 m. The studs i.e., the vertical pieces on which the sheathing or planking is nailed, should be of such a size and be so spaced as to prevent the boards between them from springing.

  - For square columns three sides of the form should be bolted or clamped together and the fourth  side left open and braced at intervals sufficient  to keep the box or casing in shape. This open side should  be gradually built up  a little advance of the concrete laying. For this purpose, strong timbers fixed with wedges behind vertical  runners should be used.

  - Other  shapes of  columns  must all be filled from an open side in the form to allow  the steel to be kept in its proper position, the structure  to be gradually  built up and the concrete properly and thoroughly consolidated.

  - A completely enclosed form should not be used as it will necessitate filling  in of concrete   from a considerable height and prevent the casting of concrete  from being properly inspected or compacted.

- Laying concrete – (A)  General (i)  Paragraph 20-21 of I.S. 456  shall apply.

  - Usually it will be found preferable to completely fix the reinforcement for the whole floor, before beginning  to place concrete.

  - Suitable wooden plugs may be placed in the centering for electric fittings. Similarly clamps for hanging  ceiling fans or sewing  planks should also be fixed in their correct positions. In all such cases where fixed blocks are embedded, care should be taken to see that the strength or effective cover of any part of the structure  is not reduced below the standard required by this specification.

  - Electrical  Conduits to be placed in Concrete Beams and Slabs – Where conduits are to be cast in concrete  slabs, the boxes and conduits are installed and held in position by blocks and iron wire fastened to the reinforcing bars. The concrete is then  poured and tamped. For structural

reasons these runs of conduit are usually  close to the bottom surface  or near the central portion of a floor slab. If a great number conduits  must be embedded it may be necessary to increase the slab thickness. This refers of course, to conduits being installed in a structural slab. In  many instances, the structural slab is covered with a concrete  topping or fill in which the conduit may be installed  without affecting the integrity of the slab. In all cases, local building codes should  be consulted  for limitations, or imbedded conduits. In any  event the top of any conduit shall be least 18mm below the finished  floor surface in order to prevent cracking. When  heavy trucking is expected, this allowance  should increased 1 ½ minimum

    - Conduits shall have an OD not greater 1/3rd of thickness as measured at its thinnest point

    - Conduits  running parallel to each  other shall be spaced not than 3 times the OD of the largest conduit center to center.

    - Conduits  running parallel to bean axis, shall not run above  beams

    - Conduit crossings shall be as near to a right angle as possible.

    - Minimum cover over conduits shall be 18 mm.

      - Handling – Para 20: 1-3 and 20:1-4 of I.S. : 456  shall apply

      - Placing and vibration of concrete – I.S. 456   shall apply

- Where the sections of concrete  members are thin and very heavily reinforced, carefully executed vibration, either alone combined with tamping, causes  the concrete to flow uniformly round bars. Moreover, the increased fluidity due to vibration occurs that with mixes which would  otherwise be regarded as too dry for reinforced concrete work.  In consequence, in work where  a low water cement ratio  would be necessary, with hand tamping, higher strength may be obtained  by vibration  due to the  drier concrete,  at can be used and compacted with these machines.

- All beams and slabs shall be filled  to the top surface in one continuous operation, that is from the  bottom  of beam to top of floor.

- For vertical joints  in long buildings – Part 19, of I.S 456   shall apply

- In cases where the outer faces of buildings are plastered the reinforced concrete  roofs should be as a rule, be carried through to the outside of walls and a small parapet built over them. In cases where the outer faces of buildings are pointed, the reinforced concrete roofs may be stopped short for the face by half a brick or a full brick and the face so left on the wall built up with bricks to suit the pointing over the rest of outer face. If leakage through the joint  between the end of roof  slab and masonry is anticipated the joint at weep holes shall be filled with an asphaltic composition and covered  with  flat tiles before  building on the parapets.

(D)  Treatment at bearings of reinforced concrete  slabs and beams

- The principle  underlying the treatment  at bearings of R.C.C. slabs and beams, is to effectively insulate the R.C.C. structures from the masonry or other supports on which they bear.

- The vertical face of the masonry rebate at bearings should also be plastered smooth with cement  mortar 1:3. A gap 12mm wide should  be left between the vertical face of the masonry and the R.C. work  and thin gap  should be filled with a mastic. Composition of bitumen for caulking purposes given in the I.S. 1580 shall apply. Composition specification for mastic given in I.S. 5057 and for bitumen felt in           I.S. 1322  shall apply. The object of fillings is to minimize chances of stone chips or other  hard materials inadvertently falling into the gap and getting wedged between  the R.C.C and masonry, thus obstructing free movement. Where the spaces will not be easily

accessible for mastic filling after the concrete is laid , as for example, at sides of a tee-beam  with slab extending to full length of the beam at the bearing, pre cast pads of the mastic  12mm. thick could be inserted  before laying the concrete. The mastic filling of pad should  not extend to full depth of  the gap but a space of say 25 mm to 40 mm should be left vacant at top in order to admit of the mastic being squeezed when the slab or beam moved against it. Similarly, the mastic should be  stopped about 40mm away from the vertical face of walls to ensure  the walls being finished with plaster.

(E)  (a) Finishing the concrete surface – Best results at least cost are obtained when the finishing is undertaken while the concrete is still green. The fins and any rough projections can then be rubbed down and the whole surface brought to an even finish by rubbing with a wooden float using a mortar at the same time filling the voids. A neat cement work shall then be applied to give  a smooth surface. If the concrete has set hard,  the fins and rough projection, if any, shall be removed by using corborandum brick or a paved grinding machine by chipping, before finishing off with the smoothing wash. If the work  of chipping is not done with care or if  the surface exposed  after removal of the forms can not be satisfactorily dealt with in this manner due to bad from work or for  other reasons, a coat or cement plaster of 1:2 of thickness as ordered by the Executing Engineer shall be applied. No extra payment will be given for finishing concrete surfaces as instructed above in this clause. However, for work where the finishing of R.C.C.  surface with plaster coat is found necessary  for asthetic beauty, the authority calling for tender should furnish a supplemental specification indicating  the thickness and proportion of mortar to be used for the plaster coat over the R.C.C. concrete  surface and also the method of payment in the schedule of tender notice.

(b)  	Weathering course over the R.C. slab – In the case of buildings, where there is no storey over  it, the R.C. slab is to be  protected against alternative shrinkage and expansion. For this a weathering  course of brick jelly concrete is laid over the slab. After fifteen days of the laying of R.C. slab, for every 1 m3  of weathering course a layer of brick concrete 100mm  thick with 0.96 m3 broken brick 20mm gauge and 0.375 m3 slaked lime ( no sand ) shall be laid over and well beaten to 75 mm thickness with wooden hand beaters. Necessary slope to drain the rain water shall also be given in the weathering course itself. The beating shall continue until the concrete is well  consolidated and the beater makes no impression and readily rebounds from the surface when struck on it, the whole shall be constantly wetted by sprinkling lime water. Should this surface, during the process of beating become so uneven that water lodges, it shall be picked up and fresh  concrete added as may be necessary.

( After six days or after the concrete laid has hardened, one course of pressed tiles of size 20 cm. X 20 cm X 20 mm shall be laid in oiled mortar in C.M. 1:3 and rubbed smooth

- Payment for treatment at bearings of reinforce concrete slabs and beams should  be made separately and not included in rate for R.C. Work

Joints in buildings : - The various of joints that may have to be installed in buildings are as follows :

      - Expansion joints : Joints provided to accommodate the expansion of adjacent parts and relieve compressive stress that may otherwise develop.

      - Construction Joint – Joint installed at location whose construction  stops for any reason.

      - Contraction Joints : These are separation or plans of weakness introduced in concrete structure to localize shrinkage movements

      - Sliding joint or vertical joints : - When variation in temperature, moisture content, or loading results in unequal settlement or movement in different parts  of the building vertical joints are formed so as provide as slip plane.

      - Spacing of expansion joint : - (1) Roof R.C.C. 20mm interval and at changes in direction

      - Vertical joints may be located at 30 m intervals in long uniform structures and at \junction of L.H.  and T shaped structures.

      - Joints may be provided at every 10 m for parapets compound wall, chajja and balconies

      - Expansion joint in R.C.C. roof slab or floor slab with or without R.C. beams =- The expansion joint should span in the same direction as that of the main reinforcement. In case the site  conditions require  the joints to be spanned across the main reinforcement, necessary beams should be introduced at the joints to support the slab. A typical sketch of the expansion joint in R.C.C. roof slab is enclosed. The same can be adopted  for floor slabs by omitting the raised portion above the R.C.C. slab. The avoid cracks in the supporting masonry below and in the cases where it is not possible to provide a vertical joint in the masonry, it is preferable to provide cement concrete bed block on the bearing. In the case of joints in the floor slabs over R.C.C. column the copper sheet can be perforated with slotted holes or groves and inserted over the  reinforcement and the joint  finished as usual.

      - “Method of formation” Type ‘A’ – While laying the slab, the water bar is placed in its position and tied with the slab  reinforcement grill. While concreting  the ends of the slab the joints are given  increased thickness by giving a slope of 1 in 12 for 60 cm width, on both  sides and raised to 30 cm as shown. The top and sides are finished smoothly. ( it is not advisable to raise this portion by masonry , as masonry will separate from R.C. slab during expansion and there is likelihood of rain water leaking through the separate crack )

The weathering course are then laid and furnished as shown in the raised portion. At  the top of one side of the raised portion an asphalt pad or tar felt is placed. The joint is then covered at top  with two courses of pre cast cement concrete slabs of mix. 1:2:4 and 2.5 cm thick in staged manner. The top course of slabs is given a slope of 1cm ( in 50 cm width ) by adjusting the thickness of mortar between the two course of cover slabs as shown in the sketch.

A.C. sheet is fixed at the bottom of the joint  with suitable  screws and plugs. The ends of the joint are also finished smoothly.

Type ‘B’ –While concreting, the ends of slab  at the joint are given increased thickness, by giving  a slope 1 in 12 for 60 cm width, on both sides, from the edge of the joint  and raised to 30 cm. As shown . The top and sides are finished smoothly. ( It is note advisable to raised this portion by masonry, as masonry will separate from R.C slab during expansion and there is likelihood of rain water leaking through the separated crack )

Weathering course (viz) brick jelly lime concrete and flat tile or pressed tiles as the case may be, are then laid and pointed as shown in the sketch.

A.C. Sheet is then fixed at the bottom of the joint with suitable screws and plugs.   The joint is then filled up with suitable back up materials like, plastics, cork strips, granulated cork, board, dehydrated cork, fiber board , flexible foam, or rope, etc. as shown in the sketch, up to I cm below the top of the raised portion of the joint.  A bond breaker such as polyethylene strip should be use in case the sealant has adhesive tendency to the bottom of the joint or to the back-up material.  The sides of the joint should be absolutely free from moisture, oil, dust, loose particles, etc., Damp joints may be dried by the use of gas torch, or hot air blower.   Dust may be removed by wire brush. The primer is a two component system with a base and a hardener, mixed at a suitable proportion mentioned by the manufacture.  Adequate time as stipulated by the manufacturer may be allowed for the primer to set.

The space at the top over the backup material and between the dried primer sides is the  filled up with the sealant.  Sealant compound application should be completed within the life period stipulated by the manufacturer.

At the top of one side of the raised portion an asphalt pad or felt is placed.  The joint is then  at the tope with two courses of pre cast cement concrete slabs of mix.  1:2:4 and 2.5 cm thick in a staggered manner.

The top course off slab is given a slope of I cm (in 50cm) width by adjusting the thickness of mortar between the two courses of cover a slabs as shown in the sketch”.

SECTION  V.

BRICK MASONRY.

INDIAN STANDARDS EXTRACTS.

| I.S. Numbers | Description |
| --- | --- |
| 4082 | Recommendations on Stacking and Storage of Construction Materials at site |
| 1077 | Specification for Common Burnt Clay Building Bricks |
| 3102 | Classification of Burnt Clay Solid Bricks |
| 2691 | Specification for Burnt Clay Facing Bricks |
| 2212 | Code of practice for Brick Work |
| 1905 | Code of practice for Structural Safety of Buildings ; Masonry Walls |
| 2117 | Code of practice for Manufacture of Hand Made Common Burn Building Bricks |
| 1200 | Method of Measurement of Building and Civil Engineering Works (Part-III) Brick Works |

BRICK MASONRY GENERAL

- Wall bricks : -

  - Specification for building bricks and classification are covered  I.S. 1077 / 1970 and 3102 / 1971.

N.B. Vide descriptive specification  sheet to be attached to agreement regarding  kilns from which bricks are to be obtained.

- Bricks are to be well soaked in water before use. For  further instructions regarding soaking and size of bricks to be used I.S. 2212 / 1962, and I.S. 1200 Part III / 1970 shall apply.

- Regarding making of bricks I.S. 2117 / 1967 shall apply. Where a large number of brick  has to be manufactured it would be prudent to mould and burnt a small number on trial ensure the certainty of the output before starting operations on a large scale.

  - All fuel for brick  burning should be dry and used when comparatively fresh.  Decayed  timber should not be  used as fuel., Tamarind or babul is the best wood to use in burning. Specification for facing brick is covered by I.S. 2691 / 1972.

- Mortar : - Specification is covered by I.S. 2212 / 62. Should the mortar perish, i.e., become dry white or powdery through neglect of watering, the work shall be pulled down and rebuilt  at the contractor’s expense, or should the contractor  fail to watch the work to the satisfaction of the officer – in  - charge of the work the latter  may supply the requite men to water the work  properly and charge the cost to the contractor.

  - All masonry  shall be washed down on completion and all stains – lime or otherwise – removed from the face

- Bats : No bats or cut bricks shall be used except  where absolutely required  for obtained the dimensions  of the different courses for obtaining the specified bond and where in the opinion  of the Executing Engineer the work is of too petty a nature to warrant the special  moulding of bricks of the shape required,

Setting bricks in mortar, bond and raking of joints are covered by I.S. 2212 .

- Non – plastered  face work :  Finishing joints is discussed in para 5.5. of I.S. 2212. Where is no specification superseding the standard specification “Pointing – Remarks on “ non – schedule provision made for extra payment for pointing, the exposed joint shall be finished as described under clauses 1 to 4 of  “Pointing remarks on “ and  shall be well rubbed with a piece of 10mm rod suitably bent.

7.1. No extra payment will be made  for this manner of finishing brick work face joints and it shall be included by the contractor in his unit rate for brick work.

- Racking back _ When circumstances render it necessary to carry on the same section of t he building in uneven course  the bricks shall be raked back so as to maintain an uniform  and effectual bond.

- Iron, stone concrete and other fixtures, buttresses, etc., Vide clause 16 under standard specification for stone masonry general which is to complied with.

- Wet formations : - vide clause 17 under stone masonry general which is to be complied with.

- Openings – This is covered  by paras 10.2, 10.4, of IS 2212

11.1. The contact unit rates for brick work are inclusive of quoin and jambs.

- Treating – This is covered by paras 5.12 and 5.12 of I,S, 2212 .

- Treatment at ends of beams and joists – The ends of all wooden beams  wooden roof trusses, etc., shall rest in recess having 40 mm  space for the free circulation of air all – round  them and provided with perforated zinc sheeting.

- Measurement of brick work is covered by I.S.  1200 part – III.

Measurements _

- Brickwork shall  generally be measured in cubic metres unless other-wise specified.

- Walls half brick in width and less shall be measured separately in square metres stating thickness

- Brick walls of width over half brick shall be measured in multiplies of half brick which shall be deemed to be inclusive of mortar joint, irrespective  of excess of executive  width.

- The following shall be taken as  half brick measurement.

For bricks 19 x 9 X 9 cm – 10 cm

For bricks 19X9 X 5.7 – 10 cm.

- Wooden bricks – No extra piece will be paid for wooden bricks and plugs built  into masonry, the wooden bricks and plugs themselves are to be supplied free of charge. Clause 14 under stone masonry “General” is to be complied with.

- Stacking – This is dealt with in I.S., 4082

- Handling  bricks – Bricks shall not be handled  in baskets or in other manner which will destroy the sharpness  of their edges.

- Checking of levels – The Sub Divisional Officer will personally verify all levels with a leveling instrument  when the work reaches the level of the plinth and again at floor and roof levels.

- Rounding corners – (a) Corners  of rooms or pillars whether  interior or project shall not be rounded but in exceptional cases, where it is so desired to round the corners shall be done in plaster for the re – entrant corners but will require chiseling of projecting angles before plastering.

- A separate addendum specification will be issued in case specially moulded rounded corner bricks are to be used in jambs aches and projecting and reentrant corners so as to eliminate the sharp angles.

- The rate tendered for plastering and brick work in the case (a) and (b) above shall be held to cover the cost of labour and materials for complying with this clause.

- Brick on edge coping, etc: PARA 10.1 OF I.S. 2212 shall apply, No extra payment will be made for this work, over the contract rate for brick work.  The contract rate for brick work shall include the same .

- Plinth off-sets on the interior faces only should be kept 15cm below floor level (unless finished thickness of floor to be laid will determine, instead of the 15cm) to allow for the floor paving which is done subsequently coming upto the face of the superstructure.   Similarly retaining walls of verandahs, etc., must be built with due regard to the slope allowed for the floors.

- Parapets: All parapets will be measured under the masonry below them and with which they are continuous in a floor, the parapet has to be reckoned with the lower floor (Circular Memo. 6105/A6/53-2, dated 2nd December 1953).

- Anti-termite constructional methods: At places of foundation where the existence of colonies of subterranean termites are found the probability of internal attack of termites through floor and plinth fillings is great.  In such places the constructional practices for protection against subterranean termites originating both internally  from plinth and externally from the area surrounding the building as laid down in IS 6313 (Part-I)  be followed.

- Applicability of General specification: General clauses shall apply to all the following sub-specification of brick  masonry.

  - If bricks, specially moulded to the radial lines of the arch, are demanded for arch work it will be so specified. If any case, the bricks shall be specially selected and shall be free from defects of any sort.

N.B. Specially moulded bricks are always preferable  as cutting and rubbing of bricks to give proper  radial joints remove the hard surfaces skin which protects the bricks from decay and weathering. The cost of labour, required for rubbing the bricks, will in important works come to as much as providing specially moulded bricks, in which cases it will be advisable  for the contractor to provide, the same whether specified or left to his option in supply.

  - The bricks shall be laid  in concentric  half – brick rings. The voussoir joints  shall be properly summered. The thickness of joints shall be the same as that specified in the standard specifications for the same class of brickwork in mortar. In all arches, the voussoir  joints shall be truly  perpendicular to the tangent of the curve at those points. In setting the bricks in arches, they shall be well pressed into their beds so as to compress the mortar to full bearing and leave the joint thin.

  - Bricks forming skew-back joints shall be specially mould or cut, so as to radiate truly, and defects in this particular shall not be remedied by the extravagant use of mortar, nor shall patching up by chips, etc., be permitted

  - Great care shall be  taken that the rings are bonded together properly wherever the joints of any of the rings come to a summering or planer face. Joints in two consecutive  rings shall  not  come in the same radial plane. The arch work shall be evenly and quickly done and kept  thoroughly moist so that no portion of the arch hardens or sets before the whole arch is completed

  - Centers shall comply with clause 4 for Centers under the standard specifications for “Ashlar Arching”.

  - Arch work over doors and windows shall be built  to this specification, segmental arches being given the rise indicated  in the following table :

  - When flat arches are specified, a relieving arch is to be built over the flat arch. The chamber for the flat  arch will be 10mm per metre of span. The relieving arch must first be built with above precautions regarding centers, etc. and not simply built with above precautions regarding centers, etc., and not simply built into the wall over the masonry laid to the segmental shape. Skew – back  of the flat arch. The space between reliving arch and a flat arch or lintel is not to be filled until the wall  has been completed, unless specially ordered  by the Executive Engineer. Flat  arches will be formed of carefully rubbed  brick in header and stretcher bond with all voussoir  joints  converging to the apex of an equilateral triangle  described  on and below the soffit  of the arch. Flat arches will usually only be used for pointed  face work.

BRICK WORK IN CLAY

Mud mortar of the same specification as described in the standard specification “Random Rubble in Clay” shall be used. The bricks shall be the best of the class specified

Work shall be carried out to the Standard specification with the exception  that clay replaces mortar. Joint thickness should not exceed 1.25 cm. The bricks also require  comparatively little wetting prior  to use nor need they be subsequently kept damp as specified for brick in lime mortar. As the back work will usually  be plastered less labour will be required for finishing off joints of the brick work.

It is customary with this class of masonry  in Buildings, to execute  the top 30 cm.  alround doors and windows with brick in lime mortar or cement mortar.

HONEY COMB WORK  WITH BRICKS  AND PLASTERED ONE  COAT 10MM LIME MOTAR INCLUDING WHITE WASHING TWO COATS

- Modular  bricks shall be used unless other class of bricks are expressly specified in the schedule item description. The bricks shall be thoroughly bedded  with lime mortar and have a bearing of 20mm on each side.

- The wall shall be 20 cm thick. The bond used  shall be a heading bond throughout the wall with rectangular  shaped holes.

- The faces and sides of the holes shall be plastered with lime  mortar one coat to that standard specification only with thickness 10mm,and the rate is to include plastering, All joints and edges shall be struck flush to give even surface on all sides

- Watering shall be done  for one week after completion of the masonry and prior to white washing

- Two coat  of white wash shall be finally applied to both faces of the work and also  to the interior of the holes. This included in the unit rates. The wall shall be carried up regularly  course by course

Note : Cement  mortar can also be used instead of lime mortar

HONEY WORK WITH  WASHED PAN TILES

  - Standard specification pan tiles are to be used or such other size as may be specified  or instructed  to be used by the executive Engineer. In no case tiles of different dimensions be used on the same work

  - The tiles  shall be soaked  in water for 24 hours and allowed to dry, After they are quite  dry they shall be dipped in standard specification white wash of the consistency of  cream and again allowed to dry .

  - The tiles should then be laid in regular rows with concave surface downwards with sufficient mortar at the joints to prevent the tiles from displacement. The tiles in each row shall be built in so that each tile covers the joint between two tiles in the row immediately below. Broken tiles shall never be used on the work.

REINFORCED BRICK WORK WALLS

- The walls shall be built of modular  bricks in Standard Specification cement mortar being in the proportion of 1 of cement to 3 of sand. The reinforcement shall consists  of hoop iron  40mm  wide and 15 gauge or such other size as may be specified laid flat in the middle of the brickwork. ( The bond of the hoop iron with the brickwork will be greatly  improved if the hoop iron is  punched at intervals of say 15 cm. So as to form burrs  on both sides of the reinforcement. The punch hole shall be 6 mm diameter ) .

- The strip of hoop iron shall be continuous and if it is not available for the full length  it shall be revert – jointed with an overlap of not less than 8 cm.  AT the ends the strips shall be folded over at the function to atleast half the depth of the course. It shall be placed at every alternate course or as may be otherwise specified or as shown in the drawing Before laying the hoop iron it should be exposed to the weather to remove its bluish smooth surface and hereby increase its adhesion to the mortar.

- Two slots at least 10 cms. Deep should be cut in  the walls between which the partition has to be built and the partition  shall be built into these slots, the slots being flushed full  joint with cement mortar 1:3 as the work proceeds.

- The thickness of the wall will be width of a standard  specification size brick, unless other size brick is permitted by the Executing Engineer to be used – Plus  the plaster thickness for the two  faces.

- The joints should not be more than 6 mm except  where the  hoop iron is to be laid, where the joint should be 25 mm., thick to ensure at least a cover of 12 mm between the reinforcement and the bricks. The face joints  shall be raked out to 12mm. depth before the mortar  completely.

- The wall shall be finished with standard specification cement plastering 1:5, 12 mm thick on both  faces and watered for three weeks.

- Two coats of standard specification ‘White washing’ shall then be applied.

- The rate shall be for 10 square  metres inclusive of reinforcement cutting slots in end walls, plastering, white washing etc., completely finished in place.

NOTE : (1) These walls are intended to be introduced in places where there are no walls or beams or such other foundation underneath and they are generally expected to act as a beam by themselves and to be held up by the side  walls.

2.  For the purposes of correct reinforcement it is necessary to lay most of the iron in the bottom courses farthest from the neutral axis of the wall treating  it as a beam and to reduce the reinforcement proportionately  upto the neutral  axis and to have no iron in the compression courses above the neutral axis but  to render the wall safe against any lateral shock of stress, reinforcement is taken upto be top.

Note  on protection against – X –Rays in Institutions –

- General – The following instructions shall be followed in the construction of all radiological institutions  where it is necessary to provide adequate protection to the operators and other personnel engaged in the X-ray work from the harmful effects of X-Rays. In view of the danger to the health of occupants, if any mistake are committed in applying the protective measures, it is specially warned that the precautions should be carefully studied and put into effect. The extent of protection that is required will depend upon the potentiality of the equipment to be housed. Requirements  or various forms of X-Ray work are laid down in terms of thickness of lead  in millimeters the report of the X-Ray and Radium Protection committee.

|  | X-Ray generated by peak voltage in KV | Minimum equivalent thickness of lead In millimeters |
| --- | --- | --- |
| Not Exceeding | 75 | 1.0 |
| ” | 100 | 1.5 |
| ” | 125 | 2.0 |
| ” | 150 | 2.5 |
| ” | 175 | 3.0 |
| ” | 200 | 4.0 |
| ” | 250 | 6.0 |
| ” | 300 | 9.0 |
| ” | 350 | 13.0 |
| ” | 400 | 17.0 |
| ” | 500 | 26.0 |
| ” | 600 | 35.0 |
| ” | 700 | 44.0 |
| ” | 800 | 53.0 |
| ” | 900 | 62.0 |
| ” | 1000 | 70.0 |

  - These requirements shall be  met by special requirements in construction as detailed below :

    - Walls – Walls shall be constructed of special barium impregnated bricks. These bricks shall be made  by  mixing dry 8 parts by volume of clay absolutely free from any pebbles and vegetable matter and one part by volume of barium sulphate. In order to ensure proper mixing , not  more than 0.5 m  of  clay shall be taken up in a heap for mixing at a time. All such heads are then collected together into a big heap and a pit formed in it. The requisite quantity of water shall then be poured in the pit slowly and the whole allowed to remain for 12 hours  after which the mixture shall thoroughly  plugged.  Bricks shall then be moulded in a mould of standard size but having special arrangement to leave V-shaped grooves, at the least 3 mm. deep  on all the four edges of the bricks to distinguish them from ordinary bricks.  The bricks so prepared shall be burnt as ordinary bricks and shall be laid in lime mortar to which barium sulphate is added in the proportion of 8:1 by volume. The thickness of barium – impregnated brick walls shall be as required by the graph appended to this specification.  For wall 30 cm thick barium impregnated bricks will be used for the full thickness and in walls of more than 30 cm thickness. barium impregnated bricks  will be used for 30 cm. And 40 cm. Widths, in alternate courses and the balance in thickness will be built with ordinary bricks in ordinary mortar.

    - Wall – Plastering – All the walls of barium impregnated bricks shall be plastered with  special mortar  of the following mix, viz., cement 1, sane 1, barium sulphate 3,  by volume. The thickness  of plaster shall be 15 mm to 20mm lid in the same manner  as two coats of ordinary  lime plastering

    - Floor and ceiling – Floor projection shall be given by  paving with barium bricks  laid flat in cement barium mortar ( 1 cement 1 sand and 3 barium sulphate by volume). The top of the floor shall be finished in good wood, rubber, linolium or any other  non – conducting material. Protection in ceiling may be given  by finishing with 25 mm thickness  of cement barium mortar and  painted over with white glossy  paint

    - Partitions – Partitions shall be built  in special partition bricks moulded in the proportion  of 2 clay  and 1 of barium sulphate by volume and provided with a groove to accommodate  reinforcement of mild  steel rods. The protection afforded by these bricks against X-rays is given in the graph appended. The partitions shall be finished with cement  barium plaster 15 mm to 20mm thickness laid in two coats.

    - Protection in doors, windows and shutters – Full protection shall always be provided for in all openings like doors, windows and shutters, etc. In the sides  of door frames the lining of the barium impregnated bricks shall be done to the full thickness of the wall. Door frames shall be wrapped round with 28.2 Kg / m2  ( i.e., 2.5mm thick ) lead sheet before being fixed in masonry. Protection in door shutters shall be given  by sand weighing a 56.7 kg / m2 ( 5.0 mm thick ) sheet lead between the door proper and a covering panel of wood. Similar  protection shall be given in all windows and shutters except observation windows which shall have lead glass shutters. Windows shall also be placed at a height of about 1.5 metres from floor level so that no direct radiation will  pass out of the  room.

EXTRACT FROM I.S. 4082

Recommendations  on stacking and storage of construction

Materials at site ( first revision )

  - Bricks :

    - brick shall not be dumped at site. They shall be stacked in regular tiers as and when they are unloaded to minimize breakage and defacement of bricks

    - In the case of brick made from clays containing the KANKAR, the bricks  in stack should be thoroughly soaked in water (docked) to prevent lime bursting.

    - Brick stacks shall be placed close to the site of work so that least efforts is required to unload and transport the bricks again by loading on pallets or in barrows. Building  bricks shall be loaded or unloaded a pair at a time unless  palletized. Unloading of building bricks or handling in any other way likely to damage the corners or edges or other parts of bricks shall not be permitted.

    - Bricks shall be stacked on dry firm ground. For proper inspection of quality and ease in counting. The stacks shall be 50 bricks  long and 10 bricks high, the bricks being placed on edge and preferably, the width of each stack shall be two bricks. Clear distance between adjacent stack shall not be less than 0.8.m

    - Bricks  of different types and classification shall be stacked separately,.

EXTRACT FROM I.S. 1077 – 1970

Specification for common burnt clay building Bricks

3. Classification

  - The common burnt clay bricks shall be classified on the basis of their minimum compressive strength. The brick of  compressive strength 50 Kg / cm2  shall be classified as 50.

    - Each class of bricks  shall be further divided into two sub-classes . A and B based on tolerances and shape. The brick  of classification 50 shall have sub-classification  50 A and 50 B.

Note :  For convenience, detailed  classification of bricks covered by this standard as well as in IS : 2180  ( specification for heavy duty burnt clay building bricks ) (First revision ) has been prescribed in IS : 3102 classification of burnt clay building bricks, (First revision )

4.	General  Quality

  - Bricks shall be hand – or machine – moulded they shall be free from cracks and flaws and nodules of free lime. Brick of 9cm. Height shall be moulded with a frog 1or 2 cm. Deep on one of its flat sides; the shape and size of the frog shall conform to either pig. 1A or Fig. 1b. Bricks of 4cm. Height and those made by an extrusion process may not be provided with frogs.

  - The bricks of sub-class A shall smooth rectangular faces with sharp corner and emit ringing sound when struck. The bricks of sub-class B may be permitted to have slight distorted and rounded edges provided no difficulty shall arise on this account in laying of uniform courses.

5.    Dimension and tolerances

5.1 	The standard sizes of common building bricks shall be as follows:

Length.        		Width.      		Height.

Cm.     		 cm.			  Cm.

19			  9			   9

19			  9			   4

  - Tolerances- The dimension of bricks when tested in accordance with 5.2.1 shall be within the following limits:-

a. Length 	368 to 392 cm

Sub Class A 	b. Width		174 to 186 cm

c. Height 	174 to 186 cm (in the case of 9cm, high bricks )

77 to 83 cm ( in the case of 4 cm  high bricks )

a. Length 	350 to 410 cm

Sub Class B 	b. Width		165 to 195 cm

c. Height	165 to 195 cm (in the case of 9cm, high bricks )

74 to 86 cm ( in the case of 4 cm  high bricks )

5.2.1. Twenty ( or more according to the size of stack ) whole bricks shall be selected at random from the same selected under 7. All blisters, loose particles of clay and small projects shall be removed. They shall then be arranged upon a level surface successively  as indicate in Figure 2A, 2B and 2C in contact with each other and  in a straight line. The overall length  of the assembled bricks shall be measured with a steel tape or other suitable in extensible measure sufficiently long to measure the whole row at one stretch. Measurements by repeated application of a short rule or measure shall not be permitted. If for any reason it is found  impracticable to measure bricks in one row, the sample may be divided into all the  rows of 10 bricks, which shall be measured separately to the nearest millimeter. All these dimensions shall be added together.

- Physical  properties

6.1. Compressive strength : Common building bricks shall have a minimum compressive strength of

50 KG / Sq.cm . When tested in accordance with  the  procedure laid down in table 1 of IS :       3495 –  ( See not under 1-1 )

6.1.1. The compressive strength of any individual brick shall  no fall below the average compressive  strength specified for the corresponding  class of brick by more than  20 per cent.

  - Water absorption – When  tested in accordance with the procedure laid down  in table 2 of IS : 3495 ( Method  of sampling and testing of clay building bricks ) the average water absorption of common building Bricks shall not be more than 20 per cent upto class 125 ( see 3-1) and 15 per cent for higher  class,  by weight  after immersion in cold  water for 24 hours.

  - Efflorescent – When common  building bricks are tested  in accordance with the procedure laid down in Table 3 of IS : 3495  (Method of sampling  and testing  of clay  building bricks the rating of efflorescence shall  not be more than “moderate” upto Class  125 and ‘slight’ for higher classes.

EXTRACT FROM I.S.  : 3012 –

Classification of burnt clay solid  bricks

3.1. The classes and sub – classes of burnt clay solid  bricks shall be as given in table I

TABLE – I  CLASSES BURNT  CLAY SOLID BRICKS AND THEIR  PRINCIPAL REQUIREMENT

| Type of brick | Class designation (see note below) | Compressive strength Kg / cm2 Min. | Water absorption (24 hr. Immersion percentage max, ) | Efflorescence |
| --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) |
| Heavy duty (see IS:2180 ) | 450 |  |  |  |
|  | 400 | 450 | 10 | Nil |

| Common burnt clay building bricks see I.S. (1077) | 350 | 400 | 10 | Do |
| --- | --- | --- | --- | --- |
|  |  | 350 | 15 | Slight |
|  | 300 | 200 | 15 | Do |
|  | 250 | 250 | 15 | Do |
|  | 200 | 200 | 15 | Do |
|  | 175 | 175 | 15 | Do |
|  | 150 | 150 | 15 | Do |
|  | 125 | 125 | 20 | Moderate |
|  | 100 | 100 | 20 | Do |
|  | 75 | 75 | 20 | Do |
|  | 50 | 50 | 20 | Do |

NOTE :  Each class of bricks  shall further be divided into sub Classes A, B  etc., based on the following :

Sub – Class A – Tolerance limit shall be  3 per cent and shall have smooth rectangular faces with sharp corners and emit clear ringing sound.

Sub – class B – Tolerance limit shall be  8 percent and shall be permitted to have slight distortion  and rounded edges, provided no difficulty shall arise in laying of  uniform courses.

Specification for burnt clay facing bricks (First Revision )

- Classification

  - The facing bricks shall be  of  two classes

- Class I ; and

- Class II

- General Quality :

  - The facing bricks shall be made of clay, shale or mixture  of  these materials with or without admixtures and burnt  to meet the requirements of this standard. The colouring material added to the  clay shall be suitable ceramic materials  and shall be well distributed throughout the body. The brick  shall be of uniform colour.

  - The bricks  shall be free from cracks, flaws  and modules of free  lime and of even texture. These shall be thoroughly burnt  and shall have plane rectangular faces  with parallel sides and sharp  straight right angled edges.

- Dimensions  and tolerances

  - The standard sizes  of the facing bricks shall be 19X9X9 cm. And 19X9X4 cm

  - The permissible tolerances shall as under :

| Dimension | Tolerance |  |
| --- | --- | --- |
|  | Class I | Class II |
| Cm | mm | mm |
| 19 | ±3 | ±5 |
| 9 | ±2 | ±3 |
| 4 | ±1.5 | ±2 |

- Physical requirements

  - The average compressive strength obtained in accordance with the procedure laid down in Table I of IS : 3495  (Method of sampling and testing clay  building bricks ) shall not be less than 50 Kg / cm2   for 50 Class designation bricks,  75 Kg / cm2 for class II and 100 kg / cm2  for Class I

  - The water absorption requirement when tested in accordance  with the procedure laid down  in Table 2 of IS : 3495  for 24 h immersion shall not exceed 15 per cent.

  - when tested in accordance with the method  specified in Table 3 of IS : 3495  efflorescence requirements  shall be ‘Nil’ for both classes.

  - When measured in accordance with the method  specified in Table 4 of IS : 3495  the warpage for both  classes shall not exceed 2-5 mm.

EXTRACT OF IS : 2212

Code  of Practice for Brick Work

Nominal thickness of a wall – This is the thickness of wall that is stated in the estimates for calculation of quantities. It is a ‘fictitious’ dimension, which is neither the actual thickness of wall excluding surfaces finishes like plaster, rendering, etc., nor necessarily the overall thickness  including such  finishes. The following example will illustrate this point : -

|  | Nominal Thickness | Actual thickness |
| --- | --- | --- |
| For traditional brick of 9 in length (with allowance of ¼ in. for mortar joint ) |  |  |
| One brick wall | 9in | 8 ½ to 8 ¼ in |
| 1 ½ brick wall | 13 ½ in | 13 to 13 ¼ in. |
| 2 brick wall | 18 in | 17 ½ to 17 ¾ |

| For modular brick (with allowance of 1 cm. For mortar joint ) |  |  |
| --- | --- | --- |
| One brick wall | 20 cm | 19 cm |
| 1 ½ Brick wall | 30 cm | 29 cm |
| 2 Brick wall | 40 cm | 39 cm |

    - Bricks – Unless otherwise specified, burnt – clay  bricks shall conform to the requirements of IS :1077  specification for common burnt  clay building bricks, and shall be of the specified class.

    - Mortars – Mortars for  masonry  shall be prepared in accordance with IS : 2250  Code of   Practice  or preparation and use of masonry mortars.

The materials used for mortar shall conform to the requirements specified 4-2-1 to 4-2-4.

    - Cement – Cement used for brick masonry shall be ordinary  or rapid – hardening  Portland cement conforming to IS : 269 Specification for ordinary, Rapid – hardening and Low Heat Portland Cement ( Revised ) or blast furnace slag cement conforming to IS : 455, Specification for port land Blast – furnace slag Cement ( Revised )

    - Lime – Lime used for brick masonry shall conform to the requirements of  IS 712, Specification for Building Limes. This may be used in the form of either  hydrated lime or lime putty. Field slaking of lime shall be done in accordance with IS : 1635  Code of Practice for Field slaking of lime and preparation of Putty.

    - Fine aggregate – Sand  shall conform  to IS : 2116 Specification for sand for masonry Mortar.

    - Water – Water used for masonry mortar shall be clean and  free from injurious amounts of deleterious materials.

5.  Design Consideration

  - Selection of bricks

    - For use in various situations of brick masonry, the brick shall be selected in accordance with Table - I

    - When the requirements for strength of masonry  predominate in the particular situation  of use, the bricks  shall be of such grade ( see IS:  1077 ) as to give the required strength for masonry and shall be selected in accordance with their relevant  provisions of  IS : 1905 – Code of Practice for Structural Safety of buildings Masonry  Walls.

  - Selection of mortars :

5.21.   Mortars used for Brick Masonry  shall conform  generally to IS : 2250  – Code of

Practice for Preparation and use of Masonry  Mortars.

TABLE I : SELECTION OF BUILDING BRICKS

(Clause  5-1.1 )

| Serial number and situation of use | Type of brick to be used | Special consideration | Remarks |
| --- | --- | --- | --- |
| 1. Facing | Class I Common bricks (SEE IS : 1077) | Bricks shall be free from minor defects such as chips at the edge of corners. Colour and texture may also be specified. If so required | … |
| 2.a. Subject to very heavy loading b. Requiring a high resistance to water penetration | Heavy duty (See IS : 2180 ) Do | … … | … |
| 3. (a) Plinths and foundations below damp-proof course ground well drained and no chance of continual wetting in foundations | Class I or Class II common bricks (See IS : 107) | … | … |
| b) Plinths and foundations below damp – proof course subsoil water table at a high level | Do | The bricks shall be free from efflorescence. These shall also not have any salt content which will affect the mortar of the masonry. The bricks may preferably be the densest available with the minimum water absorption |  |

SECTION VI

STONE MASONRY

| I.S. Number | INDIAN STANDARDS EXTRACTS |
| --- | --- |
| 1127 | Recommendations for dimensions and workmanship of natural building stones for masonry works |
| 2250 | Code of practice for preparation and use of masonry mortars |
| 1597 | Code of practice for construction of stone masonry – Part I rubble stone masonry |
| 1597 | Part II – Ashlar masonry |
| 4515 | Code of practice for boulder lining for canals |
| 1200 | Part IV – Method of measurement of building and civil engineering works – Stone masonry |
| 1123 | Method for petrographical examination of natural building stones |
| C.B.R.I. | Pre cast stone masonry block walling – Data Sheet No.8 |
| 6042 | Code of practice for construction of light weight concrete block masonry |

SECTION VI

STONE MASONRY

STONE MASONRY

  - General – The following instructions are to be complied with for all classes for stone masonry

  - Requirements for building stone – Building stones  shall comply with specification No.3 and shall be  obtained from the quarries defined in the agreement  descriptive specification  sheet – for detailed specification of stones I.S. 1597 Part 1 and I.S. 1123  shall apply. Stone  not suited for the particular class of works  defined  in the schedule items shall be rejected and rejected stone shall be removed once by the contractor from the works spot vide “General Conditions of contract”.

- Method of laying  stone – Lay on broadest face which gives better opportunity to fill the spaces between stones. For detailed specification I.S. 1597 Part I shall apply.

- Bond – For detailed instructions I.S. 1597 Part I 1967 shall apply.

- Watering – For detailed instruction I.S. 1597 Part I shall apply.

  - Mortar – Should be mortar perish i.e., become dry, white or powderly through neglect of watering, the work shall be pulled down and rebuilt at the contractor’s expense or should the contractor fail to water the work to the satisfaction of the officer in charge of the work, the latter may supply the requisites men to water the work properly and charge the cost to the contractor.

  - All masonry shall be washed down on completion and all stains – lime or otherwise – removed from the face

- Moving stone after it has been placed upon the mortar bed. –

5.1. If it is necessary to move stone after it has been placed on the mortar bed, it should be lifted  clear and be reset. Attempt  must never be made to slide it over  stone already laid.  Care must be taken not to  disturb joints already laid  when handling or moving stone.

- Bed Plates – Bed plates are to be laid in all cases under the ends of beams, girders, roof trusses, etc. The bed plates are to be of sizes specified or ordered by the Executive Engineer.  In all important cases cut stone  or reinforced  concrete  blocks will be used and the size quantity detailed and included under the relevant item of work. In all other cases where cut stone or reinforced concrete  bed plates are not demanded, the contractor shall supply large stone of size and dressing as ordered by the Executing Engineer and he will not be paid any extra rate above the contract rate for the wall masonry for such bed plates.

- Recesses for ends of beams, girders, etc., corbels under heavy beams and cornice under joists- The ends  of all beams, girders, roof trusses, etc., shall be situated  in a recess  having 40mm. space alround them to permit free circulation of air and provided  with perforated zinc sheeting

  - Joists and rafters will usually be built in the masonry and tarred for that portion. When it is approved by the Executing Engineer to build the ends of steel joists or other steel work into masonry the masonry for atleast 15 cm. Alround the  embedded steel shall be in cement mortar. No extra payment will be made for this treatment.

- Stones which are not  to be plastered over – All lintels and inside stones which are not to be plastered  over should be to the full width of the walls in which they are placed including the thickness of the plaster.

- Plinth off-sets on the interior faces only should  be kept 15cm. below flood level (unless finished thickness  of the floor to be laid, will  determine, instead  of the 15cm.), to allow the floor paving , which is done  subsequently, coming up to the face of the superstructure. Similarly  retaining walls  of verandahs, etc.  must be  built  with due  regard  to the slope allowed for the floors.

- Jambs for door and window openings.- Jambs for door and  window openings shall be formed with  quoins of the full height of the course in the case of  coursed masonry.  The quoins shall be in breadth atleast one and a half times the depth of the course and in length at least twice the depth. For each side of door openings three and  for window  openings two, of these quoins  shall be stones  of the full thickness of the walls. Unless rebating of the whole frame into the masonry is specially specified, chisel dressing to the width of door and window frames shall be done to secure  a close fit between  the frame shall be done to secure a close fit between the frame and the masonry. A rebate shall be made in the frame  where plastering is to  join with  the frame as shown in the standard designs for doors and windows and the plaster shall be keyed well into the rebate to form a neat joint. Jambs shall be played as described in the standard specification for “Brick Masonry – General”. Contract rates for all classes of masonry  are inclusive of quoins, jambs, and chisel  dressing as demanded by the above  and by the following masonry sub specifications.

- Contractor to supply sample stones when tendering his rates – In the case of cut stone masonry or rubble masonry of any kind, the contractor shall supply a sample stone when tendering his rates, showing the class of dressing and face  finish that he will execute for each rate and these sample stones will be initialed and kept by the Executive Engineer. Failure of the contractor to execute all work of  the particular clause up to the standard  of his sample supplied, will be dealt with as contractor’s  violation of the contract

- Mortar and materials to conform to standard specification : - The mortar to be used for each class of masonry shall conform to the particular standard  specification for that  class or mortar and the standard specifications for the materials used therein. For example,  for “Cut stone in cement mortar” the Portland cement,  sand and cement mortar shall conform  to the three  respective standard specification for these items.

- Dowels and cramps – When so specified  or instructed by the Executive Engineer, copings will be dowelled or cramped – and  courses of pillars, skew-backs and similar work joggled to the stone below it using dowels and cramps  of the hardest  and toughest  stone procurable or of copper and set in pure lime or cement as ordered Iron cramps are not  to be used.  Extra payment  for such dowels or cramps will be fixed  by prior agreement before execution of the work.

- Holes for passage of water, etc., to be left and wooden plugs and wooden bricks to be built in during construction – Care shall be taken to see  that holes for passage of water wherever necessary as from terraces, bath rooms, recesses for downfall pipes, etc. are  provided during construction and neatly finished off by pointing or plastering as ordered. Well – seasoned wooden plugs and bricks of sizes and quality approved by the Executing Engineer are to be built in as the work proceeds for fixing door and window shutter appliances coat hangers, picture rails, and for any other purpose decided necessary by the Executing Engineer are to be built in as the work proceeds for fixing door and window shutter appliances coat hangers, picture rails, and for any other purpose decided necessary by the Executive Engineer. Holes are also to be cut in the sill stones to receive  the uprights  of door – frames, in cases where  there is  no threshold  plate to the frame. No extra payment will be made to the contractor for the supply and fixing  of the wooden plugs and bricks forming holes, etc. as described  in this clause, for any class masonry vide also relevant clauses in the Standard “Preliminary specification”.

- Laterite : - In districts where laterite is used  as a building stone, the work shall be executed according to the relevant masonry sub specification which will be described in the schedule item. The stone shall be the best procurable and free from any admixture of the white earth. It shall be dug out  from the quarry  a sufficient time in advance, to harden well, before being placed in the work. The least thickness of stone shall be 17-5 cm and other dimensions shall be as laid down in the relevant sub-specification , the stones being laid header and stretcher alternatively, breaking joints by atleast   7.5. cm

- Iron, stone, concrete or other fixtures and buttresses etc., - All iron, stone, , concrete or other fixtures, returns buttresses, counter forts, etc., shall be built and bonded into the work as it proceeds and not inserted or joggled on after the masonry is advanced. No extra payment will be made for labour involved in building in any fixtures, holdfasts etc., which are given to the contractor for fixing in place.

- Wet foundation – In wet foundations, the excavation shall be kept free of water by the contractor, while  the masonry is in progress and until the Executing Engineer considers the mortar has sufficiently set.

- Scaffolding and tools and plant – Vide relevant clauses under the conditions of contract

- Chisel drafting corners and rounding interior corners of room – The Vertical exterior corners of buildings and  pillars of buildings in coursed rubble in mortar, first and second sort and random rubble in mortar (except  for clay mortar masonry ) shall be chisel drafted 40mm. width on either side, or such other width as may be specified. AT the entrance of doors, windows, archways and other openings, the corners against which doors and windows will open  shall be chamfered 25 mm. width and chisel-drafted beyond the chamfer 40mm. as above ( when parliamentary hinges are used chisel drafting only is necessary )

  - All interior corners  of rooms and projecting angles shall be rounded to 25 mm radius. The projecting angle will in such cases be chamfered as necessary and the rounding for both interior corners and projecting angles will then be done in the plaster, Similarly rounded, chamfered and chisel drafted corners shall be done wherever else they may be shown on the drawings

  - All chisel drafting  and chamfering referred to under this clause shall be included by the contractor in the tender unit rate for masonry and rounding of  corners in his tender unit rate for plastering. No extra allowance will be made for labour and materials involved in compliance with this clause, unless it is so expressly stated in the agreement specification.

  - Sketches illustrative of the nature of work to be done under this clause and clause 10 preceding are appended.

- Pointing : Clause 10 of the standard specification for “Brick Masonry” “General” shall apply to this specification.

- Anti-termite constructional methods :- At places of foundation where the existence of colonies of subterranean termites are found the probability of internal attack of termites  through floors and plinth fitting is great.  In such places the constructional practices for protection against subterranean termites originating both internally from within the plinth and externally from the area surrounding the building as laid down in IS 6313(Part-I)  shall be followed.

- This general specification applicable to sub-specifications : The clauses of this “General” Specification shall be applicable in all pertinent points to the masonry sub-specifications followings and the contract unit rate for each particular item of work is inclusive of  compliance with all clauses of the “General” specification and relevant sub-specifications.

CUT STONE IN MORTAR

Additional clauses to “General” preceding

Cut stone : I.S. 1597 Part II 1967 para 6.1.1. shall apply. By fine tooled dressing or  fine dressing is meant the finest surface which can be given to a stone with a chisel and without rubbing.

  - In the absence of further definition cut stone masonry shall be executed with a fine dressing for the face vide the accompanying figures.

  - Masonry shall be laid as per I.S. 1597 Part II – 1967 and I.S. 1127 – 1970.

  - In walls 75 cm thick and under the headers shall run right through the wall.

- Fixing measurement and payment for certain features instructions in I.S. 1200 Part iv – 1970 shall apply.

- All fine dressed cut stone work shall be protected by means wood boxing immediately after fixing at the contractors expense.

CUT STON IN SURKI MORTAR

- The specification shall be the same as specified above. The mortar shall be as per I.S. 2250 – 1965.

CUT STONE STRING COURSE BOND AND OTHER CORNICES ETC. IN MORTAR

ASHLAR ARCHING

Additional clauses to “General” preceding

- The arch stones shall be closely of the same size and carefully and accurately wrought, giving the proper radiating joints, that is the arch stones shall be dressed full and true to their proper shapes and shall be carefully set in good fine mortar. Thickness of ring  and class of mortar will be defined in the relevant schedule item.

  - The arch stones shall not be less than 25 cm on their least dimension  and header joints shall break joint atleast 22.5 cm. In arches upto 60 cm in thickness, the stones shall all be of the full thickness of ring. For deeper arch rings, the Executing Engineer will supply instructions regarding bond, face stones and key stones.

  - Exact uniformity will be required in the thickness of the arch ring, and in oblique or skew arches, great care is to be taken to dress the beds to the required winding.

  - All arch work  stones should be of special large stones. The breadth of each stone at top and bottom will be determined at the time of execution according to the span and number of stones.

- Dressing : The header and side radial joints shall be fine tooled to exact fit on all beds ; the keying course shall be accurately fitted in and  driven into its place with heavy wooden beaters.

  - The side or radial joints shall be perpendicular to the tangent of the curve of the arch at each point and a right angles to the face. The soffit shall be dressed to the true curve of the arch and the back or upper surface of the arch stone shall be dressed to the true curve ofextrados or each stone shall have it supper surface dressed to a plane flat surface according as the drawing may indicate. The thickness of each joint shall not exceed 3 mm.

- Face stones : The face  stones shall be fine tooled, or rock faced as specified or shown  on the drawings. It will also be separately as specified if chamfering is to be done to the exposed radial joints of face stones and horizontal joints of the soffit.

- Centres : - The contractor shall furnish for the approval  of the Executive Engineer, plans for the centers of all arches up to 4.5 m. pan. For larger span arches, the Executing Engineer will prepare centering plans and furnish the same  to the contractor, except in the case of lump sum contracts, where the contactor shall prepare and submit the plans of the centers proposed to be used for prior approval by the Executive Engineer. For spans larger than 1.8 m timber centers are to be used and are to be provided with hardwood wedges for slackening, the centers  sand boxes shall be used for slackening  for spans over 6m. In all centers, the arrangement shall be such that the slackening can be effected without any vibration being transmitted to the arch and in the case of a series of arches that the centers can all be  slackened simultaneously. If the centers are not of clear span, butt rest on intermediate brick pillars, the brick pillars, shall be well and strongly built, the top of each pillar being capped with wood and connected up with other pillars by a continuous  piece of timber from abutment or abutment.

  - In all centers, the upper bearing surface shall be very correctly formed to the curve of the intrados of the arch.

  - The contractor shall obtain instructions from the Executive Engineer, regarding the number of hours after completion of the arch that the centers shall be sufficiently  slackened to compress the joints (re-tightening at once) and also  the time for complete removal. When using a lime mortar centers will ordinarily be slackened within 24 hours after completion of the arch. Care shall be taken however that the centers are not slackened while the mortar in the last finished joints is still so soft that it will be squeezed out, but at the same time, the centers shall be slackened while the mortar is still moist, so as to allow the arch to compress itself and bring all the joints to fair bearing. In the case of a segmental arch care shall be taken  to see that the skew backs are secure        (they are better to be given a week’s time to set and in the case of semi 0- circular, elliptical, pointed or other arches springing from a horizontal joint, that the adjacent wall has been built up to two thirds of the height of the arch, before  slackening centers. With a quick - setting cement mortar, centers should be left will the mortar has finally set.

  - During the progress of the work, care must be taken to distribute the load on the centers in order to obtain a true curve at the completion of the work.  Before  turning arches exceeding 12m. span the middle half of the center shall be loaded with all the material which is to be used on that portion of the arch ring.

  - The rate for arch work is to include the provision of proper centers as described above – together with sand boxes setting up easing and removing the same irrespective of the size of the  arch. ( centers or sand boxes are to be supplied by the department to the contractor, then such will be entered  in the tender notice and the rates shall be tendered accordingly )

- Removal of bad work – If any arch settles unduly or becomes unsightly through  carelessness, bad workmanship, centering or bad material, it shall be removed  or re-built at the contractor’s expense.

- Measurement – Measurement of arch  work  shall be the mean of the lengths of extrudes,  the full breadth of the arch  and the full thickness of the ring.

RUBBLE  ARCHING

Additional clauses to “General” proceeding

- Rubble arching  shall consist of  flat bedded slabs  laid solidly and flush in mortar  with their radial  joints perpendicular to the tangent to the curve of the arch at each point and the arch stone shall properly  break joint a heading joints. The mortar to be used shall be standard specification mortar of the class described in the Schedule item  wording and the thickness of arch ring will be defined therein. Each  slab shall be hammer dressed approximately to the proper shape,  with the necessary summering so that the arch stone may bear fairly one upon another for the full thickness of the arch ring, and should parts for the full thickness of their arch ring, and should  parts of the backs  of the stones be open they shall be solidly wedged up with spalls and chips of stones set in mortar. The exposed faces of the face or quoin stone shall be bushed or plain dressed as specified with face perpendicular to the length inwards. The soffit shall  be dressed to a plane  surface, true at all points to the curve of the arch.

- The joints on face and soffit shall not exceed 12mm. in thickness.

- The arch quoins shall be of the same thickness as the sheeting of the course, but they shall be superior selected stones, in all cases extending right through  from intrados to extrados and alternately not less  than 37.5 cm and 5.3 cm long perpendicular to the face of  the arch.

- Interior arch stone in the sheeting shall not be less than 75 mm  in least dimension, and the stones shall break joint  with each other 150mm. In arch  rings 37.5 cm. thick  and under, all stones shall be of full thickness of the ring.

- In arch rings more than 37.5 cm thick, the stones will be laid in alternate courses of headers and stretchers, unless otherwise specify. Not more than  two stones going to make up  the thickness of the arch ring.

RUBBLE IN MORTAR ( FIRST  SORT )

1. I.S. 1597 / Part I shall apply

  - The stones shall be solidly bedded set full in mortar with joints not exceeding 12mm  thickness and with no pinning whatever on the face and shall extend well back into the hearting

  - In all walls upto a width 60 cms. Bond  stones running through the wall shall be provided of intervals o f1.8 metres clear in every course. For walls thicker than  1.8 metres a line of headers each header overlapping the other by 30 cms. Or more shall be provided from front to back at 1.8 metres interval in every course. Care shall be taken not to place the bond stones of successive course  over  each other. The position  of bond stones shall be  marked on both the faces for identification and verification ( Tar marking where plaster is to be applied  will suffice )

- under-pinning of any description to be allowed and hollow bedding and flushing such gaps with  mortar are to be carefully avoided.

- The  work on the interior face shall be precisely the same as on the exterior face unless the work is to be plastered, in which case the side joints need not be vertical.

- The rate per cubic metre shall be for  finished work inclusive  of all dressing, chisel drafting etc. The contractor shall therefore tender his rate to cover the dressing required to comply with the specification.

RUBBLE IN MORTAR ( SECOND SORT )

1.  I.S. 1597 Part – I and I.S. 127 shall apply

  - The whole width of the wall will be paid for as coursed rubble in mortar second sort unless it is specified that a certain thickness will be paid for as coursed rubble and the balance as random rubble

N.B. In thick  walls such as lock walls, etc., the Executing Engineer will determine and specify in the tender  notice the width which will be paid for under each class of work.

RANDOM  RUBBLE IN MORTAR

- For details of random rubble work I.S. 1127 and I.S. 1597 Part I  shall apply.

- No pinning whatever shall be used on the face “Bushings” shall not project  beyond the face of the wall more than 12mm. in the faces proposed to be plastered.

- Measurement of stone masonry is covered by I.S. 1200 Part IV  shall apply.

RANDOM RUBBLE IN CLAY

- the mud to be used for mortar shall be  prepared from carefully selected earth  of tenacious nature  to which sand shall be added in such quantity that a dried lump  of mixture shall not show signs of cracking. The mud shall be well trodden and worked into the consistency of a thick paste  by the addition  of sufficient water. The mixture thus made shall be kept for a week allowing water to stand on the top of the mud in a shallow pool. The mud  thus stored up shall be well tamped with water by treading when it is to be used on the work, care being taken to remove all clods and stones.

- The work shall be executed to the same specification as the standard  specification  for random rubble  in mortar with the exception  the clay  of the above description is to be used instead of mortar.

N.B. It is customary with this class of masonry in building to executive top 30 cm. of walls and     30 cm. around doors and windows with random

PRECAST STONE MASONRY  BLOCK WALLING

  - The concrete  used for moulding the pre cast stone block shall  be as specified  in Specification No.28.

  - The stones shall be of such  variety conforming to the requirements of building stones as laid down in Specification No.35.

  - The method of production, dimensions of blocks and moulds, casting or blocks shall be done as specified in Data Sheet No.8 of C.B.R.I. Roorkee.

  - The average compressive strength for various mix proportions  shall be as in Table 2 of Data sheet mentioned above

  - The  blocks shall be used for load bearing and non – load bearing walls. The permissible stresses in the masonry shall be taken from I.S. Code 1905 – 69 “Structural Safety of Buildings Masonry Walls “ depending upon the mortar to be used for laying and compressive strength of the blocks. Masonry bonds  construction and curing shall be  as laid  down in the Data Sheet mentioned above.

  - The data furnished by C.B.R.I. Roorkee shall  be followed for guidance only.

CONCRETE BLOCK MASONRY

- Clerical Blocks

  - Materials – The concrete is a light weight material of uniform porous structure obtained as a result  of aerating the green concrete mix with gas. The main ingredients shall be sand quick lime, cement, fly ash aluminium powder and gypsum.

  - Properties of concrete shall be as given under.

| 1. Density (dry ) | 2,750 Kg / m3 |
| --- | --- |
| 2. Specific gravity | 0.75 |
| 3. Crushing Stress | 50 to 60 Kg. / cm3 |
| 4 Tensile stress | 4 Kg / cm2 |
| 5. Shear stress | 7.5 Kg / cm 3 |
| 6. Bond Stress | 4 Kg / cm2 |
| 7. Modulus of elasticity | 1.96 to 2.25 X 104 Kg. / cm2 |
| 8. Absorption test | Not exceeding 45 per cent to volume in 48 Hours |
| 9. Capillary action | 3 cm in 24 hours |
| 10. Shrinkage | 0.30 mm / m.for variation of 80 per cent humidity |
| 11. Thermal conductivity | 0.155 K cal/sq.mh. (degree c/m ) |
| 12. Fire resisting | Good |

  - Block sizes and finish.- The sizes of  blocks used shall be as follows:-

For load bearing walls  	(1) 59 cm X 24 cm X 19 cm

(2) 59 cm X 15 cm X 19 cm

For Partition walls 	(1)  59 cm X 12 cm X 19 cm

(2) 59 cm X 10 cm X 19 cm

Maximum variation in dimensions shall not be more than ±   mm for height and breadth and ± 3mm. for length. The faces of the blocks shall be rectangular opposite faces shall be  parallel and all arises shall be square and straight.  The angles of the blocks shall be right angles and the edges shall be sharp unless otherwise specified.

Allowable compressive and tensile stress – The maximum compressive stress shall be 50 to 60 Kg / cm2  . The tensile stress shall be 10 to 14 per cent  of the allowance compressive stress.

  - Stacking and handling – Celcrete blocks shall be stock – piled on planks or other supports free from contact with the ground. The blocks shall be  handled with care  and damaged units shall be rejected.

- Celcrete  masonry :

  - Mortar – Cement mortar 1:6 ( one cement and six sand shall be used. For detailed specifications for cement mortar I.S. : 2250 shall apply

  - Strength and stability – Minimum thickness of load bearing walls shall be 15 cm. and minimum nominal thickness of non – load bearing partition wall shall be 10cm. thick

Celcrete masonry shall be  designed in accordance with I.S. : 1905 .

  - Laying – Laying of concrete block  masonry in superstructure shall be as in paragraph  10 of I.S. : 6049 ( code of practice for construction of light weight concrete block masonry)      ( extract  furnished )

  - Finishing – External plastering shall be done if so specified. The specification No. 56 shall apply for plastering with cement mortar.

  - Concrete masonry shall be  constructed for  superstructure only and shall not be used in foundation.

LAYING INSITU CEMENT CONCRETE  FLOORING WITH FLOOR  TOPPING

- I.S. 2371 and 5491  shall apply

For  granolithic concrete floor topping the mix proportion given in I.S. 5491 shall be adopted and as regards specifications on size of panel  laying floor topping etc. the same specifications as given in I.S. 2571 for cement concrete  flooring shall be followed.

NOTES : To make a coloured floor the following materials should be mixed in the quantities stated, with every cubic  metre of top layer cement  concrete.

Red : 	One twelveth  cubic metre red oxide iron powder

Black : 	One sixth cubic metre manganese dioxide

Baff : 	One sixth cubic metre of yellow Ochre of Gopi.

FLOORING WITH 100MM CONCRETE AND  20MM CEMENT CONCRETE

SURFACE ELLIS PATTERN

- The 100 mm concrete base shall be prepared as detailed before under “General” For preparing the base I.S. : 2571  also shall apply

- On the clean wet surface of the concrete base, before it has set, will be laid a layer of cement concrete  to give a finished depth of 20mm over the lime concrete.

The cement concrete will consist of standard specification hard broken stone chippings, graded from gauges 3mm to 10 / mm and well mixed with standard specification. Cement neat, in he proportion 3 broken stone to 1 cement, or as otherwise specified  instructed by the Executive Engineer.

The cement concrete shall be spread  immediately it has been mixed, using a straight edge. The concrete must be well beaten with 2.5 kg wooden ‘thapies’ until cement slurry comes to the surface say for not less than 15 minutes – and it is very important that the consolidation should be carried out quickly, as otherwise the initial set of the cement is likely to spoil the work , if the beating is continued too long or is not sufficiently through. When the mortar has come to the surface, the floor shall be polished with trowels.

- To prevent large areas of cement concrete  from cracking due to construction bonding setting, the floor shall be divided into strips extending across the width of the room, or into squares or rectangles 1.2 m. to 2.5 m in width. The edge of each section into which the floor is divided should be defined by flat bars  of steel wood, their depth being the same as that proposed for the finished floor. They should  be white washed in order to prevent them from adhering to the concrete When the slabs have set, the bars should be removed and the joints filled in, with  standard specification cement mortar 1:2.

- If it is desired to have a fine finish cement may be sprinkled over the surface of the concrete which has set  and rubbed over with polishing stones.

- After the floor has been completed, it should be covered with two inches of grass, sand or saw  dust and kept wet for three weeks. It is better not to be brought  into use for a month after laying.

Notes : (a) Then notes given under 39 F shall apply.

(b) Special care is necessary that  the filling under the lime concrete is wetted and thoroughly consolidated in accordance with the standard  specification for “filling” in basement” – as otherwise the whole floor is liable to crack.

FLOORING WITH CONCRETE  100 MM THICK AND PLASTERING OVER WITH CEMENT MORTAR 1:3 – 12 MM THICK

- For flooring with cement concrete I.S. 2571  shall apply.

  - Standard specification cement plastering 1:3:12mm thick is to be applied to the floor before the concrete  has set in order to obtain a good bond  with the base.

2.  Cement plaster gauges or other gauges should be put on the concrete  floor about 3 metresapart to ensure even thickness.

3.   Cement plastering must be done  in squares or strips or also cracks will appear, if a large surface is done.

FLOORING WITH RED CEMENT PLASTER, POLISHED SURFACE

AND THREAD LINED

  - The concrete base shall be well cleaned brushed free from any dust or loose concrete materials and then wetted. 1 Kg. of  red oxide  of iron per square metre of flooring shall be added to the cement mortar 1:3 and well incorporated during mixing and one coat of the mixture applied to the floor, otherwise conforming to the standard specification” Plastering with cement mortar”. The plaster flooring shall be thread lined in squares diagonal to the sides of the room  and size as instructed by the departmental officer in-charge of the work. The squares  should be of equal size and finished off regularly at the edge or border around the room either at a corner or midline of the square. The surface shall be brought to a fine polished finish by the use of polishing stones.

- The precautions against cracking and instructions for watering given in the standard specification for “Plastering with cement mortar” shall be followed

TERRAZO ( MARBLE CHIPS ) FLOORING LAID IN SITU

The thickness of the under layer shall be measured correct to a mm. the thickness of the top layer shall not be less than that specified.

  - Under layers : Cement concrete of specified mix shall be used and the specifications given below shall apply.

This shall be prepared by mixing graded stone or brick – aggregate of nominal size as specified proportions with required quantity of water.

The grading and quality of aggregates shall be  such as to give minimum compressive strength of 140 Kg. cm and 210 Kg / cm  at 7 days and 28 days respectively in case of mix 1:2:4.

One sample consisting of 6 cubes 15X15X15 cm. shall be taken for every 10 cubic metre or part thereof of cement concrete 1:2:4. The cube tests shall not be carried out in the case the quantity of cement concrete 1:2:4 placed on any day is less than 10 cu. m. unless otherwise specified. For other details refer “R.C.C. work”.

The panels shall be of uniform size, not exceeding 2 sq.m in area  and 2m. in length for inside situations. In exposed situations the length of any side of the panel shall not be more than 1.25 metres cement slurry  at 2.00 Kg. per sq.m. shall be applied before laying of under layer  over the cement concrete  / R.C.C. Surface.

3. Top layer

  - Mortar – the mix for terrazzo topping shall consist cement with or without pigment, marble powder, marble aggregate (Marble chips ) and water. The cement and marble powder  shall be mixed in the proportion of 3 parts of cement to one part marble  powder by weight. For every part of cement marble powder mix the proportion  of  aggregate by volume shall be as follows : -

| Size of aggregates | Proportion of aggregates to binder mix |
| --- | --- |
| (1) | (2) |
| For grade 00,0 and 1 | 1.75 parts |
| For grade 2 and 3 | 1.50 parts |
| For grade 4 and 5 | 1.25 parts |
| Mixed size aggregate | 1.50 parts |

The marble chips shall be white or pink Makrana, black Bhainslana Chittor black, Jaisalmer Yellow, Baroda green, Denardun white,  Chittor pink, yellowPatam Chera (Madras), grey Gadu (Surant) Chittor green and yellow and Alwar black or as specified. It shall be hard should, dense and homogeneous in texture, with  crystalline and coarse grains. It shall be uniform  in colour and  free from strains, cracks, decay and weathering. The maximum thickness of the top layer  for various sizes  of marble aggregates ( marble chips ) shall be  as under :

TABLE – 2

| Grade Number | Size of aggregate in (mm) | Minimum thickness of top layer (mm) |
| --- | --- | --- |
| 00 | 1-2 | 6 |
| 0 | 2-4 | 9 |
| 1 | 4-7 | 9 |
| 2 | 7-10 | 12 |

Where aggregate of size larger than 10 mm are used the minimum thickness of topping shall  not be less than 1 1/3  times the maximum size of the chips. Where large size chips such as 20mm or 25 mm. are  used they shall be used only with a flat shape and bedded on the flat face so as to keep the minimum thickness of wearing layer.

Before starting the work, the contractor shall  get the sample of marble chips approved  by the Executive Engineer. The cement to be used shall be ordinary gray cement, white cement, coloured cement or cement with admixture of colouring matter of approved quality  in the ratio  specified in the description of the item or  in the ratio to get the required shade as ordered by the Executive Engineer. Colouring matter where  specified, shall be mixed  dry thoroughly with the cement and marble powder and then marble chips added and mixed as specified above. The full quantity of dry mixture of mortar  required for a room shall be prepared in a lot in order to ensure a uniform colour. This mixture shall be stored in a dry place and will covered and protected from moisture. The dry mortar shall be mixed with water in the usual way and  when required. The mixed mortar shall be homogeneous  and stiff and contain just sufficient water to make it workable.

The terrazzo topping shall be laid while  the under layer is still plastic, but has hardened  sufficiently to prevent cement from rising to the surface, this is normally achieved between 18 to 24 hours after the under layer has been laid. A cement slurry preferably of the same colour as the topping shall be brushed on the surface immediately before laying is commenced. It shall be laid  to a uniform thickness  slightly more than that specified in order to get the specified finished thickness after rubbing. The surface of the top layer shall be trowelled  over, pressed and brought true to required  level by a straight edge and steel gloats in such a manner  that the maximum  amount of marble  chips  come up and are spread uniformly over the surface.

  - Polishing, curing and finishing – polishing shall be done by machine. About 36 hours after laying the top layer. The surface shall be watered and ground evenly with machine fitted with special rapid cutting grit blocks ( carborundum stone ) of coarse grade ( No.60 ) till the marble chips are  evenly exposed and the floor is smooth. After the first grinding  mud and covered with a grout of cement or / and colouring matter in same mix and proportion as the topping in order to fill any pin holes that appear. The surface shall be allowed to cure for 5 to 7 days and then ground with machine fitted with fine grit blocks  ( No.120 ). The surface is cleaned and repaired as before and allowed to cure again for 3 to 5 days. Finally the third grinding fall be done with machine fitted with  fine grade  grit blocks (No.320) to get  even and smooth surface without pin holes. The finished surface should show the marble chips evenly exposed.

Where use of machine for polishing is not feasible or possible, rubbing, and polishing shall be done by lard, in the same manner as specified for machine polishing except that carborundum  store of coarse grade ( No.60 ) shall be used for the 1st rubbing, stone of medium grade   (No.80 ) for second rubbing and stone of fine grade (No.120) for final  rubbing and polishing.

After the final polish either by machine or by hand, oxalic acid shall be dusted over the surface at 33 gm per square metre sprinkled with water and rubbed hard with a namdah block (Pad of woolen rags ) . the following day, the floor shall be wiped with a moist rag and dried with a soft cloth  and finished clean.

Curing shall be done by suitable means, such  a  laying moist  saw dust or ponding water.

- Precautions : Flooring in lavatories and bathrooms shall be laid after fixing of water closet and squatting pans and floor  traps. Traps shall be plugged, while laying the floors and opened after the floors are cured and cleaned. Any damaged done to W.Cs squatting pans and floor traps  during the execution of work shall by made good of the Contractor.

During cold weather, concreting shall not be done when the temperature falls below 4o C. The concrete placed shall be protected against frost by suitable covering. Concrete damaged by frost  shall be removed and work redone. During hot weather, precautions shall be taken to see that the temperature  of wet concrete does not exceed 38oC. No concreting shall be laid within half  an hour of the closing time of the day, unless permitted by the executive Engineer.

The floor shall be protected from any damage during the execution of work.

5.  Measurements

  - Length and breadth  shall be measured correct to a c.m. before laying skirting , dado or wall plaster. The area as laid shall be calculated in square metre correct to two places of decimal. No deduction shall be made nor extra paid for any opening  in floor of  area upto 0.1. square (10dm2 ). Nothing extra shall be paid for paying the floor at  different levels in the same room or courtyard  mortar, neat gray cement slurry or honey like consistency shall be spread over the mortar at the rate of 4.4 kg of cement per square  metre over each thread. Tiles shall be washed clean and shall be  fixed in this grout one at another, each tile being gently tapped with a wooden mallet till it is properly  bedded, and in level and line with the adjoining tiles. The joints shall be kept as thin as possible  and in straight lines. The surface shall be checked with a straight edge  during laying to obtain a true surface.

12-2.2 : The square end of the tile shall, as far as possible butt against  the riser face of the concrete or brick tread and in any case shall be embedded under the side wall plaster skirting of dado and under the riser tile or other finish  to a depth of not less than 10 mm.

12.2.4 : After the tiles have been laid surplus cement grout shall be cleaned off.

  - Curing polishing and finishing – The specifications shall be as described in 39-J-73 except that polishing of the treads nosing and chequered grooves, after laying, may be done by hand in the same  manner specified under terrazzo tile flooring. Special care shall be taken polish the nosing and the grooves in such a manner as to get a uniform section for the grooves and the nosing  and their finish shall match  with the finish of the flat portion of the tiles.

  - Measurements – Chequered tiles on stair treads shall be insured  in square  metre correct to two places of decimal. Length shall be measured from the finished face of skirting, dado or all plaster as the case may be, correct to a cm. Width shall be measured from the outer edge of the tread to the finished face of riser or in the case of the edge tiles of the landing and wide steps, to the near edge of the chequered stair tread tile.

  - Rates – the rate shall include the cost of all material sand labour involved in all operations described above. No extra shall be payable for cutting the tiles to the size of trends and also for nosing.

BEST TERRACED FLOORING WITH BRICK ON EDGE 75 mm

CONCRETE AND 12 mm. CEMENT PLASTERING 1:3  TO TOP AND BOTTOM

1.1. I.S. : 2199  shall apply.

2. 	The flooring will be carried into the walls for 15 cm. except for  interior cross walls where the flooring will be carried  through the width of the wall and continued in the adjoining room in cases where such rooms are terrace floored.

  - The finished surface of the flooring must be perfectly level unless a slope for drainage is specified or instructed to be given by the Executive Engineer. The portion of flooring in set into walls  will be paid for as walling and not as flooring

BEST TERRACED FLOORING  WITH BRICK ON EDGE , 75mm

CONCRETE 12 mm CEMENT PLASTERING 1:3 TO BOTTOM

AND ON TOP PRESSED TILES SET IN LIME MORTAR AND POINTED WITH CEMENT MORTAR 1:3

  - I.S. : 2119 2 and I.S. : 1237  and I.S. : 1443  shall apply,

FLOORING WITH GRAVEL 15 cm THICK

  - After completion of the bed  in accordance with the standard specification for “Filling in basement” level or sloped as shown in the sanctioned plans  or ordered by the Executive Engineer, standard specification gravel shall be mixed with sand in the proportion of 9:5 ( nine gravel and five sand ) and with water and worked with mammooties till it can be formed into stiff  plastic balls. It shall then be laid in one thickness and well rammed with flat iron or wooden rammers the finished depth of the gravel shall be 15 cm.

Teakwood Flooring 40 mm Thick

  - I.S.: 3670 and N.B. Code Parts V,VI and VII of appendix shall apply.

Extract form I.S. 5766

Code of practice for laying burnt clay brick flooring.

6.3 Laying the bricks.

6.3.1. Soaking of Bricks – To reduce excessive section, the bricks before being laid shall be soaked in clean water and then allowed to drain until they are surface dry.

6.3.2. The bricks shall be laid in plain, diagonal, herringbone or other suitable patterns. The bricks shall either be laid flat or on edge. Damaged bricks shall not be used. Broken bricks shall not be used in flooring except for closing a line.

Bricks shall be  laid on lime mortar or cement mortar bed and each brick shall be properly bedded flat or on edge and set by gentle rapping with handle of trowel or wooden mallet. The inside faces of the bricks shall be smeared with mortar, before the next brick is laid and pressed against it. on completion of a portion of flooring, the vertical joints shall be fully filled with mortar from the top. The surface of the flooring during laying, shall be frequently checked with a straight edge at least 2m  long, so as to obtain a true surface  with the required slope. In case of flat brick flooring, bricks shall be laid with frog down. When the laid flat is plain courses, the units shall be break joints at half the length of the bricks.

  - Joints – Bricks shall be so laid that all the joints are full of mortar. The thickness of the joints shall be between 8 to 10 mm. The joints shall be flush pointed after being raked out 10mm. deep while the mortar is still green.  The raked joints shall be well brushed to remove dust and loose particles and well wetted; and shall be refilled with cement mortar ( 1 cement : 2 sand).

6.5 Curing – The flooring shall be kept wet for at least 7 days after completion. In case of cement lime mortar, curing shell commence two days after the laying and shall continue for seven days.

Code of practice for laying in situ granolithic concrete floor topping.

  - forward.

0.2 Granolithic concrete floor topping is adopted for floors of heavy engineering factories, workshops, garages, warehouses, etc., where the floor is subjected to heavy loads and severe abrasion combined with impact. The granolithic concrete made with specially selected aggregate of high hardness, surface texture and particle shape suitable for use as a wearing finish to floors. Although plain concrete as laid conforming to I.S.2571 would be satisfactory for many purposes granolithic concrete is chosen because of its high abrasion resistance and used for floor toppings wherever abrasion combined with impact is likely to be severe.

4.4. Mix proportions – Mix proportion for the granolithic concrete floor topping for different types of floor finish shall be 1:1:2 (Cement : fine aggregate : coarse aggregate, by volume).

7.1 Floor Topping laid Monolithically with the suspended slab.

Note(1). – In the monolithic method of construction the granolithic concrete floor topping may be regarded as contributing to the structural strength of the suspended floor.

Note (2) – The monolithic construction of granolithic concrete floor topping presents certain difficulties in construction due to other activities, such as plastering of walls and ceilings, fixing of joinery, movement of scaffoldings, ladders, etc., which are likely to damage the floor finish.

7.3. Floor Topping laid over cushioning layer ( See Table 3):

Note – Where lime and good quality of bricks are not available

1:4:8 cement concrete may be used.

Table 3 – DIFFERENT TYPES OF GRANOLITHIC CONCRETE FLOOR TOPPING.

(Clauses 6.1., 6.2 and 7.1 to 7.3)

| Serial Number and Type floor topping. | Thickness of granolithic concrete floor topping in mm, min. | Remarks. |
| --- | --- | --- |
| (1) | (2) | (3) |
| A. Floor topping laid over base concrete on ground. |  |  |
| 1. Floor topping laid monolithically with | 20 | Granolithic base concrete floor topping shall be laid monolithic with the bas concrete |
| 2. Floor topping laid separately on hardened base concrete. | 40 | Granolithic concrete floor topping shall be laid separately over specially prepared surface of set and hardened base concrete. |

| Serial Number and Type floor topping. | Thickness of granolithic concrete floor topping in mm, min. | Remarks. |
| --- | --- | --- |
| B. Floor topping laid over suspended slabs. |  |  |
| 3.Floor topping laid monolithically with the suspended slab. | 20 | The granolithic concrete topping shall be laid and finished monolithic with the suspended slab. For this purpose the topping shall be laid immediately after the structural concrete of the suspended slab has stiffened enough ( but is still green ) to tread over it by placing ranks. |
| 4. Floor topping laid over the hardened suspended slab. | 40 | The topping shall be laid separately over specially prepared surface of set are hardened suspended slab. |
| 5. Floor topping laid over cushioning layer | 40 | Cushioning layer of lime concrete of 40 to 50 mm thickness shall be laid over the prepared surface of suspended slab. Granolithic concrete topping shall be laid as in 4. |

8. Curing :

8.1. Immediately after the flooring surface is finished it shall be protected from rapid drying, by erecting barriers  against  win of draught and strong sunlight. As soon as the surface  had hardened sufficiently to prevent damage to it shall be kept continuously  moist for at least ten days by means of wet gunny bags, 50 mm thick layer of damp  and spread over the surface or pooling water on the surface : During this period  the flooring  shall not be exposed  to any traffic premature exposure  to traffic will lead to damage  to the surface  which cannot be satisfactorily  repaired.

EXTRACT FROM 1.S. 2571

Code of practice  for laying in situ cement concrete flooring

(First Revision )

0.1   Forward

0.2. In situ cement concrete flooring consists essentially of rich  cement concrete , and possesses good wearing properties and facility  of easy cleaning and maintenance which make it suitable for use in houses,  schools, hospitals and light industrial buildings. Depending upon  the amount of loading and the degree of wear resistance needed, the floor finish has to be laid  in various thickness  and a careful  selection  has to be made regarding mix proportions  panel sizes and number of layers. Also the laying operations have to ensure  the proper  bonding of the finish to the base of sub-floor . This code provides necessary guidance in the selection of materials and laying and finishing of cement concrete  flooring for obtaining satisfactory performance.

4.2.1.1. Coarse aggregate : - The grading of  graded coarse aggregate  for cement concrete flooring  mix shall be  within the limits given in table 1. The coarse  aggregate shall generally be the following sizes : -

- Base concrete ( lean Graded from 40 mm and below cement concrete or lime concrete )

- Cement  concrete topping Graded from 16 mm and below of thickness 40 mm and above.

- Cement concrete  topping Graded from 12.5 mm and below  of thickness 25 mm.

- Under layer of cement Grade from 12.5 mm and below concrete  topping two layers.

- The tiles used were 20 cm. 12.5 cm + 10 cm X 9 mm – with

2

a curvature of 30 mm measured at the center of the title. The number of these tiles required for one square metre of   roofing on the above basis was counted and found to be 1.650 without allowance for breakings.

- For slopes of longer lengths than noted above, it will usually be advisable to adopt  other type of roofing or break the slope, by a drop in the roof, carrying away the run – off from the upper parted the roof by guttering.

BEST TILED ROOFING WITH MANGALORE  TILES INCLUDING TEAK OR COUNTRY WOOD  REFERS

  - Clauses additional to “General” Preceding I.S. 2858  shall apply.

  - If it is specifically stated in the Schedule item description or in the specifications that ventilating tiles are to be provided, then two ventilating tiles to every 10 square metres of finished roof surface shall be provided at places pointed out by the  Executive Engineer. The ventilating tiles shall be of Standard pattern and make approved by Executing Engineer and shall be the same colour as the other tiles and well burnt. They shall fit close and tight with the other tiles ; the rate for the roofing shall include the cost of the ventilating tiles in such  case  as they are specified to be provided.

  - The roofing rate does not include the cost of  galvanized iron sheet but it shall include labour, etc. for fixing.

TERRACED ROOFING WITH BRICK ON EDGE, 75 MM. CONCRETE, TWO COURSES OF FLAT TILES IN C.M. 1:3 TO TOP AND ONE COAT OF CEMENT PLASTER 1:3 TO BOTTOM.

- Clauses additional to General Preceding. I.S. 2119 / 62 . 3036 / 65 and N.B.C. Parts V, VII Appendices shall apply.

- After six days or after the concrete laid has preened two courses of flat tiles shall be laid in cement mortar 1:3 mixed with crude oil. The mortar shall be prepared as instructed in S.S. 45 except that the crude oil is to be in the proportion of 10 per cent of the weight of the cement. The tiles shall be laid first course diagonally and the second course square to the parapet thus breaking Joint. The side Joint of tiles shall not be more than 6 mm thick set full mortar. The mortar layer bed over the roof slab shall not be less than 9 mm finished thickness after receiving the first layer of tiles and 9 mm thick  mortar bed between the two layers of tiles. The flat tiles shall be immersed in water for two hours before being used. The tiles shall be laid and finished off as instructed in SS 44 – H.

- Before the work dries up completely, the tile joints shall be raked out and pointed over with cement mortar 1:3 mixed with crude oil prepared as instructed in clause 2 Supra. The joints shall be well rubbed over thin bar trowels and the excess of mortar scrapped off until the surface of the pointing attains a black polish and becomes hard. The ceiling shall be plastered with one coat of Standard specification “Plastering with cement mortar 1:3,12 mm. thick – S.S. No.56. Clauses relating to the perishing of mortar and providing proper openings of drains in S.S. 44 – G shall apply to this S.S. also.

REINFORCED  CONCRETE ROOFING AND TWO COURSES OF

FLAT TILES TO TOP

- Clauses additional to “General Preceding”

The reinforced concrete  roofing shall be laid as per I.S. 456 and N.B. Code Part VI 	Section 5. For extracts of I.S. See Section 4.

- The reinforced concrete roof slab will itself be laid wherever possible at the required slope a secure proper  drainage of the roof. This slope will be defined in the relevant plans or specified by the Executive Engineer. When the required slope  has to be given by means of camber concrete over girders, this concrete – of the same mix as the floor slab -  will be formed on the R.S. beams or on the Tee beams, as the case may be, along with the roof slab. The forms of the roof slab and the girders shall be so adjusted as to enable the moulding  of the camber concrete en-mass with the roof slab.  This camber concrete shall be considered as included in the square metre rate for the roofing unless other method of payment is in t he Schedule.

- As soon as  the setting period for the roof slab is over the top of the slab shall be thoroughly cleaned and wetted. Two courses of 20 mm thick flat tiles shall be laid in cement mortar 1:3 mixed with crude oil and pointed in the same  manner as instructed in clauses 2 and 3 of S.S. No. 44-G.

- As the work proceeds it shall be kept  thoroughly wetted until the mortar has set firm and hard. Watering shall be continued for three weeks after construction.

- Clauses relating to the perishing of mortar and leaving proper  opening for drains in S.S. 44 – F shall apply to this specification also.

- Payment shall be per 10 sq. metres  superficial area on the completed roofing and the rate shall be either inclusive or exclusive of R.C. slab concrete as the case may be according to conditions in the tender schedule – Wide also remarks  in clause 2 supra regarding camber concrete, the bearing of the slab shall be not less than 15 cm. or the depth of the slab without the surface, whichever is greater. This area of bearing only shall be paid for at the roof slab rate irrespective of the actual  bearing when this is greater, unless otherwise specifically defined in writing before a contract  is entered  into. Corresponding deduction for wall masonry work will be made. If the roof slab is carried continuous over the wall  to form a cornice or chajja, then the roof slab shall be measured as instructed and payment for the cornice or chajja portion will be defined in Schedule A of the contract on the lineal metre basis, or as may be otherwise specified, and shall include the portion bearing on the wall apart from that specified above payment as roof slab.

- If the parapet wall is of 20 cm. thick and more than two courses of flat tiles shall be laid continuous upto 50 mm. through the parapet wall. In addition to this , along the junction of the parapet and roof, tiles shall be laid inclined at 45 to the face of the parapet after laying necessary brick jelly concrete ( wide Fig 8-10. The inclined tiles also should be taken inside the wall for a depth of 50mm.

- If the parapet wall  is of 10 cm. thick and more than two courses of flat tiles shall be laid continuous upto 50mm. through the parapet wall. In addition to this, along the junction of the parapet and roof, tiles shall be laid inclined at 45 to the face of the parapet after laying necessary brick jelly concrete ( Vide Fid. 8-1 ). The inclined tiles also should be taken inside the wall for a depth of 50 mm.

- Where  facia work in concrete is adopted instead of parapet wall, then a 20 cm. thick brick work shall be built behind the facia work for a height of 30 cm. Laying of tiles  shall be similar to the specification described above for 20 cm wall ( Fig. 8-4)

- Plastering  of the parapet shall be done  only after the tiles are laid.

- The area of the tiles for which payment is to be made shall be the exposed area of the tiles at top. The contract shall therefore quote his unit rate accordingly.

- The additional brick jelly concrete and tiles at the junction of the parapet with roof shall be measured and paid for separately.

SECTION IX

PLASTERING, POINTING, WHITE WASHING, COLOUR WASHING AND DISTEMPERING

INDIAN STANDARD EXTRACTS

| I.S. Number |  |
| --- | --- |
| 5410 | Specification for Cement Paint, Colour as required |
| 427 | Specification for Distemper, Dry Colour as required |
| 428 | Specification for Distemper, Oil emulsion, Colour as required |
| 1200 | Method of Measurement of Building and Civil Engineering Works – Part XII – Plastering pointing |
| 1597 | Code of practice for Construction of Stone Masonry – Part I - Rubble Stone Masonry |
| 1661 | Code of practice for Cement and Cement – lime Plaster Finishes on Walls and Ceilings |
| 2394 | Code of practice for application of Lime Plaster finish |
| 2402 | Code of practice for external rendered finishes |
| 6278 | Code of practice for White Washing and Colour Washing |

PLASTERING WITH CEMENT MORTAR

  - For plastering stone masonry, the specifications laid down in I.S. 1661 shall  apply.

- Should the mortar crack or perish  through neglect of watering or for other fault of the contractor, the work shall be removed and redone at the contractor’s expense or should the contractor fail to water the work to the satisfaction of the officer – in – charge of the work the latter may supply the requisite men to water to the work properly and charge the cost  to the contractor.

- Where thread lining is to be done, it shall be done to the instructions of the Executive Engineer.

- Vertical joints will be finished with a hard skin similar to the horizontal joints, only flush  and not struck vide  sketch below for brick work : -

- Random rubble joints  will be finished flush in the absence of any modifying specification or schedule item, i.e. pressing the mortar well into the joint  and rubbing smooth as in clause 2  above, restricting the mortar to the width of the joint – finished joint surface being flush with the face of the wall.

- No extra payment over the rate for  masonry will be paid for the method of finishing  joints described above

- Finishing joints as described above is all that is necessary  and done while the work is in progress, before the mortar has set, the joints look as well and neat as any pointing.

POINTING WITH CEMENT MORTAR ( 1 : 3)

  - the work shall be executed as specified for “Pointing with mortar “ except that Portland  Cement mortar using 1 Portland Cement and 3 sand, and conforming to the standard specification for Portland Cement mortar, shall be used instead of lime mortar. The work shall  be executed as rapidly as possible  ( and not again touched, after it has begun to set ) and kept wet  for 7 days thereafter. If other proportion of cement mortar  is to be used, it will be so defined in the relevant schedule item.

WHITE WASHING

  - Specification laid down in I.S. 6278 shall apply.

- Each coat of the white wash is to be allowed to dry and should be inspected by the Subordinate in charge before applying the next coat.

- The contractor shall take precaution to prevent white wash being splashed on the wall, floor and other surfaces and articles not to be white washed. The contractor shall clean all such items to the satisfaction of the Executing Engineer at his own cost. Ladders, poles, scaffolding, etc., shall be shoed with gunny bags to prevent damage or scratching of the walls and floor.

COLOUR WASHING

  - Specifications  laid down in I.S. 6278 shall apply.

DISTEMPERING

  - specification for distempers laid down in I.S. 427  and I.S. 428  shall apply.

  - Wall Surface : - Plaster should present a fine polished surface and be absolutely dry before distempering is commenced. The walls must  be well – cleaned down as no pleasing effect can ever result from the use of distemper unless the ground to be worked upon is perfectly clean. Inequalities and small holes such as nail holes shall be filled with gypsum, and allowed to set hard before the distemper is applied. No extra payment  will be given for this cleaning and  filling, unless  there is separate schedule provision  made for the same, in view  of the magnitude of the work to be done

- Mixing -  Unless  otherwise ordered by the Executing Engineer the mixing of the distemper shall be carried out in accordance with the instructions  issued by the manufacturers of the particular brand of  distemper that is specified  to be used.

- Application

  - One coat shall be applied unless otherwise defined in the relevant schedule item or otherwise ordered by the Executive Engineer. In applying, dip the brush in the distemper and stroke crosswise on the wall  and immediately stroke perpendicularly, then stop.

  - It is recommended that two men shall work together one going round the room applying the distemper from the ceiling downwards as far as he can reach; and the second man following him applying the distemper below.

  - In this way, it is easier to join up and avoid patchy overlaps.

- Brushes

  - The contractor shall use proper  distemper brushes as supplied by firms dealing in distemper.  The brushes shall, each day after work, be washed in hot water and hung up to dry.

  - Old brushes caked with dry distemper shall not be allowed on the work

Notes on distempers

- None of the distempers will stand exposure to heavy rain, nor can any of them be described as washable. No attempt should be made  to wash distempered walls during the monsoon.

- Distempers should, however, only be used  in dry climates as they give poor results in wet localities. To get the full advantage out of these distempers, it is necessary to apply a priming coat as recommended by the makers. Distempers grow dark with age.

- Distempers should not be mixed in a larger quantity than is actually required for day’s work  and hot water should be used in preference to cold  water in preparing the mixture.

- On new lime plastered walls, distempers should be applied in two coats over one coat of priming

- On old lime plastered walls covered with one or two coats of hard dry white wash free from efflorescence or “kalar”. One coat of distemper without priming should be used but only distempers which are lime-proof should be used.

- When the lime plaster is very smooth and no priming coat is used, a coating of warm glue should first be applied, but if rough, a coat of Spanish white or chalk mixed with a solution of glue  is employed to render the surface smooth, the coating when dry, being rubbed as clean and as even as possible.

- For local made distempers a priming coat of milk ( 0.75 kg . per 10 m2) may be used with good results.

- Distempers should be applied in dry weather with a broad  stiff brush, and the first  sort coat should be allowed to thoroughly dry  before the next is laid on. The first coat should always be of a lighter tint and should be applied with care. The brushes  used should never be allowed to rest on the bristles and after  use they should be carefully and thoroughly cleaned.

CEMENT PAINTING

- Specification for cement paint laid down in I.S. 5410 shall apply.

- Surface preparation – Any previous coating, lime work, loose materials and dirt shall be removed  thoroughly be scrapping and washing. In case  where cement paint was previously used, only a slight scrapping with sand paper is necessary to remove all fungus, or organic matters which may be present

The surface shall be wet thoroughly with clean water. When the surface starts drying and there is no sign of water on the wall. ( The surface must feel damp to touch ) the surface is ready for application.

NOTE : On a new surface or in very dry atmosphere where the water is either sucked in or evaporated very fast,  wet the surface with water until a uniform drying is noted. Concentrate only on those parts which are drying earlier than the rest carry on until  uniformity in drying is observed.

- Mixing – Unless otherwise ordered by the Executing Engineer the mixing of the cement paint shall be carried out in accordance with the instructions issued by the manufacturers of the particular brand of cement  paint that is specified to be used.

- Application – The water mixed cement paint shall be kept well stirred before and during use and shall be applied within one hour of preparation. Use flat brush and more horizontally or vertically to ensure perfect covering. While applying the first coat brush hard into the surface to seal up all cavities or pores which may be present. For the second coat apply similarly but finish  with a vertical movement taking care to see that the paint is spread  evenly over the surface. During application frequent  stirring should be done to ensure homogeneous composition.

- Curing – 24 hours after the first cost has been applied the surface shall be saturated with water. The second coat shall be applied when the surface is damp to touch. Re watering the surface shall be done with ample water after 24 hours to ensure  perfect setting of the paint film.

The number of coats necessary shall be as decided by the Executive Engineer

- Weather – Wherever possible, the application of cement paint shall take place in the shade. If this  is impossible, the painted surface shall be wetted with a fine water spray after 4 to 5 hours. Painting during rainy seasons shall be done taking precaution that the painted surface are not hit  by rain for a minimum of 3 to 4 hours as the paints are liable to wash off if rain occurs, within two to three hours.

EXTRACT FROM I.S. 5410

Specification for cement paint, colour as required

- Scope

  - This standard prescribes the requirements and the methods of sampling and test for cement paint, colour as required. The material is intended for use  on walls, ceilings and as an interior or exterior decorative and protective finish.

- Terminology

  - For the purpose  of this standard, the definitions given in, I.S. : 1303 shall apply

- Requirements : -

  - Composition – The material, on analysis as prescribed in appendix A, shall show a chemical composition as given below. The composition of the bulk supply shall be similar to that of registered sample

|  | Percent by weight |
| --- | --- |
| Portland cement | 60 min |
| Hyderated lime | 25 mix |
| Alkali resistance pigment | 3 to 7 |
| Water repellants hygroscopic salts fungicides | In suitable proportions to produce a material to satisfy the requirements of this standard |

#### SECTION X

PAINTING, VARNISHING, ETC.

INDIAN STANDARD – EXTRACTS

| I.S. Number |  |
| --- | --- |
| 75- | Specification for Licensed Oil Raw and Refined |
| 77 | Specification for Licensed Oil Boiled for Paints |
| 78 | Specification for Licensed Oil Boiled for Paints |
| 86 | Specification Oil Paste for Paints to Indian Standard Colours |
| 110 | Specification for Ready mixed Paint, Brushing, Frey filler, for Enamels, for use over Primers |
| 212 | Specification for Crude Coal tar for General Use |
| 337 | Specification for Varnish, Finishing Interior |
| 338 | Specification for Varnish, under coating, Exterior, Natural Resin |
| 340 | Specification for Varnish Mixing |
| 347 | Specification for Varnish, Shellac for General Purposes |
| 533 | Specification for Gum Spirit of Turpentine (Oil of Turpentine ) |
| 1200 | Method of Measurement of Building and Civil Engineering Works, (Part XV) Painting |
| 1477 | Code of practice for Painting of Ferrous metals in Buildings and Allied finishes (Part I ) Pretreatment |
| 1477 | (Part II ) – Painting |
| 2338 | Code of practice for finishing of Wood and Wood Based Materials (Part I ) Operations and Workmanship |
|  | (Part II ) Schedules |
| 2395 | Code of Practice for Painting, Concrete Masonry and Plaster Surfaces (Part I ) Operations and Workmanship |
| C.P.W.D. | Specification for painting |

SECTION X

PAINTING, VARNISHING, etc.

PAINTING

- Materials – (a) All paints, oils and other materials and labour for the painting shall be provided by the contractor and included in his rate per 10 square metre, unless otherwise defined in the scheduled  item wording or relevant supplementary specification.

NOTE : In most cases, the Executing Engineer will issue the paints, raw and boiled licensed oil and other ingredients at a place and price to be noted in the tender  notice and the tenderer will be asked to quote  his price for finished work based on the above departmental supply.

  - When the contractor is specifically permitted to obtain the paint by direct purchase and not by purchase from the department the paint shall be obtained by the contractor, and inspected by the Public Works Department officer in charge of the work, in the original containers as issued by the manufacturer, before the containers are opened by the containers are opened by the contractor.

  - When the contractor is instructed or permitted to mix paints from stiff paint pastes, he shall proceed the follows :

Preparing a pot of paint – Take sufficient stiff paint from the cask or tin, and cover what is left in the cask or tin with a layer of water. Place it in a clean paint pot with linseed  oil ( raw, boiled, or pale boiled, as the case may be _ and mix up with a stick, then add the other ingredients. Then (When a colour is desired ) add the necessary stainer or pigment and mix up again thoroughly. Then take a second clan paint pot, cover it with canvas tied on tightly and work the mixed paint through this canvas, with a brush, into this second paint pot. Take off the canvas cover and the paint is then ready for use, except when a flat finish is  desired for the final coat, when a little more turpentine may  be added.

  - Specifications for oil paste  laid in I.S. 86 to IS 100  shall also apply

  - Specifications for linseed oil laid in I.S. 75 , IS 77  and IS 78 shall also apply.

  - Specifications for turpentine laid  in IS  533  shall apply.

2. Preparing the surface for painting  - (i) For new wood work – Specifications for painting new wood work laid down in IS 2338 (Part I ) shall apply (Particularly ref. Para . 6 )

(ii) For old wood work – Specifications for “Maintenance Painting “ laid down in IS 2338  ( Part II ) shall apply. (Particularly ref. para 4.2. and Table 3 )

If the old painted surface is blistered of flaked, it will be necessary to completely remove the old paint, before repainting.

This will  be specified in the tender notice, and separate schedule rate provided, unless the painting item expressly  states that the rate to be quoted shall be inclusive of removing old paint.

Old paint shall be removed in the manner specified  in the tender notice. When no specification for method of removals mentioned in the tender notice, it shall be removed by burning with an ordinary painters ‘ blow – lamp, or by covering  with kerosene oil or other paint remover and then burning ; afterwards scrapping off the paint. It can also be removed by applying solution composed of

Soft soap 	..	..	½ part

Potash 		..	..	1 part

Quicklime	..	..	½ to 1/3 part

The soap  and potash are to be first dissolved by boiling in water. The lime is then added and the whole applied hot covering the paint, which will be removed with the solution. The solution must be left on for 12 to 34 hours after which the paint may be removed by washing  with hot water.

Another method is as follows : The solution with the ingredients noted below should be mixed in the proportion  given against each and stirred until the bubbling ceases – appalakaram 700 grams, washing soda 700 gm, slaked shell lime 1/70 cubic metre and sufficient water to make it into liquid form to flow freely.

The surface from which the paint is to be removed should be coated thickly using a whitewash brush and the solution left on for about 3 hours or so. The surface should then be washed first with water using sand and a coconut husk brush and afterwards washed clean with a good washing soap and water, using a boot brush. The surface should be allowed to completely dry up before repainting.

When the old paint  has been thoroughly removed, the surface shall be washed, rubbed, down, and holes, etc., stopped, as specified above for new wood work, If the solvent for removal contains alkali a weak acid such a dilute vinegar should be used in washing down to neutralize the alkali. Care must be taken that the wood  is not burnt in spots if a blow lamp is used. All greasy  places shall be brushed over with turpentine, and then washed with soap and water.

(iii) For new iron and steel work – Specifications for  painting new iron and steel work laid down in IS 1477 Part  I  shall apply.

(iv) For old painted surface, iron and steel work – Specifications for “maintenance painting ‘ for steel work laid down in IS 1477 Part I shall apply.

(v) For plastered surface – Specifications for painting on plastered surface laid down in IS 2395 Part I/  and Part II  shall apply.

- Application – No paint shall be applied until the articles have been passed for painting by the Public Works Department Officer in immediate charge of the work.

The paint shall be laid on evenly and properly by means of crossing and laying off, the latter in the direction of the grain for wood work and care shall be taken that the paint is of such consistency that  it runs easily from the brush. The paint must not be allowed to settle in the cans : to prevent this, each painter shall have in his paint can a small smooth stick, with which the paint shall be stirred occasionally. If a skin has formed  on the top of the pain, it shall be carefully removed before the paint is used. To prevent mixed paints from drying of skinning over, they should be kept constantly covered with a think film of water. The paint shall not be stirred with the painting brush.

Specifications laid down in Is 1477 Part II  shall also apply (Particularly ref. Appendix A )

No hair marks from the brush or clogging of paint shall be left on the work or puddles in the corners  of panels, angles of mouldings, etc.,

In painting ( or varnishing ) doors and windows, the putty round the glass shall also be painted ( or varnished, as the case may be ) but the glass must not be smeared. Stains of paint in glass  panes, etc., shall be carefully removed by applying a little turpentine and the whole work left in a clean condition.

4.  General For painting, varnishing, wood – oiling etc., All painting and varnishing shall be neatly and carefully finished. Any damage done by dropping or smearing paint or varnish shall be made  good by the contractor at his own cost. Superfluous  patches or stains of paint  or varnish shall be removed  by means of turpentine.

Method of measurement laid down in IS . 1200 Part XV shall apply.

5. Rate – The contract rate for painting per 10 sqm  shall include the cost of paint, preparing surface to be painted, brushes, cleaning materials and labour, the whole complete in accordance with this specification for the number of coats specified ( including scaffolding, etc. Vide General conditions or contract ) Unless other method of payment is specified  or defined in the relevant schedule item

### PAINTING PRIMING COAT ON WOOD, IRON OR PLASTERED SURFACES

- Materials

  - The priming coat for wood work, iron work or plastered surface shall be as specified in the description of the item.

  - The priming coat shall be prepared at the site of work, or ready made primer of approved brand and manufacture shall be used.

  - Where primer  for wood work is to be mixed at site, it shall be prepared from a mixture of red lead, white lead and double boiled linseed oil in the ratio  of 0.7 kg : 0.7 kg  = 1 litre.

  - Where primer for steel work is to be mixed at site, it shall be prepared from a mixture of red lead, raw linseed oil  and turpentine in the ratio of 2.8 kg : 1 litre

  - The specifications for the base material and thinner for mixed on site primers shall be as follows :

- White lead : The white lead shall be pure and free from adulterants like barium  sulphate and whiting. It shall conform to IS : 103  as amended from time to time.

- Red lead : This shall be in powder form and shall be pure and free from adulterants like brick dust etc. it shall conform to IS : 103  as amended from time to time.

- Red linseed oil – Raw linseed oil shall be lightly viscous 5 but clear and of yellowish colour with light brown tinge. Its specific gravity at a temperature of 30o  C shall be between 0.923 and 0.928. the oil shall be mellow and sweet to the tests with very little smell. The oil shall be of sufficiently matured quality. Oil, turbid or thick, with acid and bitter taste and rancid odour and which remains sticky for a considerable time shall be rejected. The oil shall conform in all respects to IS : 75 as amended from time to time. The oil shall be of approved brand manufacture.

- Double boiled linseed oil – This shall be more viscous than the raw oil, have a deeper colour and specific  gravity between 0.931 and 0.945 at a temperature of 30o C. It shall dry with a glossy surface. It shall conform in all respects to IS : 77  as amended from time to time. The oil shall be of approved brand and manufacture

- Turpentine – Mineral turpentine i.e., petroleum distillate which has the same rate of evaporation as vegetable turpentine (distillate product of Oleoresin of Conifers) shall be used. It shall have no grease or other residue when allowed to evaporate. It shall conform to IS : 83 as amended from time to time.

  - All the above materials shall be of approved manufacture and brought to site in their original packings  in sealed condition.

- Preparation of surface

  - Wood work : The wood work to be painted shall be dry and free from moisture.

The surface shall be thoroughly cleaned. All unevenness shall be rubbed down smooth with sand paper and shall be well dusted. Knots, if any shall be covered with preparation of red  lead made by grinding red lead in oil mixing with strong glue size and used hot.

The surface treated for knotting shall be dry before painting is applied. After the priming coat is applied, the holes and indentation on the surface shall be stopped with glazier’s putty or wood putty ( for specifications for glazier’s putty and wood putty see sub-paras 9’33 and 13.37.1’2 CPWD specification respectively ) . Stopping shall not be done before the priming coat is applied as the wood will absorb the oil in the stopping and the later is therefore liable to crack.

  - Iron and steel work – all rust and scales shall be removed by scraping or by brushing with steel wire brushes. Hard skin of oxide formed on the surface of wrought iron  during rolling which becomes loose by rusting , shall be removed

All dust and dirt shall be thoroughly wiped away from the surface.

If the surface is wet,  it shall be dried before priming coat is undertaken.

  - Plastered surface – The surface shall ordinarily not be painted until it has dried completely. Trial patches of primer shall be laid at intervals and where drying is satisfactory, painting shall be taken  in hand. Before  primer is applied, holes and undulations, shall be filled up with plaster paris and rubbed smooth.

- Application – The primer shall be applied with brushes, worked well into the surface and spread even and smooth. The painting shall be done by crossing and laying off.

- Other details – The specification for “Painting (General) shall hold good so far as they are applicable.

PAINTING WITH SUPERIOR QUALITY READY MIXED PAINT OVER G.I. SHEET

  - Paints – superior quality ready mixed paint, suitable in painting over G.I. sheets, of approved brand and manufacture and of the required shade shall be used.

  - Preparation of surface

  - New work – (a) The painting of new G.I. sheets shall not usually be done till the sheets have weathered for about a year. When new sheets are to be painted before they have weathered, they shall be treated  with a mordant solution prepared by mixing 38 gm. of copper acetate in a litre of soft water or 13 gm. hydrochloric acid in a solution of 13 gm. each of copper chloride, copper nitrate and ammonium chloride dissolve in a litre soft water. This quantity of solution is sufficient for about 235 to 280 sqm. of area and is applied for ensuring proper adhesion of paint. The painting with the mordant solution will be paid for separately.

(b) 	Before painting weathered G.I. sheets, rust patches shall be completely cleaned with coarse emery paper  and brush. All grease marks also  shall be removed  and the surface washed and dried. And rushed surface shall be touched with red mixed paint of red lead.

  - Old work – (a) IF the old paint is firm and sound, it shall be cleaned of grease, smoke, etc., The surface shall then be rubbed down with sand paper and dusted. Rusty patches shall be cleaned up and touched with red lead.

- If the old paint is blistered and flaked, it shall be completely removed. Such removal shall be paid for separately and painting shall be treated as on new work.

- Application – The number of coats to applied shall be as in the description of item. In the case of C.G.I.. sheets, the crown of the corrugations shall be painted first and when these get dried the general coat shall be given to ensure uniform finish over the entire surface without the crowns showing signs of thinning.

The second or additional coats shall be applied when the previous coat has dried

Other details – The specifications for “Painting (General)” shall hold good  so far as they 	are applicable.

SPRAY PAINTING WITH FLAT WALL PAINT ON NEW WORK INCLUDING PRIMING COAT

1. Materials – The priming coat shall consist of “Distempering primer” or “Cement primer” as specified in the description of the item. They shall be approved brand and manufacture.

The flat wall paint shall be of approved brand and manufacture and of the required shade.

2.  Preparation of the surface for priming coat – The surface shall be thoroughly  cleaned of dust, old white or colour wash by washing scrubbing. The surface shall then be allowed to dry for atleast48 hours. It shall then be sand papered to give a smooth and even surface. Any unevenness shall be made good by applying putty, made of plaster of paris, mixed with water on the entire surface including filling up the undulations and then sand papering the same after it is dry.

3.  Application

  - Primer Coat – The specified primer shall be painted or sprayed over the surface in an even and uniform  layer

Note : Flat wall paint shall normally be applied on walls 12 months after their completion, in which case, Distemper primer will suffice. If the walls are to be painted earlier, the primer coat shall consist of cement primer.

  - Painting Coal – When the surface is dry, the spray painting with the wall paint in uniform and even layers will be done to the required number of  coasts. Each coat shall be allowed to dry over night and lightly rubbed with very fine grade of sand paper and loose particles brushed off before the next coat is sprayed.

If after the final coat of wall paints, the surface obtained is not upto the mark, further one or more coats as required shall be given after rubbing down the surface and dusting off all the loose particles to obtain a smooth and even finish.

If the primer  or wall paint gets thickened during the application, I shall be thinned suitably with the thinner  recommended by the manufacturer. Adequate ventilation shall be provided to disperse spray fames. Fitments and floor shall be protected from the spray.

Other details – The specifications for “painting (general) “ shall hold good as far as applicable.

SPECIFICATION

1.   Paint – The specification shall be same as in paragraph 12.28.1.

2.  General – Where the old paint is in sound condition, renewal shall be carried  out as given in para   3 to 5. Otherwise the old paint shall be completely stripped and spray painting shall be carried out as over new work. Such removal shall be paid for separately.

3.  Preparation of surface – The surface shall be washed to remove dust and dirt. A mild detergent  solution like soap water shall be used for washing and the surface shall also be rubbed down lightly with abrasive paper when dry. Any patches appearing on the surface shall first be touched up with a coat of paint. These shall be allowed to dry and then rubbed down lightly.

- Painting – The paint shall then be applied with spraying machine  in uniform and even layer. A second coat shall be applied if considered necessary  by the Engineer – in – charge but only after the first coat is complete and dry hard.

- Other details – The specifications for “Painting (General) “ shall hold good as far as they are applicable.

PAINTING WITH ACID PROOF PAINT

  - Paint – Acid proof paint of approved brand and manufacture and of the required shade shall be used.

  - Preparation  of surface and application – The specifications shall be same as for “Panting with ordinary or superior quality ready mixed paints on new work or old work”

  - Other details – The specifications for “Painting (General)” Shall hold good as for as they are applicable.

PAINTING CAST IRON RAIN WATER, SOIL WASTE AND VENT PIPES AND FITTINGS

- Painting cast iron rain water, soil, waste and vent pipes and fittings including priming coat, on new work.

  - Paints – (a) Primer – The primer shall be prepared on site or shall be approved brand and manufacture as specified in the item.

PERSONNEL PROTECTIVE EQUIPMENT

# NYLON SAFETY BELT

- It shall be made out of Nylon  Webbing  of width  of  45mm.

- It shall be provided with Friction Buckles and  Semi triangular Block Ring

- It shall be provided with the Life line of 25ft. length of 10 mm dia

- The Nylon  rope with one end directly spliced to the back and the other end thimbleful and spliced with the Hook..

- It shall be provided with ISI  mark/supplied with test certificate.

# ACID AND ALKALI RUBBER GOGGLE

- It shall be made out of good quality thick Rubber with sponge lining inside.

- It shall be suitable to fit on the face of any person.

- It shall be fitted with clear, Non shatter able Lenses, or Toughened Lenses, in size 50 mm  dia.

- It shall give protection from splashing of chemicals and acid fumes.

- It shall be provided with adjustable Elastic Band.

## SPECTACLE TYPE SAFETY GOGGLES

- The Frame should be of conventional type used with good quality cellulose Acetate material with reinforcement on the sides and opaque (black in colour)

- The Toughened Lens should be an imported on (English, White Toric, plain, Optically  Neutral) and should withstand  the steel-ball  Impact Test  with ISI mark.

- The thickness of the Lens shall be 2.8 mm.

- The Hinged- type Foldable Wire mesh side – pieces with good Rivets shall be provided.

- Good; durable, Screws and Hinges on the side – legs with  good workmanship must be ensured.

- There should not be any defect either in the frame, or in the Lens, and the Goggles should be of  smooth and fine surface quality.

- The size of the goggles  is to be  marked with Nos. 50 & 52 respectively on the  inner side of one side –  leg.

- The initials of the Maker shall be embossed on the inner side of the other Leg, to identify the firm.

- The goggles should be packed in a good , Safety Case. Completely enclosing the goggles with a cleaner.

- Subjected to 100% inspection.

## GUM BOOTS

- Gum Boots shall be made up of Rubber/PVC material of Duck Back/Tarzan make

- It shall have inner lining of good quality cotton/cloth

- It shall have the carbon steel toe cap to the thickness of 1.5mm and should withstand the impact of 14Kgs/m  and provided with ISI mark/supplied with test certificate

- It shall be covered up to the knee and give protection from splashing of chemicals, oil, water, etc.

- It shall have the resistance to acids, alkali and oils

- It shall be provided with ISI mark/supplied with test certificate.

## GAS WELDING GOGGLES

- They shall be made of Bakelite material with smooth finish

- They shall have the vents on either side for allowing adequate ventilation, but preventing light and dust.

- They shall have a pair of cups and screw cap arrangements to hold the filter lenses

- They shall have a pair of filter lenses in size 50mm dia, shade no.6 of  DIN specification to filter the IR radiation from the gas welding/cutting operation

- They shall have two pairs of clear, protective cover(clear glass) in size 50mm dia.

- They shall have a pair of spring type elastic band with adjustable leather strap

## ACID AND ALKALI PROOF RUBBER GLOVES

- 1.It shall be made up of good quality, thick Rubber in size 14” and 16”.  Black in colour

- .It shall give protection against acid and alkali

3.    It shall be of five finger type easy wear.

## HEAVY DUTY RESPIRATOR

- It shall be made of soft PVC material so as to withstand normal usage, exposure to moisture

- The construction shall be suitable to fit the faces of men

- It shall have a mouth piece with White Cartridge

- The cartridge shall be containing Sodium Hydroxide with Potassium Hydroxide

- It shall absorb gas and afford protection against the following Vapours, gases

1.Acetic acid			2. Bromine

3.Carbolic acid			4. Carbon di oxide

5. Chlorine			6. Hydrochloric acid

7. Hydrogen Chloride		8. Iodine

9. Nitrous gases		10. Nitrogen di oxide

11.Phosgene			12. Sulphur di oxide

13.Sulphurated Hydrogen     	14. Sulphur Trioxide

- It shall be provided with a dust mouth piece provided with cotton pad to be provided to filter the respiration dust

- It shall have a non return inlet valve at the  mouth piece, and side outlet valves

- It shall have adjustable Elastic Strap not less than ¾” width for holding in position without discomfort

- It shall be light in weight

- It shall be provided with 3 nos. spare cotton pad along with each respirator

- It shall be provided with ISI mark/supplied with test certificate

### DUST RESPIRATOR

- It shall be made of soft PVC material so as to withstand normal usage, exposure to moisture

- The construction shall be suitable to fit the faces of  men

- It shall have a mouth piece with cotton pad to be provided to filter the respiration dust

- It shall have Non-return inlet valve at the mouthpiece and side outlet respiration

- It shall have adjustable Elastic Strap not less than ¾” width for holding in position without discomfort

- It shall be light in weight

- It shall be provided with 3 nos. spare cotton pad along with each respirator

- It shall be provided with ISI mark/supplied with test certificate

### LIGHT FUME  RESPIRATOR

1.   It shall  be  made of soft  PVC material  so as to  withstand  normal usage, exposure to

moisture

2.   The construction shall be suitable to fit the faces of  men

- It shall have a mouth piece with cotton pad  and black cartridge containing activated charcoal to filter fumes from paint, oil etc.

- It shall have Non-return inlet valve at the mouthpiece and side outlet respirable

- It shall have adjustable Elastic Strap not less than ¾” width for holding in position without discomfort

- It shall be light in weight

- Spare cotton pad 3 nos. to be provided along with each respirator

- It shall be provided with ISI mark/supplied with test certificate

## INDUSTRIAL HELMETS

- The Helmet shall be made of Fiber Glass Di-electric material with narrow brim

- The surface of the shell shall be natural finish and Light blue colour

- It shall have a Nape strap adjustable Head band equipped with sweat resisting lining.  The width shall not be less than 30mm

- Inner cushioning which is flexible and non absorber of water shall be provided between the head band the shell

- It shall have ventilation gap of 10mm to 20mm

- Inside, the dimension of the head band in the longitudinal direction shall be 20 to 25% greater than the inside dimension of the cross wise direction

- The shell shall have a clearance of less than 30mm between the inside of the top of the crown and top of the wearer’s head

- The wearing height shall not be less than 80 mm

- The head band shall be fitted with at least four crown straps each extending from side of the head band to the other.  The width of the crown strap shall be 20mm

- It shall have the leather chin strap , It must with stand the penetration resistance test and shock absorption test  provided with ISI mark/supplied with test certificate

SAFETY  PRACTICES  DURING  CONSTRUCTION

CONTENTS

- Introduction

2.0	Scope

3.0	Definitions

4.0	General Duties

4.1	General Duties Of Execution Agencies

4.2	General Duties Of Owners

5.0	Safety Practices At  Work Places

5.1	General Provisions

5.2	Means Of Access And Egress

5.3	Housekeeping

  - Precautions Against The Fall Of Materials And Persons,

and Collapse Of Structures

5.5	Prevention Of Unauthorised Entry

5.6	Fire Prevention And Fire Fighting

5.7	Lighting

5.8	Plant, Machinery, Equipment and Hand Tools

6.0	Construction Activities

6.1	Excavation

6.2	Scaffolding, Platforms & Ladders

6.3 	Structural Work, Laying Of Reinforcement & Concreting

6.4	Road Work

6.5	Cutting/Welding

6.6	Working In Confined Spaces

6.7	Working At Heights

6.8	Handling And Lifting Equipment

6.9      Vehicle Movement								Demolition

  - Masonry works.

7.0	First Aid								8.0	Documentation								Safety Awareness & Training

- List of safety codes for civil Works

1.0	INTRODUCTION

Safety in Construction Management deserves utmost attention.  Construction is widely recognized as one of the accident-prone activities. Most of the accidents are caused by inadequate planning, failure during the construction process and/or because of design deficiencies. Besides property loss, accidents also result in injuries and fatalities to the personnel, same needs to be prevented.

The reasons for accidents during construction activities are related to unique nature of the industry, human behavior, difficult work-site conditions, extended odd duty hours, lack of training & awareness and inadequate safety management. Unsafe working methods, equipment failure and improper housekeeping also tend to increase the accident rate in construction.

Ensuring good quality of materials, equipment and competent supervision along with compliance of standard engineering practices shall go a long way to in built safety into the system.

The objective of this standard is to provide practical guidance on  technical and educational framework for safety and health in construction with a view to:

- prevent accidents  and harmful effects on the health of workers arising from employment in construction;

- ensure appropriate safety during implementation of construction;

- provide  safety practice guidelines for appropriate measures of planning, control and enforcement.

- SCOPE

This document specifies broad guidelines on safe practices to be adhered to during construction activities. However, before commencing any job, specific hazards and its effects should be assessed and necessary corrective/preventive actions should be taken by all concerned. The document is intended only to supplement and not to replace or supersede the prevailing statutory requirements, which shall also be followed as applicable. For Personal Protective Equipment, PPE manual of TLC department shall be referred to.  The scope of this document does not include the design aspects and quality checks during construction.

- DEFINITIONS

Definitions of various terminology are given below:

- Adequate, appropriate or suitable are used to describe qualitatively or quantitatively the means or method used to protect the worker.

- Brace: A structural member that holds one point in a fixed position with respect to another point; bracing is a system of structural members designed to prevent distortion of a structure.

- By hand: The work is done without the help of a mechanised tool.

- Competent Authority: A stautory agency having the power to issue regulations, orders or other instructions having the force of law.

- Competent person: A person possessing adequate qualifications, such as suitable training and sufficient knowledge, experience and skill for the safe performance of the specific work. The competent authorities may define appropriate criteria for the designation of such persons and may determine the duties to be assigned to them.

- Execution agency:

Any physical or legal person, having contractual obligation with the owner, and who employs one or more workers on a construction site

- Owner:

Any physical or legal person for whom construction job is carried out.

It shall also include owner's designated representative / consultant / nominee / agent, authorized from time to time to act for and on its behalf, for supervising / coordinating the activities of the execution agency.

- Hazard: Danger or potential danger.

- Guard-rail: An adequately secured rail erected along an exposed edge to prevent persons from falling.

- Hoist: A machine, which lifts materials or persons by means of a platform, which runs on guides.

- Lifting gear: Any gear or tackle by means of which a load can be attached to a lifting appliance but which does not form an integral part of the appliance or load.

- Lifting appliance: Any stationary or mobile appliance used for raising or lowering persons or loads.

- Means of access or egress: Passageways, corridors, stairs, platforms, ladders and any other means for entering or leaving the workplace or for escaping in case of danger.

- Scaffold: Any fixed, suspended or mobile temporary structure supporting workers and material or to gain access to any such structure and which is not a lifting appliance as defined above.

- Toe-board: A barrier placed along the edge of a scaffold platform, runway, etc., and secured there to guard against the slipping of persons or the falling of material.

- Worker: Any person engaged in construction activity.

- Workplace: All places where workers need to be or to go by reason of their work.

- GENERAL DUTIES

4.1	GENERALDUTIESOF EXECUTION AGENCIES

    - Execution agency should:

- Provide means and organization to comply with the safety and health measures required at the workplace.

- Provide and maintain workplaces, plant, equipment, tools and machinery and organize construction work so that, there is no risk of accident or injury to health of workers. In particular, construction work should be planned, prepared and undertaken so that:

- Dangers, liable to arise at the workplace, are prevented;

- Excessively or unnecessarily strenuous work positions and movements are avoided;

- Organization of work takes into account the safety and health of workers;

- Materials and products used are suitable from a safety and health point of view;

- Working methods are adopted to safeguard workers against the harmful effects of chemical, physical and biological agents.

- Arrange for periodic safety inspections by competent persons of all buildings, plant, equipment, tools, machinery, workplaces and review of systems of work, regulations, standards or codes of practice. The competent person should examine and ascertain the safety of construction machinery and equipment.

- Provide such supervision to ensure that workers perform their work with due regard to safety and health of theirs as well as that of others.

- Employ only those workers who are qualified, trained and suited by their age, physique, state of health and skill.

- Satisfy themselves that all workers are informed and instructed in the hazards connected with their work and environment and trained in the precautions necessary to avoid accidents and injury to health.

- Ensure that buildings, plant, equipment, tools, machinery or workplaces in which a dangerous defect has been found should not be used until the defect has been rectified.

- Organize for and remain always prepared to take immediate steps to stop the operation and evacuate workers as appropriate, where there is an imminent danger to the safety of workers.

- Establish a checking system by which it can be ascertained that all the members of a shift, including operators of mobile equipment, have returned to the camp or base at the close of work on dispersed sites and where small groups of workers operate in isolation.

- Provide appropriate first aid, training and welfare facilities to workers as per various statutes like the Factories Act, 1948 etc. and, whenever collective measures are not feasible or are insufficient, provide and maintain personal protective equipment and clothing in line with the requirement as per PPE Manual of TLC Department. They should also provide access to workers to occupational health services.

- Educate workers about their right and the duty at any workplace to participate in ensuring safe working conditions to the extent of their control over the equipment and methods of work and to express views on working procedures adopted as may affect safety and health.

- Ensure that except in an emergency, workers, unless duly authorized, should not interfere with, remove, alter or displace any safety device or other appliance furnished for their protection or the protection of others, or interfere with any method or process adopted with a view to avoiding accidents and injury to health.

- Ensure that workers do not operate or interfere with plant and equipment that they have not been

duly authorised to operate, maintain or use.

- Ensure that workers do not sleep, rest or cook etc in dangerous places such as scaffolds, railway tracks, garages, confined spaces or in the vicinity of fires, dangerous or toxic substances, running machines or vehicles and heavy equipment etc.

- Obtain the necessary   clearance / permits as required and specified by owner

4.2	GENERAL DUTIES OF OWNERS

4.2.1	Owners should:

- co-ordinate or nominate a competent person to co-ordinate all activities relating to safety and health on their construction projects;

- Inform all contractors on the project of special risks to health and safety;

- Ensure that executing agency is aware of the owner's requirements and the executing agency's responsibilities with respect to safetry practices before starting the job.

5.0	SAFETY PRACTICES AT  WORK PLACES

  - GENERAL PROVISIONS

5.1.1	All openings and other areas likely to pose danger to workers should be clearly indicated.

    - Workers & Supervisors should use the safety helmet and other requisite Personal Protective Equipment according to job & site requirement. They should be trained to use personal protective equipment.

5.1.3	Never use solvents, alkalis and other oils to clean the skin.

    - Lift the load with back straight and knees bent as far as possible. Seek the help in case of heavy load.

    - Ensure the usage of correct and tested tools and tackles. Don't allow the make shift tools and tackles.

    - No loose clothing should be allowed while working near rotating equipment or working at heights.

5.2	MEANS OF ACCESS AND ENGRESS

Adequate and safe means of access (at least two, differently located) to and egress from all workplaces should be provided. Same should be displayed and maintained.

5.3	HOUSEKEEPING

5.3.1	Ensure:

- proper storage of materials and equipment;

- removal of scrap, inflammable material, waste and debris at appropriate intervals.

    - Removal of loose materials, which are not required for use, to be ensured. Accumulation of these at the site can obstruct means of access to and egress from workplaces and passageways.

5.3.3	Workplaces and passageways, that are slippery owing to oil, grease or other causes, should be cleaned up or strewn with sand, sawdust, ash etc.

5.4	PRECAUTIONS AGAINST THE FALL OF MATERIALS & PERSONS AND COLLAPSE OF STRUCTURES

    - Precautions should be taken such as the provision of fencing, look-out men or barriers to protect any person against injury by the fall of materials, or tools or equipment being raised or lowered.

5.4.2	Where necessary to prevent danger, guys, stays or supports should be used or other effective precautions should be taken to prevent the collapse of structures or parts of structures that are being erected, maintained, repaired, dismantled or demolished.

    - All openings through which workers are liable to fall should be kept effectively covered or fenced and displayed prominently.

    - As far as practicable, guardrails and toe-boards should be provided to protect workers from falling from elevated workplaces.

5.5	PREVENTION OF UNAUTHORISED ENTRY

    - Construction sites located in built-up areas and alongside vehicular and pedestrian traffic routes should be fenced to prevent the entry of unauthorized persons.

    - Visitors should not be allowed access to construction sites unless accompanied by or authorised by a competent person and provided with the appropriate protective equipment.

  - FIRE PREVENTION AND FIRE FIGHTING

5.6.1	All necessary measures should be taken by the executing agency and owner to:

- avoid the risk of fire;

- control quickly and efficiently any outbreak of fire;

- bring out a quick and safe evacuation of persons.

- Inform unit/fire station control room, where construction work is carried out within existing operating area.

    - Combustible materials such us packing materials, sawdust, greasy/oily waste and scrap wood or plastics should not be allowed to accumulate in workplaces but should be kept in closed metal containers in a safe place.

5.6.3	Places where workers are employed should, if necessary to prevent the danger of fire, be provided with:

- suitable and sufficient fire-extinguishing equipment, which should be easily visible and accessible;

- an adequate water supply at sufficient pressure meeting the requirements.

5.6.4	To guard against danger at places having combustible material, workers should be trained in the action to be taken in the event of fire, including the use of means of escape.

5.6.5	At sites having combustible material, suitable visual signs should be provided to indicate clearly the direction of escape in case of fire.

5.6.6	Means of escape should be kept clear at all times. Escape routes should be frequently inspected particularly in high structures and where access is restricted.

5.7	LIGHTING

5.7.1	Where natural lighting is not adequate, working light fittings or portable hand-lamps should be provided at workplace on the construction site where a worker will do a job.

5.7.2	Emergency lighting should be provided for personnel safety during night time to facilitate standby lighting source, if normal system fails.

5.7.2	Artificial lighting should not produce glare or disturbing shadows.

5.7.3	Lamps should be protected by guards against accidental breakage.

    - The cables of portable electrical lighting equipment should be of adequate size & characteristics for the power requirements and of adequate mechanical strength to withstand severe conditions in construction operations.

5.8	PLANT, MACHINERY, EQUIPMENT AND HAND TOOLS

5.8.1	General Provisions

i) 	Plant, machinery and equipment including hand tools, both manual and power driven, should:

- be of proper design and construction, taking into account health, Safety and ergonomic principles.

- be  maintained  in good working order;

- be used only for work for which they have been designed.

- be operated only by workers who have been authorized and given    appropriate training.

- be   provided   with protective   guards, shields   or   other devices as required.

- Adequate instructions for safe use should be provided.

iii)	Safe operating procedures should be established and used for all plant, machinery and equipment.

iv)	Operators of plant, machinery and equipment should not be distracted while work is in progress.

v)	Plant, machinery and equipment should be switched off when not in use and isolated before any adjustment, clearing or maintenance is done.

vi)	Where trailing cables or hose pipes are used they should be kept as short as practicable and not allowed to create a hazard.

vii)	All moving parts of machinery and equipment should be enclosed or adequately guarded.

viii)	Every power-driven machine and equipment should be provided with adequate means, immediately accessible and readily identifiable to the operator, of stopping it quickly and preventing it from being started again inadvertently.

ix)	Operators of plant, machinery, equipment and tools should be provided with PPEs, including where necessary, suitable ear protection.

    - Hand tools

- Hand tools should be repaired by competent persons.

- Heads of hammers and other shock tools should be dressed or ground to a suitable radius on the edge as soon as they begin to mushroom or crack.

- When not in use and while being carried or transported sharp tools should be kept in sheaths, shields, chests or other suitable containers.

- Only insulated or non-conducting tools should be used on or near live electrical installations.

- Only non-sparking tools should be used near or in the presence of flammable or explosive dusts or vapours.

    - Pneumatic Tools

i)	Operating triggers on portable pneumatic tools should be:

- so placed as to minimize the risk of accidental starting of the machine.

- so arranged as to close the air inlet valve automatically when the pressure of the operator's hand is removed.

ii)	Hose and hose connections for compressed air supply to portable pneumatic tools should be:

- designed  and tested for  the pressure and service for which they are intended;

- fastened securely on the pipe outlet and equipped with the safety   chain,   as appropriate.

- Pneumatic shock tools should be equipped with safety clips or retainers to prevent dies and tools from being accidentally expelled from the barrel.

- Pneumatic tools should be disconnected from power and the pressure in hose lines released before any adjustment or repair is made.

5.8.4	Electrical Tools

- Low voltage portable electrical tools should generally be used.

- All electrical tools should be earthed, unless they are "all insulated" or "double insulated" tools which do not require earthing.

- All electrical tools should get inspected and maintained on a regular basis by a competent electrician and complete records kept.

5.8.5	Engines

i)	Engines should:

- be installed so that they can be started safely and the maximum safe speed cannot be exceeded.

- have controls for limiting speed.

- have devices to stop them from a safe place in an emergency.

ii)	IC engines should not be run in confined spaces unless adequate exhaust ventilation is provided.

iii)	When IC engines are being fuelled:the engine should be shut off.

- care should be taken to avoid spilling fuel;

- no person should smoke or have an naked light in the vicinity.

- a fire extinguisher should be kept readily available.

- Secondary fuel reservoir should be placed outside the engine room.

6.0	CONSTRUCTION ACTIVITIES

The various common activities in construction are as under:

- Excavation

- Scaffolding, Platforms & Ladders

- Structural Work, Laying of Reinforcement & Concreting

- Road Work

- Cutting /Welding

- Working in Confined Space

- Working at Heights

- Handling & Lifting Equipments

- Vehicle Movement

- Demolition

- Masonry Works

The safe practices to be followed during the implementation of above construction activities are given below:

  - EXCAVATION

6.1.1	All excavation work should be planned and the method of excavation and the type of support work required should be decided considering the following:

- the stability of the ground;

- the excavation will not affect adjoining buildings, structures or roadways;

- to prevent hazard, the gas, water, electrical and other public utilities should be shut off or disconnected, if necessary;

- presence of underground pipes, cable conductors, etc.,

- the position of culvert/bridges, temporary roads and spoil heaps should be determined;

    - Before digging begins on site, all excavation work should be planned and the method of excavation and the type of support work required decided.

Safe work permit in the prescribed format 310-013 should be obtained prior to commencement.

    - All excavation work should be supervised.

6.1.4	Sites  of excavations  should be thoroughly inspected:

- daily, prior to each shift and after interruption in work of more than one day;

- after every blasting operation;

- after an unexpected fall of ground;

- after substantial damage to supports;

- after a heavy rain, frost or snow;

- when boulder formations are encountered.

    - Safe angle of repose while excavating trenches exceeding 1.5m depth upto 3.0m should be maintained. Based on site conditions, provide proper slope, usually 450,and suitable bench of 0.5m width at every 1.5m depth of excavation in all soils except hard rock or provide proper shoring and strutting to prevent cave-in or slides.

6.1.6	As far as possible, excavated earth should not be placed within one meter of the edge of the trench or depth of trench whichever is greater.

    - Don't allow vehicles to operate too close to excavated area. Maintain atleast 2m distance from edge of excavation. No load, plant or equipment should be placed or moved near the edge of any excavation where it is likely to cause its collapse and thereby endanger any person unless precautions such as the provision of shoring or piling are taken to prevent the sides from collapsing.

6.1.8	Adequately anchored stop blocks and barriers should be provided to prevent vehicles being driven into the excavation. Heavy vehicles should not be allowed near the excavation unless the support work has been specially designed to permit it.

6.1.9	If an excavation is likely to affect the security of a structure on which persons are working, precautions should be taken to protect the structure from collapse.

    - Barricade at 1m height (with red & white band/self glowing caution board) should be provided for excavations beyond 1.5m depth. Provide two entries/exits for such excavation.

    - Necessary precautions should be taken for underground utility lines like cables, sewers etc. and necessary approvals/clearances from the concerned authorities shall be obtained before commencement of the excavation job.

    - Water shall be pumped/bailed out, if any accumulates in the trench. Necessary precautions should be taken to prevent entry of surface water in trenches.

    - During rains, the soil becomes loose. Take additional precaution against collapse of side wall.

    - In case of mechanized excavation, precaution shall be taken  not to allow anybody to come within one meter of extreme reach of the mechanical shovel. The mechanised excavator shall be operated by a well-trained experienced operator. When not in operation, the machine shall be kept on firm leveled ground with mechanical shovel resting on ground. Wheel or belt shall be suitably jammed to prevent any accidental movement of the machine. Suitable precautions as per manufacturer guidelines should be taken for dozers, graders and other heavy machines.

    - In case of blasting, follow strictly IS:4081-1986 & Indian Explosive Act and rules for storage, handling and carrying of explosive materials and execution of blasting operation.

6.2	SCAFFOLDING, PLATFORMS & LADDERS

    - Metal as material of construction

- A scaffold should be provided and maintained or other equally safe and suitable provision should be made where work cannot safely be done on or from the ground or from part of a building or other permanent structure.

- Scaffolds should be provided with safe means of access, such as stairs, ladders or ramps. Ladders should be secured against inadvertent movement.

- Every scaffold should be constructed, erected and maintained so as to prevent collapse or accidental displacement when in use.

- Every scaffold and part thereof should be constructed :

- in such a way so as not to cause hazards for workers during erection and dismantling;

- in such a way so as guard rails and other protective devices, platforms, ladders, stairs or ramps can be easily put together;

- with sound material and of requisite size and strength for the purpose for which it is to be used and maintained in a proper condition.

- Boards and planks used for scaffolds should be protected against splitting.

- Materials used in the construction of scaffolds should be stored under good conditions and apart from any material unsuitable for scaffolds.

- Couplers should not cause deformation in tubes. Couplers should be made of drop forged steel or equivalent material.

- Tubes should be free from cracks, splits and excessive corrosion and be straight to the eye, and tube ends cut cleanly square with the tube axis.

- Scaffolds should be designed for their maximum load as per relevant code.

- Scaffolds should be adequately braced.

- Scaffolds which are not designed to be independent should be rigidly connected to the building at designated vertical and horizontal places.

- A scaffold should never extend above the highest anchorage to an extent which might endanger its stability and strength.

- Loose bricks, drainpipes, chimney-pots or other unsuitable material should not be used for the construction or support of any part of a scaffold.

- Scaffolds should be inspected and certified:

- before being taken into use;

- at periodic intervals thereafter as prescribed for different types of scaffolds;

- after any alteration, interruption in use, exposure to weather or seismic conditions or any other occurrence likely to have affected their strength or stability.

- Inspection should more particularly ascertain that:

- the scaffold is of suitable type and adequate for the job;

- materials used in its construction are sound and of sufficient strength;

- it is of sound construction and stable;

- that the required safeguards are in position.

- A scaffold should not be erected, substantially altered or dismantled except by or under the supervision.

- Every scaffold should be maintained in good and proper condition, and every part should be kept fixed or secured so that no part can be displaced in consequence of normal use.

    - Lifting appliances on scaffolds

- When a lifting appliance is to be used on a scaffold:

- the parts of the scaffold should be carefully inspected to determine the additional strengthening and other safety measures required;

- any movement of the scaffold members should be prevented;

- if practicable, the uprights should be rigidly connected to a solid part of the building at the place where the lifting appliance is erected.

    - Prefabricated scaffolds

- In the case of prefabricated scaffold systems, the instructions provided by the manufacturers or suppliers should be strictly adhered to. Prefabricated scaffolds should have adequate arrangements for fixing bracing.

- Frames of different types should not be intermingled in a single scaffold.

- Scaffolding shall be erected on firm and level ground.

- All members of metal scaffolding shall be checked periodically to screen out defective / rusted members.  All joints should be properly lubricated for easy tightening.

- Entry to scaffolding should be restricted.

- Erection, alteration and removal shall be done under supervision of experienced personnel.

- Use of barrels, boxes, loose bricks etc., for supporting platform shall not be permitted.

- Each supporting member of platform shall be securely fastened and braced

- Where planks are butt-joined, two parallel putlogs shall be used, not more than 100mm apart, to give support to each plank.

- Platform plank shall not project beyond its end support to a distance    exceeding  4  times   the thickness of plank, unless it is effectively secured to prevent tipping.  Cantilever planks should be avoided.

- The platform edges shall be provided with 150mm high toe board to eliminate hazards of tools or other objects falling from platform.

- Erect ladders in the “four up-one out position”

- Lash ladder secured with the structure.

- Using non-slip devices, such as, rubber shoes or pointed steel ferules at the ladder foot, rubber wheels at ladder top, fixing wooden battens, cleats etc.

- When ladder is used for climbing over a platform, the ladder must be of sufficient length, to extend at least one meter above the platform, when erected against the platform in  “four up-one out position.”

- Portable ladders shall be used for heights not more than 4mt.  Above 4mt flights, fixed ladders shall be provided with at least 600 mm landings at every 6mt or less.

- The width of ladder shall not be less than 300mm and rungs shall be spaced not more than 300mm.

- Every platform and means of access shall be kept free from obstruction.

- If grease, mud, gravel, mortar etc., fall on platform or scaffolds, these shall be removed immediately to avoid slippage.

- Workers shall not be allowed to work on scaffolds during storms or high wind.  After heavy rain or storms, scaffolds shall be inspected before reuse.

- Don't overload the scaffolding. Remove excess material and scrap immediately.

- Dismantling of scaffolds shall be done in a pre-planned sequential manner.

    - Suspended scaffolds / boatswain’s chair

- In addition to the requirements for scaffolds in general as regards soundness, stability and protection against the risk of falls, suspended scaffolds should meet the following specific requirements.

- platforms should be designed and built with dimensions that are compatible with the stability of the structure as a whole, especially the length;

- the number or anchorage should be compatible with the dimensions of the platform;

- the safety of workers should be safeguarded by an extra rope having a point of attachment independent of the anchorage arrangements of the scaffold;

- the anchorage and other elements of support of the scaffold should be designed and built in such a way as to ensure sufficient strength;

- the ropes, winches, pulleys or pulley blocks should be designed, assembled, used and maintained according to the requirements established for lifting gear adapted to the lifting of persons according to national laws and regulations;

- Before use, the whole structure should be checked by a competent person.

    - Bamboo / Casuarina Scaffolding

- In general, it should be avoided as far as possible.

- For construction and maintenance of residential and office buildings,  bamboo / Casuarinas scaffold, if used, should conform to provisions given in lS-3696 (Part 1)-1987.

  - STRUCTURAL WORK, LAYING OF REINFORCEMENT AND CONCRETING

    - General provisions

- The erection or dismantling of buildings, structures, civil engineering works, formwork, false work and shoring should be carried out by trained workers only under the supervision of a competent person.

- Precautions should be taken to guard against danger to workers arising from any temporary state of weakness or instability of a structure.

- Formwork, false work and shoring should be so designed, constructed and maintained that it will safely support all loads that may be imposed on it.

- Formwork should be so designed and erected that working platforms, means of access, bracing and means of handling and stabilizing are easily fixed to the formwork structure.

    - Erection and dismantling of steel and prefabricated structures

- The safety of workers employed on the erection and dismantling of steel and prefabricated structures should be ensured by appropriate means, such as provision and use of:

- ladders, gangways or fixed platforms;

- platforms, buckets, boatswain's chairs or other appropriate means suspended from lifting appliances;

- safety harnesses and lifelines, catch nets or catch platforms;

- Steel and prefabricated structures should be so designed and made that they can be safely transported and erected.

- In addition to the need for the stability of the part when erected, the design should explicitly take following into account:

- the conditions and methods of attachment in the operations of transport, storing and temporary support during erection or dismantling as applicable;

- Methods for the provision of safeguards such as railings and working platforms, and, when necessary, for mounting them easily on the structural steel or prefabricated parts.

- The hooks and other devices built in or provided on the structural steel or prefabricated parts that are required for lifting and transporting them should be so shaped, dimensioned and positioned as:

- to withstand with a sufficient margin the stresses to which they are subjected;

- Not to set up stresses in the part that could cause failures, or stresses in the structure itself not provided for in the plans, and be designed to permit easy release from the lifting appliance. Lifting points for floor and staircase units should be located (recessed if necessary) so that they do not protrude above the surface;

- To avoid imbalance or distortion of the lifted load.

- Storeplaces should be so constructed that:

- there is no risk of structural steel or prefabricated parts falling or overturning;

- storage conditions generally ensure stability and avoid damage having regard to the method of storage and atmospheric conditions;

- racks are set on firm ground and designed so that units cannot move accidentally.

- While they are being stored, transported, raised or set down, structural steel or prefabricated parts should not be subjected to stresses prejudicial to their stability.

- Every lifting appliance should:

- be suitable for the operations and not be capable of accidental disconnection;

- be approved or tested as per statutory requirement.

- Lifting hooks should be of the self-closing type or of a safety type and should have the maximum permissible load marked on them.

- Tongs, clamps and other appliances for lifting structural steel and prefabricated parts should:

- be of such shape and dimensions as to ensure a secure grip without damaging the part;

- be marked with the maximum permissible load in the most unfavourable lifting conditions.

- Structural steel or prefabricated parts should be lifted by methods or appliances that prevent them from spinning accidentally.

- When necessary to prevent danger, before they are raised from the ground, structural steel or prefabricated parts should be provided with safety devices such as railings and working platforms to prevent falls of persons.

- While structural steel or prefabricated parts are being erected, the workers should be provided with appliances for guiding them as they are being lifted and set down, so as to avoid crushing of hands and to facilitate the operations. Use of such appliances should be ensured.

- A raised structural steel or prefabricated part should be so secured and wall units so propped that their stability cannot be imperiled, even by external agencies such as wind and passing loads before its release from the lifting appliance.

- At work places, instruction should be given to the workers on the methods, arrangements and means required for the storage, transport, lifting and erection of structural steel or prefabricated parts, and, before erection starts, a meeting of all those responsible should be held to discuss and confirm the requirements for safe erection.

- During transportation within the construction area, attachments such as slings and stirrups mounted on structural steel or prefabricated parts should be securely fastened to the parts.

- Structural steel or prefabricated parts should be so transported that the conditions do not affect the stability of the parts or the means of transport result in jolting, vibration or stresses due to blows, or loads of material or persons.

- When the method of erection does not permit the provision of other means of protection against fall of persons, the workplaces should be protected by guardrails, and if appropriate by toe-boards.

- When adverse weather conditions such as snow, ice and wind or reduced visibility entail risks of accidents, the work should be carried on with particular care, or, if necessary, interrupted.

- Structures should not be worked on during violent storms or high winds, or when they are covered with ice or snow, or are slippery from other causes.

- If necessary, to prevent danger, structural steel parts should be equipped with attachments for suspended scaffolds, lifelines or safety harnesses and other means of protection.

- The risks of falling, to which workers moving on high or sloping girders are exposed, should be limited by all means of adequate collective protection or, where this is impossible, by the use of a safety harness that is well secured to a strong support.

- Structural steel parts that are to be erected at a great height should as far as practicable be assembled on the ground.

- When structural steel or prefabricated parts are being erected, a sufficiently extended area underneath the workplace should be barricaded or guarded

- Steel trusses that are being erected should be adequately shored, braced or guyed until they are permanently secured in position.

- Load-bearing structural member should not be dangerously weakened by cutting, holing or other means.

- Structural members should not be forced into place by the hoisting machine while any worker is in such a position that he could be injured by the operation.

- Open-web steel joists that are hoisted singly should be directly placed in position and secured against dislodgment.

6.3.3	Reinforcement

- Ensure that workers use Personnel Protective equipment like safety helmet, safety shoes, gloves etc.

- Don't place the hand below the rods for checking clear distance. Use measuring devices.

- Don't wear loose clothes while checking the rods.

- Don't stand unnecessarily on cantilever rods.

- To carry out welding/cutting of rods, safety procedures/precautions as mentioned in Item No. 6.5 to be followed.

- For supplying of rods at heights, proper staging and/or bundling to be provided.

- Ensure barricading and staging for supplying and fixing of rods at height.

- For short distance carrying of materials on shoulders, suitable pads to be provided.

- While transporting material by trucks/trailers, the rods shall not protrude in front of or by the sides of driver's cabin. In case such protrusion cannot be avoided behind the deck, then it should not extend 1/3rd of deck length or 1.5M which ever is less and tied with red flags/lights.

6.3.4	Concreting

- Ensure stability of shuttering work before allowing concreting.

- Barricade the concreting area while pouring at height/depths.

- Keep vibrator hoses, pumping concrete accessories in healthy conditions and mechanically locked.

- Pipelines in concrete pumping system shall not be attached to temporary structures such as scaffolds and formwork support as the forces and movements may effect their integrity.

- Check safety cages & guards around moving motors/parts etc. provided in concreting mixers.

- Use Personal Protective Equipment like gloves, safety shoes etc. asun OCP:FYC:002 while dealing with concrete and wear respirators for dealing with cement.

- Earthing of electrical mixers, vibrators, etc. should be done and verified.

- Cleaning of rotating drums of concrete mixers shall be done from outside. Lockout devices shall be provided where workers need to enter.

- Where concrete mixers are driven by internal combustion engine, exhaust points shall be located away from the worker's workstation so as to eliminate their exposure to obnoxious fumes.

- Don't allow unauthorized person to stand under the concreting area.

- Ensure adequate lighting arrangements for carrying out concrete work during night.

- Don't allow the same workers to pour concrete round the clock. Insist on shift pattern.

- During pouring, shuttering and its supports should be continuously watched for defects.

  - ROAD WORK

6.4.1	Site shall be barricaded and provided with warning signs, including night warning lamps at appropriate locations for traffic diversion.

    - Filled and empty bitumen drums shall be stacked separately at designated places.

6.4.3	Mixing aggregate with bitumen shall preferably be done with the help of bitumen batch mixing plant, unless operationally non-feasible.

6.4.4	Road rollers, Bitumen sprayers, Pavement finishers shall be driven by experienced drivers with valid driving license.

    - Workers handling hot bitumen sprayers or spreading bitumen aggregate mix or mixing bitumen with aggregate, shall be provided with PVC hand gloves and rubber shoes with legging up to knee joints.

6.4.6	At the end of day's work, surplus hot bitumen in tar boiler shall be properly covered by a metal sheet, to prevent anything falling in it,

    - If bitumen accidentally falls on ground, it shall be immediately covered by sprinkling sand, to prevent anybody stepping on it. Then it shall be removed with the help of spade.

    - For cement concrete roads, besides site barricading and installation of warning signs for traffic diversion, safe practices mentioned in the chapter on "Concreting", shall also be applicable.

  - CUTTING/WELDING

6.5.1	Common hazards involved in welding/cutting are sparks, molten metal, flying particles, harmful light rays, electric shocks etc. Following precautions should be taken after getting safe work permit in form  310-016 : -

- A dry chemical type fire extinguisher shall be made available in the work area.

- Adequate ventilation shall be ensured by opening manholes and fixing a shield or forced circulation of air etc, while doing a job in confined space.

- Ensure that only approved and well-maintained apparatus, such as torches, manifolds, regulators or pressure reducing valves, and acetylene generators, be used.

- All covers and panels shall be kept in place, when operating an electric Arc welding machine.

- The work piece should be connected directly to Power supply, and not indirectly through pipelines/structures/equipments etc.

- The welding receptacles shall be rated for 63 A suitable for 415V, 3-Phase system with a scraping earth. Receptacles shall have necessary mechanical interlocks and earthing facilities.

- All cables, including welding and ground cables, shall be checked for any worn out or cracked insulation before starting the job. Ground cable should be separate without any loose joints.

- Cable coiling shall be maintained at minimum level, if not avoidable.

- An energised electrode shall not be left unattended.

- The power source shall be turned off at the end of job.

- All gas cylinders shall be properly secured in upright position.

- Acetylene cylinder shall be turned and kept in such a way that the valve outlet points away from oxygen cylinder.

- Acetylene cylinder key for opening valve shall be kept on valve stem, while cylinder is in use, so that the acetylene cylinder could be quickly turned off in case of emergency. Use flash back arrestors to prevent back-fire in acetylene/oxygen cylinder.

- When not in use, valves of all cylinders shall be kept closed.

- All types of cylinders, whether full or empty, shall be stored at cool, dry place under shed.

- Forced opening of any cylinder valve should not be attempted.

- Lighted gas torch shall never be left unattended.

- Store acetylene and oxygen cylinders separately.

- Store full and empty cylinders separately.

- Avoid cylinders coming into contact with heat.

- Cylinders that are heavy or difficult to carry by hand may be rolled on their bottom edge but never dragged.

- If cylinders have to be moved, be sure that the cylinder valves are shut off.

- Before changing torches, shut off the gas at the pressure reducing regulators and not by crimping the hose.

- Do not use matches to light torches, use a friction lighter.

- Move out any leaking cylinder immediately.

- Use trolleys for oxygen & acetylene cylinder and chain them.

- Always use Red hose for acetylene and other fuel gases and Black for oxygen, and ensure that both are in equal length.

- Ensure that hoses are free from burns, cuts and cracks and properly clamped.

- Avoid dragging hoses over sharp edges and objects

- Do not wrap hoses around cylinders when in use or stored.

- Protect hoses from flying sparks, hot slag, and other hot objects.

- Lubricants shall not be used on Ox-fuel gas equipment.

- During cutting/welding, use proper type goggles/face shields.

  - WORKING IN CONFINED SPACES

6.6.1	Following safety practices for working in confined space like towers, columns, tanks and cellars should be followed in addition to the safety guidelines for specific jobs like scaffolding, cutting/welding etc.

- Entry inside the confined area and to carry out any job should be done after issuance of valid permit only, in line with form 310-017.

- Ensure proper and accessible means of exit before entry inside a confined space.

- The number of persons allowed inside the area should be limited to avoid overcrowding.

- When the work is going on in the confined space, there should always be one man standby at the nearby man way.

- Before entering inside the area -  underground or located at lower elevation, probability of dense vapours accumulating nearby should also be considered in addition to inside the vessel.

- Ensure requisite O2 level before entry in the confined space and monitor level periodically or other wise use respiratory devices.

- Check for no Hydrocarbon or toxic substances before entry and monitor level periodically or use requisite Personal Protective Equipment.

- Ensure adequate ventilation or use respiratory devices.

- Depending upon need, necessary respirator system, gas masks and suit shall be worn by everyone entering confined space. In case of sewer, manholes or in the confined area where there is a possibility of toxic or inert gas, gas masks shall be used by everyone while entering.

- Barricade the confined spaces

- Use 24V flameproof lamp fittings only for illumination.

- Use tools with air motors or electric tools with maximum voltage of 24V.

- House keeping shall be well maintained.

- Safety helmet, safety shoes and safety belt shall be worn by everyone entering the confined space.

- Don't wear loose clothing while working in a confined space.

- The gas cylinders used for cutting/welding shall be kept outside.

- All cables, hoses, welding equipment etc., shall be removed from confined space at end of each work day, even if the work is to be resumed in the same space the next day.

- To the extent possible sludge shall be cleared and removed from outside before entering.

- No naked light or flame or hot work such as welding, cutting and soldering should be permitted inside a confined space or area unless it has been made completely free of the flammable atmosphere, tested and found safe by a competent person. Only non-sparking tools and flameproof hand lamps protected with guard and safety torches should be used inside such confined space or area for initial inspection, cleaning or other work required to be done for making the area safe.

- Communication should be always maintained between the worker and the attendant.

6.7	WORKING AT HEIGHTS

6.7.1	General Provision

- While working at a height of more than 2 meters, ISI approved safety belt shall be used.

- While working at a height of more than 2 meters, permit should be issued by competent person in form 310-015, before commencement of the job.

- Worker should be well trained on usage of safety belt including its proper usage at the time of ascending/descending.

- All tools should be carried in tool kits to avoid their falling.

- If the job is on fragile/sloping roof, roof walk ladders shall be used, in addition to getting safe work permit in form 310-012.

- Provide lifeline wherever required.

- Additional safety measures like providing Fall Arrestor type Safety belt, safety net should be provided depending upon site conditions, job requirements.

- Keep working area neat and clean. Remove scrap material immediately.

- Don't throw or drop material/equipment from height.

- Avoid jumping from one member to another. Use proper passageway.

- Keep both hands free while climbing. Don't try to bypass the steps of the ladder.

- Try to maintain calm at height. Avoid over exertion.

- Avoid movements on beam.

- Elevated workplaces including roofs should be provided with safe means of access and egress such as stairs, ramps or ladders.

6.7.2	Roof Work

- All roof-work operations should be pre-planned and properly supervised.

- Roof work should only be undertaken by workers who are physically and psychologically fit and have the necessary knowledge and experience for such work.

- Work on roofs shouldn't be carried on in weather conditions that threaten the safety of workers.

- Crawling boards, walkways and roof ladders should be securely fastened to a firm structure.

- Roofing brackets should fit the slope of the roof and be securely supported.

- Where it is necessary for a person to kneel or crouch near the edge of the roof, necessary precautions should be taken.

- On a large roof where work have to be carried out at or near the edge, a simple barrier consisting of crossed scaffold tubes supporting a tubing guardrail may be provided.

- All covers for openings in roofs should be of substantial construction and be secured in position.

- Roofs with a pitch of more than 10 should be treated as sloping.

- When work is being carried out on sloping roof, sufficient and suitable crawling boards or roof ladders should be provided and firmly secured in position.

- During extensive work on the roof, strong barriers or guardrails and toe-boards should be provided to stop a person from falling off the roof.

- Where workers are required to work on or near roof or other places covered with fragile material, through which they are liable to fall, they should be provided with suitable roof ladders or crawling boards strong enough and when spanning across the supports for the roof covering to support those workers.

- A minimum of two boards should be provided so that it is not necessary for a person to stand on a fragile roof to move a board or a ladder, or for any other reason.

6.8	HANDLING AND LIFTING EQUIPMENT:

6.8.1	General Provisions

Following are the general guidelines to be followed with regard to all types of handling and lifting equipment in addition to the guidelines for specific type of equipments dealt later on.

- There should be a well-planned safety programme to ensure that all the lifting appliances and lifting gear are selected, installed, examined, tested, maintained, operated and dismantled with a view to preventing the occurrence of any accident;

- All lifting appliances shall be examined by competent persons at frequencies as specified in "The Factory act".

- Check thoroughly quality, size and condition of all lifting tools like chain pulley blocks, slings, U-clamps, D-shackles etc. before putting them in use.

- Safe lifting capacity of all lifting & handling equipment, tools and shackles should be got verified and certificates obtained from competent authorities before its use. The safe working load shall be marked on them.

- Check periodically the oil, brakes, gears, horns and tyre pressure of all moving equipments like cranes, forklifts, trailers etc as per manufacturer's recommendations.

- Check the weights to be lifted and accordingly decide about the crane capacity, boom length and angle of erection.

- Allow lifting slings as short as possible and check packing at the friction points.

- While lifting/placing of the load, no unauthorised person shall remain within the radius of the boom and underneath the load.

- While loading, unloading and stacking of pipes, proper wedges shall be placed to prevent rolling down of the pipes.

- Control longer jobs being lifted up from both ends.

- Only trained operators and riggers should carry out the job. While the crane is moving or lifting the load, the trained rigger should be there for keeping a vigil against hitting any other object.

- During high wind conditions and nights, lifting of heavy equipments should be avoided. If unavoidable to do erection in night, operator and rigger should be fully trained for night signaling. Also proper illumination should be there.

- Allow crane to move on hard, firm and leveled ground.

- When crane is in idle condition for long periods or unattended, crane boom should either be lowered or locked as per manufacturer's guidelines.

- Hook and load being lifted shall remain in full visibility of crane operators, while lifting, to the extent possible.

- Don't allow booms or other parts of crane to come within 3 meters reach of overhead electrical cables.

- No structural alterations or repairs should be made to any part of a lifting appliance, which may affect the safety of the appliance without the permission and supervision of the competent person.

6.8.2 	Hoists

- Hoist shafts should be enclosed with rigid panels or other adequate fencing at:

- ground level on all sides;

- all other levels at all points at which access is provided;

- all points at which persons are liable to be struck by any moving part.

- The enclosure of hoist shafts, except at approaches should extend where practicable at least 2mt above the floor, platform or other place to which access is provided except where a lesser height is sufficient to prevent any person falling down the hoist way and there is no risk of any person coming into contact with any moving part of the hoist, but in no case should the enclosure be less than 1mt in height.

- The guides of hoist platforms should offer sufficient resistance to bending and, in the case of jamming by a safety catch, to buckling.

- Where necessary to prevent danger, adequate covering should be provided above the top of hoist shafts to prevent material falling down them.

- Outdoor hoist towers should be erected on firm foundations, and securely braced, guyed and anchored.

- A ladder way should extend from the bottom to the top of outdoor hoist towers, if no other ladder way exists within easy reach.

- Hoisting engines should be of ample capacity to control the heaviest load that they will have to move.

- Hoists should be provided with devices that stop the hoisting engine as soon as the platform reaches its highest stopping place.

- Winches should be so constructed that the brake is applied when the control handle is not held in the operating position.

- It should not be possible to set in motion from the platform a hoist, which is not designed for the conveyance of persons.

- Winches should not be fitted with pawl and ratchet gears on which the pawl must be disengaged before the platform is lowered.

- Hoist platforms should be capable of supporting the maximum load that they will have to carry with a safety factor.

- Hoist platforms should be equipped with safety gear that will hold the platform with the maximum load if the hoisting rope breaks.

- If workers have to enter the cage or go on the platform at landings there should be a locking arrangement preventing the cage or platform from moving while any worker is in or on it.

- On sides not used for loading and unloading, hoist platforms should be provided with toe-boards and enclosures of wire mesh or other suitable material to prevent the fall of parts of loads.

- Where necessary to prevent danger from falling objects, hoist platforms should be provided with adequate covering.

- Counterweights consisting of an assemblage of several parts should be made of specially constructed parts rigidly connected together.

- Counterweights should run in guides.

- Platforms should be provided at all landings used by workers.

- Following notices should be posted up conspicuously and in very legible characters:

(a) 	on all hoists:

- on the platform: the carrying capacity in kilograms or other appropriate standard unit of weight;

- on the hoisting engine: the lifting capacity in kilograms or other appropriate standard unit of weight;

(b) 	on hoists authorised or certified for the conveyance of persons:

- on the platform or cage: the maximum number of persons to be carried at one time;

(c) 	on hoists for goods only:

- on every approach to the hoist and on the platform: prohibition of use by persons.

- Hoists intended for the carriage of persons should be provided with a cage so constructed as to prevent any person from falling out or being trapped between the cage and any fixed part of the structure when the cage gate is shut, or from being struck by the counterbalance weight or by articles or materials tailing down the hoist way.

- On each side in which access is provided, the cage should have a gate fitted with devices which ensure that the gate cannot be opened except when the cage is at a landing and that the gate must be closed before the cage can move away from the landing.

- Every gate in the enclosure of the hoist shaft which gives access from a landing place to the cage should be fitted with devices to ensure that the gate cannot be opened except when the cage is at that landing place, and that the cage cannot be moved away from that landing place until the gate is closed.

6.8.3 	Derricks

Stiff-leg derricks

- Derricks should be erected on a firm base capable of taking the combined weight of the crane structure and maximum rated load.

- Devices should be used to prevent masts from lifting out of their seating.

- Electrically operated derricks should be effectively earthed from the sole plate or framework.

- Counterweights should be so arranged that they do not subject the backstays, sleepers or pivots to excessive strain.

- When derricks are mounted on wheels:

- a rigid member should be used to maintain the correct distance between the wheels;

- they should be equipped with struts to prevent them from dropping if a wheel breaks or the derrick is derailed.

- The length of a derrick jib should not be altered without consulting the manufacturer.

Guy derricks

- The restraint of the guy ropes should be ensured by fitting stirrups or anchor plates in concrete foundations.

- The mast of guy derricks should be supported by six top guys spaced approximately equally.

- The spread of the guys of a guy derrick crane from the mast should not be more than 450 from the horizontal.

- Guy ropes of derricks should be equipped with a stretching screw or turnbuckle or other device to regulate the tension.

- Gudgeon pins, sheave pins and fool bearings should be lubricated frequently.

- When a derrick is not in use, the boom should be anchored to prevent it from swinging.

6.8.4 	Lifting ropes

- Only ropes with a known safe working capacity should be used as lifting ropes.

- Lifting ropes should be installed, maintained and inspected in accordance with manufacturers' instructions.

- Repaired steel ropes should not be used on hoists.

- Where multiple independent ropes are used, for the purpose of stability, to lift a work platform, each rope should be capable of carrying the load independently.

6.9	VEHICLE MOVEMENT

6.9.1	Park vehicles only at designated places. Don't block roads to create hindrance for other vehicles.

    - Don't overload the vehicle.

6.9.3	Obey speed limits and traffic rules.

6.9.4	Always expect the unexpected and be a defensive driver.

6.9.5	Drive carefully during adverse weather and road conditions.

6.9.6	Read the road ahead and ride to the left.

6.9.7	Be extra cautious at nights. Keep wind screens clean and lights in working condition.

    - All vehicles used for carrying workers and construction materials must undergo predictive/preventive maintenance and daily checks

    - Driver with proper valid driving license shall only be allowed to drive the vehicle

6.9.10	Routes shall be leveled, marked and planned in such a way so as to avoid potential hazards such as overhead power lines and sloping ground etc.

6.9.11	While reversing the vehicles, help of another worker should be ensured at all times

6.9.12	An unattended vehicle should have the engine switched off

6.9.13	Wherever possible one-way system shall be followed

    - Barriers/fixed stops should be provided for excavation/openings to prevent fall of vehicle

6.9.15	Load should be properly secured

6.9.16	The body of the tipper lorry should always be lowered before driving the vehicle off.

    - Signs/signals/caution boards etc. should be provided on routes .

6.10	DEMOLITION

6.10.1. General provisions

- When the demolition of any building or structure might present danger to workers or to the public:

- necessary precautions, methods and procedures should be adopted, including those for the disposal of waste or residues;

- the work should be planned and undertaken only under the supervision of a competent person.

- Before demolition operations begin:

- structural details and builders' drawings should be obtained wherever possible;

- details of the previous use should be obtained to identify any possible contamination and hazards from chemicals, flammables, etc.;

- an initial survey should be carried out to identify any structural problems and risks associated with flammable substances and substances hazardous to health.

The survey should note the type of ground on which the structure is erected, the     condition of the roof trusses, the type of framing used in framed structures and the load-bearing walls;

- a method of demolition should be formulated after the survey and recorded in a method statement having taken all the various considerations into account and identifying the problems and their solutions;

- All electric, gas, water and steam service lines should be shut off and, as necessary, capped or otherwise controlled at or outside the construction site before work commences.

- If it is necessary to maintain any electric power, water or other services during demolition operations, they should be adequately protected against damage.

- As far as practicable, the danger zone round the building should be adequately fenced off and sign posted. To protect the public a fence 2m high should be erected enclosing the demolition operations and the access gates should be secured outside working hours.

- The fabric of buildings contaminated with substances hazardous to health should be decontaminated. Protective clothing and respiratory devices should be provided and worn.

- Where plant has contained flammable materials, special precautions should be taken to avoid fire and explosion.

- The plant to be demolished should be isolated from all other plant that may contain flammable materials. Any residual flammable material in the plant should be rendered safe by cleaning, purging or the application of an inert atmosphere as appropriate.

- Care should be taken not to demolish any parts, which would destroy the stability of other parts.

- Demolition activities should not be continued under adverse climatic conditions such as high winds, which could cause the collapse of already weakened structures.

- To prevent hazards parts of structures should be adequately shored, braced or otherwise supported.

- Structures should not be left in a condition in which they could be brought down by wind pressure or vibration.

- Where a deliberate controlled collapse technique is to be used, expert engineering advice should be obtained, and:

- it should only be used where the whole structure is to come down because it relies on the removal of key structural members to effect a total collapse;

- it should only be used on sites that are fairly level and where there is enough surrounding space for all operatives and equipment to be withdrawn to a safe distance.

- When equipment such as power shovels and bulldozers are used for demolition, due consideration should be given to the nature of the building or structure, its dimensions, as well as to the power of the equipment being used.

- If a swinging weight is used for demolition, a safety zone having a width of at least one-and-a-half times the height of the building or structure should be maintained around the points of impact.

6.10.2. 	Demolition of structural steelwork

- All precautions should be taken to prevent danger from any sudden twist, spring or collapse of steelwork, ironwork or reinforced concrete when it is cut or released.

- Steel construction should be demolished tier by tier.

- Structural steel parts should be lowered and not dropped from a height.

  - MASONRY WORKS

- Ensure proper stacking of bricks as per standard practice.

- Ensure stability of scaffold and working platform before commencing the work.

- Ensure usage of relevant PPE like safety belt etc

- Ensure mortar mixing, platform sufficiently away from the work spot.

- Do not raise more than the stipulated height at a time.

- Permit further raising of structure only after adequate curing.

- Remove unused materials and debris from work spot immediately after the completion of work.

7.0	FIRST AID

First aid facilities should be provided in line with various statutory regulations like factory act etc. However following care should be taken:

- First aid, including the provision of trained personnel should be ensured at work sites. Arrangement should be made for ensuring the medical attention of the injured workers. First aid box should be as per the Factory rules.

- Suitable rescue equipment, like stretchers should be kept readily available at the construction site.

- First-aid kits or boxes, as appropriate and as per statutory requirements, should be provided at workplaces and be protected against contamination by dust, moisture etc.

- First-aid kit or boxes should not keep anything besides material for first aid in emergencies.

- First-aid kits and boxes should contain simple and clear instructions to be followed, be kept under the charge of a responsible person qualified to render the first aid and be regularly inspected and stocked.

- Emergency telephone numbers of nearby Hospitals, Police, Fire Station and Administration should be prominently displayed.

8.0	DOCUMENTATION

The intention of keeping documentation of all types of accident(s) is to prevent recurrence of similar accident(s). All accidents should be reported as per  Guidelines and Factories Act, 1948.

All accidents (major, minor or near miss) should be investigated, analyzed and recommendations should be documented along with implementation status.

All related data should be well-documented and further analysis highlighting the major cause(s) of accidents be done. This will help in identifying thrust areas and training needs for prevention of accidents.

9.0	SAFETY AWARENESS & TRAINING

Safety awareness to all section of personnel ranging from site-in-charge to workmen helps not only  preventing  the risk but also build up the confidence. Time and expenditures also get saved as a result.

Safety awareness basically seeks to persuade/inform people on safety besides supplementing skill also. Awareness programme may include followings:

- Poster: Posters with safety slogan in humorous, gruesome demonstrating manner may be used to discourage bad habits attributable to accidents by appealing to the workers' pride, self-love, affection curiosity or human aspects. These should be displayed in prominent location(s).

## Safety Sign Boards: Different type of message of cautioning, attention, notice etc. should be displayed at the appropriate places for learning/ awareness of the workmen while working at site.

- Films & Slides: Film(s) narrating the accident including the causes and possible remedial ways of preventing the recurrence of a similar accident should be displayed at regular intervals. Slides consisting main points of the film show may also be shown to workers.

# Talks, lectures & conferences: The success of these events would depend much on audience’s understandings of the speaker (s). The speakers are to be knowledgeable and good presenter. Speakers should know to hold the attention and to influence the audiences.

# Compotitions:  Organize competition(s) between the different depts.,/categories of workers.  The sense of reward/recognition alsowill improve safety awareness and result  in enhancing safety levels.

# Exhibitions: Exhibitions also make the workers acquainted with hazards and means of preventive measures.

# Safety Publication: Safety publications including pocket books dealing with ways of investigation and prevention in the field of safety and so on, may be distributed to workers to promote the safety awareness.

# Safety Drives: From time to time, an intensive safety drive by organizing a safety day or a safety week etc. should be launched.

# Training: Training for covering the hazards for different trade should be imparted. Training should also include the specific hazards related to a job in addition to the general safety training as has been dealt in various chapters and should include all workers.

10.0 LIST OF SAFETY CODES FOR CIVIL WORKS PUBLISHED BY BUREAU OF

# INDIAN STANDARDS

| Sl.No. | Code No. | Title |
| --- | --- | --- |
| 01 | IS : 818 | Code of Practice for Safety and Health Requirements in Electric and Gas Welding and Cutting Operations – First Revision. |
| 02 | IS : 875 | Code of practice for Structural safety of buildings: Masonry walls |
| 03 | IS : 933 | Specification for Portable Chemical Fire Extinguisher, Foam Type – Second Revision. |
| 04 | IS :1179 | Specification for Equipment for Eye and Face Protection during Welding – First Revision. |
| 05 | IS : 1904 | Code of practice for Structural safety of buildings: Shallow foundations |
| 06 | IS : 1905 | Code of practice for Structural safety of buildings: Masonry walls |
| 07 | IS : 2171 | Specification for Portable Fire Extinguishers, Dry Powder Type –Second Revision. |
| 08 | IS : 2361 | Specification for Building Grips – First Revision. |
| 09 | IS : 2750 | Specification for Steel Scaffoldings. |
| 10 | IS : 2925 | Specification for Industrial Safety Helmets – First Revision |
| 11 | IS : 3016 | Code of Practice for Fires Precautions in Welding and Cutting Operations – First Revision |
| 12 | IS : 3521 | Industrial safety belts and harnesses |
| 13 | IS : 3696 | Safety Code for Scaffolds and Ladders : Part I – Scaffolds. |
| 14 | IS : 3696 | Safety Code for Scaffolds and Ladders : Part II – Ladders. |
| 15 | IS : 3764 | Safety Code for Excavation Work |
| 16 | IS : 4014 | Part I & II Code of practice for Steel tubular scaffolding |
| 17 | IS : 4081 | Safety Code for Blasting and Related Drilling Operations. |
| 18 | IS : 4082 | Recommendations on staking and storage of construction materials at site |

| Sl.No. | Code No. | Title |
| --- | --- | --- |
| 19 | IS : 4130 | Safety Code for Demolition of Buildings – First Revision. |
| 20 | IS : 4138 | Safety Code Working in Compressed Air-First Revision |
| 21 | IS : 4912 | Safety requirements for Floor and Wall Openings, Railings and toe Boards –First Revision. |
| 22 | IS : 5121 | Safety Code for Piling and other Deep Foundations. |
| 23 | IS : 5916 | Safety Code for Construction involving use of Hot Bituminous Materials. |
| 24 | IS : 5983 | Specification for Eye Protectors – First Revision. |
| 25 | IS : 6922 | Structures subject to underground blasts, criteria for safety and design |
| 26 | IS : 7205 | Safety Code for Erection on Structural Steel Works. |
| 27 | IS : 7069 | Safety Code for Handling and Storage of Building Materials. |
| 28 | IS :7293 | Safety Code for Working with Construction Machinery. |
| 29 | IS : 7969 | Safety code for handling and storage of building material |
| 30 | IS : 8758 | Recommendation for Fire Precautionary Measures in construction of Temporary Structures and Pandals. |
| 31 | IS : 8989 | Safety Code for Erection of Concrete Framed Structures |
| 32 | IS : 9759 | Guidelines for de-watering during construction |
| 33 | IS : 11057 | Code of practice for Industrial safety nets |
| 34 | IS : 13415 | Code of Practice on safety for Protective barriers in and around building |
| 35 | IS : 13416 | Recommendations for preventive measures against hazards at working places |

(

SPECIAL CONDITIONS AND SAFETY PRECAUTIONS TO BE TAKEN AT WORK SITES.

1.	No lorry or road vehicles shall be operated so as to affect the safety of trains.  They should be allowed to work well outside the moving dimension.   At each of the locations, official should be posted as in charge to ensure that lorries do not infringe the schedule moving dimension any time and protect the track in case of emergency.  The facilities the driver to whistle, a whistle-board will have to be provided at the appropriate place.

2.	All vulnerable locations where constructions activity is in progress adjacent to  existing railway lines, should be off with proper barricades.  The most vulnerable locations shall be barricaded with rail barricades projecting atleast  1 M above ground.  At all other locations barricades of not less than 1.5 M height, consisting of bamboo / casuarina poles and supported horizontally by similar bamboo / casuarina poles should be provided.

3.	All the barricades are to be painted stuck on with luminous paints stripes at suitable intervals on the barricades.

4.	The entry to new bags which run alongside the existing track should be protected by barriers which can be closed and opened wherever necessary.

5.	At locations which are not vulnerable, provision of barricades can also be with (i) 0.6 m and 0.3m deep trenches or (ii) stones of minimum size 30cm x 15cm at 1M intervals and projecting 0.3M above ground level and painted white.  Trenches should be  allowed only in these locations where they do not lead to subsidence to Railway track as may be processed by the Section Engineer/ P. way.

6.	Barriers shall also be provided in the case of double lines, particularly at all existing LC where there is every possibility of road vehicles entering the finished formation.  These barriers are to be opened only for the movement of railway contractors authorised  vehicles of other Railway vehicles.

7.	Road vehicles employed by the contractor or should  have the certificate for the road worthiness and each vehicles numbered and the license particulars maintained contractors should ensure that the drivers permitted by them to work such road vehicles are identified, counseled, certified and are provided with photo identity card.

8.	Wherever the work repairs the movement of the road vehicles with in a distance of 3.5 m to 6.0 m from the centre line of the nearest track, such work shall be done only in the presence of railway employee authorised by the engineer-in-charge.   No part of road vehicles will be allowed at less than 3.5 m from track centre,.  Cost of such railway employees shall be borne by the railways.

9.	The driver of the vehicles shall always face the track when reversing the vehicles and wherever can not face the track for what ever reason shall be invariably be assisted by a helper with a whistle who should guide him and ensure safety.

10.	All work sites shall be supervised by the contractors represented as also a representative of the railway organisation whatever work of plying road vehicles with in 6.0 m.  Zone is actual in progress; look out men should invariably to available.  Look out men will have to be provided by the contractor from out of the list of persons who are authorised to carryout these duties.  Authorisation will be issued to the individuals by the representative of the engineer-in-charge.  One supervisor who shall be permanent staff (Gang man) leaned to JE/W/CN from the  respective gangs in whose beat the work is in progress  ( to be spared by the respective P.way Engineer / Open line) will monitor the availability and alertness of the look out men.   In case of non -availability of look out men the railway supervisor shall stop further activities for plying of road vehicles.  Even if no work is executed the night look out men shall patrol the best identified by the ensure the safety of running trains, specially from any infringement.

11.	The supervisor mentioned in para 10 above should be trained in protection rules and supplied with minimum equipment required for protecting the track.  Such staff should also be provided with basic communication facility a walkie-takie with communication facility the nearest station master or adjacent site so as to communicate to the nearest station in case of emergency / un usual occurrence till it is made available the supervisor shall use the nearest LC gate/telephone or other means of communication to relate the incident most speedily.

12.	Representative of Engineer-in-charge of the constructions organised will responsible for the general supervisors at each site.

13.	Working along side the track during the night hours is normally prohibited.  Such work can be done in the night only with the written permission of the Engineer-in-charge of the construction activity were night work is permitted lighting of the work as required should be done.

14.	The contractor shall be fully responsible for ensuring safety at all time and shall bear the cost of all damages incase of accidents/unusual occurrences damages to Railway property and its passengers.

The contractor should engage Technical persons and Workmen with the following qualifications for works as  detailed below  and the contractor shall carry out the Health Performance check at his cost for the workman engaged in the work through a registered medical practitioner and produce the certificate on demand.

# CHECK   FOR   HEALTH   PERFORMANCE

| Sl. No. | Activity | Hazard | Exposure Consequence | Check for | Periodicity |
| --- | --- | --- | --- | --- | --- |
| 1 | Concrete Dismantling | Emission of Dust & Noise | 1 | Lung function | Once in a Year |
| 2 | Concrete Mixing | Emission of Dust & Noise | 1 | Lung function | Once in a Year |
| 3 | Painting | Emission of Dust & fumes | 1 | Lung & throat function | Once in a Year |
| 4 | Cutting & Welding | Emission of fumes and gas. Exposure to Live wire | 3 | Eyes & Lung function | Once in a Year |
| 5 | Working on AC sheets | Emission of Dust | 3 | Lung function | Once in a Year |
| 6 | Sweeping of Roads | Emission of Dust | 2 | Lung function | Once in a Year |
| 7 | Collection and disposal of Sanitary waste | Foul smell & susceptibility to decease. | 3 | Lung function and skin irritation | Once in a Year |
| 8 | Handling of Oxygen & Acetylene Cylinders | Leakage of gas | 4 | Throat irritation | Once in a Year |
| 9 | Cleaning of Manholes | Exposure to poisonous gas | 4 | Suffocation | Once in a Year |
| 10 | Cleaning of Overhead tank | Emission of Dust | 1 | Suffocation & skin irritation | Once in a Year |

NOTE:	Exposure Consequence		1.	Slightly harmful

- Harmful

- Very harmful

- Extremely harmful