BHARAT HEAVY ELECTRICALS LIMITED

GOINDWAL-22

BILL OF QUANTITIES

NAME OF WORK:     :     Extension of shop floors and other infrastructure works under Valves Capacity

Augmentation in IVP Complex, BHEL, GOINDWAL.

| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PART - A |  |  |  |  |  |  |
| 1 |  | Dismantling / removing / demolishing as the case may be and carefully stacking the serviceable materials at site of work or conveying and handing over the same at BHEL stores and disposal of unserviceable materials by transporting and dumping in the near by low level areas within a lead of 100 metres. All as per instructions of Engineer-in-charge. |  |  |  |  |  |  |
| a) | 10.00 | Brick work in cement mortar |  |  | CUM |  |  |  |
| b) | 10.00 | Stone rubble masonry in cement mortar |  |  | CUM |  |  |  |
| c) | 100.00 | Old plastering / raking out joints in cement mortar / chipping the concrete and roughening the surface. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| d) | 15.00 | Plain Cement Concrete. |  |  | CUM |  |  |  |
| e) | 8.00 | RCC works including stacking of steel bars. |  |  | CUM |  |  |  |
| f) | 5.00 | RCC or PCC (Using jack hammer for pockets / cable trenches / industrial heavy duty flooring) |  |  | CUM |  |  |  |
| g) | 10.00 | G.I / PVC pipe lines of various dia including all specials. |  |  | R.M |  |  |  |
| h) | 20.00 | Bituminous road. |  |  | CUM |  |  |  |
| i) | 20.00 | W.B.M road including soling. |  |  | CUM |  |  |  |
| j) | 2.00 | Steel structures / steel works in built up sections including bolts, nuts, cutting rivets, welding, dismembering, preparation of DODL etc. |  |  | M.T |  |  |  |
| k) | 25.00 | A.C / GI sheets from roofs and side cladding including ridges, valleys, gutters etc. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| l) | 10.00 | S.W pipe lines upto 225 mm dia. |  |  | RM |  |  |  |
| m) | 10.00 | R.C.C pipes upto 450 mm dia |  |  | RM |  |  |  |
| n) | 20.00 | A.C / C.I / PVC rain water pipes upto 150 mm dia with fittings & clamps. |  |  | RM |  |  |  |
| o) | 10.00 | North light glasses at various levels ranging from 9 to 24 metre. |  |  | SQM |  |  |  |
| p) | 30.00 | Kerb stone |  |  | RM |  |  |  |
| 2. | 1810.00 | Clearing jungle including uprooting of vegetation, grass ,brushwood ,trees and saplings of girth upto 30cm measured at a height of 1 meter above ground level and removal of rubbish upto a distance of 50metre. |  |  | SQM |  |  |  |
| 3. |  | Earthwork excavation (TRENCHES) in soils given below and depositing the excavated earth at suitable places within the initial lead of 50 meter and intial lift of 4.5m for foundation in building works, roads, narrow drains, sewers, bridges, pipe lines etc., sectioning the subgrade to proper level and grade, watering and consolidating the subgrade to receive concrete bed, all as per standard specifications and instructions of Engineer-in-charge. |  |  |  |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| a) | 1370.00 | Hard / Dense soil |  |  | CUM |  |  |  |
| b) | 1120.00 | Ordinary rock. |  |  | CUM |  |  |  |
| c) | 600.00 | HARD ROCK(BLASTING PROHIBITED) |  |  | CUM |  |  |  |
| d) | 600.00 | HARD ROCK(BLASTING REQUIRED) |  |  | CUM |  |  |  |
| 4. |  | Earth work for surface excavation in soils given below including disposal of earth at suitable places within a lead upto 50 metres and lift upto 1.5 metre, sectioning the subgrade to proper level and grade. Disposed soil are to be leveled and neatly dressed. |  |  |  |  |  |  |
| a) | 1150.00 | Hard / Dense soil |  |  | CUM |  |  |  |
| b) | 600.00 | Ordinary rock. |  |  | CUM |  |  |  |
| 5. | 10.00 | Supply and filling with river sand in foundation, plinth, under floor etc. including watering, ramming, consolidating etc. complete. |  |  | CUM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 6. | 15.00 | Labour charges for cleaning the areas including uprooting thorny plants/other plants/heavy jungles/trees etc. of all heights and leveling the areas thereafter including all debris, all by using earth moving machinery (Poclain / JCB), all as per instructions of Engineer-in-charge. Rate includes hire and operation charges, to and fro charges of machinery, all labour and all other incidentals etc. complete. |  |  | HOUR |  |  |  |
| 7. | 1570.00 | Filling available excavated earth (excluding rock) in trenches, plinth, sides of foundation etc. in layers not exceeding 20 cm in depth, consolidating each layer by ramming, watering etc. complete. |  |  | CUM |  |  |  |
| 8. | 90.00 | Bailing or pumping out water from trenches, foundations etc., at all depths wherever necessary as per standard specifications. The Contractor shall maintain proper log book for pump operation and duly certified by the Engineer-in-charge. |  |  | HP/ HOUR |  |  |  |
| 9. | 175.00 | Providing and laying in position plain cement concrete 1:4:8 with 40 mm and downgraded nominal size hard broken granite stone jelly including consolidation, curing etc. complete, all as per standard specifications. All works upto plinth level. Rate excludes cost of centring and shuttering. |  |  | CUM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 10. |  | Providing and laying in position machine batched, machine mixed M20 Grade reinforced cement concrete using 20mm and downgraded hard broken granite stone including vibration, design mix of reinforced cement concrete of specified grade for structural elements, excluding cost of centering, shuttering and reinforcement but including admixtures if required in recommended proportions (as per IS:9103) to accelerate, retard setting of concrete, improve workability without impairing strength and durability, all as per directions of Engineer-in-charge. |  |  |  |  |  |  |
| a) | 625.00 | All works upto plinth level. |  |  | CUM |  |  |  |
| b) | 10.00 | All works above plinth level upto three storeyed level. (Each floor height - 4.5m) |  |  | CUM |  |  |  |
| 11. | 756.00 | Flooring with reinforced cement concrete of mix 1:2:4 using 20 mm and downgraded nominal size hard broken granite stone jelly laid in panels not exceeding 6 x 30 m between expansion joints, including necessary vibration using needle / board / screed vibrators, all as required including necessary shuttering, painting the sides with bitumen, but excluding fabrication charges for reinforcement, all as per standard specifications. |  |  | CUM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 12. | 35.00 | Straightening, cutting, bending, placing in position of MS/CTD bars and binding with MS binding wires of 18/20 gauge etc. for reinforcement of RCC works at all levels. Rate includes cover blocks, cost of binding wire, conveyance of MS/ CTD bars from BHEL stores to site of work, labour etc. complete. MS/CTD bars will be supplied by BHEL at Stores at free of cost. |  |  | M.T |  |  |  |
| 13. |  | Centring and shuttering including strutting, propping, assembling etc. and removal of form work upto a maximum height of 4.50 metre of each floor and also upto floor three level. |  |  |  |  |  |  |
| a) | 920.00 | Foundations, footings, bases of columns, equipment foundations, precast slabs, kerbs etc. for mass concrete. |  |  | SQM |  |  |  |
| b) | 895.00 | Walls (any thickness) including attached pilasters, plinth, stairs, chimney, shafts and string courses etc. |  |  | SQM |  |  |  |
| c) | 115.00 | Suspended floors, roofs, landings, balconies, access platform and sunshades. |  |  | SQM |  |  |  |
| d) | 95.00 | Lintels, beams, plinth beams, girders, bressumers and cantilevers. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| e) | 78.00 | Columns, pillars, piers, abutments, posts and struts. |  |  | SQM |  |  |  |
| f) | 20.00 | Pockets or holes in foundations for holding down bolts. |  |  | SQM |  |  |  |
| 14. | 2.00 | Providing, hoisting and fixing upto floor (4.50m height) three level precast reinforced cement concrete slabs of 25 to 150 mm thick of required sizes in mix 1 :2:4 using 12 to 20 mm size hard broken granite stone jelly for cupboards, shelves etc. including setting in cement mortar 1:3, cost of centring, shuttering and finishing with neat cement punning on exposed surfaces but excluding the cost of reinforcement and fabrication charges. |  |  | CUM |  |  |  |
| 15. | 40.00 | Providing 50 mm thick precast cement concrete Jali 1:2:4 using 12 to 20 mm size hard broken granite stone jelly with suitable reinforcement including centering and shuttering, roughening, cleaning, fixing and finishing in cement mortar1:3 etc. complete. Rate excludes reinforcement steel & fabrication charges. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 16. | 430.00 | Precasting, carrying and fixing in position of RCC louvers including 60 mm thick side posts, 60 mm thick intermittent lintel and 100 mm thick top lintel, 40 mm thick louver slabs lintels etc. (all precast) in cement concrete 1:2:4 using 12 to 20 mm size hard broken granite stone jelly with specified reinforcement, all as per standard drawing and instructions of Engineer-in-charge. Rate includes supply and fixing of holdfasts, moulding, scaffolding for fixing at specified levels, finishing and also filling the gap between precast lintels with insitu cement concrete of mix 1:2:4 using 10 mm and down graded hard broken granite stone jelly and filling the gap recesses of side walls with C.M 1:5, cutting and forming chases in brick masonry, finishing jambs wherever necessary and making good all damages to suit the existing work but excluding cost of fabrication charges for reinforcement steel. |  |  | SQM |  |  |  |
| 17. | 59.00 | Providing and fixing in position of 12 mm thick bitumen impregnated fibre board conforming to IS:1838 including cost of primer, sealing compound in expansion joints. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 18. | 190.00 | Precasting and fixing in position of 69mm thick RCC cable trench cover slab in mix 1:2:4 using 12 to 20 mm size hard broken granite stone jelly with top 15 mm thick finished with hardening compound of proportion one part of ironite or equivalent, four parts of cement and eight parts of 6 mm gauge hard granite stone chips (the proportion being by weight) including welding reinforcement rods to the frame work, all as per drawing, specifications and instructions of Engineer-in-charge but excluding cost of M.S frame work which will be paid separately. Reinforcement rods and M.S frames will be supplied by BHEL at stores at free of cost. |  |  | SQM |  |  |  |
| 19. | 1.00 | Providing and filling in position of bitumen mix filler of proportion 80 Kg of hot bitumen, 1 Kg of cement and 0.25 cubicmetre of coarse sand for expansion joints. |  |  | CUM |  |  |  |
| 20. | 165.00 | Random rubble masonry in foundation and basement in the mix given below including simultaneous flush pointing, all as per standard specifications. Cement mortar 1:5. |  |  | CUM |  |  |  |
| 21. | 470.00 | Brick work with F.P.S bricks (second class ground moulded chamber bricks) of class designation 50 in foundation and plinth and in superstructure in: Cement mortar 1:5. |  |  | CUM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 22. | 17.00 | Fabrication and fixing in position of M.S frames for cable trench cover slabs in required sizes using M.S angles, plates, flats etc., which will be supplied by BHEL at Stores at free of cost, all as per drawings and instructions of Engineer-in-charge. Rate includes conveyance of raw materials from BHEL stores to contractors fabrication shop, fabrication charges including cutting, welding, preparing and supplying DODL’s for BHEL's approval, supply of fabricated frames at site of work, fixing in proper position etc., complete. |  |  | M.T |  |  |  |
| 23. | 6670.00 | Fabrication and fixing in position of M.S inserts out of M.S plates, flats, rounds, angles etc., including necessary lugs made of rounds or flats, sizing and cutting to required lengths, threading, bolting and welding as required and placing the same in position in both plain and reinforced cement concrete members, including adjusting the shuttering if necessary, tying and holding to correct level and line. Rate includes cost of all materials, labour etc., complete but excluding raw steel sections which will be supplied by BHEL at Stores at free of cost. |  |  | K.G |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 24. | 1424.00 | Providing holding down bolts of required sizes with nuts, washers etc. and fixing in proper level, line, alignment etc., all as per drawings and instructions of Engineer-in-charge. Rate includes cost of steel materials as per IS 1363 & 2016, bolts, nuts, washers, fabrication and machining charges, fixing, all labour etc. complete. Bolts in all sizes & lengths. |  |  | K.G |  |  |  |
| 25. |  | Supply & fixing rolling shutters made of 80 x 1.25mm of M.S laths interlocked together through their entire length & joined together at end by end locks mounted on specially designed pipe shaft with brackets, side guides and arrangements for inside and outside locking with push and pull operation etc., complete including cost of providing and fixing necessary 27.5 cm long wire springs grade No.2 and M.S top cover of 1.25mm thick for rolling shutter of area 10 Sq.m. to 16.80 Sq.m. (gear operated type with gear box, crank, handle and ball bearings). |  |  | SQM |  |  |  |
| a) | 10.00 | Upto 8.00 Sq.m. |  |  |  |  |  |  |
| b) | 15.00 | Above 10 Sq.m. upto 16.80 Sq.m. |  |  |  |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 26. | 25.00 | Supply & fixing rolling shutters made of 80 x 1.25mm of M.S laths interlocked together through their entire length & joined together at end by end locks mounted on specially designed pipe shaft with brackets, side guides and arrangements for inside and outside locking with push and pull operation etc., complete including cost of providing and fixing necessary 27.5 cm long wire springs grade No.2 and M.S top cover of 1.25mm thickness thickness for rolling shutter of area above 16.80 Sq.m. (electrically operated with motor, gear box, crank shaft, ball bearing and required accessories). Quoted rate shall include cost of one coat of primer. |  |  | SQM |  |  |  |
| 27. | 3280.00 | Floor finish with 40 mm thick granolithick flooring over the concrete base while it is wet for proper bonding, the bottom 25 mm thick consisting of one part of cement and one part of 6 mm gauge hard broken granite stone chips (the proportion being by volume) and the top 15 mm finished with hardening compound of proportion one part of hardonate (ironite or equivalent) four parts of cement and eight parts of 6 mm gauge blue granite stone chips (the proportion being by weight) including finishing and painting the sides with bitumen etc., complete, all as per standard specifications. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 28. | 100.00 | Supply and fixing 7mm thick (approximate) ceramic tiles of approved make and colour of required sizes in skirting, walls, risers of steps and dado on 12 mm thick cement mortar 1:3 plaster and joined with white cement using appropriate colour pigment etc., complete, at all levels all as per standard specifications. |  |  | SQM |  |  |  |
| 29. | 10.00 | Providing weathering course on top of roof slab with 20 mm size brick jelly and pure lime (no sand) in the proportion of 32 : 12.5 to the required thickness, well beaten with wooden battens, so as to secure adequate slope, all as per standard specifications and instructions of Engineer-in-charge. |  |  | CUM |  |  |  |
| 30. | 100.00 | Providing weathering course on top of roof slab with one course of split tiles or pressed flat tiles of 20 mm nominal thickness and of approved size, set in cement mortar 1:3 with an admixture of crude oil at 10% by weight of cement, 12 mm thick bedding layer including pointing the tiles with the same oiled mortar etc., complete, all as per standard specifications. |  |  | SQM |  |  |  |
| 31. | 3770 | Plastering with cement mortar 1:5 ,12mm thick as per standard specifications. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 32. | 440.00 | Damp proof course with cement mortar 1:3, 20 mm thick mixed with an admixture of crude oil at 5 % by weight of cement, all labour etc. complete as per standard specifications. |  |  | SQM |  |  |  |
| 33. | 1685.00 | White washing three coats on surfaces at all levels using Janathacem or equivalent material to give an even shade including cleaning, brushing of walls and preparing surfaces in accordance with standard specifications. Rate includes cost of all materials, labour, curing etc., complete. |  |  | SQM |  |  |  |
| 34. | 500.00 | Painting walls with water proofing cement paint of approved brand and shade on surfaces to give an even shade including cleaning, preparing the surfaces, curing etc., complete. Three coats (A priming coat of cement primer & two coats of water proofing cement paint) |  |  | SQM |  |  |  |
| 35. | 0.73 | Painting three coats (A priming coat with approved primer and two coats of synthetic enamel paint) with synthetic enamel paint of approved brand and shade on surfaces to give an even shade including cleaning the surfaces at all levels of all dirt, dust and other foreign matters, sand papering and stopping, all as per standard specifications. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 36. | 1800.00 | Painting two coats with exterior emulsion paint (ACE or any other equivalent and approved brand) of approved shade on surfaces at all levels to give an even shade including preparing the surfaces, removal of dirt, thorough cleaning by watering etc., all as per standard specifications. Rate includes cost of necessary plaster of paris and putty wherever required, labour and all incidentals etc. complete. |  |  | SQM |  |  |  |
| 37. | 650.00 | Refilling the column foundation pits with 150mm size soling stones / boulders in layers including supplying of soling and packing the stones in position with sufficient spalls, so as to fill the voids as completely as possible and including filling up the voids (20% by volume of soling stones / boulders) with excavated earth, watering, consolidating complying with standard specification. All as per the instruction of the Engineer-in-charge. |  |  | CUM |  |  |  |
| 38. | 1.00 | Labour charges for fixing C.I pipes (S&S or flanged pipes) along the steel columns with suitable clamps for rain water pipes to proper alignment. Rate includes cost of pipes Jointing with CM 1:3. |  |  | M.T |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 39. |  | Supply RCC pipes (NP2 Class) of following sizes with collars at site of work, lowering to trench, laying to proper grade and alignment, joining the collars with stiff mixture of cement mortar 1 : 2, testing of joints etc., but excludes cost of cement which will be supplied by BHEL at stores at free of cost. |  |  |  |  |  |  |
| a) | 40.00 | 150 mm dia. |  |  | R.M. |  |  |  |
| b) | 320.00 | 300 mm dia |  |  | R.M |  |  |  |
| c) | 20.00 | 450 mm dia |  |  | R.M |  |  |  |
| d) | 20.00 | 600 mm dia |  |  | R.M |  |  |  |
| 40. |  | Providing and fixing G.I pipes and specials (B - Class) of approved brand with G.I fittings and clamps on walls & floors, including cutting and making good the walls & floors etc. complete. (Internal work) including testing if necessary. |  |  |  |  |  |  |
| a) | 20.00 | 15 mm dia nominal bore. |  |  | R.M |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| b) | 20.00 | 20 mm dia nominal bore. |  |  | R.M |  |  |  |
| c) | 100.00 | 25 mm dia nominal bore. |  |  | R.M |  |  |  |
| d) | 10.00 | 40 mm dia nominal bore. |  |  | R.M |  |  |  |
| e) | 10.00 | 50 mm dia nominal bore. |  |  | R.M |  |  |  |
| f) | 10.00 | 80 mm dia nominal bore. |  |  | R.M |  |  |  |
| g) | 120.00 | 100 mm dia nominal bore. |  |  | R.M |  |  |  |
| 41. |  | Providing and laying G.I pipes and specials (B - Class) of approved brand with G.I fittings in trenches. (External work) Earth work excavation for trenches and refilling will be paid separately. Testing shall be done if required. |  |  |  |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| a) | 20.00 | 15 mm dia nominal bore. |  |  | R.M |  |  |  |
| b) | 20.00 | 20 mm dia nominal bore. |  |  | R.M |  |  |  |
| c) | 20.00 | 25 mm dia nominal bore. |  |  | R.M |  |  |  |
| d) | 10.00 | 40 mm dia nominal bore. |  |  | R.M |  |  |  |
| e) | 10.00 | 50 mm dia nominal bore. |  |  | R.M |  |  |  |
| f) | 10.00 | 80 mm dia nominal bore. |  |  | R.M |  |  |  |
| g) | 10.00 | 100 mm dia nominal bore. |  |  | R.M |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 42. |  | Providing and laying PVC water pipes and specials conforming to ISS for water supply / Rain water wherever required in trenches, joining with proper adhesive as suggested by manufacturers etc. complete. (External work) Earth work excavation for trenches and refilling will be paid separately. For Rain water pipes suitable MS clamps should be supplied and fiited the pipes and specials with clamps as per the instructions of Engineer-in-charge. Rate includes all labour, materials, incidentals, etc., complete. |  |  |  |  |  |  |
| A) | 576.00 | PVC Pipe 160 mm OD (6 kg/Sq.cm) |  |  | R.M |  |  |  |
| 43. |  | Providing and fixing PVC pipes of 4 K.G / Sqcm conforming to ISS including joining with proper adhesive as suggested by manufacturers and suitable MS clamps for rain water pipes, drainage etc. Rate includes all specials like shoe, bend, tee, tee with door etc. |  |  |  |  |  |  |
| B) | 30.00 | PVC Pipe 110 mm OD |  |  | R.M |  |  |  |
| 44. |  | Providing and fixing PVC moulded fittings / accessories on walls for rain water pipes conforming to ISS including joining with seal rings and proper adhesive as suggested by manufacturers etc. complete. |  |  |  |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| a) | 2.00 | 75 x75 x 75 mm single tee with door. |  |  | EACH |  |  |  |
| b) | 2.00 | 75 x75 x 75 mm single tee without door. |  |  | EACH |  |  |  |
| c) | 2.00 | 110 x 110 x 110 mm single tee with door. |  |  | EACH |  |  |  |
| d) | 2.00 | 110 x110 x 110 mm single tee without door. |  |  | EACH |  |  |  |
| e) | 2.00 | 75 mm shoe |  |  | EACH |  |  |  |
| f) | 2.00 | 110 mm shoe |  |  | EACH |  |  |  |
| h) | 2.00 | 75 mm bend |  |  | EACH |  |  |  |
| i) | 2.00 | 110 mm bend |  |  | EACH |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| k) | 2.00 | 75mm Single ‘Y’ |  |  | EACH |  |  |  |
| l) | 2.00 | 110mm Single ‘Y’ |  |  | EACH |  |  |  |
| n) | 2.00 | 75mm Double ‘Y’ |  |  | EACH |  |  |  |
| o) | 2.00 | 110mm Double ‘Y’ |  |  | EACH |  |  |  |
| 45. |  | Providing and fixing in position of brass wheel valve / gate valve of approved brand with C.I wheel etc. complete. (ISI marked) |  |  |  |  |  |  |
| a) | 2.00 | 15 mm nominal bore |  |  | EACH |  |  |  |
| b) | 2.00 | 25 mm nominal bore |  |  | EACH |  |  |  |
| c) | 2.00 | 40 mm nominal bore |  |  | EACH |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| d) | 2.00 | 50 mm nominal bore |  |  | EACH |  |  |  |
| e) | 1.00 | 80 mm nominal bore |  |  | EACH |  |  |  |
| f) | 1.00 | 100 mm nominal bore |  |  | EACH |  |  |  |
| 46. | 5.00 | Providing and fixing in position of 15 mm nominal bore brass bib cock- 400gram weight (Medium Duty) of approved brand (ISI marked). |  |  | EACH |  |  |  |
| 47. | 2.00 | Providing and fixing in position 15 mm nominal bore C.P brass long body bibcock of approved brand (ISI marked) and weighing not less than 690 grams. |  |  | EACH |  |  |  |
| 48. |  | Providing and fixing in position brass stop cock (Medium Duty) of approved brand (ISI marked). |  |  |  |  |  |  |
| a) | 5.00 | 15 mm nominal bore. |  |  | EACH |  |  |  |
| b) | 2.00 | 20 mm nominal bore. |  |  | EACH |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| c) | 2.00 | 25 mm nominal bore |  |  | EACH |  |  |  |
| 49. |  | Providing rectangular/circular manhole as per Drawing No.D/1016065 for the following depths. Rate includes earth work excavation in all soils (except hard rock requiring blasting) including soft disintegrated rock, watering and ramming of foundation bed, providing cement concrete 1:4:8 in foundation, including necessary slope and forming channels smoothly finished in C.M 1:2, brick work in C.M 1:5 for arch ring, etc., using standard approved bricks, plastering in cement mortar 1:4 for side walls (outside and inside), cement concrete 1:2:4 using 12 to 20 mm HBGS jelly for manhole cover fixing, providing and fixing C.I steps for ladder rungs, etc., complete. Rate includes an initial lead of 10 m and all lifts for excavation and other charges etc., complete, but excludes cost of manhole cover and frame. |  |  |  |  |  |  |
| a) | 1.00 | Upto 0.90 m depth |  |  | EACH |  |  |  |
| b) | 1.00 | Above 0.90 and upto 1.20 m depth |  |  | EACH |  |  |  |
| c) | 1.00 | Above 1.20 and upto 1.50 m depth |  |  | EACH |  |  |  |
| d) | 1.00 | Above 1.50 and upto 1.80 m depth |  |  | EACH |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| e) | 1.00 | Above 1.80 and upto 2.10 m depth |  |  | EACH |  |  |  |
| f) | 1.00 | Above 2.10 and upto 2.40 m depth |  |  | EACH |  |  |  |
| g) | 1.00 | Above 2.40 and upto 2.70 m depth |  |  | EACH |  |  |  |
| h) | 1.00 | Above 2.70 and upto 3.00 m depth |  |  | EACH |  |  |  |
| i) | 1.00 | Above 3.00 and upto 3.30 m depth |  |  | EACH |  |  |  |
| j) | 1.00 | Above 3.30 and upto 3.60 m depth |  |  | EACH |  |  |  |
| 50. | 10.00 | Providing and fixing C.I cover with frame for manholes. 560 mm dia cover and frame. (Heavy duty) Total weight of cover and frame to be not less than 208 K.G. (Weight of cover 108 K.G and weight of frame 100 K.G) |  |  | EACH |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 51. |  | Conveyance of earth, sand, gravel, rough stone, broken stone, brick bats (debris) etc. by contractor’s own lorry including loading, leading, unloading, stacking or spreading, all as per instructions of Engineer-in-charge. Payment for conveyance of earth will be made based on pit measurement and for other items based on stack measurements. |  |  |  |  |  |  |
| a) | 2050.00 | Beyond 2 km but upto 3 km |  |  | CUM |  |  |  |
| 52. | 1.00 | Labour charges for fixing C.I specials such as tees, bends, collars, tapers, caps etc. along the steel columns with suitable clamps for rain water pipes to proper alignment. Rate includes cost of specials. Jointing with CM 1:3. |  |  | M.T |  |  |  |
| 53. |  | Providing and fixing glazing in aluminium door,window,ventilator ,shutters and partitions etc. with PVC /neoprene gasket etc complete as per drawing and instructions of Engineer-in-charge .(cost of aluminium beading shall be paid in basic item.) |  |  |  |  |  |  |
| a) | 28 | With glass panes 4mm thick (weight not less than 10 KG/SQM) |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 54. | 315 | Supply and stacking river sand (no voids) and gravel/red earth(no voids ) to standard gauge at site of work ,all as per instructions of Engineer-in-charge.It includes labour charges for mixing thoroughly to the required proportion and carrying the same and filing it in the excavated pits /and areas at various locations . |  |  | CUM |  |  |  |
| 55. | 540.00 | Providing and laying granular sub base as per table Nos. 400-1/400-2 and grading 1 of MORT & H - Revision IV with sand, gravel and crushed stone 6mm size in the proportion of 1 : 0.75 : 0.25 and combination these of for specified thick on prepared sub grade in accordance with the requirements of the specifications. Rate includes cost of all materials to site of work, mixing in the required proportion, spreading, compacting, rolling with contractor's power roller, all labour, other incidentals etc., complete. Payment will be made only for the finished (compacted) quantum of work. |  |  | CUM |  |  |  |
| 56. | 1200.00 | Providing and applying tack coat over the bituminuous surface using 5 K.G of bitumen per 10 Sq.m. area including heating the bitumn in boiler, labour for preparing the surface, spraying the bitumen as tack coat, hire and running charges for all tools & plants and all other incidentals etc., complete. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 57. |  | Supply, spreading and laying water bound macadam with stone aggregates of following sizes to proper line, level, camber and grade for first layer or second layer of metaling for a thickness of 75 mm including sectioning to camber, rolling dry with power roller, also supply and spreading gravel 25 mm thick for blindage, watering and consolidation with power roller, etc., complete, all as per standard specifications. Rate includes cost of all materials, labour, contractor’s power roller charges, etc., complete. |  |  |  |  |  |  |
| a) | 1200.00 | IRC graded stone aggregates of size range from 63 mm to 45 mm |  |  | SQM |  |  |  |
| b) | 1200.00 | IRC graded stone aggregates of size range from 53 mm to 22.4 mm |  |  | SQM |  |  |  |
| 58. | 60.00 | Providing 50 mm consolidated thick bituminous macadam over bituminous surface using IRC grade metal of 0.35 Cu.m of 25 to 12 mm, 0.21 cu.m of 12 to 6 mm & 0.14 Cu.m of 6 mm and below for 10 Sq.m with 40 kg of bitumen for premixing, including handling charges at CMP site, labour for loading the bitumen in bitumen tank and metal to feed units, labour for attending to paver site, heating & mixing the bitumen and metal to the required temperature in the central hot mix plant, conveying the mix by tipper to paver site, spreading the mix in required temperature & thickness with mechanical paver to the specified grade and cross section, consolidation by power roller to get the required density, including hire charges for CMP, paver finisher, tipper, power roller, bitumen boiler, all other tools and plants, incidental charges such as barricading, provision of danger lights etc., complete. Rate includes cost of metal, bitumen, hire and running charges for tools & plants and all other incidentals etc., complete. Payment will be made on pre level & post level measurements. |  |  | CUM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 59. | 50.00 | Providing semidense carpet of 40 mm consolidated thickness using machine crushed IRC size HBG metal 0.16 Cu.m of 13.20 mm, 0.256 Cu.m of 11.20 mm and 0.176 Cu.m of 5.60 mm & below for 10 Sq.m with 49.60 kg of bitumen for premixing for 10 Sq.m of 40 mm consolidated thickness of semidense carpet, including cost of bitumen, handling charges, labour for loading the bitumen in bitumen tanks & metal in feeder units, labour for attending paver units, heating & mixing the bitumen and metal to the required temperature in the central hot mix plant, conveying the mix from CMP site to work site by tipper, spreading by the paver finisher to the required grade & cross profile & consolidation by power roller, including hire and running charges for CMP, paver finisher, tipper, power roller, bitumen boiler and all other tools & plants & all other incidental charges etc., complete. Rate includes cost of metal, bitumen, labour, hire & running charges of tools & plants, barricading, providing of danger lights etc.. complete. Payment will be made based on pre level & post level measurements. |  |  | CUM |  |  |  |
| TOTAL for PART - A .. .. .. |  |  |  |  |  |  |  |  |
| (RUPEES………………………………………………………………………………………………………………………………………………………..) |  |  |  |  |  |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
|  | PART B |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 60. | 420.00 | Collection from BHEL stores and transporting the raw structural steel sections to the contractor’s site fabrication shop, fabrication of steel work including all materials going into the process of fabrication and forming an integral part of the steel work together with all shop welding including using low hydrogen electrode wherever necessary, all as per drawings and standard specifications. Rate includes delivery of fabricated steel work at erection site ready for erection, including preparation and supply of drawing office despatch list for BHEL’s approval in compact disk with two copies of printout, all charges for packing (wherever called for) handling, conveying, stacking and all other incidental charges both for raw materials and fabricated steel works. Rate excludes raw structural steel sections which will be supplied by BHEL at stores at free of cost. Rate shall include cost of providing one coat of red oxide primer to relevant IS specification over structural steel works in roof structures, columns etc., including preparing the surfaces by thoroughly cleaning oil, grease, dirt and other foreign matters and scoured with wire brushes, fine steel wool scrapers and sand paper, all as per standard specifications. |  |  | M.T |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 61. | 80.00 | Conveying raw structural steel sections from BHEL stores, transporting to contractor's fabrication shop/site, fabricating the components using submerged arc welding with single (or) multiple electrodes, and with approved quality of flux following the procedure stipulated in AWS D.1.1. (latest). The components shall be preassembled with required facilities for turning, inverting, tilting to facilitate fabrication and inspection. Edge preparation for welds shall be as per AWS D.1.1. (Latest) (or) as furnished in QCP. All welds shall be subjected to U.T./ N.D.T. test as per Q.C.P. of BHEL. Sequence of welding shall be worked out by the contractor and submitted to BHEL for approval. The components to be fabricated shall be preheated / post heated to remove residual stresses due to welding and to remove distortion using temporary fixtures as per Q.C.P. of BHEL. Erection mark and stacking the fabricated components shall be executed in accordance with standards. Payment will be made only for the members / parts which were welded together using auto welding, under this item. Any other attachment like bracings, battens, stiffeners, gussets, cleats, ribs etc., which will be welded during assembly will be paid under item (1) of Part B of this Bill of Quantities. The rate shall include cost of providing one coat of red oxide primer to relevant IS specification and delivery of fabricated steel work at erection site ready for erection including preparation & supply of DODL for BHEL’s approval in CD with two copies of printout. Quoted rate shall include cost for handling, leading, conveying, stacking, consumables hire charges for tools, plants, fuels, labours and other incidental etc. complete. The raw structural steel sections will be supplied by BHEL at stores free of cost. |  |  | M.T. |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 62. | 9.00 | Collection from BHEL stores and transporting the ERW/seamless steel tubes with necessary plates etc., for columns, trusses, purlins, tie runners etc., (all tubular) to the contractor’s site fabrication shop, fabrication of steel work including all materials going into the process of fabrication and forming an integral part of the steel work together with all shop welding including using low hydrogen electrode wherever necessary, all as per drawings and standard specifications. Rate includes delivery of fabricated steel work at erection site ready for erection, including preparation and supply of drawing office despatch list for BHEL’s approval in CD with two copies of print out, all charges for packing (wherever called for) handling, conveying, stacking and all other incidental charges both for raw materials and fabricated steel works. Rate excludes ERW/seamless steel tubes, plates, etc., which will be supplied by BHEL at stores at free of cost. Rate shall include cost of providing one coat of red oxide primer to relevant IS specification over structural steel works in roof structures, columns etc., including preparing the surfaces by thoroughly cleaning oil, grease, dirt and other foreign matters and scoured with wire brushes, fine steel wool scrapers and sand paper, all as per standard specificationS |  |  | MT |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 63. | 509.00 | Carrying from stacks at erection site to erection positions and erection of fabricated steel work including all handling, conveying, assembling, hoisting, erecting in position and site welding and or bolting, alignment of structures, all as per drawings and standard specifications. Rate includes bolts & nuts, hire for welding tools and plants, labour etc., but excluding cost of fabricated components. If required, power will be given free of cost by BHEL. Weight of fasteners will not be accounted for payment. |  |  | M.T |  |  |  |
| 64. | 344.00 | Painting two coats (excluding priming coat) on structural steel works in roof structures, columns etc., at all levels with synthetic enamel paint of approved brand and shade, brushing to give an even shade including cleaning the surfaces of all dirt, dust and other foreign matters, scaffolding charges etc., complete. All as per standard specifications. |  |  | M.T |  |  |  |
| 65. | 165.00 | Painting two coats (excluding priming coat) on structural steel works in roof structures, columns etc., at all levels with aluminium paint of approved brand, brushing to give an even shade including cleaning the surfaces of all dirt, dust and other foreign matters, scaffolding charges etc., complete. All as per standard specifications. |  |  | M.T. |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 66. | 6660.00 | Supply and fixing as required, hoisting and fixing in position of Bare Galvalume sheet with accessories such as ridge pieces, barge boards, curves etc., for roofing, side cladding etc., and fixing them in proper position with necessary Xylon coated self tapping / self drilling screws conforming to AS3566 standarads, EPDM washers etc., complete, all as per drawings, specifications and instructions of Engineer-in-charge. Rate includes cost of self tapping / self drilling screws, EPDM washer etc., and labour charges all scaffolding charges, tools and plants and all other incidental charges etc., complete and including cost of Galvalume sheet and its accessories such as ridge pieces, barge boards, curves etc. Payment will be made on covered area only. |  |  | SQM |  |  |  |
| 67. | 720.00 | Supply and, fixing as required, hoisting and fixing in position of 6mm thick multiwalled clear (UV coated) Polycarbonate sheet for north light glazing to suit the existing rows of glazing runner (inclined and vertical) and to suit the length of louver brackets including provision of suitable aluminium astrangle necessary powder coated aluminium stiffener, angle, etc., as required as per drawing and instrcutions of Engineer-in-charge. Rate shall include cost of astrangle and other fixing accessories, scaffolding, if required labour, tools and plants and all other incidental charges etc., complete and including cost of Polycarbonate sheet. Payment will be made on covered area only. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 68. | 50.00 | Supplying and fixing corrugated fibre glass reinforced polyester translucent roof light sheets of all colours including white of 4mm thick to match with the existing A.C. roof sheeting including cost of polymer coated crank bolts, hook bolts with square GI nuts, polymer T washers, polymer caps etc., complete. Payment will be made for the area of actual sheet laid. |  |  | SQM |  |  |  |
| 69. | 50.00 | Supply and fixing in position of galvanised iron plain sheets of 1.20 to 1.25 mm thick for flashing using G.I bolts, nuts, limpet washers etc. complete bent to shape and fixed in walls with cement mortar 1:3, all as per drawings and specifications. |  |  | SQM |  |  |  |
| 70. | 250.00 | Providing polymer concrete or fibre glass with UV resistant polyurethane compound lining to a min. thickness of 3mm on the MS gutters and making the gutters leak proof as per the instructions of Engineer in charge. The work consists of thoroughly cleaning the surface of the gutters and applying an under coat of resin over which the polymer concrete / polyurethane compound will be laid and coated at top with one coat of Gelcoat and polish to provide even surface. Rate shall include preparation of surface, polymer concrete, polyurethane / fibre glass with 50 microns thick lining complete all as per the instructions of Engineer-in-charge. Rate quoted shall also include cost of labour, material, scaffolding charges if any and all other incidental charges all Taxed etc., complete. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 71. | 2.00 | Collection and transporting the dismantled structural steel components, from site to contractor's fabrication yard, fabrication of the structural steel including all materials going into the process of making up the old component by additions and alterations, forming an integral part of the steel work together with all shop welding including using low hydrogen electrode wherever necessary as per drawing and standard specifications. Rate to include one coat of red oxide primer of standard specification and delivery of fabricated steel work ready for erection, at site; preparation of Drawing Office Despatch List for approval by BHEL in floppy with two copies of print out, and all charges for packing, wherever necessary, handling, leading, conveying, stacking, other incidental charges etc., for old component, raw steel and fabricated steel works. Rate to be all inclusive, except cost of raw structural steel sections which will be supplied by BHEL at stores free of cost. (Payment shall be made only to the portions of new items going into the process of fabrication). |  |  | MT |  |  |  |
| 72. | 1.00 | Grouting the H.D bolts, underneath the base plates etc., after alignment with non-shrink high strength cementitious grout of strenght not less than 30 N/Sq.mm. such as Cera Grout or Pidigrout or Flogrout or Conbextra or any other equivalent grout, all as per instructions of Engineer in charge and as per standard specifications. Rate includes cost of non-shrink grout, cleaning the sleeves/pockets, tools, labour, etc., complete. |  |  | CUM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit |  | Total Rs. P. |  |
| 73. | 0.01 | Grouting the H.D bolts, underneath the base plates etc., after alignment with wet cement grout (1:1), all as per standard specifications. Rate includes cleaning the sleeves/pockets, tools and plants etc., complete except cement which will be supplied by BHEL at stores at free of cost. |  |  | CUM |  |  |  |
| 74. | 500.00 | Carrying out LPI test wherever required as per QCP, in fabrication and erection work as per standard specification and as per the instruction of Engineer-in-charge. The rate to include supply of chemicals, qualified and experienced labour, hire charges for tools & plants, scaffolding, other accessories and all incidentals etc., complete. |  |  | R.M. |  |  |  |
| 75. | 550.00 | Carrying out MPI test wherever required as per QCP, in fabrication and erection work as per standard specification and as per the instruction of Engineer-in-charge. The rate to include supply of chemicals, qualified and experienced labour, hire charges for tools & plants, scaffolding, other accessories and all incidentals etc., complete. |  |  | R.M. |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| Sl. No. | Quantity | Description of work | Rate (both in figures and in words) |  | Unit | Total Rs. P. |  |  |
| 76. | 250.00 | Carrying out radiographic test using Iridium source for butt welds upto a thickness of40mm wherever required / as per QCP as per standard specification and direction of Engineer-in-charge. The rate to include supply of required qualified and experienced labour, films, tools, plants etc., complete. |  |  | SQ.CM. |  |  |  |
| 77. | 100.00 | Conducting ultrasonic test on welds as per standard practice and instruction of Engineer-in-charge. The rate to include cost of qualified and experienced labour, hire charges for tools & plants, accessories, incidentals etc., complete. |  |  | R.M. |  |  |  |
| 78. | 1600.00 | Supplying and fixing Cup type rail clamp with fittings, necessary bolts and nuts, etc. as per Drawing for crane rail / BG track / gantry track etc. Rate includes conveyance, welding including consumables, labour charges etc. complete as per instructions of Engineer-in-charge. |  |  | EACH |  |  |  |
| 79. | 100.00 | Conducting Ultrasonic testing at BHEL inside fabrication yard for steel plates using straight beam probe 24mm OD of 10mm 100% coverage from one major surface of plate in scanning direction –1 as per standard specification SA 435/SA435-M. The rate shall include all labour charges for marking, testing, punching of defective spots, preparation and submission of UT reports, punching of identification number and cost of consumables like couplants, cotton waste, etc. Qualified NDE personnel certified level II as per ASNT standard of latest edition shall conduct testing. Also includes all incidental charges, hire and running charges for equipments tools & plants, transportation, leading loading, conveyance, etc, complete. |  |  | SQM |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
| TOTAL for PART - B .. .. .. |  |  |  |  |  |  |  |  |
| TOTAL for PART - A .. .. .. |  |  |  |  |  |  |  |  |
| GRAND TOTAL .. .. .. |  |  |  |  |  |  |  |  |
| (RUPEES…………………………………………………………………………………………………………………………………………………….....) |  |  |  |  |  |  |  |  |
| CONTRACTOR ACCEPTING OFFICER |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | . |  |  |