| Sr.No. | ITEM NO. |  | ITEM NAME | UOM | TOTAL QTY. | RATING(A) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ELSTG190101 |  | UNIT-7 TURBINE SERVICE PMCC #7DA | EA | 1 | 4000A |
| 2 | ELSTG190102 |  | UNIT-8 TURBINE SERVICE PMCC #8DA | EA | 1 | 4000A |
| 3 | ELSTG190103 |  | UNIT-7 BOILER SERVICE PMCC #7DB | EA | 1 | 4000A |
| 4 | ELSTG190104 |  | UNIT-8 BOILER SERVICE PMCC #8DB | EA | 1 | 4000A |
| 5 | ELSTG190105 |  | UNIT-7 EMERGENCY MCC #7DG | EA | 1 | 3000 A |
| 6 | ELSTG190106 |  | UNIT-8 EMERGENCY MCC #8DG | EA | 1 | 3000 A |
| 7 | ELSTG190107 |  | UNIT-7 BLR. VALVE & DAMPER ACDB #7HB | EA | 1 | 250 A |
| 8 | ELSTG190108 |  | UNIT-8 BLR. VALVE & DAMPER ACDB #8HB | EA | 1 | 250A |
| 9 | ELSTG190109 |  | UNIT-7 BOILER ACDB #7HC | EA | 1 | 400A |
| 10 | ELSTG190110 |  | UNIT-8 BOILER ACDB #8HC | EA | 1 | 400A |
| 11 | ELSTG190111 |  | UNIT-7 ESP AREA MCC #7HD | EA | 1 | 250 A |
| 12 | ELSTG190112 |  | UNIT-8 ESP AREA MCC #8HD | EA | 1 | 250 A |
| 13 | ELSTG190113 |  | UNIT-7 TURBINE VALVE DB #7KB | EA | 1 | 250 A |
| 14 | ELSTG190114 |  | UNIT-8 TURBINE VALVE DB #8KB | EA | 1 | 250 A |
| 15 | ELSTG190115 |  | UNIT-7 UNIT SERVICE ACDB #7QA | EA | 1 | 630 A |
| 16 | ELSTG190116 |  | UNIT-8 UNIT SERVICE ACDB #8QA | EA | 1 | 630 A |
| 17 | ELSTG190117 |  | UNIT-7 AIR WASHER MCC #7SA | EA | 1 | 630 A |
| 18 | ELSTG190118 |  | UNIT-8 AIR WASHER MCC #8SA | EA | 1 | 630 A |
| 19 | ELSTG190119 |  | UNIT-7 VENT MCC #7TA | EA | 1 | 630 A |
| 20 | ELSTG190120 |  | UNIT-8 VENT MCC #8TA | EA | 1 | 630 A |
| 21 | ELSTG190121 |  | UNIT-7 ESP AC & VENT. MCC #7TB | EA | 1 | 400A |
| 22 | ELSTG190122 |  | UNIT-8 ESP AC & VENT. MCC #8TB | EA | 1 | 400A |
| 23 | ELSTG190123 |  | 220V UNIT-7 MAIN DCDB #7FA | EA | 1 | 1250 A |
| 24 | ELSTG190124 |  | 220V UNIT-8 MAIN DCDB #8FA | EA | 1 | 1250 A |
| 25 | ELSTG190125 |  | STATION SERVICE PMCC # 0DA | EA | 1 | 4000 A |
| 26 | ELSTG190126 |  | STATION MAIN ACDB # 0FA | EA | 1 | 250A |
| 27 | ELSTG190127 |  | FIRE WATER PMCC # 0DB | EA | 1 | 4000A |
| 28 | ELSTG190128 |  | STATION SERVICE MCC #0QA | EA | 1 | 1000A |
| 29 | ELSTG190129 |  | CW P/HPMCC # 0DC | EA | 1 | 3000A |
| 30 | ELSTG190130 |  | FUEL OIL UNLOADING SWBD # 0DD | EA | 1 | 1000A |
| 31 | ELSTG190131 |  | ADMIN BUILDING SWBD # 0DE | EA | 1 | 1600A |
| 32 | ELSTG190132 |  | INTAKE WATER SWBD #0DF | EA | 1 | 630A |
| 33 | ELSTG190133 |  | INTAKE WATER DCDB# 0FB | EA | 1 | 150A |
| 34 | ELSTG190134 |  | DM PLANT/CPU MCC # 0SA | EA | 1 | 1000A |
| 35 | ELSTG190135 |  | ETP/PT PLANT MCC #0SB | EA | 1 | 630A |
| 36 | ELSTG190136 |  | WORKSHOP/RAW WATER MCC #0SC | EA | 1 | 630A |
| 37 | ELSTG190137 |  | ELECTROCHLORINATION MCC # 0SD | EA | 1 | 400A |
| 38 | ELSTG190208 |  | AUX BOILER #0SE | EA | 1 | 400A |
| 39 | ELSTG190139 |  | HYDROGEN PLANT MCC # 0SG | EA | 1 | 630A |
| 40 | ELSTG190140 |  | STP MCC #0SF | EA | 1 | 250A |
| 41 | ELSTG190141 |  | FUEL OIL (LDO) MCC #0SH | EA | 1 | 400A |
| 42 | ELSTG190142 |  | CCR AC MCC #0TA | EA | 1 | 1600A |
| 43 | ELSTG190143 |  | SERVICE BUILDING SWBD #0TB | EA | 1 | 1000A |
| 44 | ELSTG190144 |  | 415V AC DBs (WALL MOUNTED) | EA | 30 | 63A |
| 45 | ELSTG190145 |  | 220V DC DBs (WALL MOUNTED) | EA | 6 | 63A |
| 46 | ELSTG190210 | SBMCC PANEL SURATGARH |  | EA | 2 | 100A |

ITEM NO.: 1        BOARD DESIGNATION:  TURBINE SERVICE PMCC # 7DA	       QTY: 1

BUSBAR RATING: 4000A      	       TYPE: DRAWOUT        		        DOUBLE FRONT

###### RATING                                MODULE TYPE                           QUANTITY

INCOMER:		4000A			            DAET(I/C)				2

###### BUSCOUPLER:	4000A			            DAET (B/C)				1

| 6A | E3 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 16A | E3 |  |  | 0 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 4 | 2 | 6 |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 |  |  | 0 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 63A | EA3 |  |  | 0 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 | 1 | 0 | 1 |
| 400A | EA3 | 1 | 1 | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E |  |  | 0 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 50.1 – 55.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 55.1 – 75.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 75.1 – 90.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 90.1 – 200.0KW | DM / PM |  |  | 0 |
| 110.0KW | DM / PM | 1 |  | 1 |
| 132.0KW | DM / PM | 2 |  | 2 |
| 160.0kW | DM / PM | 2 |  | 2 |
| 630A | DAE(O/G) | 3 |  | 3 |
| 1000A | DAE(O/G) | 2 |  | 2 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) | 2 |  | 2 |
| 3000A | DAE-TIE(O/G) | 2 |  | 2 |
|  | CS | 1 |  | 1 |
|  | SH | 1 |  | 1 |
|  | TT | 18 |  | 18 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 2        BOARD DESIGNATION:  TURBINE SERVICE PMCC # 8DA          QTY: 1

BUSBAR RATING: 4000A      	       TYPE: DRAWOUT        		        DOUBLE FRONT

###### RATING                                MODULE TYPE                           QUANTITY

INCOMER:		4000A			            DAET(I/C)				2

###### BUSCOUPLER:	4000A			            DAET (B/C)				1

| 6A | E3 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 16A | E3 |  |  | 0 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 4 | 2 | 6 |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 |  |  | 0 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 63A | EA3 |  |  | 0 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 | 1 | 0 | 1 |
| 400A | EA3 | 1 | 1 | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E |  |  | 0 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 50.1 – 55.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 55.1 – 75.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 75.1 – 90.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 90.1 – 200.0KW | DM / PM |  |  | 0 |
| 110.0KW | DM / PM | 1 |  | 1 |
| 132.0KW | DM / PM | 2 |  | 2 |
| 160.0kW | DM / PM | 2 |  | 2 |
| 630A | DAE(O/G) | 3 |  | 3 |
| 1000A | DAE(O/G) | 2 |  | 2 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) | 2 |  | 2 |
| 3000A | DAE-TIE(O/G) | 2 |  | 2 |
|  | CS | 1 |  | 1 |
|  | SH | 1 |  | 1 |
|  | TT | 18 |  | 18 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 3        BOARD DESIGNATION:  BOILER SERVICE PMCC #7DB             QTY: 1

BUSBAR RATING: 4000A      	       TYPE: DRAWOUT        		        DOUBLE FRONT

###### RATING                                MODULE TYPE                           QUANTITY

INCOMER:		4000A			            DAET(I/C)				2

###### BUSCOUPLER:	4000A			            DAET (B/C)				1

| 6A | E3 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 16A | E3 |  |  | 0 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 10 | 1 | 11 |
| 63A | E3 | 6 |  | 6 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 3 |  | 3 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 |  |  | 0 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 63A | EA3 |  |  | 0 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 | 1 |  | 1 |
| 250A | EA3 | 1 |  | 1 |
| 400A | EA3 | 1 | 1 | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 2 | 7 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 3 | 1 | 4 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 10 | 2 | 12 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E | 3 | 1 | 4 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E | 2 | 1 | 3 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E | 3 |  | 3 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E | 2 |  | 2 |
| 50.1 – 55.0KW | DK21E / PK21E / AK21E | 1 |  | 1 |
| 55.1 – 75.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 75.1 – 90.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 132.0KW | DM / PM | 6 | 1 | 7 |
| 630A | DAE(O/G) | 3 | 1 | 4 |
| 1000A | DAE(O/G) | 1 | 1 | 2 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | SH | 1 |  | 1 |
|  | TT | 21 |  | 21 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 4       BOARD DESIGNATION:  BOILER SERVICE PMCC #8DB                QTY: 1

BUSBAR RATING: 4000A      	       TYPE: DRAWOUT        		        DOUBLE FRONT

###### RATING                                MODULE TYPE                           QUANTITY

INCOMER:		4000A			            DAET(I/C)				2

###### BUSCOUPLER:	4000A			            DAET (B/C)				1

| 6A | E3 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 16A | E3 |  |  | 0 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 10 | 1 | 11 |
| 63A | E3 | 6 |  | 6 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 3 |  | 3 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 |  |  | 0 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 63A | EA3 |  |  | 0 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 | 1 |  | 1 |
| 250A | EA3 | 1 |  | 1 |
| 400A | EA3 | 1 | 1 | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 2 | 7 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 3 | 1 | 4 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 10 | 2 | 12 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E | 3 | 1 | 4 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E | 2 | 1 | 3 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E | 3 |  | 3 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E | 2 |  | 2 |
| 50.1 – 55.0KW | DK21E / PK21E / AK21E | 1 |  | 1 |
| 55.1 – 75.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 75.1 – 90.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 132.0KW | DM / PM | 6 | 1 | 7 |
| 630A | DAE(O/G) | 3 | 1 | 4 |
| 1000A | DAE(O/G) | 1 | 1 | 2 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | SH | 1 |  | 1 |
|  | TT | 21 |  | 21 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 5        BOARD DESIGNATION:  EMERGENCY MCC #7DG                      QTY: 1

BUSBAR RATING: 3000A      	       TYPE: DRAWOUT        		        DOUBLE FRONT

###### RATING                                MODULE TYPE                           QUANTITY

INCOMER:		3000A			            DAEG (I/C)				2

INCOMER:		3000A			            DAEGN (I/C)				2

###### BUSCOUPLER:	3000A			            DAEG (B/C)				1

| 6A | E3 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 16A | E3 |  |  | 0 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 15 | 1 | 16 |
| 63A | E3 | 8 | 2 | 10 |
| 100A | E3 | 5 | 2 | 7 |
| 125A | E3 | 3 | 1 | 4 |
| 160A | E3 | 2 | 1 | 3 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 | 4 | 1 | 5 |
| 400A | EA3 | 10 | 2 | 12 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 6 |  | 6 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 10 | 1 | 11 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 15 | 2 | 17 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E | 15 | 1 | 16 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E | 2 |  | 2 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 5 | 1 | 6 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E | 3 |  | 3 |
| 50.1 – 55.0KW | DK21E / PK21E / AK21E | 1 |  | 1 |
| 55.1 – 75.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 75.1 – 90.0KW | DK21E / PK21E / AK21E | 5 | 1 | 6 |
| 75.1 – 90.0KW | K21 |  |  | 0 |
| 0.0 – 5.5KW | K3 | 4 | 1 | 5 |
| 5.6 – 7.0KW | K3 |  |  | 0 |
| 7.1 – 13.0KW | K3 |  |  | 0 |
| 13.1 – 24.0KW | K3 |  |  | 0 |
| 24.1 – 29.9KW | K3 |  |  | 0 |
| 132.0KW | DM / PM | 3 | 1 | 4 |
| 630A | DAE(O/G) | 9 | 2 | 11 |
| 1000A | DAE(O/G) |  |  | 0 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 37 |  | 37 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 6        BOARD DESIGNATION:  EMERGENCY MCC # 8DG                      QTY: 1

BUSBAR RATING: 3000A      	       TYPE: DRAWOUT        		        DOUBLE FRONT

###### RATING                                MODULE TYPE                           QUANTITY

INCOMER:		3000A			DAET(I/C)				2

###### BUSCOUPLER:	3000A			            DAET (B/C)				1

| 6A | E3 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 16A | E3 |  |  | 0 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 15 | 1 | 16 |
| 63A | E3 | 8 | 2 | 10 |
| 100A | E3 | 5 | 2 | 7 |
| 125A | E3 | 3 | 1 | 4 |
| 160A | E3 | 2 | 1 | 3 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 | 4 | 1 | 5 |
| 400A | EA3 | 10 | 2 | 12 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 6 |  | 6 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 10 | 1 | 11 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 15 | 2 | 17 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E | 15 | 1 | 16 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E | 2 |  | 2 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 5 | 1 | 6 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E | 3 |  | 3 |
| 50.1 – 55.0KW | DK21E / PK21E / AK21E | 1 |  | 1 |
| 55.1 – 75.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 75.1 – 90.0KW | DK21E / PK21E / AK21E | 5 | 1 | 6 |
| 75.1 – 90.0KW | K21 |  |  | 0 |
| 0.0 – 5.5KW | K3 | 4 | 1 | 5 |
| 5.6 – 7.0KW | K3 |  |  | 0 |
| 7.1 – 13.0KW | K3 |  |  | 0 |
| 13.1 – 24.0KW | K3 |  |  | 0 |
| 24.1 – 29.9KW | K3 |  |  | 0 |
| 132.0KW | DM / PM | 3 | 1 | 4 |
| 630A | DAE(O/G) | 9 | 2 | 11 |
| 1000A | DAE(O/G) |  |  | 0 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 37 |  | 37 |
|  | G1 | 2 |  | 2 |

ITEM NO.:7  BOARD DESIGNATION:  BLR. VALVE & DAMPER ACDB # 7HB        QTY: 1

BUSBAR RATING: 250A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		250A				DAE (I/C)				2

| 32A | E3 | 4 | 2 | 6 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 160A | E3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DN1/ PN1/ AN1 | 124 | 10 | 134 |
| 5.6 – 7.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 7.1 – 13.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 13.1 – 24.0KW | DN1/ PN1/ AN1 | 10 | 1 | 11 |
| 24.1 – 30.0KW | DN1/ PN1/ AN1 | 2 | 1 | 3 |
|  | CS-A | 1 |  | 1 |
|  | SH-A | 1 |  | 1 |
|  | TT | 17 |  | 17 |
|  | G1 | 1 |  | 1 |

ITEM NO.:8  BOARD DESIGNATION:  BLR. VALVE & DAMPER ACDB # 8HB             QTY: 1

BUSBAR RATING: 250A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		250A				DAE (I/C)				2

| 32A | E3 | 4 | 2 | 6 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 160A | E3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DN1/ PN1/ AN1 | 124 | 10 | 134 |
| 5.6 – 7.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 7.1 – 13.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 13.1 – 24.0KW | DN1/ PN1/ AN1 | 10 | 1 | 11 |
| 24.1 – 30.0KW | DN1/ PN1/ AN1 | 2 | 1 | 3 |
|  | CS-A | 1 |  | 1 |
|  | SH-A | 1 |  | 1 |
|  | TT | 17 |  | 17 |
|  | G1 | 1 |  | 1 |

ITEM NO.: 9 BOARD DESIGNATION:  BOILER ACDB #7HC	                              QTY: 1

BUSBAR RATING: 400A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		400A				DAE (I/C)				2

###### BUSCOUPLER:	400A				DAE (B/C)				1

| 32A | E3 | 9 | 6 | 15 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 11 | 4 | 15 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 | 3 | 14 | 17 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 |  |  | 0 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 10 | 1 | 11 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 5 | 1 | 6 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 20 |  | 20 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 10 BOARD DESIGNATION:  BOILER ACDB #8HC	QTY: 1

BUSBAR RATING: 400A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		400A				DAE (I/C)				2

###### BUSCOUPLER:	400A				DAE (B/C)				1

| 32A | E3 | 9 | 6 | 15 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 11 | 4 | 15 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 | 3 | 14 | 17 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 |  |  | 0 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 10 | 1 | 11 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 5 | 1 | 6 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 20 |  | 20 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 11  BOARD DESIGNATION:ESP AREA MCC #7HD                                  QTY: 1

BUSBAR RATING: 250A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		250A				AE (I/C)				2

| 16A | E3 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 4 | 2 | 6 |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 2 | 1 | 3 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 | 2 | 1 | 3 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 3 | 8 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 4 | 2 | 6 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 0.0 – 5.5KW | DN1/ PN1/ AN1 | 16 | 5 | 21 |
| 5.6 – 7.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 7.1 – 13.0KW | DN1/ PN1/ AN1 | 8 | 3 | 11 |
| 13.1 – 24.0KW | DN1/ PN1/ AN1 |  |  | 0 |
|  | CS-A | 1 |  | 1 |
|  | SH-A | 1 |  | 1 |
|  | TT | 20 |  | 20 |
|  | G1 | 1 |  | 1 |

ITEM NO.: 12  BOARD DESIGNATION:ESP AREA MCC #8HD                                 QTY: 1

BUSBAR RATING: 250A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		250A				AE (I/C)				2

| 16A | E3 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 4 | 2 | 6 |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 2 | 1 | 3 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 | 2 | 1 | 3 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 3 | 8 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 4 | 2 | 6 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 0.0 – 5.5KW | DN1/ PN1/ AN1 | 16 | 5 | 21 |
| 5.6 – 7.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 7.1 – 13.0KW | DN1/ PN1/ AN1 | 8 | 3 | 11 |
| 13.1 – 24.0KW | DN1/ PN1/ AN1 |  |  | 0 |
|  | CS-A | 1 |  | 1 |
|  | SH-A | 1 |  | 1 |
|  | TT | 20 |  | 20 |
|  | G1 | 1 |  | 1 |

ITEM NO.: 13  BOARD DESIGNATION:TURBINE VALVE DB #7KB                        QTY: 1

BUSBAR RATING: 250A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		250A				DAE (I/C)				2

| 6A | E3 | 2 | 1 | 3 |
| --- | --- | --- | --- | --- |
| 32A | E3 | 4 | 2 | 6 |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 2 | 1 | 3 |
| 0.0 – 5.5KW | DN1/ PN1/ AN1 | 154 | 1 | 155 |
| 5.6 – 7.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 7.1 – 13.0KW | DN1/ PN1/ AN1 | 28 | 1 | 29 |
| 13.1 – 24.0KW | DN1/ PN1/ AN1 | 3 | 1 | 4 |
| 24.1 – 30.0KW | DN1/ PN1/ AN1 | 15 | 1 | 16 |
|  | CS-A | 1 |  | 1 |
|  | SH-A | 1 |  | 1 |
|  | TT | 17 |  | 17 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 14  BOARD DESIGNATION:TURBINE VALVE DB #8KB                          QTY: 1

BUSBAR RATING: 250A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		250A				DAE (I/C)				2

| 6A | E3 | 2 | 1 | 3 |
| --- | --- | --- | --- | --- |
| 32A | E3 | 4 | 2 | 6 |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 2 | 1 | 3 |
| 0.0 – 5.5KW | DN1/ PN1/ AN1 | 154 | 1 | 155 |
| 5.6 – 7.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 7.1 – 13.0KW | DN1/ PN1/ AN1 | 28 | 1 | 29 |
| 13.1 – 24.0KW | DN1/ PN1/ AN1 | 3 | 1 | 4 |
| 24.1 – 30.0KW | DN1/ PN1/ AN1 | 15 | 1 | 16 |
|  | CS-A | 1 |  | 1 |
|  | SH-A | 1 |  | 1 |
|  | TT | 17 |  | 17 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 15 BOARD DESIGNATION:  UNIT SERVICE ACDB # 7QA                      QTY: 1

BUSBAR RATING:  630A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                                MODULE TYPE                            QUANTITY

INCOMER:		630A				DAE (I/C)				2

###### BUSCOUPLER:	630A				DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 10 | 5 | 15 |
| 16A | E1 | 10 | 5 | 15 |
| 25A | E1 |  |  | 0 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 20 | 10 | 30 |
| 63A | E3 | 10 | 5 | 15 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 15 | 1 | 16 |
| 32A | EA3 | 30 | 2 | 32 |
| 63A | EA3 | 8 | 2 | 10 |
| 125A | EA3 | 10 | 1 | 11 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 | 10 | 1 | 11 |
| 400A | EA3 |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 18 |  | 18 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 16  BOARD DESIGNATION:  UNIT SERVICE ACDB # 8QAQTY: 1

BUSBAR RATING:  630A      	         TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                                MODULE TYPE                           QUANTITY

INCOMER:		 630A				  DAE (I/C)				2

###### BUSCOUPLER:	 630A				  DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 10 | 5 | 15 |
| 16A | E1 | 10 | 5 | 15 |
| 25A | E1 |  |  | 0 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 20 | 10 | 30 |
| 63A | E3 | 10 | 5 | 15 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 15 | 1 | 16 |
| 32A | EA3 | 30 | 2 | 32 |
| 63A | EA3 | 8 | 2 | 10 |
| 125A | EA3 | 10 | 1 | 11 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 | 10 | 1 | 11 |
| 400A | EA3 |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 18 |  | 18 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 17                BOARD DESIGNATION:  AIR WASHER MCC # 7SA               QTY: 1

BUSBAR RATING:  630A      	         TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                                MODULE TYPE                            QUANTITY

INCOMER:		 630A				  DAE (I/C)				2

###### BUSCOUPLER:	 630A				  DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 10 | 5 | 15 |
| 16A | E1 | 10 | 5 | 15 |
| 32A | E3 | 60 | 1 | 61 |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 3 | 1 | 4 |
| 160A | E3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 4 | 2 | 6 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 12 | 2 | 14 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 12 | 5 | 17 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E |  |  | 0 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 5 | 2 | 7 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E | 3 | 2 | 5 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E | 5 | 1 | 6 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 20 |  | 20 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 18                BOARD DESIGNATION:  AIR WASHER MCC # 8SA               QTY: 1

BUSBAR RATING:  630A      	         TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                                MODULE TYPE                            QUANTITY

INCOMER:		 630A				  DAE (I/C)				2

###### BUSCOUPLER:	 630A				  DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 10 | 5 | 15 |
| 16A | E1 | 10 | 5 | 15 |
| 32A | E3 | 60 | 1 | 61 |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 3 | 1 | 4 |
| 160A | E3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 4 | 2 | 6 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 12 | 2 | 14 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 12 | 5 | 17 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E |  |  | 0 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 5 | 2 | 7 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E | 3 | 2 | 5 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E | 5 | 1 | 6 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 20 |  | 20 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 19                BOARD DESIGNATION:  VENT MCC # 7TA                          QTY: 1

BUSBAR RATING:  630A      	         TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                                MODULE TYPE                            QUANTITY

INCOMER:		 630A				  DAE (I/C)				2

###### BUSCOUPLER:	 630A				  DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 10 | 1 | 11 |
| 16A | E1 | 10 | 1 | 11 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 30 | 5 | 35 |
| 63A | E3 | 4 | 1 | 5 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 14 | 3 | 17 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 9 | 2 | 11 |
| 0.0 – 5.5KW | K3 | 10 | 10 | 20 |
| 5.6 – 7.0KW | K3 |  |  | 0 |
| 7.1 – 13.0KW | K3 |  |  | 0 |
| 13.1 – 24.0KW | K3 | 15 | 2 | 17 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 13 |  | 13 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 20                BOARD DESIGNATION:  VENT MCC # 8TA                        QTY: 1

BUSBAR RATING:  630A      	         TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                                MODULE TYPE                            QUANTITY

INCOMER:		 630A				  DAE (I/C)				2

###### BUSCOUPLER:	 630A				  DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 10 | 1 | 11 |
| 16A | E1 | 10 | 1 | 11 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 30 | 5 | 35 |
| 63A | E3 | 4 | 1 | 5 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 14 | 3 | 17 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 9 | 2 | 11 |
| 0.0 – 5.5KW | K3 | 10 | 10 | 20 |
| 5.6 – 7.0KW | K3 |  |  | 0 |
| 7.1 – 13.0KW | K3 |  |  | 0 |
| 13.1 – 24.0KW | K3 | 15 | 2 | 17 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 13 |  | 13 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 21 BOARD DESIGNATION:  ESP AC & VENT. MCC # 7TB                    QTY: 1

BUSBAR RATING: 400A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		400A				AE (I/C)				2

###### BUSCOUPLER:	400A				AE (B/C)				1

| 6A | E1 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- |
| 16A | E1 | 1 | 1 | 2 |
| 25A | E1 | 2 | 1 | 3 |
| 32A | E1 |  |  | 0 |
| 63A | E1 |  |  | 0 |
| 125A | E1 |  |  | 0 |
| 6A | E3 |  |  | 0 |
| 16A | E3 | 1 | 1 | 2 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 4 | 2 | 6 |
| 63A | E3 | 8 | 3 | 11 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 2 | 1 | 3 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 | 4 | 2 | 6 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 3 | 8 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 5 | 2 | 7 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 20 |  | 20 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 22BOARD DESIGNATION:  ESP AC & VENT. MCC # 8TB                      QTY: 1

BUSBAR RATING: 400A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		400A				AE (I/C)				2

###### BUSCOUPLER:	400A				AE (B/C)				1

| 6A | E1 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- |
| 16A | E1 | 1 | 1 | 2 |
| 25A | E1 | 2 | 1 | 3 |
| 32A | E1 |  |  | 0 |
| 63A | E1 |  |  | 0 |
| 125A | E1 |  |  | 0 |
| 6A | E3 |  |  | 0 |
| 16A | E3 | 1 | 1 | 2 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 4 | 2 | 6 |
| 63A | E3 | 8 | 3 | 11 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 2 | 1 | 3 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 | 4 | 2 | 6 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 3 | 8 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 5 | 2 | 7 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 20 |  | 20 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 23                BOARD DESIGNATION: MAIN DCDB # 7FA                         QTY:  1

BUSBAR RATING:      1250A     		     TYPE: FIXED		         DOUBLE FRONT

###### RATING                             MODULE TYPE                            QUANTITY

INCOMER

FROM MAIN DCDB:      1250A                                    CH (I/C)				2

BUSCOUPLER:             1250A                                    CH (B/C)				1

| 6A | X | 30 | 1 | 31 |
| --- | --- | --- | --- | --- |
| 16A | X | 30 | 2 | 32 |
| 32A | X | 125 | 20 | 145 |
| 63A | X | 12 | 10 | 22 |
| 125A | X | 8 | 4 | 12 |
| 200A | X | 6 | 2 | 8 |
| 250A | X | 4 | 1 | 5 |
| 400A | X | 6 | 2 | 8 |
| 500A | X | 4 | 1 | 5 |
|  | CS | 2 |  | 2 |
|  | SH | 1 |  | 1 |
|  | TT | 15 |  | 15 |
|  | S | 2 |  | 2 |

ITEM NO.: 24                     BOARD DESIGNATION: MAIN DCDB # 8FA               QTY:  1

BUSBAR RATING:      1250A     		     TYPE: FIXED		         DOUBLE FRONT

###### RATING                             MODULE TYPE                            QUANTITY

INCOMER

FROM MAIN DCDB:      1250A                                    CH (I/C)					2

BUSCOUPLER:             1250A                                    CH (B/C)					1

| 6A | X | 30 | 1 | 31 |
| --- | --- | --- | --- | --- |
| 16A | X | 30 | 2 | 32 |
| 32A | X | 125 | 20 | 145 |
| 63A | X | 12 | 10 | 22 |
| 125A | X | 8 | 4 | 12 |
| 200A | X | 6 | 2 | 8 |
| 250A | X | 4 | 1 | 5 |
| 400A | X | 6 | 2 | 8 |
| 500A | X | 4 | 1 | 5 |
|  | CS | 2 |  | 2 |
|  | SH | 1 |  | 1 |
|  | TT | 15 |  | 15 |
|  | S | 2 |  | 2 |

ITEM NO.: 25 BOARD DESIGNATION:STATIONSERVICE PMCC #0DA            QTY: 1

BUSBAR RATING: 4000A      	              TYPE: DRAWOUT                     DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		4000A				DAET (I/C)				2

BUSCOUPLER:         4000A                                     DAET(B/C)			            1

| 32A | E3 | 6 |  | 6 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 4 | 2 | 6 |
| 100A | E3 | 4 |  | 4 |
| 125A | E3 | 1 | 1 | 2 |
| 160A | E3 | 4 |  | 4 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 |  |  | 0 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 63A | EA3 |  |  | 0 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 | 6 | 2 | 8 |
| 250A | EA3 | 2 | 1 | 3 |
| 400A | EA3 | 4 |  | 4 |
| 90.1 – 200.0KW | DM / PM |  |  | 0 |
| 132.0KW | DM / PM | 2 | 1 | 3 |
| 630A | DAE(O/G) |  |  | 0 |
| 1000A | DAE(O/G) | 4 | 1 | 5 |
| 1600A | DAE(O/G) | 2 | 1 | 3 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 18 |  | 18 |
|  | G1 | 2 |  | 2 |

ITEM NO.:26 BOARD DESIGNATION: STATION MAIN ACDB # 0FA                	QTY: 1

BUSBAR RATING: 250A      	          TYPE: DRAWOUT                          DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		250A				CH (I/C)				2

###### BUSCOUPLER:	250A				CH (B/C)				1

| 6A | X | 1 | 1 | 2 |
| --- | --- | --- | --- | --- |
| 16A | X | 1 | 1 | 2 |
| 32A | X | 10 | 1 | 11 |
| 63A | X | 5 |  | 5 |
| 125A | X | 8 |  | 8 |
| 200A | X | 2 |  | 2 |
| 250A | X | 2 |  | 2 |
| 400A | X | 2 |  | 2 |
| 500A | X | 2 |  | 2 |
|  | CS | 1 |  | 1 |
|  | SH | 1 |  | 1 |
|  | TT | 8 |  | 8 |
|  | S | 2 |  | 2 |

ITEM NO.:27 BOARD DESIGNATION: FIRE WATER PMCC # 0DB                      QTY: 1

BUSBAR RATING: 4000A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		4000A				DAET (I/C)				2

###### BUSCOUPLER:	4000A				DAET (B/C)				1

| 6A | E1 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 16A | E1 |  |  | 0 |
| 25A | E1 |  |  | 0 |
| 32A | E1 | 5 | 1 | 6 |
| 63A | E1 | 2 | 1 | 3 |
| 125A | E1 |  |  | 0 |
| 6A | E3 |  |  | 0 |
| 16A | E3 |  |  | 0 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 2 |  | 2 |
| 63A | E3 | 1 | 1 | 2 |
| 100A | E3 | 1 | 1 | 2 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 2 | 1 | 3 |
| 6A | EA1 |  |  | 0 |
| 16A | EA1 |  |  | 0 |
| 25A | EA1 |  |  | 0 |
| 32A | EA1 |  |  | 0 |
| 63A | EA1 |  |  | 0 |
| 125A | EA1 |  |  | 0 |
| 6A | EA3 |  |  | 0 |
| 16A | EA3 | 1 | 1 | 2 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 |  |  | 0 |
| 63A | EA3 |  |  | 0 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 |  |  | 0 |
| 400A | EA3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 |  |  | 0 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 1 | 1 | 2 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 1 |  | 1 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E | 3 | 2 | 5 |
| 50.1 – 55.0KW | DK21E / PK21E / AK21E | 2 | 1 | 3 |
| 55.1 – 75.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 75.1 – 90.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 0.0 – 5.5KW | K2 |  |  | 0 |
| 5.6 – 7.0KW | K2 |  |  | 0 |
| 7.1 – 13.0KW | K2 | 3 | 2 | 5 |
| 13.1 – 24.0KW | K2 |  |  | 0 |
| 24.1 – 29.9KW | K2 |  |  | 0 |
| 30.0 – 37.0KW | K21 | 2 | 1 | 3 |
| 37.1 – 50.0KW | K21 |  |  | 0 |
| 50.1 – 55.0KW | K21 |  |  | 0 |
| 55.1 – 75.0KW | K21 |  |  | 0 |
| 75.1 – 90.0KW | K21 |  |  | 0 |
| 0.0 – 5.5KW | K3 | 1 |  | 1 |
| 5.6 – 7.0KW | K3 | 1 |  | 1 |
| 7.1 – 13.0KW | K3 | 1 |  | 1 |
| 13.1 – 24.0KW | K3 | 1 |  | 1 |
| 24.1 – 29.9KW | K3 |  |  | 0 |
| 30.0 – 37.0KW | K31 |  |  | 0 |
| 37.1 – 50.0KW | K31 | 1 | 1 | 2 |
| 50.1 – 55.0KW | K31 | 1 | 1 | 2 |
| 55.1 – 75.0KW | K31 |  |  | 0 |
| 75.1 – 90.0KW | K31 |  |  | 0 |
| 0.0 – 5.5KW | DN1/ PN1/ AN1 |  |  | 0 |
| 5.6 – 7.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 7.1 – 13.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 13.1 – 24.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 24.1 – 30.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 90.1 – 200.0KW | DM / PM |  |  | 0 |
| 110.0KW | DM / PM |  |  | 0 |
| 132.0KW | DM / PM |  |  | 0 |
| 160.0kW | DM / PM |  |  | 0 |
| 630A | DAE(O/G) | 3 |  | 3 |
| 1000A | DAE(O/G) |  |  | 0 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) |  |  | 0 |
| 3000A | DAE-TIE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 13 |  | 13 |
|  | G1 | 2 |  | 2 |

ITEM NO.:28 BOARD DESIGNATION: STATION SERVICE MCC # 0QA                 QTY: 1

BUSBAR RATING: 1000A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		1000A				DAE (I/C)				2

###### BUSCOUPLER:	1000A				DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 5 | 2 | 7 |
| 16A | E1 | 5 | 2 | 7 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 50 | 1 | 51 |
| 63A | E3 | 10 | 1 | 11 |
| 100A | E3 | 8 | 1 | 9 |
| 125A | E3 | 6 | 1 | 7 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA1 |  |  | 0 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 | 5 | 2 | 7 |
| 400A | EA3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 15 | 1 | 16 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 4 | 2 | 6 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 3 | 1 | 4 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E |  |  | 0 |
| 50.1 – 55.0KW | DK21E / PK21E / AK21E |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 18 |  | 18 |
|  | G1 | 2 |  | 2 |

ITEM NO.:29 BOARD DESIGNATION:  CW P/H PMCC # 0DC                     	QTY: 1

BUSBAR RATING: 3000A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		3000A				DAET (I/C)				2

###### BUSCOUPLER:	3000A				DAET (B/C)				1

| 32A | E3 | 4 | 2 | 6 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA1 |  |  | 0 |
| 32A | EA3 | 2 | 1 | 3 |
| 63A | EA3 | 2 | 1 | 3 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 |  |  | 0 |
| 400A | EA3 | 2 | 1 | 3 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 2 | 1 | 3 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 | 3 | 1 | 4 |
| 132.0KW | DM / PM | 3 | 1 | 4 |
| 630A | DAE(O/G) |  |  | 0 |
| 1000A | DAE(O/G) |  |  | 0 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 13 |  | 13 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 30 BOARD DESIGNATION:  FUEL OIL UNLOADING SWBD # 0DD     QTY: 1

BUSBAR RATING: 1000A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		1000A				DAET (I/C)				2

###### BUSCOUPLER:	1000A				DAET (B/C)				1

| 32A | E3 | 4 | 2 | 6 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 1 | 6 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 | 5 | 2 | 7 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 2 | 2 | 4 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 5 | 2 | 7 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 2 | 2 | 4 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E | 2 | 1 | 3 |
| 0.0 – 5.5KW | K2 | 5 | 1 | 6 |
| 5.6 – 7.0KW | K2 | 5 | 1 | 6 |
| 7.1 – 13.0KW | K2 |  |  | 0 |
| 13.1 – 24.0KW | K2 |  |  | 0 |
| 24.1 – 29.9KW | K2 |  |  | 0 |
| 30.0 – 37.0KW | K21 | 5 | 1 | 6 |
| 37.1 – 50.0KW | K21 |  |  | 0 |
| 50.1 – 55.0KW | K21 |  |  | 0 |
| 55.1 – 75.0KW | K21 |  |  | 0 |
| 75.1 – 90.0KW | K21 |  |  | 0 |
| 0.0 – 5.5KW | K3 |  |  | 0 |
| 5.6 – 7.0KW | K3 | 2 | 1 | 3 |
| 7.1 – 13.0KW | K3 | 2 | 1 | 3 |
| 13.1 – 24.0KW | K3 |  |  | 0 |
| 24.1 – 29.9KW | K3 |  |  | 0 |
| 30.0 – 37.0KW | K31 | 5 | 2 | 7 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 13 |  | 13 |
|  | G1 | 2 |  | 2 |

ITEM NO.:31 BOARD DESIGNATION:  ADMIN BUILDING SWBD # 0DE              QTY: 1

BUSBAR RATING: 1600A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		1600A				DAET (I/C)				2

###### BUSCOUPLER:	1600A				DAET (B/C)				1

| 32A | E3 | 4 | 2 | 6 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 1 | 1 | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 1 | 6 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 | 5 | 1 | 6 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 1 |  | 1 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 1 |  | 1 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E | 1 |  | 1 |
| 0.0 – 5.5KW | K2 | 1 |  | 1 |
| 5.6 – 7.0KW | K2 | 1 |  | 1 |
| 7.1 – 13.0KW | K2 |  |  | 0 |
| 13.1 – 24.0KW | K2 |  |  | 0 |
| 24.1 – 29.9KW | K2 |  |  | 0 |
| 30.0 – 37.0KW | K21 | 1 | 1 | 2 |
| 37.1 – 50.0KW | K21 |  |  | 0 |
| 50.1 – 55.0KW | K21 |  |  | 0 |
| 55.1 – 75.0KW | K21 |  |  | 0 |
| 75.1 – 90.0KW | K21 |  |  | 0 |
| 0.0 – 5.5KW | K3 |  |  | 0 |
| 5.6 – 7.0KW | K3 | 1 | 1 | 2 |
| 7.1 – 13.0KW | K3 |  |  | 0 |
| 13.1 – 24.0KW | K3 |  |  | 0 |
| 24.1 – 29.9KW | K3 |  |  | 0 |
| 30.0 – 37.0KW | K31 | 2 | 1 | 3 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 11 |  | 11 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 32 BOARD DESIGNATION:  INTAKE WATER SWBD #0DF     	QTY: 1

BUSBAR RATING: 630A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		630A				DAET (I/C)				2

###### BUSCOUPLER:	630A				DAET (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 2 | 1 | 3 |
| 16A | E1 | 2 | 1 | 3 |
| 32A | E3 | 1 | 1 | 2 |
| 63A | E3 | 3 | 1 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 150A | EA3 | 2 |  | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 1 | 6 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 | 5 | 2 | 7 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 2 | 2 | 4 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 1 | 1 | 2 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 1 | 1 | 2 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E | 1 | 1 | 2 |
| 0.0 – 5.5KW | K2 | 5 | 1 | 6 |
| 5.6 – 7.0KW | K2 |  |  | 0 |
| 30.0 – 37.0KW | K21 | 2 | 2 | 4 |
| 37.1 – 50.0KW | K21 |  |  | 0 |
| 50.1 – 55.0KW | K21 |  |  | 0 |
| 55.1 – 75.0KW | K21 |  |  | 0 |
| 75.1 – 90.0KW | K21 |  |  | 0 |
| 0.0 – 5.5KW | K3 |  |  | 0 |
| 5.6 – 7.0KW | K3 | 3 | 2 | 5 |
| 7.1 – 13.0KW | K3 | 2 | 2 | 4 |
| 13.1 – 24.0KW | K3 |  |  | 0 |
| 24.1 – 29.9KW | K3 |  |  | 0 |
| 30.0 – 37.0KW | K31 | 10 | 5 | 15 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 13 |  | 13 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 33 BOARD DESIGNATION:  INTAKE WATER DCDB# 0FB	          QTY: 1

BUSBAR RATING: 150A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		150A				CH (I/C)				2

###### BUSCOUPLER:	150A				CH (B/C)				1

| 6A | X | 4 | 1 | 5 |
| --- | --- | --- | --- | --- |
| 16A | X | 2 | 2 | 4 |
| 32A | X | 50 | 5 | 55 |
| 63A | X | 12 | 10 | 22 |
| 125A | X | 4 | 2 | 6 |
| 200A | X | 2 | 1 | 3 |
| 250A | X | 4 | 1 | 5 |
| 400A | X | 6 | 2 | 8 |
| 500A | X | 4 | 1 | 5 |
|  | CS | 2 |  | 2 |
|  | SH | 1 |  | 1 |
|  | TT | 8 |  | 8 |
|  | S | 2 |  | 2 |

ITEM NO.: 34 BOARD DESIGNATION:  DM PLANT/CPU MCC # 0SA     	QTY: 1

BUSBAR RATING: 1000A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		1000A				DAE (I/C)				2

###### BUSCOUPLER:	1000A				DAE (B/C)				1

| 32A | E3 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 1 | 1 | 2 |
| 100A | E3 | 1 | 1 | 2 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA1 |  |  | 0 |
| 400A | EA3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 2 | 1 | 3 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 2 | 1 | 3 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 2 | 1 | 3 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 132.0KW | DM / PM |  |  | 0 |
| 630A | DAE(O/G) | 2 | 1 | 3 |
| 1000A | DAE(O/G) |  |  | 0 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 11 |  | 11 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 35 BOARD DESIGNATION:  ETP/PT PLANT MCC #0SB	                QTY: 1

BUSBAR RATING: 630A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		630A				DAE (I/C)				2

###### BUSCOUPLER:	630A				DAE (B/C)				1

| 32A | E3 | 2 | 2 | 4 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 2 | 1 | 3 |
| 100A | E3 | 2 | 1 | 3 |
| 125A | E3 | 2 | 1 | 3 |
| 160A | E3 | 2 | 1 | 3 |
| 400A | EA3 | 1 | 1 | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 3 | 1 | 4 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 | 2 | 1 | 3 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 1 |  | 1 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 1 | 1 | 2 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E | 1 |  | 1 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 11 |  | 11 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 36 BOARD DESIGNATION:  WORKSHOP/RAW WATER MCC #0SC       QTY: 1

BUSBAR RATING: 630A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		630A				DAE (I/C)				2

###### BUSCOUPLER:	630A				DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 1 | 1 | 2 |
| 16A | E1 | 1 | 1 | 2 |
| 32A | E3 | 1 | 1 | 2 |
| 63A | E3 | 1 | 1 | 2 |
| 100A | E3 | 1 | 1 | 2 |
| 125A | E3 | 1 | 1 | 2 |
| 160A | E3 | 1 | 1 | 2 |
| 400A | EA3 | 1 | 1 | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 2 |  | 2 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 | 1 |  | 1 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 1 |  | 1 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 1 |  | 1 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 1 | 1 | 2 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 11 |  | 11 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 37 BOARD DESIGNATION:  ELECTROCHLORINATION MCC # 0SD      QTY: 1

BUSBAR RATING: 400A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		400A				DAE (I/C)				2

###### BUSCOUPLER:	400A				DAE (B/C)				1

| 32A | E3 | 5 | 1 | 6 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 6 | 1 | 7 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 3 | 3 | 6 |
| 160A | E3 | 3 | 1 | 4 |
| 400A | EA3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 5 | 1 | 6 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 | 2 | 1 | 3 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 3 | 1 | 4 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 2 |  | 2 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E | 1 |  | 1 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 15 |  | 15 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 38  BOARD DESIGNATION: AUX BOILER #0SE                                     QTY: 1

BUSBAR RATING: 400A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		400A				DAE (I/C)				2

###### BUSCOUPLER:	400A				DAE(B/C)				1

| 6A | E3 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- |
| 16A | E3 | 1 | 1 | 2 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 1 | 1 | 2 |
| 63A | E3 | 4 | 2 | 6 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 | 1 | 1 | 2 |
| 160A | E3 | 3 | 1 | 4 |
| 400A | EA3 | 3 | 1 | 4 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 |  |  | 0 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 2 | 1 | 3 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 | 2 | 1 | 3 |
| 0.0 – 5.5KW | DN1/ PN1/ AN1 | 2 | 1 | 3 |
| 5.6 – 7.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 7.1 – 13.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 13.1 – 24.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 24.1 – 30.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 132.0kW | DM / PM | 2 | 1 | 3 |
|  | CS | 1 |  | 1 |
|  | SH | 1 |  | 1 |
|  | TT | 15 |  | 15 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 39 BOARD DESIGNATION:  HYDROGEN PLANT MCC # 0SG	      QTY: 1

BUSBAR RATING: 630A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		630A				DAE (I/C)				2

###### BUSCOUPLER:	630A				DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 1 | 1 | 2 |
| 16A | E1 | 1 | 1 | 2 |
| 32A | E3 | 1 | 2 | 3 |
| 63A | E3 | 1 | 1 | 2 |
| 100A | E3 | 1 | 1 | 2 |
| 125A | E3 | 2 | 3 | 5 |
| 160A | E3 | 2 | 1 | 3 |
| 400A | EA3 | 2 | 1 | 3 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 2 | 3 | 5 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 1 | 2 | 3 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 1 |  | 1 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 1 | 1 | 2 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E | 1 |  | 1 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 11 |  | 11 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 40 BOARD DESIGNATION:  STP MCC #0SF	                                        QTY: 1

BUSBAR RATING: 250A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		250A				DAE (I/C)				2

###### BUSCOUPLER:	250A				DAE (B/C)				1

| 32A | E3 | 2 | 1 | 3 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 1 | 1 | 2 |
| 100A | E3 | 1 | 1 | 2 |
| 125A | E3 | 1 |  | 1 |
| 160A | E3 | 1 |  | 1 |
| 400A | EA3 | 1 |  | 1 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 1 |  | 1 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 1 |  | 1 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 0.0 – 5.5KW | DK2E / PK2E / AK2E | 1 |  | 1 |
| 5.6 – 7.0KW | DK2E / PK2E / AK2E | 1 |  | 1 |
| 7.1 – 13.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 13.1 – 24.0KW | DK2E / PK2E / AK2E |  |  | 0 |
| 24.1 – 29.9KW | DK2E / PK2E / AK2E |  |  | 0 |
| 30.0 – 37.0KW | DK21E / PK21E / AK21E | 1 |  | 1 |
| 37.1 – 50.0KW | DK21E / PK21E / AK21E |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 8 |  | 8 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 41 BOARD DESIGNATION:  FUEL OIL (LDO) MCC #0SH	                 QTY: 1

BUSBAR RATING: 400A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		400A				DAE (I/C)				2

###### BUSCOUPLER:	400A				DAE (B/C)				1

| 6A | E3 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- |
| 16A | E3 | 1 | 1 | 2 |
| 25A | E3 |  |  | 0 |
| 32A | E3 | 2 |  | 2 |
| 63A | E3 | 2 |  | 2 |
| 100A | E3 | 2 |  | 2 |
| 125A | E3 | 1 |  | 1 |
| 160A | E3 | 1 |  | 1 |
| 400A | EA3 | 1 |  | 1 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 |  |  | 0 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 50.1 – 55.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 55.1 – 75.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 75.1 – 90.0KW | DK21 / PK21 / AK21 | 2 | 1 | 3 |
| 0.0 – 5.5KW | DN1/ PN1/ AN1 | 1 | 1 | 2 |
| 5.6 – 7.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 7.1 – 13.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 13.1 – 24.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 24.1 – 30.0KW | DN1/ PN1/ AN1 |  |  | 0 |
| 132.0kW | DM / PM | 2 | 1 | 3 |
|  | CS | 1 |  | 1 |
|  | SH | 1 |  | 1 |
|  | TT | 8 |  | 8 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 42 BOARD DESIGNATION:  CCR AC MCC #0TA	QTY: 1

BUSBAR RATING: 1600A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		1600A				DAE (I/C)				2

###### BUSCOUPLER:	1600A				DAE (B/C)				1

| ISOLATING TRANSFORMER-25kVA |  | 2 |  | 2 |
| --- | --- | --- | --- | --- |
| 6A | E1 | 4 |  | 4 |
| 16A | E1 | 2 |  | 2 |
| 32A | E3 | 5 | 3 | 8 |
| 63A | E3 | 1 | 1 | 2 |
| 100A | E3 | 1 | 1 | 2 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 6A | EA1 |  |  | 0 |
| 25A | EA3 |  |  | 0 |
| 32A | EA3 | 1 | 1 | 2 |
| 63A | EA3 |  |  | 0 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 |  |  | 0 |
| 400A | EA3 | 1 | 1 | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 1 |  | 1 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 | 2 | 1 | 3 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 30.0 – 37.0KW | DK21 / PK21 / AK21 | 1 | 1 | 2 |
| 37.1 – 50.0KW | DK21 / PK21 / AK21 |  |  | 0 |
| 7.1 – 13.0KW | K2 |  |  | 0 |
| 13.1 – 24.0KW | K2 |  |  | 0 |
| 24.1 – 29.9KW | K2 |  |  | 0 |
| 30.0 – 37.0KW | K21 | 1 | 1 | 2 |
| 37.1 – 50.0KW | K21 |  |  | 0 |
| 50.1 – 55.0KW | K21 |  |  | 0 |
| 55.1 – 75.0KW | K21 |  |  | 0 |
| 75.1 – 90.0KW | K21 |  |  | 0 |
| 0.0 – 5.5KW | K3 | 1 | 1 | 2 |
| 5.6 – 7.0KW | K3 | 1 | 1 | 2 |
| 630A | DAE(O/G) | 1 | 1 | 2 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 10 |  | 10 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 43 BOARD DESIGNATION:  SERVICE BUILDING SWBD #0TB             QTY: 1

BUSBAR RATING: 1000A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		1000A				DAE (I/C)				2

###### BUSCOUPLER:	1000A				DAE (B/C)				1

| 25A | E3 |  |  | 0 |
| --- | --- | --- | --- | --- |
| 32A | E3 | 3 | 2 | 5 |
| 63A | E3 | 2 | 2 | 4 |
| 100A | E3 | 3 | 1 | 4 |
| 125A | E3 |  |  | 0 |
| 160A | E3 | 3 | 1 | 4 |
| 32A | EA3 | 2 | 1 | 3 |
| 63A | EA3 |  |  | 0 |
| 125A | EA3 |  |  | 0 |
| 200A | EA3 |  |  | 0 |
| 250A | EA3 | 1 | 2 | 3 |
| 400A | EA3 | 1 | 1 | 2 |
| 0.0 – 5.5KW | DK2 / PK2 / AK2 | 1 | 1 | 2 |
| 5.6 – 7.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 7.1 – 13.0KW | DK2 / PK2 / AK2 |  |  | 0 |
| 13.1 – 24.0KW | DK2 / PK2 / AK2 | 5 | 2 | 7 |
| 24.1 – 29.9KW | DK2 / PK2 / AK2 |  |  | 0 |
| 630A | DAE(O/G) |  |  | 0 |
| 1000A | DAE(O/G) |  |  | 0 |
| 1600A | DAE(O/G) |  |  | 0 |
| 3000A | DAE(O/G) |  |  | 0 |
|  | CS | 1 |  | 1 |
|  | CS-A |  |  | 0 |
|  | SH | 1 |  | 1 |
|  | SH-A |  |  | 0 |
|  | TT | 10 |  | 10 |
|  | G1 | 2 |  | 2 |

ITEM NO.: 44 BOARD DESIGNATION:  415V AC DBs (WALL MOUNTED)	QTY:30 NOS

BUSBAR RATING: 63A      	TYPE: DRAWOUT        		DOUBLE FRONT

###### RATING                               MODULE TYPE                            QUANTITY

INCOMER:		63A				H3 (I/C)				1

BUSCOUPLER:             -                                              -                                                         -

| 63A | E1 | 16 |  | 16 |
| --- | --- | --- | --- | --- |
| 63A | E3 | 4 |  | 4 |

ITEM NO.: 45 BOARD DESIGNATION: 220V DC DBs (WALL MOUNTED)           QTY:6 NOS

BUSBAR RATING:           			TYPE: FIXED 			         WALL MOUNTED

###### RATING                             MODULE TYPE                                 QUANTITY

INCOMER                    63A                                     H2 (I/C)                                                   1

BUSCOUPLER:             -                                              -                                                         -

| 6A | X | 16 |  | 16 |
| --- | --- | --- | --- | --- |
| 16A | X | 4 |  | 4 |

ITEM NO.: 46  BOARD DESIGNATION:SBMCC PANEL SURATGARH           QTY:2 NOS

BUSBAR RATING:           			TYPE: DRAWOUT		         WALL MOUNTED

###### RATING                             MODULE TYPE                                 QUANTITY

INCOMER                    100A                                     EM3(I/C)                                                 2

BUSCOUPLER:           100A                                     EM3(B/C)                                               1

NOTE: Board should be manufactured as per the Drawing No. : 3-96-186-14633/03 TO 3-96-186-14636/01, 3-96-186-14637/00 TO 3-96-186-14645/00