BHARAT HEAVY ELECTRICALS LIMITED

UNIT: JHANSI

(WORK. ENGG. & SERVICES DEPTT.)

SCHEME                		 :           Phase-II

EQUIP	MENT			:	Core Building Platforms

ITEM NO			:	LE- 1/7/6268 & 6269

LOCATION			:	Power Assy. Shop (Bay-9)

QUANTITY			:	2 Nos

SPECIFICATION NO.	:	10579/Rev-02

| SL.NO. | DESCRIPTION FOR BHEL REQUIREMENT | Confirmation/Deviation. | OFFERED |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | by vendor |  |  |  |
| 1 | DESCRIPTION & WORKPIECE MATERIAL |  |  |  |  |
| 1.1 | Core building platform will be used for making of transformer core. The platform shall be in two parts attached to each other. The main platform is placed on the floor area while the titling platform is bolted with supporting beam and it can be tilted at 90 degree from the horizontal surface. Both platforms shall have suitable lifting arrangements. The movement from horizontal to vertical is to be by EOT crane and is to be smooth. The distance between the core stacking beams is to be adjustable. Main building platform & tilting platform have to be machined to the required accuracy. Platform is to be designed so that the cores having 2, 3 & 5 limbs can be made on the same platform. | Vendor to confirm |  |  |  |
| 2 | Work Piece Material :- Material used is silicon steel sheets stacked together to form Transformer Core having weight of upto 100 Tonnes | Vendor to confirm |  |  |  |
| 3 | SCOPE: | Vendor to confirm |  |  |  |
| 3.1 | Complete design, manufacturing, erection & commissioning, testing of core building platform |  |  |  |  |
| 4 | CONFIGURATION& TECHNICAL SPECIFICATION: | Vendor to confirm |  |  |  |
| 4.1 | Configuration:-The core building plateform shall be fabricated from steel plate channels & I beams. |  |  |  |  |
|  | The equipment shall be of robust and stress relieved steel structure. The equipment shall consists of the following- |  |  |  |  |
| 4.2 | The building plateform shall be fabricated from MS plate channels, I beams etc having building area 5000x9900 mm with ladder arrangement. You may refer drawing BHEL/JHS/WE&S/A1-01739 for the purpose of general arrangement only. Thicknesses and other sizes of steel plates indicated in the drg are for reference only.Supplier can decide the values based on his own design however values should not be less than what are indicated in the drgs. BHEL/JHS/WE&S/A1-01740 col-A,B and BHEL/JHS/WE&S/A1-01741 or wherever indicated.Building plateform should be strong enough to take suitable loading of hydraulic clamping of core by hydraulic jack of 20 tonnes. Thickness of the steel plates after machining shall not be less than the specified thickness in the drawing . Supporting stiffeners below the steel strips shall be 5nos. in place of 3 nos. specified in the drawing no. BHEL/JHS/WE&S/A1-01740 it.22. The various dimensions of plates channels, I-beams, and tie rods mentioned are minimum. Supplier can select as per their design requirement to handle core of weight 100 tonnes as per clause 1.1. | Vendor to confirm the |  |  |  |
| 4.3 | Tilting plateform shall be of size 2430 x 9900 mm to drg. BHEL/JHS/WE&S/A1-01740 item 1 to 18 Black steel plate .Thickness 32/25/20 mm minimum. Tilting plateform shall be supported by clamping I-beam and tie rods. The size of I beam to be decided suitably as per loading requirement. | Vendor to confirm the |  |  |  |
| 4.4 | Main building plateform & Tilting plateform have to be machined to the required accuracy as per clause 10. |  |  |  |  |
| 4.5 | Plateform shall be equiped with following accessories. |  |  |  |  |
| 4.5.1 | Supporting bridges for frame - 2100 high to drg. No. BHEL/JHS/WE&S/A1-01743( it.1,2 4 to 7) Qty.10 Nos. |  |  |  |  |
| 4.5.2 | Supporting bridges for frame - 1700 high to drg. No. BHEL/JHS/WE&S/A1-01743( it.1,2 4 to 7) Qty.22 Nos. |  |  |  |  |
| 4.5.3 | Core support to drg. BHEL/JHS/WE&S/A1-01743 (it. 8 to 11 ) Qty.15 Nos. |  |  |  |  |
| 4.5.4 | Clamping pads for supports to drg. BHEL/JHS/WE&S/A1-01743 ( it.12 & 13) Qty.30 Nos. |  |  |  |  |
| 4.5.5 | Screw jacks to drg. No. BHEL/JHS/WE&S/A1-01744 (It. 12 to 14) Qty.20 Nos. |  |  |  |  |
| 4.5.6 | Clamping pads for screw jacks drg. No. BHEL/JHS/WE&S/A1-01743 (It 12 to 14) Qty. 80 Nos. |  |  |  |  |
| 4.5.7 | Clamping bolts for clamping pads Drg. No. BHEL/JHS/WE&S/A1-01743(It.15 to 16 ) Qty.110 nos. |  |  |  |  |
| 4.5.8 | Axle for core supports Drg.No. BHEL/JHS/WE&S/A1-01743 (It.17) Qty.15 Nos. |  |  |  |  |
| 4.5.10 | Studs for axles Drg BHEL/JHS/WE&S/A1-01743( it.18) Qty 30 Nos. |  |  |  |  |
| 4.5.11 | Cross pieces for frame 670 Long Drg. BHEL/JHS/WE&S/A1-01743(It.20)Qty.2 Nos. |  |  |  |  |
| 4.5.12 | Cross pieces for frame 1210 Long Drg. BHEL/JHS/WE&S/A1-01743(It.19)Qty.2 Nos. | Vendor to confirm |  |  |  |
| 4.5.13 | Angle cross pieces for adjusting as per Drg. BHEL/JHS/WE&S/A1-01744 (It.1 to 5) Qty.3 Nos. Angle cross pieces for staking to Drg. BHEL/JHS/WE&S/A1-01744 (It.6 to 11) Qty.3 Nos. |  |  |  |  |
| 4.5.14 | Core support 2400 Long to drg. BHEL/JHS/WE&S/A1-01743 (it.15 to 16 )Qty.8 Nos. Core support 2000 Long to drg. BHEL/JHS/WE&S/A1-01743 (it.17 to 18) Qty.8 Nos. Rocker rails item 20 to drawing BHEL/JHS/WE&S/A1-01739. |  |  |  |  |
| 5 | LEVELING & ANCHORING SYSTEM |  |  |  |  |
| 5.1 | Complete anchoring system including foundation bolts, anchoring materials, fixators,leveling | Vendor to confirm |  |  |  |
|  | shoes etc. shall be supplied by vendors. |  |  |  |  |
| 5.2 | Special tools & equipment required for erection of the plateform shall be brought by the vendor. |  |  |  |  |
| 5.3 | Necessary tools like Torque Wrench, Spanners, Keys, grease guns etc. for operation and maintenance of the plant should be supplied. List of such tools should be submitted with offer. |  |  |  |  |
| 6 | OPTIONAL ITEM | Vendor to confirm |  |  |  |
| 6.1 | Assy of core staking bed & intermediate member to drg. BHEL/JHS/WE&S/A1-01746 having over all dimn. 4400x561x1000 width each consisting of following items Qty - 5 sets |  |  |  |  |
| 6.2 | Staking bed made of 250x125x1000 RS beam, 100x50x414 RS channel, 16tk / 10tk steel plates having over all dimn.1000x1000x415 as per drg. BHEL/JHS/WE&S/A1-01747 var-00 Qty- 2 Nos. |  |  |  |  |
| 6.3 | Staking bed made of 250x125x1000 RS beam, 100x50x414 RS channel, 16tk/ 10tk steel plates having over all dimn.1000x415x500 as per drg. BHEL/JHS/WE&S/A1-01747 var-01 Qty - 2 Nos |  |  |  |  |
| 6.4 | Staking bed made of 250x125x250 RS beam 100x50x414 RS channel,16tk /10tk steel plates having over all dimn.1000x415x250 as per drg. BHEL/JHS/WE&S/A1-01747 var-02 Qty - 2 Nos. |  |  |  |  |
| 6.5 | Staking bed made of 250x125x125 RS beam 100x50x414 RS channel, 16tk/10tk steel plates having over all dimn.1000x415x125 as per drg. BHEL/JHS/WE&S/A1-01747 var-03. Qty - 2 Nos. |  |  |  |  |
| 6.6 | Staking bed made of 100x50x414 RS channel, 16tk/10tk steel plates having over all dimn. 570x445x125 as per drg. BHEL/JHS/WE&S/A1-01747 var-04.RS beam 300x140x125 Qty - 2 Nos. |  |  |  |  |
| 6.7 | Assy of intermediate member made of 125x70x4400, 125x70x425 RS beam 12tk steel plates 20tkx30x100 steel stopper having over all dimn.4400x570x146 as per drg. BHEL/JHS/WE&S/A1-01747 var-05 The assy shall be machined with tolerence Ra.20 Qty - 1 No. |  |  |  |  |
| 7 | DOCUMENTATION :- Three set of following documents (Hard Copies) in English Language should be supplied alongwith plate form | Vendor to confirm |  |  |  |
| 7.1 | Operating manual of platform |  |  |  |  |
| 7.2 | Detailed maintenance manual of platform with all manufacturing drawings of platform assembly / subassembly/parts. |  |  |  |  |
| 7.3 | Detailed design and manufacturing drawings of all supplies such as assemblies / parts. |  |  |  |  |
| 8 | FOUNDATION :- Vendor shall submit the preliminary layout drawing for getting BHEL's approval within one month from the date of letter of Intent (LOI)/P.O. | Vendor to confirm |  |  |  |
| 9 | ERECTION & COMMISSIONING :- | Vendor to confirm |  |  |  |
| 9.1 | Supplier to take full responsibility for carrying out the erection , commission & testing of equipment, other requirements like crane and lifting tackles etc. shall be provided by BHEL. Details of these requirements should be informed by vendor in advance in the offer. All tools tackles & manpower for E &C shall be provided by vendor. Only civil work according to vendor’s drawing shall be done by BHEL. |  |  |  |  |
| 9.2 | Successful proving on BHEL Transformer Core of 100 + 25%( 125 )Ton by the supplier shall be considered as part of commissioning.(The core weight of 100 Tonnes ) |  |  |  |  |
| 9.3 | Charges of Erection and Commissioning shall be submitted with the offer. |  |  |  |  |
| 9.4 | Charges, duration , terms & conditions for E&C should be furnished in detail separately by vendor along with offer |  |  |  |  |
| 10 | ACCURACY TESTS :- Complete prove out of core assembly platform with proper leveling of 0.2 mm per meter length should be carried out to the satisfaction of BHEL engineers. | Vendor to confirm |  |  |  |
| 11 | PACKING :- Standard & rigid packing for all items of complete plant, all accessories and other supplied items to avoid any damage/loss in transit . When plant is dispatched in containers, all small loose items shall be suitably packed in boxes. | Vendor to confirm |  |  |  |
| 12 | GURANTEE :- 24 months from the date of acceptance of the machine irrespective of supply, erection & commissioning |  |  |  |  |
| 13 | GENERAL :- The vendor should submit the following information. | Vendor to confirm |  |  |  |
| 13.1 | Model of the platform. |  |  |  |  |
| 13.2 | Total Tilting and lifting load in Tons. |  |  |  |  |
| 13.3 | Floor area required (Length, Width, Height) for complete platform & accessories. |  |  |  |  |
| 13.4 | Painting of Platform & Panels : Jasmine Yellow as per IS -5/1961 |  |  |  |  |
| 13.5 | Total weight of the part / platform |  |  |  |  |
| 13.6 | Vendor to submit, alongwith offer, the reference of customers where similar platforms have been supplied mentioning the customer, platform model, major specification of the supplied platform year of supply etc. |  |  |  |  |
| 14 | BHEL reserves the right to verify information submitted by vendor .In case the information is found to be false / incorrect , the offer shall be rejected. | Vendor to confirm |  |  |  |

15. MANDATORY DOCUMENTS REQUIRED TO BE SUBMITTED WITH THE OFFER-

- Dimensioned outline general arrangement drawing with write up of offered platform with list of standard accessories.

- Details of safety devices / guards provided.

3.   Quotation with item wise prices of spares for 2 years normal working.

- Details & list of standard & optional accessories offered. All standard accessories should be         supplied with the platform without extra cost.

- Catalogues / leaflets of machine offered .Operation & maintenance manual ( 3sets) & tool kit to be

provided with machine.

6.  Charges for the Erection & commissioning of the machine at BHEL Jhansi should be

quoted in offer separately.

16. GENERAL CONDITIONS-

- The machine should be painted with one coat of red oxide primer & two coats of finishing paint before dispatch. Colour of the finishing paint should be Jasmine Yellow to IS 5/1961.

- Machine will be inspected & tried by our engineer prior to dispatch. Provision is to be made for deputation of commissioning engineer for erection & commissioning of the machine at BHEL , Jhansi.

- The performance of the machine should be demonstrated by the supplier after its successful commissioning at our works. Training to our operators should be given by supplier’s representative.

4.	BHEL reserves thr right to verify information submited by vendor.In case the information is found to be false / incorrect , the offer shall be rejected.

17. QUALIFYING CONDITIONS:-

Only those vendors, who have well established Design ,fabrication , machining & lifting facilities for more than 15 tons  with them and are in field of Designing, fabricating & machining of heavy component ( min.15 Ton) since  last 5 years  OR   Who have supplied similar type of core building platform for power transformers in past can be considered. Documentary proof along with copy of purchase orders executed in support of above is essentially required before we consider the offer technically.

- Name of the customer/ company where similar platforms  is installed

- Complete postal address of the customer

- Year of commissioning

- Name and designation of the contact person of the customer

- Phone , Fax No. and e-mail address of the contact person of the customer

- Performance certificate from the customer regarding satisfactory performance of  the platforms

supplied to them. However if arranging performance certificate not possible commissioning report can be submitted.

| Drafted By | Checked By | Approved By |  |  |
| --- | --- | --- | --- | --- |
| A. Singh Dy.Mgr(WEX) | I. Mathur Mgr(WEX) | S.J.Chakraborty DGM(WEX) | A.K.Srivastava AGM(FW) | V.K.Agrawal AGM(TRM/TRP) |

Rev02- 1)Hydraulic arrangement for lifting deleted.

2) Qualifying condition modified.

3) Drawing numbers mentioned.