SPECIFICATION

OF

EPM LIFTING MAGNETS

- Application:-

EPM lifting magnets (1T & 2T capacity) will be used to ensure smooth & safe handling of gas cut / machined components in fabrication shops.

- Technical Specification:-

  - Type			:	Battery operated electro permananent

magnetic lifter (EPM lifting magnet).

  - Capacity		:	Lifting capacity  - 1T (8 nos.)

Lifting capacity – 2T (8 nos.)

  - No. of Poles		:	2 pole

  - Lifting/Testing capacity:	1T/3T (8 nos)

:	2T/6T (8 nos)

  - Battery			:	Maintenance free complete by sealed

re-chargeable batteries.

- Technical Features:

  - Light weight & robust construction.

  - Battery power will be needed only to switch ON & OFF.

  - No battery power will be needed to keep EPM lifting magnet ON.

  - With  full charged battery the lifter should work for around 300 times switch ON & OFF.

  - Warning signal  when battery charge is low.

  - In – built battery charger (operating at 230VAC) with over charge protection feature.

  - Built –in space for keeping battery charging  cable/ socket.

  - Front panel should have following features/display:-

- ON/OFF switches.

- Mag / Demag lamp.

- Audio alarm.

- Battery condition monitor.

- Charge indicator.

  - EPM lifting magnet should work on un-machined mill  supplied plates.

  - Fitted with lifting Eye & Protection cover for front panel.

- Following information  to be provided by vendors with the offer:-

  - Chart showing holding power of lifting magnets w.r.t.  thickness.

  - Change in holding power w.r.t. roughness of contact surface (As rolled, machined etc.).

  - Change in holding power w.r.t. composition of steel like low carbon, medium carbon, high carbon, low alloy steel, cast iron etc.

  - Calculation of lifting capacity of EPM lifting magnet for a particular case like :

Thickness x Surface roughness x composition of steel x rated

capacity= Lifting capacity.

- Scope of Supply( In accordance with clause no.2.0,3.0 & 4.0 of specification)

  - EPM lifting magnets fitted with maintenance free batteries.

  - Remote control pendant for  ON & OFF control.

  - Complete in ready to use condition.

  - Three sets of operation & maintenance manuals consisting of:-

- Operating instructions.

- Maintenance instructions.

- Electrical circuit  diagrams of control PCB, interconnection  diagrams  of various components (e.g. relays, push buttons, interfacing with the PCBs etc.)

- Battery charging instructions.

- Storage life of battery with or without charge.

- Battery size, specification & make.

- Holding power charts w.r.t. thickness, contact surface roughness, material grade etc.

- Dos & Don’ts.

  - Recommended spares for 2 years smooth & trouble free working indicating identification no. of each item.

- Pre-dispatch inspection at vendor’s works by BHEL personnel.

- Commissioning at BHEL works by vendor.

- 24 months  guarantee for smooth & trouble free working. Replacement of defective components free of cost during guarantee period.

- List of customers to whom same or higher capacity EPM lifting magnets have been supplied & working satisfactory.

P.K.Srivastava				 Nishan Singh			   H.P.Roy

DGM(Bl-II-Maint)	     		 AGM (Bl-II)			SDGM (WT)