TECHNICAL SPECIFICATION OF HORIZONTAL MILLING MACHINE  EMT-6758

- PURPOSE:

The machine shall be used for machining of small components of various insulating materials such as epoxy based glass epoxy laminates, phenolic based cotton fabric laminates.

- MACHINE SPECIFICATIONS:

2.1      SPINDLE

2.1.1   Spindle taper                                  :  ISO 40

2.1.2   Spindle motor power                      :  5.5 KW or more

2.1.3  Spindle speed range                         :  45 to 2000 r.p.m. (approx.)

2.1.4    No’s of speed                                 :  10 minimum

2.1.5    Minmum/Maximum distance         :   35/440 mm

between spindle axis to

table surface.

2.1.6    Minmum/Maximum distance         :  300mm

Between inner face of table

To vertical coulmn of guide ways

2.1.7      Minmum/Maximum distance        :  160 mm

between spindle axis to bottom

surface of over arm.

2.2       TABLE

2.2.1    Overall clamping area                      :  1350 mm x 320 mm approx.

2.2.2   Maximum Permissible load on table :   275 kg minimum.

2.2.3    The table top should have 3 longitudinal T-slots. Size 14x63

POWER OPERATED TRAVERSE

2.2.4    Longitudinal traverse                        : 750 mm minimum

2.2.5  Vertical traverse                                :   375  mm minimum

2.2.6    Cross traverse                                   :  265 mm minimum

2.3                   FEED RATES

2.3.1    No’s of feed                                       :  10 minimum

2.3.2    Feed rates

Longitudinal & cross                         :  14 -900 mm /minute (approx.)

Vertical                                               :  4 -250 mm/minute(approx.)

2.3.3   Rapid traverse rate

Longitudinal & cross                         : 2800 mm/minute or more

Vertical                                              :  800 mm/minute or more

- GENERAL FEATURES OF MACHINE:

  - The machine must be rigid structure and vibration free construction. All gears

must be case hardened and ground. All the slides should be rigid to

withstand heavy stock removal. It should have independent main drive and feed

drive motors.

  - A suitable manual lubricating system for lubrication of all moving / rotating

elements should be there. Open lubrication parts should be protected

from glass dust generated during machining of insulating laminates as

referral in sl.no.1.0.

3.3   Each axis should have mechanical stopper.

.         3.4   Total power requirement of the machine to be specified by the supplier.

3.5   Overall space required for the installation of the machine to be specified

by the supplier.

3.6    Standard test chart for accuracy test, is to be submitted with the offer.

- ATTACHMENT & ACCESSORIES

Qty

4.1  Machine vices with fixed base, width of jaws 160 mm.                      1 No.

4.2  Universal swivel vices with fixed base , width of jaw 160 mm.          1 No.

4.3 DRAW BOLT TYPE REDUCTION MILLING SOCKETS

4.3.1  ISO40( B40) / MT 2                                                                 3 No.

4.3.2    ISO 40( B40 ) / MT 3                                                              3 No.

##### 4.4  STUB ARBORS WITH KEY

###### 4.5.1 ISO40X32X500                                                                                  3 No.

##### 4.5.2 ISO40X22X500                                                                                  3No.

4.6 Work piece illumination light.                                                                         1No.

4.7  Foundation parts.                                                                                            1set

5.0           POWER SUPPLY / ENVIRONMENT CONDITION

5.1  The machine should be suitable for operation in the following conditions:-

5.2   Power supply for the machine : 415 V + 10% / -15% ,3 phase, 50 Hz +/-3%

5.3  Temperature :     5 to 45 deg.C.

5.4   Relative humidity :    95% max.

5.5  Compressed air :  4 to 6 Kg / sq.cm

5.6    Other conditions: Similar to tropical country.

6.0              MECHANICAL AND ELECTRICAL SPARES

6.1	Lead screw and nuts

6.1.1          x-axis                                                                              1 No.

6.1.2	 y-axis                                                                              1 No.

6.1.3	 z-axis                                                                               1. No.

6.2	 Wipers                                                                             1set

6.3             indicating lamps and push buttons                                  5 No’s each type

6.4             Relays and contactors                                                      1No each type

6.5             Limit Switches                                                                  2  No’s each type

Vender should include the items of spares( Sl.No.6.1.1 to 6.5 ) alongwith the

other recommended spares.

7.0    OPERATION AND MAINTENANCE MANUALS

7.1   4 copies of Operation & Maintenance manuals in English language shall be supplied.The operarion & maintenance manual, shall contain the following details alongwith the other details.

7.1.1    Electrical circuit diagramme indicating the cables, terminal strip etc with part

list and proper identification no.to be supplied.

7.1.2   Assembly drawings of each unit with marked list of each component with its

specification & make will be provided.

7.1.3    Detail catalogue of bought out items with its specification & make will be

provided.

8.0 GENERAL ARRANGMENT DRAWING OF HORIZONTAL.MILLING M/C

8.1 General arrangement drg. is to be submitted for BHEL approval within 4 weeks after placement of purchase order

8.2 General arrangement drg. should include the following points.

8.2.1 Foundation drawing of the m/c.

8.2.2 Total electrical load.

8.2.3 Overall  size of the m/c.

9.0    COLOUR OF THE MACHINE

9.1 The colour of the machine should be apple green as per ISO.

10.0     PREACCEPTANCE OF THE MILLING M/C AT VENDOR WORKS

10.1The vertical milling m/c will be preliminary inspected at the vendor’s works by BHEL Engineers for accuracy test, feed rate and rapid traverse etc. before dispatch.

11.0  ERRECTION AND COMMISSIONING OF THE VERETICAL MILLING

M/C AT BHEL WORKS

11.1  M/C will be erected by BHEL as per supplier’s foundation drg. and

commissioning will be done under the supervision of supplier’s representative.

11.2   Vendor will prove out the accuracy test at BHEL works as per the chart

supplied by the vendor with tender document.

##### 12.0	GUARANTEE

12.1 The machine shall be guaranteed for  18 months from the date of  commissioning and job proving at BHEL works. Any spares required during guarantee period shall have to be arranged by the vender, free of cost( upto BHEL Haridwar stores ) any duty levied have to be borne by the customer.

13.0      PACKING

13.1  Packing box will be rigid for all the items of the machine, all accessories and other supplied items in wooden boxes to avoid any damage/loss in transit.  All small and loose items like spares will  be suitably packed in wooden boxes. Packing list shall to be attached with each packing box.