ANNEXURE II

SPECIFICATIONS / COMPLIANCE

Technical Prequalification Compliance

Note:

- The vendor shall state in the “Compliance” column Y (yes) or N (no)

- Wherever specific documents / proofs are required, the same shall be given by the vendor without fail.

| Sl No | Criteria | Compliance(Y/N) | Remarks |
| --- | --- | --- | --- |
| 1 | The bidder should be either OEM of Network Active Components who is willing to undertake total scope of work or an authorized system integrator of the active OEM having direct purchase and support agreement with OEM for last 3 years. Authorization certificate from OEM to be submitted along with the bid. |  |  |
| 2 | Bidder shall be an existing network & security integrator of similar enterprise network setups for the last 3 years. |  |  |
| 3 | The bidder should have an office in Andhra Pradesh, preferably Hyderabad and the project should be coordinated by an official from this office throughout the lease period. |  |  |
| 4 | The Bidder should have successfully executed Enterprise/Business networks (educational institutions and pure IP phone networks to be excluded) of the OEM whose products are being offered to BHEL, in India, during the last 3 years as on 30-06-2009. The number and value of such orders should satisfy one of the below: - Three Enterprise Networks, each order value of not less than the 150 lakhs on Lease Rental or 125 lakhs on outright purchase basis Or - Two Enterprise Networks, each order value of not less than 250 lakhs on Lease Rental / 200 lakhs on outright purchase basis Or - One Enterprise Network order of value not less than 450 lakhs on Lease Rental / 400 lakhs on outright purchase basis. Evidence (POs and Completion Certificates) in support of the above should be produced. |  |  |

TECHNICAL SPECIFICATIONS

3.0  General Specifications

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| 1 | All switches shall be of enterprise makes, not SOHO products. The vendor shall ensure all the components work seamlessly and in tandem together without any interface problems. The switches offered should be suitable for continuous operation round the clock 365 days a year. |  |  |
| 2 | The active components must be UL listed / certified, passive components should be certified by UL / ETL / Pyramid / equivalent. Proof of the same should be submitted. |  |  |
| 3 | The makes and models / brands offered for network switches, passive components, racks shall be of OEMs who possess ISO / NQA (National Quality assurance) certification. |  |  |
| 4 | Entire switching solution excluding DSLAM, Router, Firewall & IPS appliance should be from one OEM. |  |  |
| 5 | The OEMs of active and passive components should not have filed for bankruptcy or bankruptcy protection. |  |  |
| 6 | The active and passive systems, components offered should not have been declared end-of-life during the time of delivery. The respective OEM should support the product for at least five years from date of delivery and letter from OEM in support of this should be attached. |  |  |
| 7 | All products supplied shall be genuine, new and procured directly from OEM or through their authorized channels after the date of placement of order. Refurbished / counterfeit products, products procured from unauthorized sources or grey market should not be supplied. Documents shall be provided to verify the source and date of purchase. |  |  |
| 8 | Entire range of structured cabling components should be of the same make. |  |  |
| 9 | Quantities mentioned against passive, work and service components are approximate. They will be billed as per actual quantities and measurements observed on site after completion of the project execution. |  |  |
| 10 | The bidder should have a 24 X 7 operational Technical Assistance Centre for operating in India. Tie ups with third parties not acceptable. |  |  |

3.1  Core / Distribution Switches

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements common to all Core / Distribution Switches |  |  |  |
| 1 | Architecture Modular or Fixed Configuration Multilayer Rack mountable Switch of Wire speed and non blocking architecture. Rack-mount kit to be provided along with the switch. Fixed Configuration switch if offered should support stacking for future expansion. Modular Switch should have at least one vacant slot. |  |  |
| 2 | Switch Redundancy The Switch should support Redundant Power Supplies |  |  |
| 3 | Flash & Memory The proposed switch should have enough Memory (Flash and RAM) to hold the latest Software Release. It should support all features of switch and parameters like MAC Address Table, IP Routing Tables, VLANs, ACLs, etc. at their peak values as claimed in the Data Sheets of the Switch. |  |  |
| 4 | Switching Fabric Capacity The Switch should support a minimum of 24 Gbps of Switching Fabric Capacity. |  |  |
| 5 | Switch Forwarding Rates (Layer 2) The Switch should Support Minimum Switching (Layer 2) Performance of 17 Mpps. |  |  |
| 6 | Routing Capability (Layer 3) The Minimum Routing (Layer 3) Performance should be 17 Mpps . |  |  |
| 7 | 1000BASE –TX / LX / SX Should support at least 12 nos. of 1000 Base TX / LX / SX Ports either built-in or through SFPs or Modules so that Fiber/Copper connectivity can be configured to suit user requirement by procuring required number Fiber/Copper modules. These modules will be included in the BOM. |  |  |
| Connectivity & Filtering |  |  |  |
| 8 | 802.3ad Should support Industry Standard Port/Link Aggregation for All Ports. |  |  |
| 9 | Jumbo Frames Jumbo Frames support up to 9K Bytes on Gigabit / 10 G Ports |  |  |
| 10 | Storm Control Support for broadcast/multicast/unicast storm control to prevent degradation of switch performance from faulty end stations |  |  |
| Layer 2 Functionality |  |  |  |
| 11 | 802.1Q Should support 802.1Q protocol |  |  |
| 12 | Type of VLANs Should support port, protocol and subnet based VLANs |  |  |
| 13 | Voice VLAN Should Support Voice VLAN for separating and prioritising VoIP traffic |  |  |
| 14 | VLAN Trunking Protocol or Equivalent Should support VTP or equivalent protocol to reduce administrative burden of configuring VLANs on multiple switches |  |  |
| 15 | No. of VLANs Should support minimum 1000 active VLANs |  |  |
| 16 | MAC Addresses No. of MAC Address Supported: 10000 |  |  |
| Layer 3 Functionality |  |  |  |
| 17 | RIP/RIP2 Should support RIP/RIP2 |  |  |
| 18 | No. of Route Entries Should support minimum 1000 Route entries |  |  |
| 19 | Access Control Lists Should support Standard and Extended ACLs |  |  |
| 20 | Various type of ACLs Should support various types of ACLs like port based/time based. |  |  |
| 21 | Detection of Attacks Should support real time detection of DoS attacks, Hacker attacks from Internal/external sources |  |  |
| 22 | MAC Address Filtering Should Support MAC Address Filtering based on source and destination address |  |  |
| 23 | 802.1X Should support 802.1X Network Security and Authentication, Mac address / IP based port filtering support |  |  |
| 24 | RADIUS Should have support for RADIUS |  |  |
| 25 | NAC Should support integration with NAC solutions. Please provide list of NAC appliances those can be integrated. |  |  |
| Switch Redundancy |  |  |  |
| 26 | Redundancy in Hardware Should support Redundant for Power Supply |  |  |
| Network Protocols |  |  |  |
| 27 | STP Should Support Spanning Tree Protocol, Rapid Spanning Tree Protocol, Multiple Spanning Tree Protocol |  |  |
| 28 | DNS, TFTP, NTP Should support DNS, TFTP and NTP protocols |  |  |
| Quality of Service |  |  |  |
| 39 | QoS Scheduling Should support QoS scheduling with queues supported in hardware |  |  |
| 30 | Queues per port Should support minimum 4 queues per port |  |  |
| 31 | Traffic Classification Should support policy based traffic classification based on Type of Service (ToS), IP Precedence mapping, Layer 2/3/4 defined traffic flows, MAC address, VLANs |  |  |
| Multicasting |  |  |  |
| 32 | Multicast Should support H/W based IPv4 Multicasting |  |  |
| 33 | IGMP Should Support IGMP v1, v2 , v3 |  |  |
| 34 | PIM Should support Protocol Independent Multicast - Sparse Mode |  |  |
| Management |  |  |  |
| 35 | Network monitoring/ management Switch should be manageable through NMS on per port/switch basis with common interface for all manageable devices on the network. Should Support syslog, SNMP, RMON/RMON-II, SSH, telnet, web management |  |  |
| 36 | Port Mirroring Should support port mirroring feature for monitoring network traffic of a particular port/VLAN/group of ports/entire switch. |  |  |
| Software |  |  |  |
| 37 | Software Version Version of software for supplied switch should be latest release to support all required features |  |  |
| 38 | Software Updates Software updates should be bundled for the entire contract period |  |  |
| IEEE Standards Compliance |  |  |  |
| 39 | IEEE Standards |  |  |
|  | Ethernet: IEEE 802.3, 10BASE-T |  |  |
|  | Fast Ethernet: IEEE 802.3u, 100BASE-TX, 100BASE-FX |  |  |
|  | Gigabit Ethernet: IEEE 802.3z, IEEE 802.3ab, 1000BASE-X (mini-GBIC/SFP), 1000BASE-SX, 1000BASE-LX/LH |  |  |
|  | IEEE 802.1D Spanning-Tree Protocol |  |  |
|  | IEEE 802.1S & 1W for Rapid Spanning tree convergence |  |  |
|  | IEEE 802.1p CoS Prioritization |  |  |
|  | IEEE 802.3x Flow Control |  |  |
|  | IEEE 802.3ad Link Aggregation |  |  |

3.11  Full Layer 3 Distribution Switch (Core Switch at ITX and Backup at CFD)

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Additional Hardware & Performance Requirements Full Layer 3 Distribution switches |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Redundant Power Supply Should be supplied with internal / external redundant power supply |  |  |
| 4 | Full Layer 3 Features RIPv1, RIPv2, Built in OSPF or OSPF through Software upgrade |  |  |

3.12  Basic Layer 3 Distribution Switch

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Additional Hardware & Performance Requirements basic Layer 3 Distribution Switches |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Basic Layer 3 Features RIPv1, RIPv2 |  |  |

3.2. Aggregation Switches

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Architecture Fixed Configuration Basic Layer 3 Rack mountable Switch. Rack mount kit to be provided along with the switch. Switch should support Redundant Power Supply |  |  |
| 4 | Flash & Memory The proposed switch should have enough Memory (Flash and/or any other) to hold the latest Software Release. The Memory should be large enough to support fully populated ports, all features of switch and parameters like MAC Address Table, VLANs etc. at their peak values as claimed in the Data Sheets of the Switch |  |  |
| 5 | Switching Fabric Capacity The Switch should support a minimum of 30 Gbps of switching Fabric Capacity. |  |  |
| 6 | Switch Forwarding Rates (Layer 2) The Switch should Support Minimum Switching (Basic Layer 3) Performance of 35 Mpps. |  |  |
| 7 | No. of Ports The Switch should have minimum 24 nos. 10/100/1000BaseTX ports and 2 nos. of Dual Purpose or SFP based Gigabit Uplinks. One of the fiber uplink ports should be loaded with 1000 Base LX module and other should have 1000 Base TX Connectivity. |  |  |
| 8 | Type of ports All Autosensing, Auto-negotiating, Auto-MDIX on all UTP ports |  |  |
| Connectivity & Filtering |  |  |  |
| 9 | 802.3ad Should support Industry Standard Port/Link Aggregation for All Ports. |  |  |
| 10 | Jumbo Frames Jumbo Frames support up to 9K Bytes |  |  |
| 11 | Storm Control Support for broadcast/multicast/unicast storm control to prevent degradation of switch performance from faulty end stations |  |  |
| Layer 2 Functionality |  |  |  |
| 12 | 802.1Q Should support 802.1Q protocol |  |  |
| 13 | Type of VLANs Should support port, protocol and subnet based VLANs |  |  |
| 14 | Voice VLAN Should Support Voice VLAN for separating and prioritising VoIP traffic |  |  |
| 15 | VLAN Trunking Protocol or Equivalent |  |  |
| 16 | No. of VLANs Should support VTP or equivalent protocol to reduce administrative burden of configuring VLANs on multiple switches |  |  |
| 17 | MAC Addresses No. of MAC Address Supported: 8000 |  |  |
| Security Features |  |  |  |
| 18 | MAC Address Filtering Should Support MAC Address Filtering based on source and destination address |  |  |
| 19 | DHCP Snooping Should support DHCP Snooping to filter untrusted DHCP messages |  |  |
| 20 | 802.1X Should support 802.1x Network Security and Authentication |  |  |
| 21 | 802.1x with VLAN Assignment/ Guest VLAN Should support 802.1x with VLAN Assignment/Guest VLAN features |  |  |
| 22 | RADIUS Should have support for RADIUS |  |  |
| 23 | NAC Should support integration with NAC solutions |  |  |
| Network Protocols |  |  |  |
| 24 | STP Should Support Spanning Tree Protocol, Rapid Spanning Tree Protocol, Multiple Spanning Tree Protocol |  |  |
| 25 | PortFast/ UplinkFast Support for fast convergence features like PortFast/ UplinkFast |  |  |
| Quality of Service |  |  |  |
| 26 | QoS Scheduling Should support QoS scheduling |  |  |
| 27 | Queues per port Should support minimum 4 queues per port |  |  |
| 28 | Rate Limiting Switch should support rate limiting (bandwidth control). Rate limiting should be supported on the Ingress or Egress of any port without sacrificing additional ports. |  |  |
| Multicasting |  |  |  |
| 29 | Multicast Should support H/W based IPv4 Multicasting |  |  |
| 30 | IGMP Should Support IGMP v1, v2 , v3 |  |  |
| 31 | Multicast Entries Should support 4000 entries |  |  |
| 32 | PIM Should support Protocol Independent Multicast - Sparse and dense mode |  |  |
| Management |  |  |  |
| 33 | Network monitoring/ management Switch should be manageable through NMS on per port/switch basis with common interface for all manageable devices on the network. Should Support syslog, SNMP, RMON/RMON-II, SSH, telnet, web management |  |  |
| 34 | Port Mirroring Should support port mirroring feature for monitoring network traffic of a particular port/VLAN/group of ports/entire switch. |  |  |
| Software |  |  |  |
| 35 | Software Version Version of software for supplied switch should be latest release to support all required features |  |  |
| 36 | Software Updates Software updates should be bundled for the entire contract period |  |  |
| IEEE Standards Compliance |  |  |  |
| 37 | IEEE Standards |  |  |
|  | Ethernet: IEEE 802.3, 10BASE-T |  |  |
|  | Fast Ethernet: IEEE 802.3u, 100BASE-TX, 100BASE-FX |  |  |
|  | Gigabit Ethernet: IEEE 802.3z, IEEE 802.3ab, 1000BASE-X (mini-GBIC/SFP), 1000BASE-SX, 1000BASE-LX/LH |  |  |
|  | IEEE 802.1D Spanning-Tree Protocol |  |  |
|  | IEEE 802.1S & 1W for Rapid Spanning tree convergence |  |  |
|  | IEEE 802.1p CoS Prioritization |  |  |
|  | IEEE 802.3x Flow Control |  |  |
|  | IEEE 802.3ad Link Aggregation |  |  |

3.3  Access/Edge Switches

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements Common to all Access Switches |  |  |  |
| 1 | Architecture Fixed Configuration Layer 2 Rack mountable Switch. Rack mount kit to be provided along with the switch |  |  |
| 2 | Flash & Memory The proposed switch should have enough Memory (Flash and/or any other) to hold the latest Software Release. The Memory should be large enough to support fully populated ports, all features of switch and parameters like MAC Address Table, VLANs etc. at their peak values as claimed in the Data Sheets of the Switch |  |  |
| 3 | No. of Uplink Ports 2nos. of dual purpose uplink ports (Choice of 10/100/1000 BaseTX and SFP based Gigabit port) to be used for copper or fiber uplink for those switches which require fiber connectivity or 2 nos. of 1000 Base TX uplink ports for copper only uplinking. |  |  |
| 4 | Type of ports All Autosensing, Auto-negotiating, Auto-MDIX on all UTP ports |  |  |
| 5 | Redundancy Should support redundant power supply except in 8 port Switch |  |  |
| Connectivity & Filtering |  |  |  |
| 6 | 802.3ad Should support Industry Standard Port/Link Aggregation for All Ports. |  |  |
| 7 | Jumbo Frames Jumbo Frames support up to 9K Bytes |  |  |
| 8 | Storm Control Support for broadcast/multicast/unicast storm control to prevent degradation of switch performance from faulty end stations |  |  |
| Layer 2 Functionality |  |  |  |
| 9 | 802.1Q Should support 802.1Q protocol |  |  |
| 10 | Type of VLANs Should support port, protocol and subnet based VLANs |  |  |
| 11 | Voice VLAN Should Support Voice VLAN for separating and prioritising VoIP traffic |  |  |
| 12 | VLAN Trunking Protocol or Equivalent Should support VTP or equivalent protocol to reduce administrative burden of configuring VLANs on multiple switches |  |  |
| 13 | No. of VLANs Should support minimum 250 active VLANs |  |  |
| 14 | MAC Addresses No. of MAC Address Supported: 8000 |  |  |
| Security Features |  |  |  |
| 15 | MAC Address Filtering Should Support MAC Address Filtering based on source and Destination address |  |  |
| 16 | DHCP Snooping Should support DHCP Snooping to filter untrusted DHCP messages |  |  |
| 17 | 802.1X Should support 802.1x Network Security and Authentication |  |  |
| 18 | 802.1x with VLAN Assignment/ Guest VLAN Should support 802.1x with VLAN Assignment/Guest VLAN features |  |  |
| 19 | RADIUS Should have support for RADIUD |  |  |
| 20 | NAC Should support integration with NAC solutions. |  |  |
| Network Protocols |  |  |  |
| 21 | STP Should Support Spanning Tree Protocol, Rapid Spanning Tree Protocol, Multiple Spanning Tree Protocol |  |  |
| 22 | PortFast/ UplinkFast Support for fast convergence features like PortFast/ UplinkFast |  |  |
| Quality of Service |  |  |  |
| 23 | QoS Scheduling Should support QoS scheduling |  |  |
| 24 | Queues per port Should support minimum 4 queues per port |  |  |
| 25 | Rate Limiting Switch should support rate limiting (bandwidth control). Rate limiting should be supported on the Ingress or Egress of any port without sacrificing additional ports. |  |  |
| Multicasting |  |  |  |
| 26 | Multicast Should support H/W based IPv4 Multicasting |  |  |
| 27 | IGMP Should Support IGMP v1, v2 , v3 |  |  |
| 28 | Multicast Entries Should support 4000 entries |  |  |
| 29 | PIM Should support Protocol Independent Multicast - Sparse and dense mode |  |  |
| Management |  |  |  |
| 30 | Network monitoring/ management Switch should be manageable through NMS on per port/switch basis with common interface for all manageable devices on the network. Should Support syslog, SNMP, RMON/RMON-II, SSH, telnet, web management |  |  |
| 31 | Port Mirroring Should support port mirroring feature for monitoring network traffic of a particular port/VLAN/group of ports/entire switch. |  |  |
| Software |  |  |  |
| 32 | Software Version Version of software for supplied switch should be latest release to support all required features |  |  |
| 33 | Software Updates Software updates should be bundled for the entire contract period |  |  |
| IEEE Standards Compliance |  |  |  |
| 34 | IEEE Standards Ethernet: IEEE 802.3, 10BASE-T |  |  |

3.31  48 port 10/100/1000 Access Switch

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Switching Fabric / Back Plane capacity - 32 Gbps |  |  |
| 4 | Forwarding rate - 39 Mpps |  |  |
| 5 | 10/100/1000 Mbps ports - 48 |  |  |

3.32  48 port 10/100 Access Switch with 2 nos. of Gigabit copper uplinks

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Switching Fabric / Back Plane capacity - 13.6 Gbps |  |  |
| 4 | Forwarding rate - 10 Mpps |  |  |
| 5 | 10/100 Mbps ports - 48 |  |  |
| 6 | 1 Gbps Uplink ports 2 nos. of 1 Gbps Dual Purpose / SFP ports loaded with UTP modules. Required Fiber modules will be included in BOM suitably |  |  |

3.33  24 port 10/100 Access Switch  with 2nos. of  Gigabit fiber uplinks

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Switching Fabric / Back Plane capacity - 8.8 Gbps |  |  |
| 4 | Forwarding rate - 6.5 Mpps |  |  |
| 5 | 10/100 Mbps ports - 24 |  |  |
| 6 | 1 Gbps Uplink ports 2 nos. of 1 Gbps Dual Purpose / SFP ports loaded with UTP modules. Required Fiber modules will be included in BOM suitably |  |  |

3.34  24 port 10/100 Access Switch with 2 nos. of Gigabit copper uplinks

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Switching Fabric / Back Plane capacity - 8.8 Gbps |  |  |
| 4 | Forwarding rate rate - 6.5 Mpps |  |  |
| 5 | 10/100 Mbps ports -- 24 |  |  |
| 6 | 1 Gbps Uplink ports 2 nos. of 1000 Base TX |  |  |

3.35  8 port 10/100 Access Switch with 1 no. of Gigabit fiber uplink

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Switching Fabric / Back Plane capacity - 3.6 Gbps |  |  |
| 4 | Forwarding rate - 2.5 Mpps |  |  |
|  | 10/100 Mbps ports - 8 |  |  |
|  | 1 Gbps Uplink ports 1 no. of 1 Gbps Dual Purpose / SFP port. Required Fiber module will be included in BOM suitably |  |  |

3.4   Router

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Architecture The router shall be modular in design for scalability |  |  |
| 4 | Flash & Memory 256 MB DRAM upgradeable up to 1GB and 64 MB Flash upgradeable up to 256MB |  |  |
| 5 | Ports & Interfaces |  |  |
|  | Router shall have 2 Nos. of 10/100/1000 Base TX Ethernet ports as per 802.3z specification. The ports shall support both half duplex and full duplex. |  |  |
|  | The Router shall support WAN ports such as V.35 and G.703 (Clear Channel) interfaces for connecting to long haul carrier network. |  |  |
|  | The Router shall support ISDN BRI, E1, E1 G703, E1 - DI (Drop & Insert), ADSL, SHDSL, Analog Modem interface, Ethernet Switch Interface |  |  |
|  | Should have minimum of 5 slots and should have three free slots after the current port requirement |  |  |
|  | Router should scalable up to twenty V.35 Ports |  |  |
|  | Should have at least 2 internal slots and 3 DSP slots apart from the above to save the interface slots |  |  |
|  | The router shall have Two V.35 WAN ports |  |  |
|  | Should have minimum of 2 USB ports |  |  |
|  | Router shall support a console port with RS-232 interface for configuration and diagnostic purposes. |  |  |
| 6 | Performance |  |  |
|  | The Router shall support aggregate packet forwarding rate up to 150 kpps for a packet length of 64 bytes. |  |  |
|  | Should support hardware assisted encryption up to 35 Mbps |  |  |
| 7 | Availability |  |  |
|  | Shall support dual-redundant GE connection to Campus LAN |  |  |
|  | Shall support Redundant Power Supply |  |  |
|  | Shall support link aggregation using LACP as per IEEE 802.3ad |  |  |
|  | It shall support fast reboot for minimum network downtime. |  |  |
|  | It shall support boot options like booting from TFTP server, Network node and Flash Memory. |  |  |
| 8 | QoS Features |  |  |
|  | Classification and Marking: Policy based routing, IP Precedence, DSCP, MPLS exp bits |  |  |
|  | Congestion Management: WRED, Priority queuing, Class based weighted fair queuing |  |  |
|  | Traffic Conditioning: Committed Access Rate/Rate limiting |  |  |
|  | Signalling: RSVP for Bandwidth reservation |  |  |
| 9 | Protocols |  |  |
|  | Shall support Routing protocols like RIP ver1 &2, OSPF ver2 , OSPF on demand, BGP4 |  |  |
|  | Should have Dual Stack IPv4 & IPv6 protocols. |  |  |
|  | Shall support IPv6 features (DHCPv6, IPv6 QoS, IPv6 Multicast support, BiDirectional PIM, Multicast VPN, PIM SSM (Source Specific Multicast), IPv6 PIMv2 Sparse Mode, IPv6 PIMv2 Source-Specific Multicast). |  |  |
|  | Shall support RIPng and OSPFv3 for IPv6 |  |  |
|  | Shall support MPLS Provider /Provider Edge functionality. |  |  |
|  | MPLS VPN, MPLS mVPN (Multicast VPN), Support for VRF-Aware Services (NAT, FW, IPsec, Syslog), Carrier Supporting Carrier (CsC), DiffServ Tunnel Modes, MPLS TE (Fast re-route), DiffServ-Aware TE, Inter-AS VPNs). |  |  |
| 10 | Security Features |  |  |
|  | Should have GRE Tunneling, NAT, L2TP tunneling, without any major performance impact. |  |  |
|  | Should support IPSEC Site-to-Site and Remote Access VPNs. Hardware based encryption. Any Office to Any other office, dynamic establishment of VPNs so that the configuration & management of IPSEC VPNs becomes easier. IPSEC VPNs should be able to carry data, voice, video multicast traffic. |  |  |
|  | Should support Firewall, IPS & support for SSL VPN features. Support for customized IPS signatures. |  |  |
|  | Support for tunnel less VPNs for very high scalability with support for routing protocols and multicast traffic |  |  |
|  | MD-5 route authentication for RIP, OSPF, IS-IS and BGP |  |  |
|  | SNMPv3 authentication, SSHv2 |  |  |
|  | AAA support using Radius and/or TACACS |  |  |
|  | DoS prevention through TCP Intercept, DDoS protection |  |  |
|  | IEEE 802.1x support for MAC address authentication |  |  |
| 11 | Multimedia Support |  |  |
|  | Shall support Voice capabilities |  |  |
|  | Shall support Voice pass-through |  |  |
| 12 | Diagnostics Support |  |  |
|  | Router shall have built in power-on diagnostics system to detect hardware failures. |  |  |
|  | Support for monitoring of Traffic flows for Network planning and Security purposes |  |  |
|  | Display of input and output error statistics on all interfaces |  |  |
|  | Display of Dynamic Arp table |  |  |
|  | Trace-route, Ping and extended Ping |  |  |
|  | Should have extensive support for SLA monitoring for metrics like delay, latency, jitter, packet loss, and MOS |  |  |
|  | Support for MPLS OAM |  |  |
| 13 | Management |  |  |
|  | Shall have support for Web based management, CLI, Telnet, SNMP-V1, V2 and V3. |  |  |
|  | Shall support Secure Shell for secure connectivity |  |  |
|  | Shall support Out of band management through Console and external modem for remote management |  |  |
|  | Should have accounting features |  |  |
|  | Shall support RMON, MIB-I & MIB-II |  |  |
|  | Shall support all MIBs pertaining to Routing protocols such as OSPF, BGP4 etc. |  |  |
| 14 | General |  |  |
|  | Router shall support 19” rack mountings. |  |  |
|  | Router shall support software upgrade through Flash Memory |  |  |
|  | Router shall support live software reconfiguration on the fly while on working, to implement without rebooting. |  |  |
|  | Router shall be capable of working with 170 – 240 volts AC nominal at frequency 50±2 Hz. |  |  |
|  | Router shall have LEDs or LCD panel for diagnostics to check the status of ports and modules. |  |  |

3.5   Firewall & IPS

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Performance Requirements |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Hardware |  |  |
|  | The Firewall should be Hardware based, Reliable, purpose-built security appliance with hardened operating system that eliminates the security risks associated with general-purpose operating systems with minimum 4 interfaces |  |  |
|  | The Firewall should be Hardware based, Reliable, purpose-built security appliance with hardened operating system that eliminates the security risks associated with general-purpose operating systems with minimum 4 interfaces The Appliance should be Full-featured, high-performance firewall and IP Security, Secure Sockets Layer (IPSec, SSL) VPN technologies deliver robust application security, user- and application-based access control, worm and virus mitigation, malware protection, and remote user/site connectivity. |  |  |
|  | Should support Active/Active or Active/Standby Firewall for High Availability & Scalability |  |  |
|  | Firewall Throughput of Up to 300 Mbps |  |  |
|  | Concurrent Threat Mitigation Throughput (Firewall and IPS Services) upto 150Mbps |  |  |
|  | VPN Throughput of Up to 170 Mbps |  |  |
|  | Concurrent Sessions of at least 50,000 |  |  |
|  | Virtual Interfaces (VLANs) support for at least 100 VLANs for forming Secure server Farms and DMZs |  |  |
|  | Scalability through VPN clustering and load balancing |  |  |
| 4 | Application Security |  |  |
|  | The Firewall should have Integrated specialized inspection engines for protocols like HTTP, FTP, ESMTP, DNS, SNMP, ICMP, SQL*Net, NFS, H.323 Versions 1-4, SIP, MGCP, RTSP, TAPI and JTAPI over CTIQBE protocol, GTP, LDAP, ILS, RPC. |  |  |
|  | The Firewall should provide advanced inspection services to detect and optionally block instant messaging, peer-to-peer file sharing, and other applications tunneling through Web application ports |  |  |
|  | It should be able to block popular instant messaging applications such as AOL Instant Messenger, Microsoft Messenger, and Yahoo Messenger |  |  |
|  | Should be able to stop peer-to-peer file sharing applications such as KaZaA and Gnutella. Should be able to thwart tunneling applications such as GoToMyPC |  |  |
|  | Support for SNMP version filtering for all SNMP traffic attempting to flow through security appliance, supporting filtering of SNMP versions 1, 2, 2c, and 3 |  |  |
|  | Support for port-hopping UNIX applications through stateful inspection and NAT services for Sun RPC and NIS+ sessions transactions that use Portmapper v2 or RPCBind v3 or v4 |  |  |
|  | Advanced H.323 inspection services that support versions 1-4 of the protocol along with Direct Call Signaling (DCS) and Gatekeeper Router Control Signaling (GKRCS) to provide flexible security integration in a variety of H.323-driven voice-over-IP (VoIP) environments |  |  |
|  | NAT and PAT support for H.323 services, including advanced features such as fax over IP (FoIP) using the T.38 protocol, an ITU standard that defines how to transmit FoIP in real time |  |  |
|  | Support for foundational capabilities to assist in detecting protocol and application layer attacks |  |  |
|  | To provide TCP stream reassembly and analysis services to help detect attacks that are spread across a series of packets |  |  |
|  | Should offer TCP traffic normalization services for additional techniques to detect attacks, including advanced flag and option checking, TCP packet checksum verification, detection of data tampering in retransmitted packets, and more |  |  |
| 5 | IPS Features |  |  |
|  | Protects your network from policy violations, vulnerability exploitations, and anomalous activity through detailed inspection of traffic in Layers 2 through 7 |  |  |
|  | Deliver a wide array of inline packet drop actions, including the ability to drop single malicious packets, all packets within a flow that contains multiple malicious packets, and all packets from the attacker's IP address |  |  |
|  | Should support Traffic anomaly detection to provide anomaly identification for attacks that may cover multiple sessions and connections, using techniques based on identifying changes in normal network traffic patterns. |  |  |
|  | Should support Protocol anomaly detection to identify attacks based on observed deviations in the normal RFC behavior of a protocol or service. |  |  |
|  | Should support stateful pattern recognition to identify vulnerability-based attacks through the use of multipacket inspection across all protocols. |  |  |
|  | Should provide traffic normalization, IP defragmentation, TCP stream reassembly. |  |  |
|  | Should provide protection against Reconnaissance activity, misuse activity, file-sharing threats, DoS activity, Back Orifice, failed login attempts, and TCP hijacking |  |  |
| 6 | Network Containment and Control |  |  |
|  | Robust stateful inspection firewall services that track the state of all network communications |  |  |
|  | Inbound and outbound access control lists (ACLs) for interfaces, time-based ACLs, and per-user or per-group policies for improved control over network and application usage |  |  |
|  | Convenient troubleshooting tool that allows administrators to test and fine-tune ACLs without the need to remove and replace ACL entries |  |  |
|  | Creation of security policies based on interface name instead of IP address, a feature that is especially useful in broadband environments where the external interface is typically assigned a dynamic IP address |  |  |
|  | Powerful reporting and troubleshooting capabilities that help enable collection of detailed statistics on which ACL entries are triggered by network traffic attempting to traverse a security appliance |  |  |
| 7 | Secure Connectivity |  |  |
|  | IPSec remote-access VPN concentrator services for up to hundreds of simultaneous remote software- or hardware-based VPN clients |  |  |
|  | Should be able to Push VPN policy dynamically to Easy VPN Remote-enabled solutions upon connection, eliminating the need to manage each client separately and therefore helping ensure enforcement of the latest corporate VPN security policies |  |  |
|  | Should provide convenient method for authenticating VPN users through native integration with popular authentication services, including Microsoft Active Directory, Microsoft Windows Domains, Kerberos, LDAP, and RSA SecurID (without requiring a separate RADIUS server to act as an intermediary) |  |  |
|  | Support for Internet Key Exchange (IKE) and IPSec VPN standards with hub-and-spoke or meshed VPN configurations |  |  |
|  | Support for SSL VPN for mobile users, remote sites |  |  |
|  | Scalable to support for SSL VPNs for up to 250 peers |  |  |
| 8 | High Availability |  |  |
|  | Support for Active/Active & Active/Standby failover |  |  |
|  | Support for bidirectional state sharing between Active/Active failover pair members for support of advanced network environments with asymmetric routing topologies, allowing flows to enter through one Firewall appliance and exit through the other, if required |  |  |
|  | Maximizing VPN connection uptime with Active/Standby stateful failover for VPN connections |  |  |
|  | Support for Synchronizing all security association state information and session key material between failover pair members, providing a highly resilient VPN solution |  |  |
|  | Enable geographic separation of Security appliances in a failover pair by allowing failover information to be shared over a dedicated LAN connection between failover pair members |  |  |
| 9 | Intelligent Networking |  |  |
|  | Enable creation of multiple security contexts (virtual firewalls) within a single security appliance, with each context having its own set of security policies, logical interfaces, and administrative domains |  |  |
|  | Support for upto 5 number of virtual firewalls on the same hardware firewall. Providing a convenient way of consolidating multiple firewalls into a single physical appliance or failover pair, while retaining the ability to separately manage each of these virtual instances. |  |  |
|  | Support for deployment of the Firewall in a secure Layer 2 bridging mode, providing rich Layer 2-7 firewall security services for the protected network while remaining "invisible" to devices on each side of it. Simplified appliance deployments in existing network environments by not requiring businesses to readdress the protected networks |  |  |
|  | Support for multiple virtual interfaces on a single physical interface through VLAN trunking and multiple VLAN trunks per appliance |  |  |
|  | Facilitate a wide range of multicast applications by including support for Internet Group Management Protocol (IGMPv2) and stub multicast routing, including NAT and PAT and the ability to build ACLs for multicast traffic |  |  |
|  | Access control and deep inspection firewall services for native IPv6 network environments and mixed IPv4 and IPv6 network environments through dual-stack support |  |  |
|  | IPv6-enabled inspection services for applications based on HTTP, FTP, SMTP, ICMP, TCP, and UDP |  |  |
|  | Support for SSHv2, Telnet, HTTP and HTTPS, and ICMP-based management over IPv6 |  |  |
| 10 | Management |  |  |
|  | Should support automatic updates to provide most up to date protection. |  |  |
|  | Support for Built-in Management Software for simple, secure remote management of the security appliances through integrated, Web-based GUI |  |  |
|  | Should provide a wide range of informative, real-time, and historical reports that give critical insight into usage trends, performance baselines, and security events |  |  |
|  | Accessible through console port, Telnet, and SSHv2 |  |  |
|  | Strong authentication of users through the Firewall appliance through a local user database or through integration with enterprise databases, either directly using RADIUS or indirectly with AAA Server |  |  |
|  | Prevention of unauthorized access to sensitive configuration data, certificates, and key material stored on the security appliance by automatically wiping flash memory contents if an asset recovery or password reset procedure occurs |  |  |

3.6   DSLAM

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| Hardware & Standards |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Minimum 2 Nos. 100 BaseTX Ports |  |  |
| 4 | Subscriber Line Interfaces 12 or more ports, ADSL2+ with integrated POTS Splitter |  |  |
| 5 | ADSL/ADSL+ Standards G.994.1, G.997.1, G.992.1, G.992.3, G.992.5 |  |  |
| Protocol Support |  |  |  |
| 6 | IP Host and gateway support |  |  |
| 7 | Bridging 802.1D support, RFC 1483/2684 bridged encapsulation |  |  |
| 8 | DHCP Relay |  |  |
| 9 | VLAN 802.1p/q |  |  |
| 10 | RADIUS Authentication |  |  |
| Management |  |  |  |
| 11 | In-Band IP |  |  |
| 12 | Out-band IP over 10/100BT Ethernet or V.24 RS-232 serial for async terminal |  |  |
| 13 | Manageable through CLI, telnet, SNMP, Web Interface |  |  |
| Other Requirements |  |  |  |
| 14 | Bandwidth/Distance Data Rate Upto 24 Mbps, Distance Maximum 7 Km |  |  |
| 15 | Rack Mount Should be Rack Mountable, Rack Mount Kit should be provided |  |  |
| 16 | Misc Should come with necessary manuals, cables for ADSL input/POTS output and CDs, 230V internal power supply. |  |  |
| 17 | Software Updates Software updates should be bundled for the entire contract period |  |  |

3.7   ADSL CPEs

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | WAN Interface 1 No. RJ11 |  |  |
| 4 | LAN Interface 1 No. RJ45 10/100BaseT |  |  |
| 5 | POTS Splitter Built-in or External POTS Splitter to be provided |  |  |
| 6 | ADSL/ADSL+ Standards G.994.1, G.997.1, G.992.1, G.992.2, G.992.3, G.992.5 |  |  |
| 7 | Standards Support |  |  |
|  | Routing/ Bridge Support |  |  |
|  | Transparent Bridge Support |  |  |
|  | RIP1 and RIP2 |  |  |
|  | DHCP Relay/Server/Client |  |  |
|  | VLAN 802.1p/q |  |  |
|  | PPPoA, PPPoE, PAP/CHAP Authentication |  |  |
| 8 | Security NAT, Ingress/Egress Packet Filtering, MAC Filtering, Stateful Packet Inspection |  |  |
| 9 | Management Manageable through CLI, telnet, SNMP, Web Interface |  |  |
| 10 | Syslog Syslog Support |  |  |
| 11 | Misc Should come with necessary manuals, cables and CDs, 230V internal/external power supply. |  |  |
| 12 | Software Updates Software updates should be bundled for the entire contract period |  |  |

3.8   Network Management System

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| 1 | Make Should be of same make of the offered Switches |  |  |
| 2 | Model (specify) |  |  |
| 3 | Software Requirement |  |  |
|  | Multiple administrative access logins should be possible |  |  |
|  | Provide tools for configuring, managing, monitoring and troubleshooting of the Campus Area Network devices. |  |  |
|  | Provide a flexible framework to address the device management needs of network converging voice, video and data. |  |  |
|  | Automatic discovery process for networked devices that create a network topology map using a color-coded, hierarchical view of the network for IP networks. |  |  |
|  | Tools to simplify Device configuration and management for Routers and Switches. |  |  |
|  | Have tools for creating, deleting and editing VLANs. |  |  |
|  | Should supports automated fault detection that recognizes common problems in the network |  |  |
|  | Threshold management features that can be set for many performance variables to generate an alarm or event notification if threshold are exceeded. |  |  |
|  | Path trace tool for layer 2 and layer 3 path analysis |  |  |
|  | Provide functionality to correlate MAC address and IP address to switch port |  |  |
|  | Support for Inventory Management |  |  |
|  | The system must manage and deploy configuration changes to multiple network devices. Changes can be downloaded immediately or run as scheduled operations |  |  |
|  | Provide capabilities for software updates to be performed against multiple network devices in the network. |  |  |
|  | Pro-active & real-time monitoring and tracking of key information and data relating to device performance, traffic, and environment, with metrics such as utilization percentage, frames transmitted and received, errors, and a variety of other device-specific indicators |  |  |
|  | Provide troubleshooting and diagnostics tools including Ping, Trace Route, Connections, Statistics, and Hostname/Address Lookup |  |  |
|  | Should be possible to Integrate with other management tools and third party applications, like HP OV, IBM Tivoli etc. |  |  |
|  | New device support should be easily added |  |  |
|  | Web Interface for managing the network |  |  |
|  | Should be able to manage devices using inventory- and device-change management, network-configuration and software-image management, network availability, and syslog analysis. |  |  |
| 4 | OS Compatibility NMS software should be Windows OS compatible |  |  |
| 5 | Hardware Requirement Suitable hardware for running NMS shall be provided along with NMS without additional cost. Minimum Hardware: Dual Core Xeon 3.6 GHz Based CPU, 4 GB RAM, RAID controller, 140 GB Available Disk Capacity in RAID Configuration, DVD Writer, 2 10/100/1000 NIC, Redundant Power Supply, Rack mounting Kit, 17" TFT color monitor, Hardware make: HP/IBM/Dell. Suitable Windows Server 2003/2008 OS license should also be provided without additional cost |  |  |
| 6 | Performance Requirement A single Installation should support minimum 200 devices. License if any shall be given for minimum 200 devices. |  |  |
| 7 | Software Updates Software updates should be bundled for the entire contract period |  |  |

3.9. UPS Systems

3.9.1  1 KVA UPS

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Rating - 1 KVA or Higher |  |  |
| 4 | Technology - Line-Interactive |  |  |
| 5 | Nominal Input Voltage - 230V Single Phase |  |  |
| 6 | Input Voltage Range - 175V AC to 275V AC |  |  |
| 7 | Input Frequency - 50 Hz +/- 3 Hz |  |  |
| 8 | Nominal Output Voltage - 230 VAC +/- 5% |  |  |
| 9 | Output Frequency - 50 Hz +/- 1 Hz |  |  |
| 10 | Output power capacity - 600 Watts or Higher |  |  |
| 11 | Automatic Voltage regulation Automatic Voltage Regulation to be provided |  |  |
| 12 | Transfer time - <6ms |  |  |
| 13 | Protection Surge, brownout, sag, short circuit etc protection to be provided |  |  |
| 14 | Battery Type & quantity All batteries whether external or internal should be recognised by UPS. UPS should be able to test all batteries periodically. UPS should be able to show actual backup time considering all batteries. Specify Quantity. |  |  |
| 13 | Battery make OEM/ Panasonic / Global Yuasa / Rocket |  |  |
| 14 | Power Sockets Output power sockets should be 5/15A standard Indian type or necessary Interface power cords should be provided to connect to network devices |  |  |
| 15 | Battery Refresh The bidder shall replace all the batteries after every 2.5 years and whenever the batteries get faulty during the entire contract period |  |  |
| 16 | Communication Port RS232C or USB port & necessary communication cables to be provided |  |  |
| 17 | Backup Min. 5 minutes on full load |  |  |
| 18 | Power Management Software The UPS should come with Power Management Software to view & monitor UPS status, backup time & shutdown scheduling for Windows XP/2003/Vista systems |  |  |
| 19 | Warranty OEM warranty (labour/parts/on-site) for the entire contract period |  |  |

3.9.2   5 KVA UPS

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| 1 | Make (specify) |  |  |
| 2 | Model specify) |  |  |
| 3 | Rating - 5 KVA or Higher |  |  |
| 4 | Technology - True Online, Double Conversion |  |  |
| 5 | Nominal Input Voltage - 230V Single Phase |  |  |
| 6 | Input Voltage Range - 175V AC to 275V AC |  |  |
| 7 | Input Frequency - 50 Hz +/- 3 Hz |  |  |
| 8 | Nominal Output Voltage - 230 VAC +/- 5% |  |  |
| 9 | Output Frequency - 50 Hz +/- 1 Hz |  |  |
| 10 | Output power capacity - 3.5 KW or Higher |  |  |
| 11 | Battery Type and Quantity - Sealed, maintenance-free. If external, proper casing to be provided. Specify Quantity. |  |  |
| 12 | Battery make - OEM/ Panasonic / Global Yuasa / Rocket |  |  |
| 13 | Desired Backup - Min. 30 minutes backup on full load |  |  |
| 14 | Battery Management All batteries whether external or internal should be recognised by UPS. UPS should be able to test all batteries periodically. UPS should be able to show actual backup time considering all batteries |  |  |
| 15 | Power Sockets Output power sockets should be 5/15A standard Indian type or necessary Interface power cords should be provided to connect to network devices |  |  |
| 14 | Cold Start Cold start on 100% load |  |  |
| 15 | Communication Port RS232C or USB port & necessary communication cables to be provided |  |  |
| 16 | SNMP Card SNMP Card to be provided for connecting UPS to Ethernet LAN to monitor and manage the UPS with a standard Web browser, while simultaneously providing shutdown & restart for multiple windows, Linux & Unix computer systems over LAN |  |  |
| 17 | Power Management Software The UPS should come with Power Management Software to view & monitor UPS status, backup time & shutdown scheduling for Windows 9x/NT//200/XP & Red Hat Linux 7.2 & above systems |  |  |
| 18 | Battery Refresh The bidder shall replace all the batteries after every 2.5 years and whenever the batteries get faulty during the entire contract period |  |  |
| 19 | Warranty OEM warranty (labour/parts/on-site) for the entire contract period |  |  |

3.10  Network Racks

3.10.1  42 U Network Rack

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| 1 | Make & Model - Enterprise makes like APW/RITTAL/ VALRACK (specify) |  |  |
| 2 | Mounting - Floor Standing |  |  |
| 3 | Dimensions Dimensions (Width X Depth ) - 800 x 1000 mm, 42U Height |  |  |
| 4 | Doors Lockable front door of toughened tinted glass & lockable vented rear door of steel |  |  |
| 5 | Side Panels Side Panels vented with Slam Latches and Key Locks |  |  |
| 6 | Top & Bottom Covers Top & Bottom Covers with cable entry gland plates and cut outs. Removable and vented top cover for allowing heat conduction with provision for mounting fan housing unit and cable entry provision from rear side. The top cover should have provision for mounting a fan Housing unit for hosting the fans |  |  |
| 7 | Stationary Shelves 3 No. of Stationary Shelves |  |  |
| 8 | Equipment Mounting Angles One pair of Equipment Mounting Angles to provide 19" mounting positions |  |  |
| 9 | Cooling Fans Min. 4 Nos. of Cooling Fans (230VAC, 90 CFM) in top mounted Fan Housing Unit |  |  |
| 10 | Mounting Hardware Captive Mounting Hardware (10 Pkts) |  |  |
| 11 | Power Distribution Min. Two independent & redundant vertical or horizontal power strips each containing 10 Nos. of 5/15A power sockets, a fuse, indicator lamp and 15A Switch |  |  |
| 12 | Castors 4 castors with foot operated brakes |  |  |
| 13 | Earthing Kit Copper earthing kit (bars, straps, continuity kit, etc) |  |  |
| 14 | Cable Management accessories Two horizontal, two vertical cable managers & two vertical Cable Channels with cabling loops. Cable entry - Bottom and Top |  |  |

3.10.2  15 U Network Rack

| Sl No | Criteria |  | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- | --- |
| 1 | Make & Model - Enterprise makes like APW/RITTAL/ VALRACK (Specify) |  |  |  |
| 2 | Mounting - Wall mounting |  |  |  |
| 3 | Dimensions Dimensions (Width X Depth ) - 600 x 600 mm, 15U Height |  |  |  |
| 4 | Doors Lockable front door of toughened tinted glass |  |  |  |
| 5 | Top & Bottom Covers Sealed cable entrance with access holes with gland plates top and bottom |  |  |  |
| 6 | Equipment Mounting Angles One pair of Equipment Mounting Angles to provide 19" mounting positions |  |  |  |
| 7 | Cooling Fans Min. 2 Nos. of Cooling Fans in top mounted position |  |  |  |
| 8 | Mounting Hardware Captive Mounting Hardware (2 Pkts) |  |  |  |
| 9 | Power Distribution Min. One horizontal power strip containing 5 Nos. of 5A power sockets, a fuse, indicator lamp and 5A Switch |  |  |  |

3.10.3  12 U Network Rack

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| 1 | Make & Model - Enterprise makes like APW/RITTAL/ VALRACK (specify) |  |  |
| 2 | Mounting - Wall mounting |  |  |
| 3 | Dimensions Dimensions (Width X Depth ) - 600 x 600 mm, 12U Height |  |  |
| 4 | Doors Lockable front door of toughened tinted glass |  |  |
| 5 | Top & Bottom Covers Sealed cable entrance with access holes with gland plates top and bottom |  |  |
| 6 | Equipment Mounting Angles One pair of Equipment Mounting Angles to provide 19" mounting positions |  |  |
| 7 | Cooling Fans Min. 2 Nos. of Cooling Fans in top mounted position |  |  |
| 8 | Mounting Hardware Captive Mounting Hardware (2 Pkts) |  |  |
| 9 | Power Distribution Min. One horizontal power strip containing 5 Nos. of 5A power sockets, a fuse, indicator lamp and 5A Switch |  |  |

3.11. Structured Cabling Components

| Sl No | Criteria | Compliance(Y/N) | Details/Remarks |
| --- | --- | --- | --- |
| General Specifications |  |  |  |
| 1 | The structured cabling solution shall be from Tyco/ Systimax/Molex/Panduit/Krone/R&M/Belden/DLink/Digilink. |  |  |
| 2 | All passive cabling components shall be from single OEM. |  |  |
| 3 | All UTP Cabling shall be Category 6 type. |  |  |
| 4 | The vendor shall install all passive components through a registered system integrator of cabling OEM. |  |  |
| 5 | The vendor shall label all cables and cords, LIUs, jack panels, SMBs etc according to industry standards. |  |  |
| 6 | The cabling system installed by the vendor shall meet the specifications as prescribed in ANSI/EIA/TIA, ISO 11081 standards and to that effect shall submit a certificate after the completion of the work that the work has been done as per standards. |  |  |
| 7 | Quantities specified in the bill of material given below FOR passive components and labour are approximate. The quantities used / measured in the site after the completion of net work will be used for payments. |  |  |
| 8 | Outdoor fiber cabling will be laid by vendor along the finalized route between the buildings. The laying work covers road-cutting wherever needed, digging the trenches (80cm deep X 30cm wide), laying of cable inside the HDPE pipes, covering with bricks or granite stones and sand, filling the trenches with excavated soil/sand and finishing. Wherever the roads are cut, these have to be cemented after refilling. Care is to be taken for providing smooth curved pipe with acceptable radius at the bends as per standards. |  |  |
| 9 | Vendor shall observe the bending radius and pulling strength requirements for both UTP and fiber. |  |  |
| 10 | The vendor shall be responsible for removing and replacing all ceiling/floor tiles (in case of suspended ceiling/raised floor) required for installation of the wiring. Any damage to tiles shall be made good by the vendor. |  |  |
| 11 | Any opening made in the racks as well as in LIUs for cable entry shall be closed to control damages by rodents. |  |  |
| CAT 6 UTP Cable |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Shall meet minimum Category 6 requirements |  |  |
| 4 | The conductors shall be 4 Pair 23 AWG |  |  |
| 5 | Frequency shall be characterised to 250 MHz |  |  |
| 6 | Complies to standard TIA/EIA 568B, ISO Class E 11801-2002 |  |  |
| 7 | Shall meet or exceed Gigabit Ethernet Requirements at 100 meters |  |  |
| 8 | These cables are UL Listed and UL & ETL verified |  |  |
| CAT 6 Face Plate with 8 Position RJ45 I/O and Surface Mount Box |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Shall meet minimum Category 6 requirements |  |  |
| 4 | Shall have integrated dust cover |  |  |
| 5 | Shall provide strain relief for terminated cables |  |  |
| 6 | Shall meet or exceed TIA/EIA 568B, ISO Category 6 Performance Requirements |  |  |
| 7 | Shall meet or exceed Gigabit Ethernet Requirements at 100 meters |  |  |
| 24 Port Cat 6 UTP Jack Panel 1U Height |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Shall meet minimum Category 6 requirements |  |  |
| 4 | Shall be loaded with 24 nos. Individually removable/Replaceable RJ45 I/Os |  |  |
| 5 | Dimensions - 19" Width, 1U Height |  |  |
| 6 | Shall include labels and clear label covers |  |  |
| 7 | Shall meet or exceed TIA/EIA 568B, ISO Category 6 Performance Requirements |  |  |
| 8 | Shall meet or exceed Gigabit Ethernet Requirements at 100 meters |  |  |
| Cat 6 4-Pair UTP Patch Cord - 3 Feet |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Shall meet minimum Category 6 requirements |  |  |
| 4 | Length - 3 Feet |  |  |
| 5 | Shall include snagless, strain relief boots at both ends |  |  |
| 6 | Shall meet or exceed TIA/EIA 568B, ISO Category 6 Performance Requirements |  |  |
| 7 | Shall meet or exceed Gigabit Ethernet Requirements at 100 meters |  |  |
| Cat 6 4-Pair UTP Patch Cord - 7 Feet |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Shall meet minimum Category 6 requirements |  |  |
| 4 | Length - 7 Feet |  |  |
| 5 | Shall include snagless, strain relief boots at both ends |  |  |
| 6 | Shall meet or exceed TIA/EIA 568B, ISO Category 6 Performance Requirements |  |  |
| 7 | Shall meet or exceed Gigabit Ethernet Requirements at 100 meters |  |  |
| 24 Port Rack Mount Fiber LIU |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Shall be 19" rack mounted with 1U height, Rubber grommets shall be provided at the cable entry points, for tight sealing. |  |  |
| 4 | Shall be complete with 24 port SC Couplers, Connectors, Pigtails etc. for SM Fiber |  |  |
| 5 | Shall have adhesive labeling for easy port identification |  |  |
| 12 Port Rack Mount Fiber LIU |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Shall be 19" rack mounted with 1U height, Rubber grommets shall be provided at the cable entry points, for tight sealing. |  |  |
| 4 | Shall be complete with 24 port SC Couplers, Connectors, Pigtails etc. for SM Fiber |  |  |
| 5 | Shall have adhesive labeling for easy port identification |  |  |
| 6 Port Rack Mount Fiber LIU |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Shall be 19" rack mounted with 1U height, Rubber grommets shall be provided at the cable entry points, for tight sealing. |  |  |
| 4 | Shall be complete with 24 port SC Couplers, Connectors, Pigtails etc. for SM Fiber |  |  |
| 5 | Shall have adhesive labeling for easy port identification |  |  |
| 1G/10G Fiber Patch Cord |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Shall have SC on one side and LC connector on other side |  |  |
| 4 | Minimum Length shall be3 meters |  |  |
| 5 | Shall be Factory polished and tested |  |  |
| 6 | Insertion loss shall be less than 0.35dB per connector for single mode fiber |  |  |
| 7 | Type of fiber shall be Single Mode |  |  |
| Single mode Fiber 6 Core |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Type of fiber shall be 6 Core single mode, 9/125 micron Loose Tube, armored |  |  |
| 4 | Shall support Outdoor/ Underground application |  |  |
| 5 | Shall be compliant to new ISO/IEC 11801 Standard |  |  |
| Single mode Fiber 12 Core |  |  |  |
| 1 | Make (specify) |  |  |
| 2 | Model (specify) |  |  |
| 3 | Type of fiber shall be 12 Core single mode, 9/125 micron Loose Tube, armored |  |  |
| 4 | Shall support Outdoor/ Underground application |  |  |
| 5 | Shall be compliant to new ISO/IEC 11801 Standard |  |  |