TECHNICAL SPECIFICATIONS- FULLY AUTOMATED FIVE PART HAEMATOLOGY ANALYSER

- Fully automated analysis of RBC’s, WBC’s and platelets with five part leucocyte differential count with high precision and accuracy. Minimum parameters: Haemoglobin concentration, total leukocyte count, neutrophill percentage, absolute neutrophill count, eosinophill percentage, absolute eosinophill count, basophill percentage, absolute basophill count, lymphocyte percentage, absolute lymphocyte count, monocyte percentage, absolute monocyte count, platelet count, MCV, MCH, MCHC, RDW, hct.

- Throughput  equal to or more than 50 samples/hour with whole blood samples.

- Large colour LCD display of text and histograms.

- Report printouts with external laser printer. External laser printer compatible with the equipment is to be supplied along with the equipment.

- Automatic internal and external sample probe cleaning.

- Fully automatic calibration.

- Bar code identification of samples as well as manual patient data entry by key board.

- Autoloader facility for samples with vaccutainer piercing. Twenty or more samples to be put at a time.

- Large data storage capacity with patient data search function.

10-Power connection requirements- compatible with 220 volts AC,

50 hertz.